3GPP TSG RAN WG1 #109-e	                         			  R1-2203745

e-Meeting, May 9st – 20th, 2022

Agenda Item: 	9.11.2

Source: 	Sony

Title: 	Considerations on capacity enhancements techniques for XR

Document for: 	Discussion / Decision

# Introduction

In RAN1#95e meeting, a revised study item on XR enhancements for NR was approved [1]. The study is to be based on the outcome of Release 17 study item [2], on corresponding Release 17 work from SA4 [3] and on Release 18 work from SA2 [4]. The study item addresses XR-awareness in RAN, XR-specific power saving, and XR-specific power saving.  The objectives on XR-specific capacity improvements (RAN1, RAN2):

- Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms:

  - SPS and CG enhancements

  - Dynamic scheduling/grant enhancements.

This contribution provides our initial views on the mechanisms to improve capacity, including SPS and CG enhancements for the transmission of XR service that has specific characteristics as described above.

# Discussion

## Non-integer periodicity aspect of XR traffic

XR traffic may require a transmission with non-integer periodicity due to the required video frame rate in XR traffic. The typical video frame rate are 30, 60, and 90 fps. An XR transmission with 60 fps requires a transmission with 16.67 ms. Furthermore, XR is a delay sensitive application. A certain delay in the transmission can reduce user experience. Large size of data buffering which could be used to re-arrange the transmission may not be possible as it may introduce significant delay.

The legacy 5G NR typically uses Semi-Persistent Scheduling (SPS) and Configured Grant (CG) to handle a periodic transmission (e.g., voice call). Hence, the usage of control channel can be minimized and resulting in improving capacity. However, the legacy SPS periodicity is using a selected integer number as shown below:

SPS-config = periodicity ENUMERATED {ms10, ms20, ms32, ms40, ms64, ms80, ms128, ms160, ms320, ms640,spareX}

The current SPS configuration is not designed to accommodate quasi-static periodicity of XR transmissions. This is illustrated in the following Figure 1. Here, a mismatch between the SPS allocation pattern and the XR traffic pattern is illustrated. Furthermore, the mismatch gets accumulated over time.

Figure 1: Mismatch of SPS allocation pattern and XR traffic pattern

Observation 1: Depending on the XR traffic pattern, there can be a mismatch of periods between NR SPS/CG configuration and XR traffic.

We consider to enhanced the existing SPS/CG configuration so that it can dynamically adjust the transmission to accommodate the XR traffic pattern. Some of the transmission based on SPS/CG configuration can be slightly adjusted (delayed) so that basically all transmission is transmitted in an integer slot number. In case of DL-SPS transmission The UE should be aware that the transmission has been dynamically adjusted. We consider the adjustment can be implicit or explicitly indicated to the UE. Implicit indication can be supported by providing the adjustment rules defined in the specification. Hence, the amount of signalling between gNB and UE can be minimized. Explicit indication can be supported by providing some new parameters representing the adjustment value. This may require some new parameters in SPS / CG configuration.

Proposal 1: Consider Dynamic SPS/CG configuration to dynamically adjust the transmission in order to accommodate the XR traffic pattern.

A possible simple dynamic SPS/CG configuration is using pseudo periodic SPS/CG as shown in Figure 2, where CG configuration for XR traffic with 30 FPS is selected for illustration purpose. Since the period of 30 FPS traffic is 33.33 ms, the basic period of pseudo periodic can be set to 33 ms. With an extra 1 ms inserted in the beginning of every 3  CG occasions, the time gap between traffic generation and transmission resources can be limited to no longer than 1ms. This could make the CG configuration matching with XR traffic every 100ms. Similarly, this pseudo periodic CG configuration can be applied to SPS transmission for the downlink case.

Figure 2: A schematic/illustration of pseudo periodic CG

Proposal 2: Pseudo-periodic SPS/CG configuration can be considered for XR traffic with non-integer period.

## Multiple flows aspect of XR traffic

The legacy downlink control information (DCI) for SPS/CG is scrambled by CS-RNTI. The DCI is used for the indication of SPS/CG type II activation and release. The associated SPS/CG can only support periodic traffic with fixed packet size. Moreover, only one CS-RNTI is supported in the legacy NR. XR service could contain multiple UL and DL streams/flows (pairs) with variable packet size. To cover the indication of SPS/CG operations for each stream/flow (pair), multi-TB SPS/CG and dynamic update of SPS/CG configurations was proposed [5]. In this section, we would like to propose another scheme with multi CS-RNTIs configuration as a complement to solve the issue.

An example of the multi CS-RNTIs scheme is illustrated in Figure 3, with the assumption of AR scenario with two DL (I-stream and P-stream) and three UL streams (I-stream, P-stream and pose/control stream). Firstly, two CS-RNTIs associated with different stream (pairs) will be configured and sent by RRC signalling, e.g. CS-RNTI1 will indicate the operations of SPS/CG for I-stream pair (green) and CS-RNTI2 for P-stream pair (purple). The third periodic UL stream (yellow) can be served by CG type I without CS-RNTI. Through this configuration and scheduling procedure, the operations of SPS/CG for four streams with two I-steam and P-stream pair can be dynamically and flexibly indicated.

Figure 3:Multi CS-RNTIs configuration for multi streams in XR.

For comparison purpose with any other potential solutions, we enumerate couples of potential shortcomings of multi-TB SPS/CG scheme [5] that have been proposed before. Firstly, a larger specification impact on DCI format design to support multi TB is needed, as well as relative reconsiderations on DCI budget and blind detection in search space set (SSS). Secondly, DCI supporting multi TB scrambled by CS-RNTI can only indicate either multi UL streams or multi DL streams, not able to indicate DL and UL streams simultaneously since one DCI can only be a DL DCI or a UL DCI. Thirdly, if gNB only plans to indicate one stream, other TBs in multi TB DCI need to be configured as special/default/reserved value, which will waste DCI payload. Fourthly, if switching back to single TB DCI is supported for the above case with only one stream indication from gNB, an additional bit on stream id is needed in DCI otherwise UE does not know which stream this DCI is targeting. While our proposed multi CS-RNTIs scheme just needs multi CS-RNTIs configuration in RRC and low complexity on multi CS-RNTIs descrambling and CRC checking, as well as no increasing of PDCCH blind detection per DCI, which had less specification impact and more flexible/dynamic indication of SPS/CG per stream.

Proposal 3: Multi CS-RNTIs configuration for multi flows in XR can be considered.

Considering another scenario on VR DL with video and audio/data streams, where the video stream is modelled as truncated Gaussian distribution in clause 5.1.1 in TR 38.838 [x], and the audio/data stream is modelled as a periodic traffic with fixed size packet as Table 5.1.2.2-1 in TR 38.838 [x], a SPS type I configuration similar to that in CG type I configuration can be considered for the audio/data stream. In that way, the blind detection of DCI activation/release could be saved for power saving.

Proposal 4: A new SPS type configuration that is similar to CG type I configuration can be considered to support multi-flows in XR.

## Jitter aspect of XR traffic

Although the packet arrival for some  XR services may be periodic, the actual arrival time of the packet may experience jitter causing it to arrive randomly within a jitter time window, TJitter.  An example is shown in Figure 4, where an XR application has periodic traffic with a periodicity of PApp.  However, this traffic experiences jitter and so the actual packet arrival falls within a time window TJitter.  In this example, the first jitter time window starts at time t0 where the packet can arrive within this time window between t0 to t4 and here the packet arrives at time t1.  The next packet arrives after a time PApp later, starting at time t5, and here once again the packet can arrive at any time within the jitter time window between t5 to t9.  In the second instance, the packet arrives at time t8, which is towards the end of the jitter time window TJitter.

Figure 4: Jitter time window

SPS configuration provides PDSCH resources to the UE with a deterministic periodicity which can be from 1 to 640 slots.  It is recognized that such deterministic periodicity configuration is not suitable for traffic experiencing jitter.  In order to cater for jitter, multiple SPS configurations are used, where each SPS configuration is activated with a different starting offset, i.e. different K0, as indicated in the DCI field “Time Domain Resource Assignment” (TDRA).  That is, the SPS resource is over-configured to support jittering.  In the example in Figure 4, the UE can be configured with 4 SPS PDSCH configurations, such that these 4 SPS fall within the jitter time window and each has a periodicity of PAPP.  An example is shown in Figure 5, where 4 SPS configurations labelled as #1, #2, #3 and #4 with periodicity PApp, but different offsets, are configured for a UE.  Hence, by configuring multiple SPS resources, the UE is therefore provided with PDSCH resource whenever the packet data arrives within the jitter time window.

Figure 5: Over-configuring SPS for jittering traffic

The SPS resources configured within a jitter time window may not need to be adjacent to each other and there can be gaps between two SPS instances.  An example is shown in Figure 6, where four SPS instances, SPS#1, SPS#2, SPS#3 and SPS#4, are configured to handle jitter and here we have a gap between SPS#2 and SPS#3.  It should also be appreciated that over-configuration of SPS resources is not limited to only traffic types with jitter but can also be used for traffic types that do not have a periodicity that matches those that are configurable by RRC.

Figure 6: SPS with gaps between instances within a jitter time window

Since UE needs to monitor all SPS regardless of whether they contain any actual PDSCH or not, over configuration of SPS to serve a single traffic consumes unnecessary UE power.  Hence the issue to be solved is to provide SPS resources that is suitable for quasi-periodic traffic with minimal additional UE monitoring. An enhancement of the legacy SPS is needed.

Observation 2: SPS enhancements are required to support XR services with the benefit to improve the capacity and minimize unnecessary UE power consumption.

We consider this issue can be solved by configuring the UE to monitor a subset of configured SPS in a group of configured SPS. Particularly, that is in a configured group of NSPS SPS (SPS Group), the UE needs only monitor configured MSPS SPS resources within a period, where MSPS < NSPS.  The value of MSPS and the exact SPS in the group of SPS that are monitored can be different in different period.  This recognizes that in quasi-static periodic traffic, typically only one of configured SPS in a group of over configured SPS resources, contains actual PDSCH as shown in Figure 5 and Figure 6.

In Figure 7, the figure on the left is the case when the UE only monitor SPS#4 and SPS#5. In this case, it covers most of transmission affected by the jitter. However, it does not cover the case with small probability when the transmission occurs at the outside of jitter standard deviation. The UE can also be configured to monitor the last configured SPS, in this case the UE monitors SPS#4, SPS#5, and SPS8. Furthermore, the UE can stop monitor the remaining SPS in case the UE receives the packet in the earlier SPS. For example, if the UE receives the packet in SPS#4 then the UE stops monitoring the remaining SPS (i.e.. SPS#5, and SPS#8).

Figure 7: Illustration of UE monitoring a subset of configured SPS

Proposal 5: Consider the UE to monitor a subset of configured SPS in a group of configured SPS to handle the jitter of XR traffic.

## Scheduling Enhancements for XR traffic

Legacy UL CG is mainly intended for traffic with low data rate which arrives with predictable periodicity and where resource allocation is relatively small as specified in URLLC/IIoT in NR Rel-16/17. However, traffic with high data rate and with low latency, like XR, large resource allocations would be required. In this case, a UE can be pre-configured large resources (e.g., CG) for uplink data transmissions. However, pre-configured large resources for uplink data transmissions have some issues:

- The resources are always reserved in advance regardless of whether a UE has data or not.  Even though a UE is able to release the resource after finishing the UL data transmissions, the concern is that re-allocating/re-activating of the resources will come from the network which results in some unbearable delays for a variety of services like Heavy uplink URLLC. In addition, this will involve signalling from the UE to request resources in advance either via SR or RACH or resources being configured for idle periods (if not released).

- The UE must use/occupy all the resources whenever it has data to transmit because the gNB and UE must be aligned for the allocated resources. This may mean that the UE must add padding bits (in the PDU) in order to fill the remaining resources. This is not desirable as it increases UE power consumption unnecessarily and generates interference for other UEs located in the same cell or neighbouring cells.

- The UE may not be able to control completely the link adaptation parameters, like frequency-domain scheduling to choose the best frequency resources (PRBs) in a BWP, modulation and coding scheme (MCS), etc.

Observation 3: Pre-configured large resources have some issues: a) reserved in advance regardless of whether a UE has data or not, b) when the UE has data to transmit all the resources must be filled/occupied each time, and c) the UE is unable to control completely the link adaptation parameters.

Therefore, some enhancement for UL scheduling will be required for some services, like XR in 5G-Advanced. Below is listed a set of requirements for enhanced UL scheduling:

- Immediate transmission of the UL data in order to reduce the latency, for example for heavy UL data with low latency.

- Choosing appropriate link adaptation parameters, for example the best frequency resources (PRBs), MCS, transmission power.

- Flexible resource allocation scheme, for example FDRA and or TDRA in a slot

- Efficient way of identifying UE and its resource allocation dynamically at the gNB receiver

- Improving spectral efficiency of the cell, so when a UE is not using the resources, another UE can be assigned the resources with minimum latency.

In order to enhance the UL scheduling for XR, it is envisioned here to introduce a new mode of UE-based scheduling where a UE controls/decides its scheduling assignments within the pre-configured uplink resources. The benefit is that the UE will use and allocate just enough resources each time in order to avoid the interferences, reduce latency, as well as choosing appropriate link adaptation parameters, etc.

Proposal 6: Consider introducing a new mode of UE-based scheduling for heavy UL traffic where a UE controls/decides its scheduling assignments within the pre-configured uplink resources.

# Conclusion

In this contribution, we made the following observations and proposals:

Observation 1: Depending on the XR traffic pattern, there can be a mismatch of periods between NR SPS/CG configuration and XR traffic.

Observation 2: SPS enhancements are required to support XR services with the benefit to improve the capacity and minimize unnecessary UE power consumption.

Observation 3: Pre-configured large resources have some issues: a) reserved in advance regardless of whether a UE has data or not, b) when the UE has data to transmit all the resources must be filled/occupied each time, and c) the UE is unable to control completely the link adaptation parameters.

Proposal 1: Consider Dynamic SPS/CG configuration to dynamically adjust the transmission in order to accommodate the XR traffic pattern.

Proposal 2: Pseudo-periodic SPS/CG configuration can be considered for XR traffic with non-integer period.

Proposal 3: Multi CS-RNTIs configuration for multi flows in XR can be considered.

Proposal 4: A new SPS type configuration that is similar to CG type I configuration can be considered to support multi-flows in XR.

Proposal 5: Consider the UE to monitor a subset of configured SPS in a group of configured SPS to handle the jitter of XR traffic.

Proposal 6: Consider introducing a new mode of UE-based scheduling for heavy UL traffic where a UE controls/decides its scheduling assignments within the pre-configured uplink resources.

# References

- RP-220285, “Revised SID : Study on XR Enhancements for NR”, RANP #95e, March 2022

- TR 38.838

- SP-210043

- SP-211166

- RP-211655, RAN93e-R18Prep-05__XR_evolution.