3GPP TSG RAN WG1 #109-e		 R1-2203749

e-Meeting, May 9th – 20th, 2022

Source:	Panasonic

Title: 	Sidelink channel access on unlicensed spectrum

Agenda Item:	9.4.1.1

Item for:	Discussion

Introduction

A work item on NR sidelink enchantment was approved [1]. One of objectives of this work item is the support of sidelink on unlicensed spectrum. One important aspect is the channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation. Following is the relevant part of the objective.

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

This document provides our view on Sidelink channel access on unlicensed spectrum.

Discussion

Background

NR-U

The existing NR-U channel access framework shall be reused for sidelink as much as possible. In NR-U, LBT (Listen Before Talk) are defined to check whether channel is busy or idle before transmission as shown in Table 1. For determining a COT (Channel Occupancy Time), if a transmission gap is less than or equal to 25 us, the gap duration is counted in the COT. A COT can be shared for transmission between an gNB and the corresponding UE(s) [2].

Table 1 Channel access procedures

|  | Sensing interval | Applicable gap duration |  |
| --- | --- | --- | --- |
| Type 1 (cat 4 LBT) | Based on counter N depending on priority class (TS 37.213) |  | Start of gNB initiated COT Start of UE initiated COT (configured grant, SRS, PUCCH, RACH) |
| Type 2A (cat 2 LBT) | 25us | - | Discovery burst (SS/PSBH block) |
|  |  | 25us | Inside COT |
| Type 2B (cat 2 LBT) | 16us | 16us | Inside COT |
| Type 2C (cat 1 LBT) | Without channel sensing | <16us | Inside COT |

In Type 1, there are 4 channel access Priority Class (p) as shown in Table 2 (Table 4.1.1-1 in [2]). When p=0 and no collision was detected in previous transmission, the sensing duration is randomly selected from 25,34,43 and 52us as calculated from 16+9*mp+9*Ninit.  Ninit is a random number uniformly distributed between 0 and CWp .

Table 2　 Channel Access Priority Class (CAPC)

| Channel Access Priority Class (p) | mp | CWmin,p | CWmax,p | Tmcot,p | allowed CWpsizes |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3 | 7 | 2 ms | {3,7} |
| 2 | 1 | 7 | 15 | 3 ms | {7,15} |
| 3 | 3 | 15 | 63 | 8 or 10 ms | {15,31,63} |
| 4 | 7 | 15 | 1023 | 8 or 10 ms | {15,31,63,127,255,511,1023} |

NR Sidelink slot format

NR sidelink slot format is shown in Figure 1. PSCCH is transmitted in the same slot as the associated PSSCH. PSCCH and PSSCH start from a second symbol that is available for sidelink transmission in a slot. PSFCH is transmitted in second symbol counted from the end of the sidelink slot when PSFCH is configured in the sidelink slot. First symbol is copy of second symbol for AGC (Auto gain control) and before PSFCH, copy of PSFCH is transmitted for AGC. The length of PSSCH is (pre-)configured by sl-LengthSymbols. PSSCH is not transmitted in symbols are configured for use by PSFCH if PSFCH is configured in the sidelink slot. The one symbol for guard period is located on the last symbol in the sidelink slot and between PSSCH/PSFCH to switch TX and Rx.

Figure 1 Sidelink slot format

Sidelink unlicensed operation

When all 14 symbols are (pre-)configured for a sidelink slot, one symbol for guard period is located between sidelink slots. The symbol durations for SCS 15kHz, 30kHz and 60kHz are show in Table 3.

Table 3. Symbol duration of each SCS

|  | Symbol duration |
| --- | --- |
| 15kHz | 66.67 us |
| 30kHz | 33.33 us |
| 60kHz | 16.67 us |

PSCCH/PSSCH

For Type 1 channel access procedures, in 60 kHz SCS, one symbol is shorter than smallest sensing duration as 25 us. In 15 kHz and 30 kHz SCS, depending on the priority value p and random value Ninit, one symbol is shorter than sensing duration. To have enough sensing duration, there are two approaches as shown in Figure 2. First approach is that the shorter length of PSSCH is (pre-)configured by sl-LengthSymbols in the resource pool in unlicensed band. To have 52 us sensing duration, 2 symbols/4 symbols are necessary for sensing duration in 30 kHz/60 kHz SCS. One symbol of guard period is also used for sensing duration, then additional guard period as 1 symbol/3 symbols are necessary for sensing duration in 30 kHz/60 kHz SCS respectively. It is useful for shorter sensing duration in lower priority class as p=1. Second approach is that UE can get COT when previous slot is idle. It is useful for longer sensing duration in higher priority class as p=3,4.

Figure 2　Type 1 channel acess prcedures

For Type 2 channel access procedures, in 15kHz and 30kHz SCS, the guard period is longer than 25us. Therefore, the COT cannot be continued between slots. If multiple sidelink slots share a COT with Type 2A/2B/2C, the guard period is shortened in Type 2A/2B and there is no guard period in Type 2C. If multiple slots share a COT, CP extension is necessary as shown in Figure 3. In this case, which of Type 2A, Type 2B or Type 2C is used should be notified in advance. The COT duration should be indicated to other UEs. When UE detects COT duration >1 in slot K and channel is idle in sensing duration, UE transmits CP extension in the last symbol in slot K.

Figure 3　Type 2 channel acess prcedures

We envisage two options to support sidelink unlicensed operation.

Option 1) Only Type 1 channel access procedure is used.

A UE obtains COT per sidelink slot by Type 1 channel access procedures. COT is not shared by multiple UEs. This option is lower spec impact.

Option 2) Type 1 and Type 2 channel access procedure is used.

A UE obtains UE initial COT by Type 1 channel access procedure and COT duration is indicated in every slot. Multiple UEs share the UE initiated COT with Type 2 channel access procedures.

The resource utilization ratio can be improved in option 2. However, there are larger spec impact as CP extension, the determination and indication of COT duration by the UE who obtains a COT, how to know priority values. We think option 1 can work in unlicensed band with shorten sl-LengthSymbols is (pre)configured in a resource pool. It has minimum specification impact. Our view is to support sidelink in unlicensed band is to investigate new usage case and the optimization is not essential for the first release. We think the amount of unlicensed operation should be limited in order to finalize 4 objectives for Rel.18 [1] in timely manner.

Proposal 1: Only Type 1 channel access procedure is used in sidelink unlicensed band. A COT is not shared among multiple UEs.

PSFCH

For PSFCH, multiple UEs transmit PSFCH on the same symbol, then timing of channel access procedure should be aligned for multiple UEs. In uplink link, before PUCCH transmission, gNB can control Type 1 or Type 2 channel access procedures of PUCCH. However, in sidelink, one UE doesn’t control all transmissions. The PSFCH in sidelink is not the feedback of the slot with PSFCH. The timing location of PSFCH is not controlled by the UE who obtains a COT. We see three options as follows.

Option 1) Only Type 1 channel access procedure is used.

When a UE uses Type 1 channel access procedures for PUCCH transmissions in UL, the UE shall use UL channel access priority class p=1 . Similarly, PSFCH in sidelink can be also priority class p=1. For PSFCH with p=1, in 15kHz, one symbol is enough for sensing duration. Then the existing guard period can be used for sensing duration.

Option 2) Type 1 and Type 2 channel access procedure is used.

For Type 2 usage, UE should detect PSCCH/PSSCH in the slot with PSFCH and COT duration > 1. Either of longer PSSCH length or CP extension of PSFCH is needed to align with Type 2 sensing duration. When UE need to switch Tx and Rx, the shorten guard duration might be not enough. When UE doesn’t detect PSCCH in the slot with PSFCH, UE uses Type 1 channel access procedure.

Option 3) PSFCH is not supported in unlicensed band.

In 30kHz and 60kHz SCS, additional guard period is necessary. For 60kHz, if both before PSFCH and end of slot has additional guard period, PSSCH length is too short.

The resource utilization ratio can be improved in option 2. However, there is larger spec impact to support indication of COT duration and shorten guard period. For 15kHz, there is no issue with Type 1 channel access procedure. Then we propose to use Type 1 for 15kHz. For 30kHz and 60kHz, option 3 of not supporting PSFCH has lower spec impact. HARQ can be supported by retransmission without PSFCH. On the other hand, for PSFCH, whether interlaced mapping is supported or not should is discussed [3]. If interlaced mapping is not supported, PSFCH is not supported for also 15kHz is one option.

Proposal 2: For PSFCH, channel access priority class is p=1.

Proposal 3: For 15kHz, Type 1 channel access procedure is used for PSFCH.

Proposal 4: For 30kHz and 60kHz, PSFCH is not supported in unlicensed band.

S-SS/PSBCH

In NR-U, Type 2A channel access procedures used for discovery burst as “Transmission(s) initiated by a gNB with only discovery burst or with discovery burst multiplexed with non-unicast information, where the transmission(s) duration is at most 1ms, and the discovery burst duty cycle is at most” [2]. As similar as discovery bust, Type 2A is used for S-SS/PSBCH.

Proposal 5: Type 2A channel access procedure is used for S-SSS/PSBCH block.

Conclusion

This document provided our view on sidelink channel access on unlicensed spectrum. Based on the discussions, we have following proposals.

Proposal 1: Only Type 1 channel access procedure is used in sidelink unlicensed band. A COT is not shared among multiple UEs.

Proposal 2: For PSFCH, channel access priority class is p=1.

Proposal 3: For 15kHz, Type 1 channel access procedure is used for PSFCH.

Proposal 4: For 30kHz and 60kHz, PSFCH is not supported in unlicensed band.

Proposal 5: Type 2A channel access procedure is used for S-SSS/PSBCH block.

Reference

[1] RP-220300, "WID revision: NR sidelink evolution"

[2] TS 37.213, "Physical layer procedures for shared spectrum channel access"

[3] R1-2203750, "Physical channel design for sidelink on unlicensed spectrum", Panasonic