3GPP TSG-RAN WG1 #109	R1-2203751

e-Meeting, 9th May – 20th May, 2022

Agenda item:		9.5.1.1

Source:	MediaTek Inc.

Title:	Scenarios and requirements for sidelink positioning

Document for:		Discussion

# Introduction

The conventional RAT dependent positioning performs the measurements between the UE and the base stations for absolute positioning. The events that happen locally may resort to the local measurements to resolve the problems. In these events, the relative positioning is more critical than the absolute positioning.

The measurements between UEs could realize the measurements locally. Furthermore, in some applications, the relative positioning maybe more crucial than the absolute positioning.

In this contribution, we provide our views for the scenarios and requirements particularly for the relative positioning under sidelink framework.

# Scenarios

The videos as shown in [1][2][3] are the accidents that can be avoided if some new technologies related to the relative positioning apply. It is observed that,

- In [1], a motorcycle seems not be able to control the speed during the turn so that it is accidently across to the opposite lane, causing the collision with a taxi

- In [2], a moving truck doesn’t keep sufficient lateral distance with a static tour bus, causing the collision

- In [3], a sedan doesn’t keep proper trajectory within a lane after lane change, further moving to across to another lane to cause the collision

In RP-210036 the reply LS from SAE to 3GPP RAN, there are several points mentioned by SAE,

- It is not only important to acquire accurate longitudinal and latitudinal coordinates but also it is important to improve the positioning service availability even when a UE cannot rely on the conventional positioning technologies that utilize cellular infrastructure, GNSS or the combination of the two

- It is necessary to improve the positioning technology so that the advanced V2X applications can work properly even in various out-of-coverage scenarios

In above cases, the sidelink based for positioning may improve the service availability that the conventional positioning for absolute positioning can’t achieve.

Fig. 2-1 shows the scenario for sidelink assisted for positioning. The UE A is under NLOS channel with some TRPs, the UE B having better channel condition may assist UE A for positioning.

Fig. 2-1

# Requirements

The considered requirements could be found from the 3GPP SA specifications and the 5GAA documents. It is seen that,

- In RP-210040 the LS reply from 5GAA to 3GPP RAN, 3 different levels of accuracy: tens of meters, lane level, and sub meters are listed together with the targeted use cases

- In TS 22.261 7.3.2.2, the requirements for horizontal and vertical positioning service levels in table 7.3.2.2-1 don't preclude any type of UE. Then it could be interpreted that the requirements are also applicable to the specific UE for v2x

- In TS 22.186,

  - [R.5.1-007] The 3GPP system shall support relative lateral position accuracy of 0.1 m between UEs supporting V2X application (for general requirement)

  - [R.5.2-003] The 3GPP system shall support relative longitudinal position accuracy of less than 0.5 m for UEs supporting V2X application for platooning in proximity (for vehicle platooning)

In our view, the accuracy is always related to the channel condition. Then for RAN1, the design goal for the accuracy requirement may take the channel condition as the side condition.

Proposal 3-1: For RAN1, the design goal for the accuracy requirement may take the channel condition as the side condition

Proposal 3-2: The relative lateral position accuracy of 0.1m and the relative longitudinal position accuracy of 0.5m could be the starting point for evaluation

# Conclusion

Proposal 3-1: For RAN1, the design goal for the accuracy requirement may take the channel condition as the side condition

Proposal 3-2: The relative lateral position accuracy of 0.1m and the relative longitudinal position accuracy of 0.5m could be the starting point for evaluation

# Reference

[1]  https://www.facebook.com/167876306720403/videos/326436622816398

[2]  https://www.facebook.com/watch/?v=4666559720056772

[3]  https://www.facebook.com/watch/?v=673797650409306