3GPP TSG-RAN WG1 #109	R1-2203752

e-Meeting, 9th May – 20th May, 2022

Agenda item:		9.5.1.3

Source:	MediaTek Inc.

Title:	The potential solutions for sidelink positioning

Document for:		Discussion

# Introduction

The sidelink positioning performs the measurement between UEs for relative and absolute positioning. The physical resources used for the measurements could be allocated at centralized and decentralized mechanism.

The relative positioning has promising applications at least for the V2X and IIOT scenarios. In this contribution, we provide the solutions to realize the measurement between UEs, especially under the decentralized mechanism.

# Design for the measurement between UEs

To realize the measurement between UEs under SL framework, the following items may be considered,

- The RS for the measurements (SL-PRS).

- The measurement types

- The steps to facilitate the measurements, transmission and reporting within the positioning flow

# The RS for the measurements (SL-PRS)

The DL-PRS adopts the Gold sequence and SRS is applied with the ZC sequence. The sequence for the SL-PRS may be chosen from Gold or ZC.

The ZC is slightly preferred than the Gold, due to the low PAPR property. The Rel-18 scope may not specify UE to measure its transmitted signal due to the bouncing back, which is the radar property. If the forward compatibility is considered, ZC seems to be a better choice than Gold.

The full staggering RS structure designed in Rel-16 considers the longer distance between TRP and UE. When the sidelink is targeted as the short range transmission, the partial staggering, or non-staggering RS structure are preferred.

Proposal 3-1: ZC is slightly preferred over the Gold as the sequence for SL-PRS, due to the low PAPR property

Proposal 3-2: Consider partial staggering or non-staggering structure for SL-PRS

Proposal 3-3: Develop the power control mechanism for SL-PRS

# The measurement types

The measurement types for NR positioning are downlink or uplink specific,

| downlink | uplink |
| --- | --- |
| DL RSTD DL PRS-RSRP DL PRS-RSRPP UE RX-TX time difference | UL RTOA UL AOA UL SRS-RSRP UL SRS-RSRPP gNB RX-TX time difference |

For the UE RX-TX time difference, the TX timing is based on the TA. The UL RTOA is relative to a reference time. In sidelink, there is no TA. Then the transmission may be relative to the RX timing. As such, the definition of RTOA seems to be more suitable for the RTT technique under sidelink.

Proposal 4-1: Slightly prefer to use the terminology of SL RTOA measurements for RTT technique under sidelink

Proposal 4-2: Define the measurement types of SL RSTD, SL PRS-RSRP, SL PRS-RSRPP, SL RTOA and SL AOA

# Steps to facilitate measurements, transmission and reporting within the positioning flow

Under the centralized control mechanism, the DL-PRS, and SRS are pre-configured before the start of the measurement and transmission, so that the UE knows when to receive DL-PRS and further to descramble it. UE also knows when to transmit SRS with the corresponding power and direction. The processing capability of UE is also reported to LMF, and the SRS transmission capability of UE is reported to the gNB.

Under the decentralized mechanism, the coordination between the UEs may be needed when a UE decides to start a positioning flow. Then some steps may contain the coordination to accomplish a positioning flow.

The positioning flow in LPP/NRPPa could be leveraged. For example the steps such as the capability transfer and the location request may be required during sidelink positioning.

The capability for sidelink positioning may contain the measurement (SL-PRS processing) capability and the transmission capability. The measurement capability may simply indicate that a period and a number of SL-PRS that a UE could process in the period. The UE may also indicate the remaining available number for SL-PRS processing so that another UE may decide whether to initiate the request.

The transmission capability may at least contain, for example,

- the number of beams for transmission under the beam-sweeping manner

- the BW for SL-PRS transmission

- the pathloss RS measurement capability for transmission, and the remaining number for pathloss RS measurement

The location request for sidelink positioning may contain two types, the measurement/providing-results request and the transmission request. The request type may depend on the positioning technique and the corresponding measurement type to use. For example, if UE A would like to adopt the technique similar to DL-TDOA and DL-AOD, UE A may request other UEs for the transmission.

If UE A would like to adopt single-RTT technique, UE A may request UE B for both the transmission and the measurement/providing-results. And if UE A decides to adopt the technique similar to UL-TDOA, UE A may request other UEs for measurement/providing-results.

The coordination for SL-PRS configuration may happen for the transmission request and the measurement/providing-results request. UE A may play the role as RX UE to provide the preferred setting during the transmission request. And during the measurement/providing-results request, UE A may play the role as TX UE to provide the SL-PRS configuration for transmission, and pre-occupy the physical resources for other UEs to send back the measurement results.

The other UEs, when receiving the requests, may also determine whether to adopt the proposed SL-PRS configuration by UE A. The other UEs may also play the role as RX UE to assess the SL-PRS configuration for transmission from UE A. The conflict resolution step may further happen as part of coordination.

The priority rule for SL-PRS transmission may be needed when there is collision between TX and RX, or between TX and TX

Proposal 5-1: RAN1 to discuss whether the capability transfer and the items are required for positioning measurement. The capability may contain the measurement and transmission capability. Once agreed, RAN2 may further deal with the corresponding signalling

Proposal 5-2: RAN1 to discuss the configured parameters for SL-PRS transmission. Once agreed, RAN2 may further handle the corresponding signalling

Proposal 5-3: RAN1 to discuss the required assistance information for SL-PRS measurement. Once agreed, RAN2 may further handle the corresponding signalling

Proposal 5-4: RAN1 to discuss whether there are pre-defined time/frequency physical resources for SL-PRS allocation. SCI could be used to indicate the occupation within the pre-defined physical resources

Proposal 5-5: RAN1 to study the coordination mechanism for SL-PRS configuration

# Conclusion

Proposal 3-1: ZC is slightly preferred over the Gold as the sequence for SL-PRS, due to the low PAPR property

Proposal 3-2: Consider partial staggering or non-staggering structure for SL-PRS

Proposal 3-3: Develop the power control mechanism for SL-PRS

Proposal 4-1: Slightly prefer to use the terminology of SL RTOA measurements for RTT technique under sidelink

Proposal 4-2: Define the measurement types of SL RSTD, SL PRS-RSRP, SL PRS-RSRPP, SL RTOA and SL AOA

Proposal 5-1: RAN1 to discuss whether the capability transfer and the items are required for positioning measurement. The capability may contain the measurement and transmission capability. Once agreed, RAN2 may further deal with the corresponding signalling

Proposal 5-2: RAN1 to discuss the configured parameters for SL-PRS transmission. Once agreed, RAN2 may further handle the corresponding signalling

Proposal 5-3: RAN1 to discuss the required assistance information for SL-PRS measurement. Once agreed, RAN2 may further handle the corresponding signalling

Proposal 5-4: RAN1 to discuss whether there are pre-defined time/frequency physical resources for SL-PRS allocation. SCI could be used to indicate the occupation within the pre-defined physical resources

Proposal 5-5: RAN1 to study the coordination mechanism for SL-PRS configuration