3GPP TSG RAN WG1 #109	R1-2203755

e-Meeting, May 9th – May 20th, 2022

Agenda item:		9.12.2

Source:	Nordic Semiconductor ASA

Title:					Disabling of HARQ feedback for IoT NTN

Document for:		Discussion and Decision

# Introduction

In RAN#95, work item in RP-220979 has been approved. The objectives regarding disabled HARQ feedback are the following

This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below: - Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2]

# On Disabling of HARQ feedback

For HARQ enabling and disabling, the following conclusions were made during the study item phase of NR NTN [1].

For NTN the network could disable uplink HARQ feedback for downlink transmission at the UE receiver e.g. to support long propagation delays. Even if HARQ feedback is disabled, the HARQ processes are still configured. Enabling / disabling of HARQ feedback is a network decision signaled semi-statically to the UE by RRC signaling. The enabling / disabling of HARQ feedback for downlink transmission should be configurable on a per UE and per HARQ process basis via RRC signaling.

Accordingly, an agreement was made in NR NTN saying that enabling/disabling on HARQ feedback for downlink transmission should be configurable per HARQ process via UE specific RRC signalling. We think that the same approach for configuration of enabling/disabling of HARQ feedback would be beneficial for IoT NTN.

Proposal 1: At least for eMTC, enabling/disabling on HARQ feedback for downlink transmission should be semi-statically configurable per HARQ process via UE specific RRC signalling

There is a certain processing latency for PDSCH processing at UE and that’s why the UE can receive a new TB for the same HARQ process only after that processing latency. For eMTC, the processing latency of 2s should be sufficient.

Proposal 2: In eMTC, for a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH scheduled for the given HARQ process that starts until 2ms after the end of the reception of the last PDSCH for that HARQ process.

In NB-IoT, with disabled HARQ feedback, a gap from the end of the last subframe of NPDSCH to the beginning of NPDCCH carrying the assignment to the same HARQ process needs to be specified. Since there are NB-IoT devices on market with a wide range of computational capabilities, it would be useful from NTN system point of view if these varied capabilities could be leveraged by allowing more flexible timing relationships for NB-IoT NTN. Thus, we propose that the gap value for a single HARQ process case could be configurable per UE and the UE specific value could be based on the capability signalling of the UE. This is not however needed for UEs supporting 2 HARQ processes since such devices are already quite capable as such, and thus the gap for those UEs, as defined from the end of last subframe of NPDSCH carrying the second HARQ process to the beginning of NPDCCH, could be fixed, for example 2ms.

Proposal 3: In NB-IoT single HARQ case, the gap from the end of the last subframe of NPDSCH to the beginning of NPDCCH is configurable by the UE-specific RRC-signalling and its value could be based on capability indicated by the UE.

Proposal 4: In NB-IoT two HARQ case, the gap from the end of the last subframe of NPDSCH carrying the TB of the second HARQ process to the beginning of NPDCCH is fixed in specification.

When the HARQ feedback is disabled in downlink, it is beneficial that eNB obtains updated CSI periodically or at least when changes in channel condition happens. In eMTC, efficient CSI reporting mechanisms already exist, but NB-IoT devices have more restricted possibilities for CSI reporting, even though support for connected mode CSI reporting other than Msg3 reporting was added in Rel-16. Therefore, it would be good if fast and frequent CSI reporting methods are considered for NTN deployment of NB-IoT. Alternatively, a one-bit feedback channel for UE to suggest an increase or decrease in PDSCH MCS or repetition value would be beneficial.

Observation 1: A new CSI reporting method or a one-bit feedback channel for UE to suggest an increase or decrease in MCS or repetition value of NPDSCH would be beneficial.

# Conclusions

In this contribution we discussed issues related to disabled HARQ feedback for IoT NTN and made the following proposals:

Proposal 1: At least for eMTC, enabling/disabling on HARQ feedback for downlink transmission should be configurable per HARQ process via UE specific RRC signalling

Proposal 2: In eMTC, for a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH scheduled for the given HARQ process that starts until 1ms after the end of the reception of the last PDSCH for that HARQ process.

Proposal 3: In NB-IoT single HARQ case, the gap from the end of the last subframe of NPDSCH to the beginning of NPDCCH is configurable by the UE-specific RRC-signalling and its value could be based on capability indicated by the UE.

Proposal 4: In NB-IoT two HARQ case, the gap from the end of the last subframe of NPDSCH carrying the TB of the second HARQ process to the beginning of NPDCCH is fixed in specification.

Observation 1: A new CSI reporting method or a one-bit feedback channel for UE to suggest an increase or decrease in MCS or repetition value of NPDSCH would be beneficial.

# References

[1] 3GPP TR 38.821 v16.1.0 Solutions for NR to support non-terrestrial networks (NTN)  (Release 16)