3GPP TSG RAN WG1 #109-e                                              R1-2203758

e-Meeting, May 9th –20th, 2022

Agenda Item:	9.12.2

Source:	CATT

Title:	         HARQ feedback disabling for IoT NTN

Document for:	Discussion and Decision

Introduction

Based on new WID RP-220979 on IoT NTN [1], HARQ feedback disabling needs to be studied in Rel-18.

Disabling of HARQ feedback has been agreed in NR NTN: Enabling/disabling on HARQ feedback for downlink transmission should be at least configurable per HARQ process via UE specific RRC signalling. Since it is considered not essential for system operation, increasing the number of HARQ processes for NB-IoT and for eMTC in NTN is recommended not to be supported in Rel-17. Considering the benefits of disabling HARQ feedback, in this contribution, we further analyze the technical issues of NB-IoT/eMTC HARQ operation and provide our views on these issues.

Discussion

  - Disabling HARQ feedback

As specified in Rel-16 TS 36.213[2], there shall be a maximum of 2 HARQ processes for NB-IoT, and a maximum of 8 HARQ processes for CEModeA and 4 HARQ processes for CEModeB. Since enhancement on TN IoT has been studied in Rel-17, maximum 14 HARQ processes for MTC UE are supported. Therefore, a unified baseline is necessary for all companies. We expect that Rel-17 is a baseline for HARQ enhancement.

Proposal 1: Rel-17 IoT functionality on HARQ is recommended as the baseline for HARQ enhancement of IoT NTN.

It was proposed that a new work item is proposed to define further enhancements for NB-IoT NTN and eMTC NTN in order to improve performance in terms of throughput in WID on IoT NTN enhancements [3]. A simple method to improve throughput is to disable HARQ feedback. Take NB-IoT as an example, if HARQ feedback enabled,

- HARQ stalling due to the large RTT may reduce UE data rates, as another NPDSCH with the same HARQ process should be scheduled after receving a feedback of the previous NPDSCH

- an uplink resource will be always needed for HARQ-ACK feedback, more power consumption is needed from UE side

- Considering HD-FDD processing for IoT UE, more UL transmission will impact DL scheduling and resource allocation in time domain and impact DL throughput/data rate, especially for large coupling losses in the uplink that necessitate large number of repetitions in the uplink.

Base on these advantages of disabling HARQ feedback, along with minor changes on specification and implementation, it is recommended to disable HARQ feedback. And it is sufficient to reuse the disabled HARQ feedback mechanism of NR NTN.

Proposal 2: Disabling HARQ feedback mechansim of NR NTN can be reused for IoT NTN.

Especially when a UE is configured with one HARQ process, if the HARQ feedback is disabled, the eNB would never know whether the downlink transmission is correctly received through the HARQ mechanism of the physical layer, including paging and system messages. Therefore, whether the disabling HARQ mechanism is introduced for the UE needs to further studied.

Proposal 3: Further study on if HARQ disabling is needed for IoT UE that supports only one HARQ process.

Conclusion

In this contribution, we analzyed potential issues of HARQ impact due to longer propogation delay in NB-IoT and eMTC over satellite. In sum, the observations and proposals are listed as follows:

Proposal 1: Rel-17 IoT functionality on HARQ is recommended as the baseline for HARQ enhancement of IoT NTN.

Proposal 2: Reuse disabling HARQ feedback mechansim of NR NTN for IoT NTN.

Proposal 3: Further study on if HARQ disabling is needed for IoT UE that supports only one HARQ process.

References

[1] RP-220979, WID on IoT NTN enhancements

[2] 3GPP TS 36.213: "LTE; Physical layer procedures"

[3] RP-212729, “New WID on IoT NTN enhancements”, AT&T, 3GPP TSG RAN Meeting #94e-b meeting, Dec. 6 - 17, 2021