3GPP TSG RAN WG1 #109-e                                            R1-2203760

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.12.4

Source:	CATT

Title:	Others issues for NR NTN

Document for:	Discussion and Decision

Introduction

In 3GPP TSG RAN #95e meeting, new NTN SID was approved and the network verified UE location is one studying point[1]. For the network verified the UE location aspect, the researching points were listed as follow:

- Study detailed regulatory requirement (e.g. accuracy, privacy, reliability, latency) for network-verified UE location for potential use cases/services (i.e. emergency call, lawful intercept, public warning, charging/billing) (at RAN plenary, from RAN#95 to RAN#96). [RAN]

  - Including further clarification on network verified UE location and its relationship to network-based positioning [RAN]

  - Study and evaluate, if needed, solutions for network to verify UE reported location information [RAN2,RAN1,RAN3]

On the other hand, the NR-NTN deployment in above 10 GHz bands was also discussed in [1]. Regarding the NR-NTN deployment in above 10 GHz bands, currently there is one bullet to define the possible work scope in RP-213690 as follows:

- Identify values for physical layer parameters chosen from the existing FR1 and FR2 sets. The following set of parameters to specify, but not necessarily limited to, are listed as follows [RAN4]:

  - time relationship related enhancement (e.g. K_offset)

  - subcarrier spacing for different UL/DL signals/channels

  - PRACH configuration index for FDD above 10 GHz.

In this document, topics of the UE location verified and the long CP issue were discussed.

Network verified UE location

  - Requirement of the UE location for network

Compared with the NR terrestrial cell, the coverage of the NTN scenarios is obviously huge, especially for GEO satellite. The footprint of single beam generally has hundreds kilometers in GEO scenarios. In order to provide favorable services, it is essential to distinguish the location of the UEs in the NTN networks. Collecting the UE’s location to network is one of the most convenient methods to resolve the location problem for the network. The requirements of network to figure out clearly the UEs’ location information are including the following factors.

- Handover: It is necessary to network indicate the UE to act the handover based on the UE’s location because the big coverage and long distance of satellite links will influence the accuracy of handover decision which just only based on RRM measurement.

- Paging: The UE location can assist to execute an easy paging for network in the large coverage of NTN satellite cells.

- Random access: It will be more efficient with acquired the UE location by network when the UE applies to access the network.

- Other requirements: The UE location can be also utilized in distinguishing the bound of service areas, especially between the two different countries.

As described above, obtaining the UE location is beneficial to network in NTN scenarios because of the large coverage of the satellites cell. Meanwhile, the validity of the UE location information is also very important. The invalidity location information not only affects the mobility management of the UE, but also brings risk to mobile management. So it is necessary to verify the validity of the UE location information for the network.

Observation 1: Obtaining the UE location is beneficial to network especially in NTN scenarios.

Proposal 1: It is essential for network to verify the validity of the UE’s location information.

  - Network verified the UE location based on UE reporting

In NTN scenarios, the UE with GNSS capability is the prerequisite. So the UE can get own location information by fixing the position based on the GNSS. In generally, the UE should report the location information to network after the UE accessed the network successfully. The network will store the UE location and use this information when the handover or paging actions are happened. Considering the unstable of GNSS signals or wireless channel of the satellite links, the UE location information with data error will be transmitted to network. So it is necessary to verify the validity of the UE location information which reported based on UE’s GNSS position.

Proposal 2: It is necessary to verify the UE location information by network even if the UE has reported the GNSS position to network.

  - Network verified the UE location without UE reporting

In the NTN networks, sometimes, the signals of GNSS would be unstable, or the UE cannot report the location to the network because of the security policy restrained. In this situation, the UE location would be verified by network without UE reporting. Hence, the network based positioning should be a solution to verify the UE location by network. The networks based positioning means that the network configures the positioning reference signals, collects the measurements of signals, and calculates the location information. Such as the RAT-dependent positioning solutions are utilized in NR networks which can calculate the UE location information by collecting the measurement to network. So when there is no location reported from UE, the network positioning is a solution to verify the UE location.

Proposal 3: Network positioning, such as RAT-dependent positioning, is a potential solution for network verified the UE location.

  - Network verified the UE location based on NR positioning solution

In the previous section, the UE reported GNSS location was discussed. In this section, the network verified the UE location with NR positioning solutions will be analyzed.

Introduction of NR positioning solutions

According to TS 38.305, the RAT-dependent based positioning solutions in NR are including the following aspects [3].

-	NR enhanced cell ID methods (NR E-CID) based on NR signals;

-	Multi-Round Trip Time Positioning (Multi-RTT based on NR signals);

-	Downlink Angle-of-Departure (DL-AoD) based on NR signals;

-	Downlink Time Difference of Arrival (DL-TDOA) based on NR signals;

-	Uplink Time Difference of Arrival (UL-TDOA) based on NR signals;

-	Uplink Angle-of-Arrival (UL-AoA), including A-AoA and Z-AoA based on NR signals.

- NR E-CID

In E-CID, the cell ID is utilized to represent the UE location. The RRM measurement is introduced in NR to enhance the accuracy of the positioning by the UE measuring the SSS-RSRP, SS-RSRQ, CSI-RSRP or CSI-RSRQ.

- NR DL-TDOA

In the NR DL-TDOA method, UE measures the DL PRS of the neighbour TPs to get DL PRS RSTD. Then, according to the location information of the neighbour TPs, the position is fixed by network or UE. It is worth noting that at least 4 TPs should be measured by UE to calculate the positioning location.

- NR UL-TDOA

In the NR UL-TDOA method, the UE transmits the reference signals, such as SRS, and the RPs measure the signals. The measurement results can be collected to the network, and the location of UE is fixed by the networks. The same to the DL-TDOA method, at least 4 RPs should be involved in the signal measuring to position the UE location.

- NR Multi-RTT

In the NR Multi-RTT method, the UE and TRPs should measure the signal of each other. The measurement results should be collected to the network. According to the RTT measurement results and the location information of the TRPs, the UE location can be fixed by the network.

- NR DL-AoD

In the NR DL-AoD method, the UE measures the DL PRS of the neighbour TPs to get the angle of the downlink signals. Then UE reports the results of the angle measurement to the network. The network calculates the location of UE by the measurements.

- NR UL-AoA

In the NR UL-AoA method, the RPs measure the signals SRS-Pos from the UE, and get the azimuth angle and zenith angle. The RPs collect the UL-AoA measurement to the network. The network determines the UE location according to the angles measured and the RPs’ location.

As descripted above, the RAT-dependent based positioning solutions in NR determine the UE location always based on the signal measurements. The measurement parameter in E-CID is the power strength of the downlink signal. The measurement parameter in both TDOA and Multi-RTT is the transmitting time or time difference. The parameters in AoD and AoA are the angles of the reference signals.

Observation 2: In the RAT-dependent based positioning solutions, the measurement parameters including signal strength, time or time difference, and arrival angles of signal are involved in calculating the UE location.

Analysis of NR positioning solution in NTN scenarios

In the NTN scenarios, especially in satellites networks, the long distance caused the degradation of the signal is serious, and the transmission delay of signal is also large. The influence of the weak signal and long delay on the RAT dependent based positioning solutions in NTN scenarios can be analyzed in the following.

- Feasibility of NR E-CID method

As descripted in the TR 38.821, the satellite beam diameter for GEO is almost 250km, 90km for LEO-1200 and 50km for LEO-600 [2]. So, the cell coverage becomes very huge even for just one beam one cell in the satellite scenarios comparing with the NR cells which generally hundred meters. The signal strength at the edge of the cell is just 3dB less than at the center of the cell. So if the NR E-CID method is applied in NTN satellite scenarios, it is difficult to achieve an accurate location of the UE because of the large scale coverage of the satellite beam. And the RRM measurement cannot improve the accuracy of the location in satellite scenarios. The network cannot distinguish the exact location of UE based on the NR E-CID method in the NTN cells.

Observation 3: The NR E-COD method cannot be straightly applied in NTN scenarios because of the large coverage of satellite beams.

- Feasibility of NR TDOA and multi-RTT method

In the NR TDOA and multi-RTT method, the transmitting time or time difference should be measured to calculate the UE’s location. As shown in TR 38.821, the CNR of both UL and DL is very low [2]. The low CNR will result in week accuracy of time measurement in these time based techniques, which affects positioning accuracy for the UE. Furthermore, in order to fix the UE’s location, it needs multi TRPs or satellites to involve. In the NR networks at terrestrial, the base stations can be deployed intensively, and the cells will be overlapping to each other. However, in the NTN scenarios, the overlapping deployment of multi satellites no matter GEO or LEO is unusual. So it will be a risk point to involve multi satellites to apply time based positioning technique. In the end, the long distance of satellite communication introduces large delay of the signals transmission. The long and different transmission delays will also affect the cooperation of the deploying PRS in multi satellites.

Observation 4: In the satellite scenarios, the low CNR, less overlapping coverage, and long various transmission delays should be seriously considered firstly when deploying the NR TDOA or multi-RTT positioning method.

- Feasibility of NR AoD or AoA method

As described in TS 38.305, the DL-AoD positioning method makes use of the measured DL-PRS-RSRP of downlink signals received from multiple TPs, at the UE, and the UL-AoA positioning method makes use of the measured azimuth and zenith of arrival at multiple RPs of uplink signals transmitted from the UE [3]. But in the satellite scenarios, the fixed beam and moving beam are supported in 3GPP. For the fixed beam, the beam coverage in earth is unchanged for a while, so the angle between beam and satellite is time-varying. It makes uneasy to measure the angle of signals. For the moving beam, the coverage of beam is always dozens kilometers, the receiver cannot exactly distinguish the angle of signals both for the satellite or the UE. So the accuracy of measuring angles is week. Furthermore, it is difficult to catch multi-measurement of angles simultaneously from different satellites because of the less overlapping areas in satellite scenarios.

Observation 5: For the NR AoD or AoA method, the specific method of measuring angles of signals should be evaluated firstly in NTN satellite scenarios.

Discussion of the major issue affecting the RAT position

As discussed above, there are serious problems when the RAT-dependent NR positioning solutions were deployed directly in NTN scenarios. The characteristic of the RAT-dependent NR positioning method applied in NTN scenarios can be summarized in the following table.

Table-1 Summary of the RAT-dependent NR positioning method for NTN scenarios

| NR positioning method | Parameter to measure | Positioning method | Problem in NTN scenarios | positioning level in NTN |
| --- | --- | --- | --- | --- |
| E-CID | CID or SNR | Network based | Huge coverage | Beam level or NTN cell level, hundreds kilometers |
| NR DL-TDOA | Time difference | UE based or network based | Less overlapping coverage, time synchronizing between the satellites, low SNR | NR cell level, hundreds meters or few kilometers under the perfect conditions. |
| NR UL-TDOA | Time difference | Network based | Less overlapping coverage, time synchronizing between the satellites, low SNR |  |
| NR Multi-RTT | time | Network based | Less overlapping coverage, low SNR |  |
| NR DL-AoD | angle | UE based or network based | Method of angle measuring, less overlapping coverage | Beam level or NTN cell level, hundreds kilometers |
| NR UL-AoA | angle | Network based | Method of angle measuring, less overlapping coverage |  |

From the table, the different method of RAT dependent position can achieve the different positioning level in NTN scenarios. The E-CID method can just only distinguish the beam level or NTN cell level accuracy of the UE location. The NR DL-AoD and UL-AoA methods also just achieve the beam level of NTN cell level accuracy of UE location because of the uneasy angles measuring. The timing-measurement based methods can obtain optimum positioning results with hundreds meters level under the perfect conditions in NTN networks. As shown in table, the major issues of applied the RAT-dependent positioning solutions in NTN scenarios can be summarized as the less overlapping coverage, the lower SNR, the frequency band, and the UE types.

- The problem of less overlapping coverage in NTN

In the NR cells, the RAT-dependent positioning solutions are applied in the overlapping areas of multi-cells, especially in TDOA, Multi-RTT, AoD and AoA methods. However, in NTN scenarios, the overlapping areas are less especially for fourfold overlapping. According to the TR 38.821, even in LEO satellites constellation scenarios, the four-fold overlapping area is very small between multi-satellites footprints. Hence, it is difficulty to receive signals simultaneously from more than 4 different satellites even in the overlapping areas. Furthermore, it is impossible to calculate the positioning location for the TDOA, Multi-RTT, AoD and AoA methods when the UEs measure the signals less than 4 satellites in the overlapping areas.

Observation 6: The less overlapping coverage makes it difficult to apply the RAT-dependent positioning solution to the NTN scenarios.

- The problem of lower SNR in NTN

In the NTN scenarios, the distances between the UEs and gNBs are always long, so the power received at UE or gNB is week. The long distance results in the huge degradation of the satellite links. The lower SNR will affect the accuracy of the positioning calculation, even resulting in an error location calculated. In order to improve the performance of position in the lower SNR, the more resource of the band and time will be utilized in RAT-dependent positioning to carry more PRS or SRS signals. The positioning signals will occupy most resource, and the service traffic load will be reduced greatly. So the efficiency of the NTN network is very week when applied the RAT-dependent positioning solutions in lower SNR conditions.

Observation 7: The lower SNR will be an inevitable issue in NTN scenarios.

- The problem of working frequency band and UE types in NTN

In the 3GPP, the RAT-dependent positioning solutions were discussed in Sub 6GHz band. But in NTN scenarios, the S band with 2GHz and Ka band with 20GHz or 30GHz had been discussed in TR 38.821 [2]. For the sub 6GHz band, it is easy to deploy gNBs to overlap the UE. And the UE’s antennas always have no direction, so it is convenient to receive signals from multi-gNBs. But for the Ka band, the beams always have exact direction. Hence, it is difficult to measure multiple signals from different satellites simultaneously. Meanwhile, the UE types also affect the application of RAT-dependent positioning solution in NTN scenarios. For the handset UE type, always working in sub 6GHz, it is possible to measure multi signals at the same time. But for the VAST UE type, which has direct antennas, it is impossible to measure multi direct signals simultaneously. Therefore, the UE types affect the application of RAT-dependent positioning solutions in NTN scenarios.

Observation 8: The frequency band and UE types will affect the application of RAT-dependent positioning solutions in NTN scenarios.

Proposal 4: The less overlapping coverage, lower SNR, frequency band and UE types are the in-negligible issue that should be seriously considered before applying the RAT-dependent positioning solutions in NTN scenarios.

Discussion of long CP in FR2

Per the NR specification, when FR2 is applied, the subcarrier spacing will be enlarged automatically. However, for NTN scenario, using larger subcarrier spacing doesn’t mean the cell coverage is shrunken. Instead, NTN cell size is still very large, which makes UL synchronization more difficult due to short CP. For 120kHz case, the short CP is only equal to 0.59us. Based on the discussion in Rel-17, the time synchronization performance is quite challenging both in FR1 and FR2. In view of unavoidable synchronization error, including GNSS error, ephemeris error or UE implementation error, extended CP is very necessary to guarantee the performance of UL synchronization. Hence, it is important to complete the specification of extended CP in RAN1 for higher subcarrier spacing. Currently RAN1 only specifies the extended CP in 60kHz subcarrier, while for 120kHz it is not defined. If no normal work to revise the extended CP in RAN1 specification, in essence, the NTN is not well workable in FR2.

In addition, it is essential to make sure that the CP should be longer than the maximum multipath delay for the OFDM systems. The NTN channel models can be attained from TR38.811 and scaling parameters can be recognized from TR38.901 [4][5]. The maximum multipath delay spread in different scaling parameters of different channel models are collected in Table-2, as shown in following.

Table-2 Maximum multipath delay spread of NTN channel model

| Channel model | Maximum multipath delay spread (us) |  |  |
| --- | --- | --- | --- |
|  | Very short delay spread | Short delay spread | Nominal delay spread |
| NTN-CDL-A / NTN-TDL-A | 0.028416 | 0.085248 | 0.28416 |
| NTN-CDL-B / NTN-TDL-B | 0.057392 | 0.172176 | 0.57392 |
| NTN-CDL-C / NTN-TDL-C | 0.148124 | 0.444426 | 1.48124 |
| NTN-CDL-D / NTN-TDL-D | 0.073340 | 0.22002 | 0.73340 |

As specified in Rel-17, the CP length for 120kHz subcarrier spacing is just only 0.59us. Compared with the maximum multipath delay shown in table above, the normal CP for 120kHz subcarrier spacing can overcome the delay spread of very short and short delay scenarios. But for the nominal delay spread scenario, the normal CP cannot satisfy the requirement of NTN-CDL-C/NTN-TDL-C or NTN-CDL-D/NTN-TDL-D channel models. So, it is beneficial to extend CP length in NTN to relieve influence involved by the delay spread of the NTN channel, especially in the 120kHz or 240kHz subcarrier spacing configurations. Because the higher subcarrier spacing is, the shorter the CP is. As a result, the shorter CP is difficult to overcome the delay spread in NTN channel.

Proposal 5: Support extended CP in more than 60 kHz subcarrier spacing in Rel-18.

Conclusion

In this contribution, we analzyed potential issues for network verified the UE location and long CP usage in NTN.

A few of observations and proposals are made as follows for network verified positioning:

Observation 1: Obtaining the UE location is beneficial to network especially in NTN scenarios.

Observation 2: In the RAT-dependent based positioning solutions, the measurement parameters including signal strength, time or time difference, and arrival angles of signal are involved in calculating the UE location.

Observation 3: The NR E-COD method cannot be straightly applied in NTN scenarios because of the large coverage of satellite beams.

Observation 4: In the satellite scenarios, the low CNR, less overlapping coverage, and long various transmission delays should be seriously considered firstly when deploying the NR TDOA or multi-RTT positioning method.

Observation 5: For the NR AoD or AoA method, the specific method of measuring angles of signals should be evaluated firstly in NTN satellite scenarios.

Observation 6: The less overlapping coverage makes it difficult to apply the RAT-dependent positioning solution to the NTN scenarios.

Observation 7: The lower SNR will be an inevitable issue in NTN scenarios.

Observation 8: The frequency band and UE types will affect the application of RAT-dependent positioning solutions in NTN scenarios.

Proposal 1: It is essential for network to verify the validity of the UE’s location information.

Proposal 2: It is necessary to verify the UE location information by network even if the UE has reported the GNSS position to network.

Proposal 3: Network positioning, such as RAT-dependent positioning, is a potential solution for network verified the UE location.

Proposal 4: The less overlapping coverage, lower SNR, frequency band and UE types are the in-negligible issue that should be seriously considered before applying the RAT-dependent positioning solutions in NTN scenarios.

For long CP issue, we propose:

Proposal 5: Support extended CP in more than 60 kHz subcarrier spacing in Rel-18.

References

- RP-220953, NR NTN (Non-Terrestrial Networks) enhancements, Thales

- TR 38.821 v16.1.0, Solutions for NR to support non-terrestrial networks (NTN) (Release 16), 3GPP

- TS 38.305v16.2.0, Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN (Release 16), 3GPP

- TR38.811 v15.2.0, Study on New Radio (NR) to support non-terrestrial networks (Release 15), 3GPP

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz (Release 14), 3GPP