3GPP TSG-RAN WG1 Meeting #109-e                                                                                                                   	                                                                                                               R1-2203777

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.16.1

Source: 	Xiaomi

Title: 	Discussion on FeMIMO UE features

Document for:	Discussion/Decision

- Introduction

There are still some remaining issues about FeMIMO UE features based on the updated RAN1 UE features list R1-2202928[1] for Rel-17 NR after RAN1 #108-e. In this contribution, we provide our views on the according FeMIMO UE features.

- Multi-Beam Operation

There is an agreement about supporting a UE feature on how many physical cell IDs can be associated with the activated TCI states as following [6]:

Agreement On Rel.17 beam indication enhancements for inter-cell beam management, for the supported Rel-17 MAC-CE-based and/or DCI-based beam indication (at least using DCI formats 1_1/1_2 with and without DL assignment including the associated MAC-CE-based TCI state activation): - Support a UE feature on how many physical cell IDs (including that of the serving cell) can be associated with the activated TCI states - FFS: If UE is configured for only one physical cell ID, decide between the following two options: - Opt1: the NW can activate TCI states associated with either the same physical cell ID as that of the serving cell or a different physical cell ID from that of the serving cell - Opt2: the NW can only activate TCI states associated with the same physical cell ID as that of the serving cell Note: The above does not necessarily mean that more than 1 physical cell ID that is not serving cell in RRC

Until now, we have discussed the maximum number of RRC-configured PCI(s) different from serving cell PCI for L1-RSRP measurement in FG 23-1-2, however this UE feature has not been discussed in any FG. Therefore, it is better to add a new feature group to deal with it which is related to beam indication for inter-cell beam management.

Proposal 2-1: Add a new feature group as follows (highlighted in blue).

| 23. NR_FeMIMO | 23-3-1j | Indication/configuration of R17 TCI states for CORESET #0 | Support of indication/configuration of R17 TCI states for CORESET #0 and the respective PDSCH reception reusing the Rel-15/16 signaling/configuration design(s) | 23-1-1 | Yes |  | Support of indication/configuration of R17 TCI states for CORESET #0 and the respective PDSCH reception reusing the Rel-15/16 signaling/configuration design(s) is not supported | Per band | n/a | n/a | n/a | [A UE that supports 23-1-1 must indicate this FG is supported] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-1-1k | beam indication for inter-cell BM | 1. The number of MAC-CE activated PCI(s) different from serving cell PCI for beam indication. | 23-1-1 |  |  |  |  |  |  |  |  |  |

- mTRP PDCCH

void

- mTRP PUSCH

The following has been agreed for mTRP PUSCH for UE feature list in RAN1#108-e meeting [1].

| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A) -codebook based | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions 3. Support of two SRS resource sets with usage set to 'codebook' 4. Supported number of SRS resources in one SRS resource set FFS: Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources | FFS | Yes |  | Multi-TRP PUSCH repetition (type A) is not supported for codebook based | per FS | n/a | n/a | n/a | Component 4 candidate values: {1,2 [,4]} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-1-2b | CSI-RS processing framework for SRS with two associated CSI-RS resources | 1. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 2. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 4. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 5. UE can process up to X CSI-RS resources associated with SRS for non-codebook based transmission simultaneously | 23-3-1-2a | Yes |  | CSI-RS processing framework for SRS with two associated CSI-RS resources is not supported | Per Band | n/a | n/a | n/a | [Component 1: {1 to 8} Component 2: {1 to 8} Component 3: {1 to 8} Component 4: {1 to 16} Component 5: {1,2}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-1 -codebook based | Multi-TRP PUSCH repetition (type B) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) for codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to ‘codebook’ |  | Yes |  | Codebook based multi-TRP PUSCH repetition (type B) is not supported | Per FSPC | No | No | No | Component 3 candidate values: {1,2[,4]} | Optional with capability signalling |

FG23-3-1: Multi-TRP PUSCH repetition (type A) -codebook based

On component 4, the UL full power transmission fullpowerMode2 should be considered. Based on the Rel-16 UE feature FG 16-5c, the candidate values of number of SRS resources in one SRS resource set can be set as {1,2,4}.

FG23-3-1-2b: CSI-RS processing framework for SRS with two associated CSI-RS resources

To align with FG 2-15b in Rel-15 (shown as follows), on components 1, 2, and 4, the current candidate values can be confirmed since there is two CSI-RS per BWP. On component 3, value 0 can be added as the candidate values, i.e., the the candidate values can be set as {0 to 8}. As for component 5, we support the current candidate values.

|  | 2-15b | CSI-RS processing framework for SRS | 1. Maximum number of periodic SRS resources associated with CSI-RS per BWP 2. Maximum number of aperiodic SRS resources associated with CSI-RS per BWP 3. Maximum number of semi-persistent SRS resources associated with CSI-RS per BWP 4. UE can process Y SRS resources associated with CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS. 5. UE can process X SRS resources associated with CSI-RS resources simultaneously across all CCs. Includes P/SP/A SRS. | 2-15a | Yes | Association between CSI-RS and SRS is not supported | Type 3 | N.A. | N.A. |  | NOTE: Other MIMO capability other than component 5 may further restrict (reduce) the number of SRS associated with CSI-RS that the UE has to simultaneously derive. |  | Component-1 candidate values: {1, 2, 3, 4} Component-2 candidate values {1, 2, 3, 4} Component-3 candidate values: {0, 1, 2, 3, 4} Component-4 candidate values: {from 1 to 8} Component-5: candidate values: {from 5 to 32} |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG: 23-3-1-1: Multi-TRP PUSCH repetition (type B) -codebook based

On component 4, similar to PUSCH repetition type A, the candidate values can be set as {1,2,4}.

Proposal 4-1: Adopt the following for Rel-17 mTRP PUSCH UE feature (highlighted in blue).

| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A) -codebook based | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions 3. Support of two SRS resource sets with usage set to 'codebook' 4. Supported number of SRS resources in one SRS resource set FFS: Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources | FFS | Yes |  | Multi-TRP PUSCH repetition (type A) is not supported for codebook based | per FS | n/a | n/a | n/a | Component 4 candidate values: {1,2 [,4]} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-1-2b | CSI-RS processing framework for SRS with two associated CSI-RS resources | 1. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 2. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 4. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 5. UE can process up to X CSI-RS resources associated with SRS for non-codebook based transmission simultaneously | 23-3-1-2a | Yes |  | CSI-RS processing framework for SRS with two associated CSI-RS resources is not supported | Per Band | n/a | n/a | n/a | [Component 1: {1 to 8} Component 2: {1 to 8} Component 3: {1 to 8} Component 4: {10 to 16} Component 5: {1,2}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-1 -codebook based | Multi-TRP PUSCH repetition (type B) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) for codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to ‘codebook’ |  | Yes |  | Codebook based multi-TRP PUSCH repetition (type B) is not supported | Per FSPC | No | No | No | Component 3 candidate values: {1,2[,4]} | Optional with capability signalling |

- mTRP PUCCH

void

- Inter-cell mTRP

Void

- mTRP BM

void

- mTRP HST-SFN

void

- SRS enhancement

The following has been agreed for SRS enhancement for UE feature list.

| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | The maximum number of configured available slots offsets for determining aperiodic SRS location based on available slot | 2-52 | Yes |  | SRS triggering offset enhancement is not supported | Per band | n/a | n/a | n/a | Candidate 1 component values: {1, 2, 4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI | 2-52 | Yes |  | Triggering SRS only in DCI 0_1/0_2 is not supported | per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | 1. Support of SRS antenna switching xTyR with y>4 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL | 2-55 | Yes |  | SRS Antenna switching for >4Rx is not supported | Per FS | n/a | n/a | n/a | Component 1 candidate values: a combination from the set {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8} Note: For any indicated value, x shall be equal to or smaller than the one associated with the largest y Component 2 candidate values: {1 to 32} Component 3 candidate values: {1 to 32} Note: Component 2 and component 3 is not reported if component 1 is reported as xTyR with x=y. | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP and 1 periodic SRS sets for antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for antenna switching | 2-53 | Yes |  | Maximum 2 SP and 1 periodic SRS sets for antenna switching is not supported | Per FS | n/a | n/a | n/a | Note1: - Applies for all supported xTyR where y<=8 - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS - The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource | 10-11, 2-52 | Yes |  | Increased repetition for SRS is not suported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS | Support of partial frequency sounding for SRS | 2-52 | Yes |  | Partial frequency sounding of SRS is not suported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial frequency SRS transmission across different SRS frequency hopping periods for periodic/semi-persistent/aperiodoc SRS | 23-8-6 | Yes |  | Start RB location hopping for partial frequency SRS is not suported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for SRS other than for positioning |  | Yes |  | comb-8 for SRS other than for positioning is not suported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-9 | Extension of aperiodic SRS configuration for 1T4R, 1T2R and 2T4R | Support of 4 aperiodic SRS resource sets for 1T4R and 2 aperiodic resource sets for 1T2R/2T4R. | 2-53, 2-55 | Yes |  | Extension of aperiodic SRS configuration for 1T4R, 1T2R and 2T4R is not supported | Per FS | N/A | N/A | N/A | [Note: When UE only supports part of {1T4R, 1T2R, 2T4R}, this FG is only applicable to the antenna switching configuration(s) that UE supports] | Optional with capability signalling |

FG23-8-10

The following has been agreed for SRS enhancement in RAN1#108 meeting.

Agreement

RPFS is applicable for both frequency hopping and non-frequency hopping cases, where support of RPFS for non-FH case is an optional UE feature for UEs supporting RPFS.

RPFS for non-FH case also requires new UE capability.

Proposal 9-1: Add FG23-8-10 to support the RPFS for non-FH.

Proposal 9-2: Adopt the following for Rel-17 SRS enhancement UE feature (highlighted in blue)

| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | The maximum number of configured available slots offsets for determining aperiodic SRS location based on available slot | 2-52 | Yes |  | SRS triggering offset enhancement is not supported | Per band | n/a | n/a | n/a | Candidate 1 component values: {1, 2, 4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI | 2-52 | Yes |  | Triggering SRS only in DCI 0_1/0_2 is not supported | per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | 1. Support of SRS antenna switching xTyR with y>4 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL | 2-55 | Yes |  | SRS Antenna switching for >4Rx is not supported | Per FS | n/a | n/a | n/a | Component 1 candidate values: a combination from the set {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8} Note: For any indicated value, x shall be equal to or smaller than the one associated with the largest y Component 2 candidate values: {1 to 32} Component 3 candidate values: {1 to 32} Note: Component 2 and component 3 is not reported if component 1 is reported as xTyR with x=y. | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP and 1 periodic SRS sets for antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for antenna switching | 2-53 | Yes |  | Maximum 2 SP and 1 periodic SRS sets for antenna switching is not supported | Per FS | n/a | n/a | n/a | Note1: - Applies for all supported xTyR where y<=8 - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS - The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource | 10-11, 2-52 | Yes |  | Increased repetition for SRS is not suported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS | Support of partial frequency sounding for SRS | 2-52 | Yes |  | Partial frequency sounding of SRS is not suported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial frequency SRS transmission across different SRS frequency hopping periods for periodic/semi-persistent/aperiodoc SRS | 23-8-6 | Yes |  | Start RB location hopping for partial frequency SRS is not suported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for SRS other than for positioning |  | Yes |  | comb-8 for SRS other than for positioning is not suported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-9 | Extension of aperiodic SRS configuration for 1T4R, 1T2R and 2T4R | Support of 4 aperiodic SRS resource sets for 1T4R and 2 aperiodic resource sets for 1T2R/2T4R. | 2-53, 2-55 | Yes |  | Extension of aperiodic SRS configuration for 1T4R, 1T2R and 2T4R is not supported | Per FS | N/A | N/A | N/A | [Note: When UE only supports part of {1T4R, 1T2R, 2T4R}, this FG is only applicable to the antenna switching configuration(s) that UE supports] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-10 | Partial frequency sounding of SRS | Support of non-frequncy hopping for partial frequency sounding of SRS | 2-52 | Yes |  | RPFS for non-FH is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

- mTRP CSI

| 23. NR_FeMIMO | 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | - Support of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Support of N=1 - Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max - CSI report mode selection of mode 1 with X=0 and/or mode 2 - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis - Maximum total number of CMRs for NCJT measurement - Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses - [A list of (Y1,Y2): UE can process Y1 NCJT CSI and Y2 sTRP CSI measurement hypothesis simultaneously in a CC] - [A list of (X1,X2): UE can process X1 NCJT CSI and X2 sTRP CSI measurement hypothesis simultaneously across all CCs] |  | Yes |  | CSI Enhancement for Multi-TRP is not supported | Per band and per BC | n/a | n/a | n/a | Component 2 candidate value set: {2, 3, 4, 5, 6, 7, 8} Component 3 candidate value set: { mode 1 with X=0, mode 2, both} Component 4 candidate values: - {2, 4, 8, 12, 16, 24, 32} - {2,3,4 … 64} - {2,3,4, …, 256} [Component 5: The list can have maximum of 16 pairs. - Y1: {1 to 4} - Y2: {1 to 8}] [Component 6: The list can have maximum of 16 pairs. - X1: {1 to 16} - X2: {1 to 32}] Note: ‘NCJT’ is not used in RAN1 specifications and will be aligned with 38.214 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-7-1b | Active CSI-RS resources and ports in the presence of multi-TRP CSI | 1. List of codebook combinations 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | 23-7-1 | Yes |  | [Active CSI-RS resources and ports in the presence of multi-TRP CSI is not supported] | Per band and per BC | n/a | n/a | n/a | Component 1 candidate values: Codebook 1 = {[‘NCJT’,] NCJT+Type 1 SP (for sTRP)} {Codebook 2, Codebook 3} = {(NULL, NULL}), {“Rel 16 combinations in FG 16-8”}, {“New Rel17 combinations in FG 23-9-5”}} Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {2, 4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Note 1: A CMR pair configured for NCJT will be counted as two activated resources, a CMR configured for sTRP will be counted as one activated resource for a triplet. Note2: This capability is relevant only when UE is configured with NCJT CSI in at least one CSI report setting in at least one CC in the band and/or band combination. | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-1a | Additional CSI report mode 1 | Maximum value of numberOfSingleTRP-CSI-Mode1 | 23-7-1 | Yes |  | CSI report mode 1 with X=1, or X=2 is not supported | Per Band | n/a | n/a | n/a | Component 1 candidate value set: { X=1, X=2} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-4 | Support of Nmax=2 for Multi-TRP CSI | Support of maximum number of CMR pairs Nmax=2 configured in NZP-CSI-RS-ResourceSet for a given CSI report setting | 23-7-1 | Yes |  | Nmax=2 for Multi-TRP CSI is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-5 | CMR sharing | Support a NZP CSI-RS resource referred by both a CMR pair configured for Rel-17 Multi-TRP CSI enhancement and a single CMR configured for Single-TRP measurement in a CSI reporting setting | 23-7-1 | Yes |  | CMR sharing is not supported | Per band | n/a | FR2 only | n/a |  | Optional with capability signalling |

According the above agreement, there are some remained issues on two FG 23-7-1 and FG 23-7-1b.  It will be disccuesed whether both Component 5 and Componet 6 for FG 23-7-1 is retained or not.  It was agreed that UE can report NCJT CSI and sTRP CSI simultaneously. In addition, the PMI search complexity is different for NJCT CSI and sTRP CSI. This implies that it cannot infer the UE capabiltity by just reporting the processing capability of NCJT CSI or sTRP CSI. Hence, the two components should be kept to indicate the processing capabiltiliy of UE when NJCT CSI and sTRP CSI are reported together.

It was also agreed that UE does not report sTRP CSI, i.e., X=0, and only NCJT CSI is fedback for NCJT. For NCJT CSI reporting, only Type I codebook is supported. Howerever, the PMI computation complexity for NCJT CSI and sTRP CSI even if both them adopt Type I codebook to report CSI. Therefore, codebook combinations on codebook1 should include NCJT and NCJT with Type 1 SP, repespectively, for Component 1 of FG 23-7-1b. According to above discussion, we provide the following proposal.

Proposal 10-1: Adopt the following for Rel-17 mTRP CSI UE feature (highlighted in blue).

| 23. NR_FeMIMO | 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | - Support of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Support of N=1 - Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max - CSI report mode selection of mode 1 with X=0 and/or mode 2 - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis - Maximum total number of CMRs for NCJT measurement - Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses - [A list of (Y1,Y2): UE can process Y1 NCJT CSI and Y2 sTRP CSI measurement hypothesis simultaneously in a CC] - [A list of (X1,X2): UE can process X1 NCJT CSI and X2 sTRP CSI measurement hypothesis simultaneously across all CCs] |  | Yes |  | CSI Enhancement for Multi-TRP is not supported | Per band and per BC | n/a | n/a | n/a | Component 2 candidate value set: {2, 3, 4, 5, 6, 7, 8} Component 3 candidate value set: { mode 1 with X=0, mode 2, both} Component 4 candidate values: - {2, 4, 8, 12, 16, 24, 32} - {2,3,4 … 64} - {2,3,4, …, 256} [Component 5: The list can have maximum of 16 pairs. - Y1: {1 to 4} - Y2: {1 to 8}] [Component 6: The list can have maximum of 16 pairs. - X1: {1 to 16} - X2: {1 to 32}] Note: ‘NCJT’ is not used in RAN1 specifications and will be aligned with 38.214 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-7-1b | Active CSI-RS resources and ports in the presence of multi-TRP CSI | 1. List of codebook combinations 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | 23-7-1 | Yes |  | [Active CSI-RS resources and ports in the presence of multi-TRP CSI is not supported] | Per band and per BC | n/a | n/a | n/a | Component 1 candidate values: Codebook 1 = {[‘NCJT’,] NCJT+Type 1 SP (for sTRP)} {Codebook 2, Codebook 3} = {(NULL, NULL}), {“Rel 16 combinations in FG 16-8”}, {“New Rel17 combinations in FG 23-9-5”}} Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {2, 4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Note 1: A CMR pair configured for NCJT will be counted as two activated resources, a CMR configured for sTRP will be counted as one activated resource for a triplet. Note2: This capability is relevant only when UE is configured with NCJT CSI in at least one CSI report setting in at least one CC in the band and/or band combination. | Optional with capability signalling |

- FDD CSI

void

- References

- R1-2202928, Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e, Moderators (AT&T, NTT DOCOMO, INC.).