3GPP TSG RAN WG1 #109-e			R1-2203779

e-Meeting, May 9th – 20th, 2022

Source: 	     Xiaomi

Title:	Discussion on UE features for NR MBS

Agenda item:    8.16.12

Document for:  Discussion

Introduction

In RAN1#108 e-meeting, UE features for NR MBS was discussed and the achieved agreements can be found in the appendix. In this contribution, we provide our views on the UE feature group for NR MBS.

Discussion

FG 33-1:

In RAN1#108 e-meeting, the following agreement on how to process MBS broadcast DCI was agreed:

Agreement Regarding the number of DCIs that a UE can process in a slot or span, MBS broadcast DCI monitored by the UE is treated as unicast DCI scheduling DL following the current feature group 3-1/3-5a/3-5b for RRC_CONNECTED UEs.

The above agreement addresses MBS UE capability related to DCI processing. It should be captured in FG 33-1. We propose to add the following component for FG 33-1 in order to address the newly achieved agreement in main session.

- Broadcast DCI is treated as unicast DCI scheduling DL following the current feature group 3-1/3-5a/3-5b.

Proposal 1: Add the following component for FG 33-1:

- Broadcast DCI is treated as unicast DCI scheduling DL following the current feature group 3-1/3-5a/3-5b.

FG 33-2:

In RAN1#107bis e-meeting, the following agreement on how to processing multicast DCI was agreed:

Agreement Regarding the number of DCIs that a UE can process in a slot or span, multicast DCI is treated as unicast DCI scheduling DL following the current feature group 3-1/3-5a/3-5b.

The above agreement addresses MBS UE capability related to DCI processing. It should be captured in FG 33-2. We propose to add the following component for FG 33-2 in order to address the newly achieved agreement in main session.

- Multicast DCI is treated as unicast DCI scheduling DL following the current feature group 3-1/3-5a/3-5b.

Proposal 2: Add the following component for FG 33-2:

- Multicast DCI is treated as unicast DCI scheduling DL following the current feature group 3-1/3-5a/3-5b.

FG 33-2a and FG33-4:

It is still open whether to define separate PUCCH resource configurations from unicast as separate FG from that of supporting shared PUCCH resources with unicast. Supporting separate PUCCH resource configurations for unicast and multicast respectively is an advanced UE capability. The default behavior for a UE should be transmitting UCI on shared PUCCH resources with unicast. Hence we have the following proposal:

Proposal 3: Define separate PUCCH resource configurations from unicast as separate FG from FG 33-2a.

Proposal 4: Define separate PUCCH resource configurations from unicast as separate FG from FG 33-4.

FG 33-5-1:

Currently, the component of FG 33-5-1 is defined as below:

- Support one SPS group-common PDSCH configuration for multicast

- Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH

One open issue is whether to separate the capability for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling. In Rel-16, whether the newly introduced DCI format 1_2 can be used for SPS release is defined as a separate UE capability. We prefer to follow the same logic as Rel-16 NR IIoT here.

Proposal 5: For FG 33-5-1, the support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling is a separate UE capability.

FG 33-5-1d:

The same logic as dynamic scheduling should be adopted here, i.e. PTP retransmission can be a separate FG compared to the basic retransmission capability supporting PTM retransmission.

Proposal 6: FG 33-5-1d should be kept as a separate FG.

We also note that the DCI format 1_0 and DCI format 1_1 are used in the feature group related to multicast. Actually the new DCI format used for multicast is already captured in TS38.212, i.e. DCI format 4_1 and DCI format 4_2. The DCI format should be aligned between UE feature list and other physical specifications.

Conclusion

In this contribution, we discuss the NR MBS UE feature group. Based on the discussion in section 2, we have the following proposals:

Proposal 1: Add the following component for FG 33-1:

- Broadcast DCI is treated as unicast DCI scheduling DL following the current feature group 3-1/3-5a/3-5b.

Proposal 2: Add the following component for FG 33-2:

- Multicast DCI is treated as unicast DCI scheduling DL following the current feature group 3-1/3-5a/3-5b.

Proposal 3: Define separate PUCCH resource configurations from unicast as separate FG from FG 33-2a.

Proposal 4: Define separate PUCCH resource configurations from unicast as separate FG from FG 33-4.

Proposal 5: For FG 33-5-1, the support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling is a separate UE capability.

Proposal 6: FG 33-5-1d should be kept as a separate FG.

Reference

  - R1-2205120, Summary on UE features for NR MBS, Moderator (NTT DOCOMO, INC.)

  - R1-2202928, Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e, Moderators (AT&T, NTT DOCOMO, INC.)

Appendix

Agreement

- The capability for support of dynamic slot-level repetition for MTCH is separated from FG 33-1.

  - maximum number of dynamic slot-level repetitions is 16

Agreement

- FG 33-2c is merged into FG 33-2a and FG 33-2d is kept as follows

| 33. NR_MBS | 33-2a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | 1) Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling 2)Support of PTM retransmission for multicast FFS shared/separate PUCCH resource configurations with/from unicast At least 33-2c or 33-2d is merged, FFS which one or both | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-2c | PTM retransmission for multicast | Support of PTM retransmission for multicast FFS whether to merge with 33-2a | 33-2a | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2d | PTP retransmission for multicast | Support of PTP retransmission for multicast FFS whether to merge with 33-2a | 33-2a | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |

Agreement

- FG 33-5-1 is updated as follows

| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support one SPS group-common PDSCH configuration for multicast - Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - FFS: HARQ-ACK feedback for SPS group-common with PDCCH scheduling and SPS release PDCCH. - Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH FFS whether/how to separate the above capabilities from FG 33-5-1 the capability for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-5-1a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, SPS group-common PDSCH activation, and SPS release PDCCH | 33-5-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1b | DCI-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signaling | 33-5-1a | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1c | PTM retransmission for SPS group-common PDSCH for multicast | Support of PTM retransmission associated with G-CS-RNTI for SPS multicast FFS whether to merge with 33-5-1a and 33-5-1f, respectively | 33-5-1a | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1d | PTP retransmission for SPS group-common PDSCH for multicast | Support of PTP retransmission for SPS multicast FFS whether to merge with 33-5-1a | 33-5-1a | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1e | Dynamic Slot-level repetition for SPS group-common PDSCH for multicast | Support up to X times dynamic slot-level repetition for SPS group-common PDSCH for multicast. | 33-5-1 | Yes |  |  | Per UE | No | No |  | Candidate values for X is: {8, 16} | Optional with capability signalling |
| 33. NR_MBS | 33-5-1f | NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling | 33-5-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1g | DCI-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signaling | 33-5-1f | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1h | Multiple G-CS-RNTIs for SPS group-common PDSCHs | Max number of G-CS-RNTIs for SPS multicast [per slot] per CC | 33-5-1 | Yes |  |  | Per UE | No | No |  | FFS max number | Optional with capability signalling |

Agreement

- The capability for support of DCI format 4_2 with CRC scrambled with G-RNTI for multicast is separated from FG 33-2.

Agreement

- FGs 33-5-1c and 33-5-1f are updated as follows

| 33. NR_MBS | 33-5-1c | PTM retransmission for SPS group-common PDSCH for multicast | Support of PTM retransmission associated with G-CS-RNTI for SPS multicast FFS whether to merge with 33-5-1a and 33-5-1f, respectively | 33-5-1a | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-5-1f | NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | 1) Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling 2) Support of PTM retransmission associated with G-CS-RNTI for SPS multicast | 33-5-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |

Agreement

- FG 33-7 is removed.

Agreement

- Separate FG of maxNumberMIMO-LayersPDSCH for multicast PDSCH is introduced

  - Note: If UE supports up to 8 layers, the UE supports TB2

  - FFS any condition for mandatory supported layers

Agreement

- Add an FG 33-3-3a for FDM-ed Type-1 and Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast

  - Note1: FDM-ed Type-1 HARQ-ACK codebook is generated by concatenating the Type-1 sub-codebook for unicast and the Type-1 sub-codebook for multicast.

  - Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast.

    - FFS value of X G-RNTIs

- Add an FG 33-3-3b for Mode 2 TDM-ed Type-1 and Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast

  - Note: Mode 2 TDM-ed Type-1 HARQ-ACK codebook is generated based on the union TDRA tables from unicast and multicast and the union of k1 sets from unicast and multicast.

  - Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast.

    - FFS value of X G-RNTIs

- FG 33-3-3b is added as a prerequisite FG for FG 33-3-4

Agreement

- Add a component for supporting Type-1 and Type-2 HARQ-ACK CB for multicast feedback only in FG 33-2a

Agreement

- Component 1 in FG 33-5-2 is updated as follows

  - Support [N>1] SPS group-common PDSCH configuration per CFR for multicast, and up to K SPS group-common PDSCH configuration in a cell group

    - FFS the value range for K

Agreement

- FG 33-2g is updated as follows

| 33. NR_MBS | 33-2g | MIMO layers for multicast PDSCH | Supported maximal number of MIMO layers for multicast PDSCH | 33-2 | Yes |  | UE supports 1 MIMO layer only for multicast PDSCH | Per FSPC | N/A | N/A |  | Candidate values:{2,4,8} Note: If UE supports up to 8 layers, the UE supports TB2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- Shared PUCCH resource configurations with unicast is included as a component of FGs 33-2a/33-4

- FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FGs 33-2a/33-4

Agreement

- FG 33-1 is updated as follows

| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. - support of higher layer configured slot-level repetition up to 8 for MTCH DCI indicated slot-level repetition up to 16 for MTCH is defined as another FG, FFS whether to merge with FG 33-3-1 |  | Up to RAN2 |  |  | Up to RAN2 | Up to RAN2 | Up to RAN2 |  |  | Up to RAN2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG 33-2 is separated to two FGs: One for PCell case, the other for SCell case

  - Type of FG for PCell case is [per band or per FSPC]

  - Type of FG for SCell case is per FSPC

  - FG for PCell is added as a Prerequisite FG for FG for SCell

Agreement

- Add a component of Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast and multicast at a UE into FG 33-6-1

- Add an FG of Support of priority configured for multicast HARQ-ACK feedback of SPS multicast

  - FG 33-6-1 is added as a prerequisite FG of this FG

- Revise the component 1 of FG 33-6-2: Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE.