3GPP TSG RAN WG1 #109-e			R1-2203781

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.16.14

Source:	Xiaomi

Title:	Discussion on UE features for IoT NTN

Document for:	Discussion

Introduction

The updated RAN1 UE features for Rel-17 NR NTN are provided in [1]. In this contribution, we provide our comments on the update RAN1 UE features.

Comments on UE features

On 2-2 and 2-2a

Agreement

For IoT NTN, the information of K_mac is carried in system information.

In RAN1 107e meeting [2], an agreement on the K_mac was made that the K_mac is a cell-specific parameter. In order to avoid any ambiguity, it is suggested to add the description “cell-specific” before the K_mac

Proposal 1: Add the description “cell-specific” before the K_mac to FG 2-2 and 2-2a.

Meanwhile, it is still FFS whether the NTN FG should be per UE or per band defined. In our understanding, it is more flexible to have per band FGs. A UE can have the flexibility to decide whether to support NTN when operating on a band which may be used by both NTN and TN systems.

Proposal 2: The IoT-NTN features are per band defined.

The detailed updates can be found in the appended table.

Conclusions

In this contribution, we present the discussion on the UE features for IoT NTN. Based on our analysis, we have the following proposals:

Proposal 1: Add the description “cell-specific” before the K_mac to FG 2-2 and 2-2a.

Proposal 2: The IoT-NTN features are per band defined.

References

- R1-2202925, Updated RAN1 UE features list for Rel-17 LTE after RAN1 #108-e, Moderators (AT&T, NTT DOCOMO, INC.), 3GPP TSG RAN WG1 #108-e e-Meeting, February 21st – March 3rd, 2022

- 3GPP RAN1 107-e meeting chairman-note, 3GPP TSG RAN WG1 #107-e, e-Meeting, November 11th – 19th, 2021

| 2. LTE_NBIOT_eMTC_NTN | 2-2 | Enhancing timing relationships using a time offset for eMTC | UE receives and applies UE specific K_offset/cell-specific K_mac in timing relationship enhancements | 2-1 , 2-3 | Yes | N/A | eMTC UE does not know the offset to apply for UL transmission | [per band] | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. LTE_NBIOT_eMTC_NTN | 2-2a | Enhancing timing relationships using a time offset for NB-IoT | UE receives and applies UE specific K_offset/cell-specific K_mac in timing relationship enhancements | 2-1b, 2-3a | Yes | N/A | NB-IoT UE does not know the offset to apply for UL transmission | [per band] | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. | Optional with capability signalling |