3GPP TSG RAN WG1 #109-e                                       	       R1-2203793

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.1.1.1

Source:	Xiaomi

Title:                     Unified TCI framework extension for multi-TRP

Document for:	Discussion and Decision

Introduction

The WI for R18 MIMO was approved in RAN-94 e-meeting, and the detailed objectives [1] are as follows:

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows:

  - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis

  - UE reporting of time-domain channel properties measured via CSI-RS for tracking

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework.

- Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM,

  - Striving for a common design between DL and UL DMRS

  - Up to 24 orthogonal DM-RS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

  - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

  - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences

  - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable)

  - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission

    - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

  - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation

    - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

- Study, and if justified, specify the following

  - Two TAs for UL multi-DCI for multi-TRP operation

  - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed.

For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

While in this contribution, we will focus on the unified TCI framework extension for M-TRP, which including the object 2, the second bullet of objective 6 and the first bullet of objective 7.

Indication of multiple DL/UL TCI states

Switching between joint or separate DL/UL TCI state configuration

During the discussion of R17 unified TCI state framework, there is a conclusion in RAN1-105 e-meeting [2] seen below. It concludes that the configuration of joint TCI or separate DL/UL TCI state is based on RRC signaling. While for M-TRP scenario, since different TRP may experience different environment, thus separate configuration of joint TCI or separate DL/UL TCI for different TRP is preferred. It means that it is possible that one TRP is configured with joint TCI and the other one is configured with separate DL/UL TCI.

Conclusion

On Rel-17 unified TCI framework, for a UE configured with both joint TCI and separate DL/UL TCI, configuration of joint TCI or separate DL/UL TCI is based on RRC signaling

- There is no consensus in RAN1 on how to support dynamic switching (either MAC-CE or codepoint based)

Proposal 1: For M-TRP unified TCI framework, support one TRP is configured with joint TCI and the other one is configured with separate DL/UL TCI.

In addition to UL to micro cell and DL/UL only TRP, separate DL/UL TCI state will be used in the case of MPE issue and the PHR related to MPE issue is indicated by MAC CE [3], thus the configuration of joint TCI or separate DL/UL TCI state is based on RRC signaling can’t handle the MPE issue with short latency, which will introduce some performance loss by using non-suitable UL TCI state. While in order to reduce UE complexity, DCI based switching is not recommended.

Proposal 2: For M-TRP unified TCI framework, support to switch between joint TCI and separate DL/UL TCI based on MAC CE.

Unified TCI state indication in S-DCI case

In R17, both MAC CE and MAC CE+DCI can be used for unified TCI state indication. Thus it is straightforward to support both mechanism for M-TRP. During the discussion of R16 M-TRP PDSCH and R17 M-TRP PUSCH, dynamic switching between S-TRP and M-TRP is supported. Which means that the DCI code point can map to TCI state of one TRP or two TRPs. Thus it is also straightforward to support dynamic switching between S-TRP and M-TRP by unified TCI state indication for S-DCI M-TRP scenario.

Proposal 3: For S-DCI M-TRP scenario, support both MAC CE and MAC CE+DCI for unified TCI state indication.

Proposal 4: For S-DCI M-TRP scenario, support dynamic switching between S-TRP and M-TRP by mapping some DCI codepoints to TCI states of two TRPs and some DCI codepoints to TCI states of one TRP.

Dynamic switching between S-TRP and M-TRP means gNB indicate TCI states for two TRPs at t1 time, and indicate TCI state for only one TRP at t2 time. For PDSCH, dynamic granted PUSCH/configured granted PUSCH type 2 and PUCCH configured with M-TRP, there is no problem on dynamic switching between S-TRP and M-TRP. If TCI states of two TRPs are indicated, UE will use TCI states of two TRPs for transmission or reception. If TCI state(s) of only one TRP is indicated, UE will use TCI state(s) of only one TRP for transmission or reception.

But for PDCCH, including M-TRP PDCCH repetition and HST-SFN scheme based PDCCH, dynamic switching between S-TRP and M-TRP is not support. While for M-TRP PDCCH, linking two SS sets by RRC configuration is supported only. When M-TRP PDCCH repetition and M-TRP PDSCH/PUSCH is supported, sometimes TCI state(s) of one TRP is indicated and sometimes TCI state(s) of two TRPs are indicated. Then for PDCCH, how does UE do when DLorjoint TCI state(s) of one only TRP is indicated? There are two alternatives for down selection when DLorjoint TCI state(s) of only one TRP is indicated.

- Alt 1: For PDCCH, support dynamic switching between M-TRP and S-TRP, so UE switches to S-TRP when DLorjoint TCI state(s) of only one TRP is indicated.

- Alt 2: For PDCCH, dynamic switching between M-TRP and S-TRP is not supported. So UE still acts as M-TRP, and for the other TRP whose DLorjoint TCI state is not indicated, UE use the indicated DLorjoint TCI state of that TRP in previous signaling.

Proposal 5: For M-TRP PDCCH repetition, to discuss whether to support dynamic switching to S-TRP when DLorjoint TCI state of only one TRP is indicated.

- Alt 1: UE switches to S-TRP.

- Alt 2: UE still acts as M-TRP by applying the indicated unified TCI state of the other TRP in the latest previous signaling.

While for S-DCI M-TRP, another problem is that there is no CORESETPoolIndex for each CORESET. If unified TCI state is used for M-TRP, there will be no index of CORESET included in the unified TCI state indication signaling. In this case, UE doesn’t know which TCI state will be applied for which CORESET. In order to solve this problem, we suggest to configure a CORESET group ID for each CORESET in S-DCI M-TRP scenario and indicate the association between CORESET group ID and the indicated TCI state.

Proposal 6: For S-DCI M-TRP scenario, support to configure CORESET group ID for each CORESET and indicate the association between CORESET group ID with unified TCI state of each TRP.

In addition, for a CORESET configured with SFN, before to discuss the CORESET group ID for such CORESET, it is better to discuss whether to support such CORESET in S-DCI M-TRP unified TCI state framework. During the discussion in R17 HST-SFN, HSF-SFN PDCCH+ R16 M-TRP PDSCH is not supported. But HSF-SFN PDCCH + R17 M-TRP PUSCH or HSF-SFN PDCCH + R17 M-TRP PUCCH is not discussed so far. If neither one is supported, that means only M-TRP PDCCH is supported by HSF-SFN, it is better to reuse TCI state indicate by MAC CE with CORESET ID for such CORESET and not to use the unified TCI state framework. But if any combination is supported, unified TCI state may be needed for M-TRP PUSCH or PUCCH. In this case, how to support the unified TCI state update for HSF-SFN PDCCH?

Proposal 7: For S-DCI M-TRP scenario, suggest to discuss whether/how to support unified TCI state indication for HST-SFN PDCCH.

For PDCCH/PDSCH/PUCCH/PUSCH, the M-TRP repetition is configured independently. For example, considering only two kinds of channels, there are many combinations and some of combinations can be seen below:

- S-TRP PDCCH+ M-TRP PDSCH

- S-TRP PDSCH+ M-TRP PDCCH

- S-TRP PDCCH+ M-TRP PUSCH

- S-TRP PUSCH+ M-TRP PDCCH

- S-TRP PUSCH+ M-TRP PDSCH

- S-TRP PDSCH+ M-TRP PUSCH

……

For S-TRP PDCCH + M-TRP PDSCH/PUSCH/PUCCH, unified TCI state of two TRP can be indicated and which one will be applied for which CORESET can be handled by above proposal 6.

For M-TRP PDCCH + S-TRP PDSCH/PUSCH/PUCCH, whether to support unified TCI state indication of M-TRP or not is needed to be discussed first. If not supported, the TCI state indication for PDCCH will reuse R15/16 MAC CE. And unified TCI state indication for S-TRP will be used for PDSCH/PUSCH/PUCCH. If supported, the TCI state of which one TRP will be used for S-TRP PDSCH/PUSCH/PUCCH when unified TCI state of two TRPs are indicated. It is possible to configure a group ID for S-TRP PDSCH/PUSCH/PUCCH and associate the TCI state of one TRP with one group ID.

Proposal 8: For M-TRP PDCCH + S-TRP PDSCH/PUSCH/PUCCH, suggest to discuss whether/how to support unified TCI state indication in S-DCI M-TRP scenario.

For M-TRP PDSCH + S-TRP PUSCH/PUCCH, M-TRP PUSCH/PUCCH + S-TRP PDSCH, M-TRP PUSCH + S-TRP PUCCH, it is better to support unified TCI state indication for S-DCI M-TRP. But for the S-TRP channel transmission, unified TCI state of which one TRP will be used. As same as the above section, the possible solution is to configure a group ID for S-TRP channel transmission and associate the TCI state of one TRP with one group ID.

Proposal 9: For UE is configured with the combination below, when unified TCI state of two TRPs are indicated, suggest to define TCI states of which one TRP will be applied for S-TRP channel transmission:

- M-TRP PDSCH + S-TRP PUSCH/PUCCH,

- M-TRP PUSCH/PUCCH + S-TRP PDSCH,

- M-TRP PUSCH + S-TRP PUCCH

Unified TCI state indication in M-DCI case

In R17, both MAC CE and MAC CE+DCI can be used for unified TCI state indication. Thus it is straightforward to support both mechanism for M-TRP.

Proposal 10: For M-DCI M-TRP scenario, support both MAC CE and MAC CE+DCI for unified TCI state indication.

For different TRP, different CORESETPoolIndex is configured for CORESET. And unified TCI state one CORESETPoolIndex will be applied for the CORESET with same CORESETPoolIndex. While for the transmission scheduled by DCI, such as PDSCH/PDSCH/dynamic grant PUSCH type 2/ aperiodic PUCCH, the indicated unified TCI state of the corresponding scheduling PDCCH will be applied.

Proposal 11: For M-DCI M-TRP scenario, for channel /signal transmission scheduled by DCI, the unified TCI state of the CORESETPoolIndex same as the corresponding scheduling CORESET will be applied.

While for transmissions not scheduled by DCI, such as periodic/ semi-persistent PUCCH /configured grant PUSCH type 1, it needs to define that the TCI state of which one TRP will be applied. The straightforward method is to configure a group ID for periodic/ semi-persistent PUCCH /configured grant PUSCH type 1. And associate TCI state of each TRP with each group ID.

Proposal 12: For M-DCI M-TRP scenario, configure CORESETPoolIndex to PUCCH and configured grant PUSCH Type 1.

In addition, for M-DCI M-TRP scenario, if unified TCI state is indicated by MAC CE+DCI, it is straightforward to indicate the unified TCI state for the CORESETPoolIndex by the CORESET with same CORESETPoolIndex. But if for CORESET#1, the performance of previous TCI state becomes bad, it may be necessary to support cross CORESETPoolIndex indication for unified TCI state. It means that indicate the unified TCI state for CORESETPoolIndex #0 by CORESET #1 and CORESET#1 is configured with CORESETPoolIndex#1.

Proposal 13: For M-DCI M-TRP scenario, support to indicate the unified TCI state for CORESETPoolIndex#1 by DCI in CORESET configured with CORESETPoolIndex#0.

Support UL DCI format to indicate unified TCI state

During the discussion in R17, for DCI format, only DCI format 1_1 or 1_2 with or without DL assignment is supported to indicate the unified TCI state. But in the scenario of separate DL/UL TCI state, when only separate UL TCI state need to be indicated, there are some disadvantages by reusing DCI format 1_1 or 1_2. For example, when separate UL TCI state is needed for PUSCH transmission, two DCIs need to be monitored and decoded correctly. It means that first DCI format 1_1 or 1_2 should be monitored and decoded to obtain the separate UL TCI state. And then DCI format 0_1 or 0_2 should be monitored and decoded to obtain PUSCH scheduling information. In this case, for each PUSCH transmission, UE need to decode two DCIs correctly, which increases the BD times and the probabilities of DCI decoding failure. Thus we prefer to indicate separate UL TCI state at least by DCI formats 0_1 and 0_2.

Proposal 14: Support to indicate at least separate UL TCI state by DCI formats 0_1 and 0_2.

Simultaneous multi-panel UL transmission

The Rel-18 WID for MIMO Evolution for Downlink and Uplink is approved [1], which includes the following objective mainly targeting UL enhancements:

Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable)

  - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission

- The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

  - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation

    - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable)

  - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission

- The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

  - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation

    - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

In Rel-16 mTRP enhancements, the focus was mostly on PDSCH reliability enhancements, while in Rel-17 the enhancements for PDCCH, PUCCH, and PUSCH was taken to ensure that both data and control for both downlink and uplink are reliable as the overall reliability of the system. For the uplink mTRP enhancements alone, in Rel-17 the transmission schemes for PUSCH and PUCCH are TDM based solutions considering the UE capabilities in FR2. PUSCH or PUCCH is transmitted on multiple repetitions in time-domain hence making these schemes suitable for UEs that can transmit on a single panel at a time. In Rel-18, the uplink mTRP enhancements mainly consider the simultaneous transmissions with multiple UE panels in FR2.

In the multi-panel transmission case, a UE has the capability to simultaneously transmit over multiple panels. Compared to single panel transmission case, the main benefit is the reduced latency and higher throughput.

In NR Rel-15/16, the spatial transmission properties for PUSCH are associated with the SRS(s) configured in a SRS resource set with usage of ‘Codebook’ or ‘non-Codebook’.  In NR Rel-17 feMIMO WI, a joint TCI state for both UL and DL or a TCI state for UL was introduced. So in Rel-18, the PUSCH multi-TRP enhancements can be extended to cover the unified TCI state framework as the Rel-17 design of unified TCI state framework is tend to be stable.

In this session, we discuss the beam indication issues for the simultaneous transmission schemes of multi-panel/multi-TRP based on the unified TCI framework.

Similar to downlink, uplink communication can also benefit from multi-TRP deployment for improving reliability and robustness. Up to Rel-16 NR, the following schemes are supported for multi-TRP operation in downlink: SDM, FDM, and TDM. The SDM, FDM, and TDM schemes are all feasible for uplink. A TDM-based solution requires low UE capabilities by paying extra latency and power, repetition in time domain is well performed since for the uplink power is limited. SDM and FDM schemes are considered to be able to achieve a lower latency. And compared with SDM, FDM has the benefits that there’s no inter-layer interference and power boosting can be utilized, while SDM normally achieves a larger spectral efficiency. On the other hand, SDM/FDM schemes require power split and may be harmful for coverage, so power control issues need to be further studied. Based on the above discussion, we propose to focus on SDM/FDM-based schemes for uplink in R18. If PUSCH/PUCCH is transmitted in SDM/FDM scheme, multiple beams are needed to transmit to different TRPs simultaneously and each beam is associated with one TRP of the two TRPs. However, the PL RS associated with each beam is also different for each TRP. Therefore, the power of both transmissions from different panels would have to be different in the same OFDM symbols within a slot.

Proposal 15: Consider both SDM and FDM schemes for the S-DCI based simultaneous multi-panel uplink transmission in Rel-18. SFN scheme can also be considered in Rel-18.

Despite similar enhancements for NC-JT considerations, coherent joint transmission (C-JT) in uplink may be feasible for multi-panel UEs, especially for CPE/FWA like devices. Coherent joint reception can be done by post processing of received signals at two TRPs.

Proposal 16: Both NC-JT and C-JT transmission schemes can be considered for the multi-panel uplink transmission.

In a multi-panel/multi-TRP uplink transmission, there may be different potential combinations of simultaneous transmission such as PUSCH/PUSCH or PUCCH/PUCCH that can be considered for reliability enhancements. A tight coordination with potentially CSI exchange between the TRPs for the co-scheduling is needed to reduce interference between the simultaneous transmissions for SDM solutions. In these operations, uplink beams needs to be carefully selected to mitigate interference towards the other TRP.

Proposal 17: Beam selection needs to be further enhanced for the coordination between panels/TRPs to mitigate the interference for the SDM schemes.

For multi-TRP based PUSCH/PUCCH transmissions, 2 beams need to be indicated (via DCI or MAC-CE) for the transmissions towards different TRPs from different UE panels. TCIs (including either joint TCI or separate UL TCI) or SRIs can be used for the detailed indication for the 2 beams. The association between the indicated TCI(s)/SRI(s) and specific TRP(s)/panel(s) should take the UE capability value [set] index into consideration.

Proposal 18: The association between the indicated TCI(s)/SRI(s) and specific TRP(s)/panel(s) should take the UE capability value [set] index into consideration.

For the multi-TRP based PUSCH/PUCCH transmissions, different beams/TCIs/SRIs are associated with different transmission parameters for each transmission schemes defined in AI 9.1.4.1. The detailed discussion should be based on the agreed simultaneous transmission schemes.

Proposal 19: Beams/TCIs/SRIs are assumed to be associated with the corresponding set of transmission layer(s)/DMRS port(s)/RBs, each corresponding to a specific TRP or a specific panel. Detailed discussion should be based on the agreed transmission schemes discussed in AI 9.1.4.1.

Power control for UL single DCI

In Rel17, we have discussed enhancements on mTRP for PUSCH/PUCCH and TDMed PUCCH/PUSCH transmission has been specified under Rel15/16 TCI framework. However, unified TCI framework have been introduced in Rel17 and it will be extended to both DL and UL mTRP use case in Rel18. Then, though TRP-specific power control (PC) is already supported in Rel-17, some necessary enhancements on power control for UL single DCI for multi-TRP operation are still needed because the indication signaling of power control parameters for UL transmission in Rel17 unified TCI framework is different with that in Rel15/16 TCI framework.

Observed from section 7 in TS 38.213[4], the PC parameters for UL transmission includes P0, alpha, closed loop index and PL-RS. In Rel17 unified TCI framework, the PC parameters {P0, alpha, closed loop index} can be associated with joint/UL TCI state or configured independently, which means without association with joint/UL TCI state. While the PC parameter PL-RS is always included in joint/UL TCI state.

If all the PC parameters is configured to be associated with joint/UL TCI state via RRC, then the PC parameters for each TRP can be decided according to the corresponding joint/UL TCI state for each TRP to support single DCI based mTRP UL transmission.

Proposal 20: For S-DCI M-TRP UL transmission, the PC parameters for each TRP are decided according to the corresponding joint/UL TCI state for each TRP when the association between PC parameters and joint/UL TCI state  is configured.

However, above methods cannot indicate the PC parameters {P0, alpha, closed loop index} to UE when these parameters are independent of joint/UL TCI state. Then, two sets of PC parameters{ P0, alpha, closed loop index} can be configured to support single DCI based multi-TRP UL transmission.

Proposal 21: Two sets of PC parameters {P0, alpha, closed loop index} can be configured via RRC to support single DCI based multi-TRP UL transmission if these parameters are not associated with joint/UL TCI state.

Conclusion

In this contribution, we discuss about the unified TCI framework extension for M-TRP. Based on above discusses, we provide the following proposals.

Indication of multiple DL/UL TCI states

Proposal 1: For M-TRP unified TCI framework, support one TRP is configured with joint TCI and the other one is configured with separate DL/UL TCI.

Proposal 2: For M-TRP unified TCI framework, support to switch between joint TCI and separate DL/UL TCI based on MAC CE.

Proposal 3: For S-DCI M-TRP scenario, support both MAC CE and MAC CE+DCI for unified TCI state indication.

Proposal 4: For S-DCI M-TRP scenario, support dynamic switching between S-TRP and M-TRP by mapping some DCI codepoints to TCI states of two TRPs and some DCI codepoints to TCI states of one TRP.

Proposal 5: For M-TRP PDCCH repetition, to discuss whether to support dynamic switching to S-TRP when DLorjoint TCI state of only one TRP is indicated.

- Alt 1: UE switches to S-TRP.

- Alt 2: UE still acts as M-TRP by applying the indicated unified TCI state of the other TRP in the latest previous signaling.

Proposal 6: For S-DCI M-TRP scenario, support to configure CORESET group ID for each CORESET and indicate the association between CORESET group ID with unified TCI state of each TRP.

Proposal 7: For S-DCI M-TRP scenario, suggest to discuss whether/how to support unified TCI state indication for HST-SFN PDCCH.

Proposal 8: For M-TRP PDCCH + S-TRP PDSCH/PUSCH/PUCCH, suggest to discuss whether/how to support unified TCI state indication in S-DCI M-TRP scenario.

Proposal 9: For UE is configured with the combination below, when unified TCI state of two TRPs are indicated, suggest to define TCI states of which one TRP will be applied for S-TRP channel transmission:

- M-TRP PDSCH + S-TRP PUSCH/PUCCH,

- M-TRP PUSCH/PUCCH + S-TRP PDSCH,

- M-TRP PUSCH + S-TRP PUCCH

Proposal 10: For M-DCI M-TRP scenario, support both MAC CE and MAC CE+DCI for unified TCI state indication.

Proposal 11: For M-DCI M-TRP scenario, for channel /signal transmission scheduled by DCI, the unified TCI state of the CORESETPoolIndex same as the corresponding scheduling CORESET will be applied.

Proposal 12: For M-DCI M-TRP scenario, configure CORESETPoolIndex to PUCCH and configured grant PUSCH Type 1.

Proposal 13: For M-DCI M-TRP scenario, support to indicate the unified TCI state for CORESETPoolIndex#1 by DCI in CORESET configured with CORESETPoolIndex#0.

Proposal 14: Support to indicate at least separate UL TCI state by DCI formats 0_1 and 0_2.

Simultaneous multi-panel UL transmission

Proposal 15: Consider both SDM and FDM schemes for the S-DCI based simultaneous multi-panel uplink transmission in Rel-18. SFN scheme can also be considered in Rel-18.

Proposal 16: Both NC-JT and C-JT transmission schemes can be considered for the multi-panel uplink transmission.

Proposal 17: Beam selection needs to be further enhanced for the coordination between panels/TRPs to mitigate the interference for the SDM schemes.

Proposal 18: The association between the indicated TCI(s)/SRI(s) and specific TRP(s)/panel(s) should take the UE capability value [set] index into consideration.

Proposal 19: Beams/TCIs/SRIs are assumed to be associated with the corresponding set of transmission layer(s)/DMRS port(s)/RBs, each corresponding to a specific TRP or a specific panel. Detailed discussion should be based on the agreed transmission schemes discussed in AI 9.1.4.1.

Power control for UL single DCI

Proposal 20: For S-DCI M-TRP UL transmission, the PC parameters for each TRP are decided according to the corresponding joint/UL TCI state for each TRP when the association between PC parameters and joint/UL TCI state is configured.

Proposal 21: Two sets of PC parameters {P0, alpha, closed loop index} can be configured via RRC to support single DCI based multi-TRP UL transmission if these parameters are not associated with joint/UL TCI state.

References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink

- RAN1-105 e-meeting Chairman notes.

- TS 38.321 h00, V17.0.0 (2022-03)