3GPP TSG RAN WG1 #109-e                                          R1-2203794

e-Meeting, May 09st – May 20st, 2022

Agenda Item:	9.1.1.2

Source:	Xiaomi

Title:	Discussion on two TAs for multi-TRP operation

Document for:	Discussion and Decision

- Introduction

The Rel-18 WI on NR MIMO evolution for downlink and uplink is approved in RAN#94-e meeting [1], which includes the following objective.

7. Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed.

In this contribution, we discuss on two TAs for UL multi-DCI for multi-TRP operation.

- Two TAs for UL multi-DCI for multi-TRP operation

  - TRP-specific TAs and TA reference timings

In Rel-16/17 M-TRP transmission, the two cooperative TRPs are not geographically co-located. The distances between TRPs and UE are typically different so that the DL/UL signal transmissions between TRPs and UE experience different propagation time delays. When using one TA and one TA reference timing based on one TRP, the UL/DL timing cannot be aligned at the other TRP. This timing misalignment results in inter-symbol interference between DL and UL signal transmissions, which leads to performance degradation.

To alleviate this timing misalignment, two TAs and two TA reference timings can be adopted in Rel-18 M-TRP, i.e., TRP-specific TAs and TA reference timings. When discussing one/two TAs and one/two TA reference timings, the following 4 cases can be considered. Without loss of generality, we assume that TRP#1 is closer to UE than the TRP#2.

- Case 1: Adopting one TA (based on TRP#1) and one TA reference timing (based on TRP#1)

- Case 2: Adopting one TA (based on TRP#1) and two TA reference timings (TRP-specific)

- Case 3: Adopting two TAs (TRP-specific) and one TA reference timing (based on TRP#1)

- Case 4: Adopting two TAs (TRP-specific) and two TA reference timings (TRP-specific)

A detailed mathematical timing analysis for TA can be found in Appendix. The above-mentioned 4 cases are illustrated in Figure 1.

Figure 1 Timing analysis for cases 1-4

From Figure 1 we can observe that only when adopting two TAs and two TA reference timings, the DL and UL signals can be aligned at two TRPs. As a result, we support to adopt TRP-specific TAs and TA reference timings in Rel-18 inter/intra-cell M-TRP operation. The RAR TAC and MAC-CE TAC are then used to indicate the initial TAs and updated TAs for M-TRP [2][3].

Observation 1: When using one TA and TA reference timing based on TRP#1, the DL and UL signals cannot be aligned at the TRP#2.

Proposal 1: Adopting TRP-specific TAs and TA reference timings in Rel-18 M-TRP operation.

  - Acquisition of two TAs

The following two cases should be considered, i.e., inter-cell M-TRP and intra-cell M-TRP.

Case 1: Inter-cell M-TRP

In inter-cell M-TRP, two TRPs adopt two independent random access processes to acquire two TAs. UE transmits random access preamble to two TRPs and receives random access responses (RARs) from two TRPs independently in a successive or parallel way.

Case 2: Intra-cell M-TRP

In intra-cell M-TRP, two TRPs adopt two independent random access processes to acquire two TAs. UE distinguishes the TRP based on that whether the beam direction of SSB contains CORESET#0. The TRP whose beam direction of SSB contains CORESET#0 is specified as the TRP#1, and the other TRP is specified as TRP#2. The UE initiates random access process and transmits random access preamble to TRP#1, and then the gNB uses PDCCH order to indicate the random access process for TRP#2 to acquire two TAs.

Proposal 2: Adopting per-TRP random access process to acquire two TAs for inter/intra-cell M-TRP operation.

  - Indication of two TAs

When TRP-specific TAs are acquired, the TAs are indicated by RAR TAC initially and updated by MAC-CE TAC.

RAR TAC:

A second RAR TAC field can be introduced. The existing and the second RAR TAC fields indicate the TA for TRP#1 and TRP#2, respectively.

The  and  can be respectively calculated as

where  is indicated by the existing RAR TAC field (12 bits), and  is indicated by the introduced second RAR TAC field (e.g., 12 bits).

Proposal 3: Using two TAC fields in RAR to indicate the two TAs for M-TRP separately.

MAC-CE TAC:

Alt.1: Adopt one MAC-CE to update two TAs for M-TRP.

Alt.1-1: A second MAC-CE TAC field can be introduced. The existing and the second MAC-CE TAC fields indicate the updated TA for TRP#1 and TRP#2, respectively.

The  and  can be respectively calculated as

where  is indicated by the existing MAC-CE TAC field (6 bits),  is indicated by the introduced second MAC-CE TAC field (e.g., 6 bits),  and  are indicated initially by RAR TAC.

Alt.1-2: A MAC-CE timing advance offset command (TAOC) field can be introduced. The existing MAC-CE TAC field indicates the updated TA for TRP#1, and the introduced MAC-CE TAOC field indicates the relative offset  for TRP#2 with respect to the updated TA for TRP#1.

The  and  can be respectively calculated as

where  is indicated by the existing MAC-CE TAC field (6 bits),  is indicated by the introduced MAC-CE TAOC field (e.g., 7 bits) in which the first bit indicates the positive or negative of ,  and  are indicated initially by RAR TAC.

Proposal 4: One MAC-CE can be adopted to indicate the updated TAs for M-TRP. One way is to introduce a second MAC-CE TAC field (e.g., 6 bits) while the existing and the second MAC-CE TAC fields indicate the updated TAs for TRP#1 and TRP#2, respectively. Another way is to introduce a new MAC-CE TAOC field (e.g., 7bits) while the existing MAC-CE TAC field indicates the updated TA for TRP#1 and the introduced MAC-CE TAOC field indicates the relative offset ΔTA for TRP#2 with respect to the TA for TRP#1.

Alt.2: Adopt two MAC-CE to update two TAs for M-TRP.

A second MAC-CE can be introduced. The TAC fields of the existing and the second MAC-CE indicate the TA for TRP#1 and TRP#2, respectively.

The  and  can be respectively calculated as

where  is indicated by the TAC field of the existing MAC-CE,  is indicated by the TAC filed of the second MAC-CE,  and  are indicated initially by RAR TAC.

Proposal 5: The second MAC-CE can be introduced to indicate the updated TAs for M-TRP. The TACs in the existing and the second MAC-CE are adopted to indicate the updated TAs for TRP#1 and TRP#2, respectively.

- Conclusion

In this contribution, we discuss about two TAs for UL multi-DCI for multi-TRP operation. According to the above discussions, we provide the following observation and proposals.

Observation 1: When using one TA and TA reference timing based on TRP#1, the DL and UL signals cannot be aligned at the TRP#2.

Proposal 1: Adopting TRP-specific TAs and TA reference timings in Rel-18 M-TRP operation.

Proposal 2: Adopting per-TRP random access process to acquire two TAs for inter/intra-cell M-TRP operation.

Proposal 3: Using two TAC fields in RAR to indicate the two TAs for M-TRP separately.

Proposal 4: One MAC-CE can be adopted to indicate the updated TAs for M-TRP. One way is to introduce a second MAC-CE TAC field (e.g., 6 bits) while the existing and the second MAC-CE TAC fields indicate the updated TAs for TRP#1 and TRP#2, respectively. Another way is to introduce a new MAC-CE TAOC field (e.g., 7bits) while the existing MAC-CE TAC field indicates the updated TA for TRP#1 and the introduced MAC-CE TAOC field indicates the relative offset ΔTA for TRP#2 with respect to the TA for TRP#1.

Proposal 5: The second MAC-CE can be introduced to indicate the updated TAs for M-TRP. The TACs in the existing and the second MAC-CE are adopted to indicate the updated TAs for TRP#1 and TRP#2, respectively.

- References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, RAN#94-e, Dec. 6-17, 2021.

- TS 38.213.

- TS 38.321.

- Appendix

Table 1 Timing analysis for case 1

|  | TRP#1 | TRP#2 |
| --- | --- | --- |
| Distance |  |  |
| TRP Tx timing |  |  |
| Propagation delay |  |  |
| UE Rx timing |  |  |
| TA reference timing |  |  |
| TA |  |  |
| UE Tx timing |  |  |
| TRP Rx timing |  |  |
| TRP Rx timing offset |  |  |
| Rx timing offset between TRPs |  |  |

Table 2 Timing analysis for case 2

|  | TRP#1 | TRP#2 |
| --- | --- | --- |
| Distance |  |  |
| TRP Tx timing |  |  |
| Propagation delay |  |  |
| UE Rx timing |  |  |
| TA reference timing |  |  |
| TA |  |  |
| UE Tx timing |  |  |
| TRP Rx timing |  |  |
| TRP Rx timing offset |  |  |
| Rx timing offset between TRPs |  |  |

Table 3 Timing analysis for case 3

|  | TRP#1 | TRP#2 |
| --- | --- | --- |
| Distance |  |  |
| TRP Tx timing |  |  |
| Propagation delay |  |  |
| UE Rx timing |  |  |
| TA reference timing |  |  |
| TA |  |  |
| UE Tx timing |  |  |
| TRP Rx timing |  |  |
| TRP Rx timing offset |  |  |
| Rx timing offset between TRPs |  |  |

Table 4 Timing analysis for case 4

|  | TRP#1 | TRP#2 |
| --- | --- | --- |
| Distance |  |  |
| TRP Tx timing |  |  |
| Propagation delay |  |  |
| UE Rx timing |  |  |
| TA reference timing |  |  |
| TA |  |  |
| UE Tx timing |  |  |
| TRP Rx timing |  |  |
| TRP Rx timing offset |  |  |
| Rx timing offset between TRPs |  |  |