3GPP TSG RAN WG1 #109-e                                          	         R1-2203795

e-Meeting, May 9th – 20th 2022

Agenda Item:	9.1.2

Source:	Xiaomi

Title:                     Discussion on CSI enhancement

Document for:	Discussion and Decision

Introduction

In 3GPP TSG RAN 94 e-meeting, the following two objectives on CSI reporting or acquisition enhancement were identified [1].

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking - Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

In this contribution, we discuss the CSI reporting enhancement for high/medium UE velocities and CSI acquisition enhancements for coherent joint transmission (CJT) respectively.

CSI reporting enhancement for high/medium UE velocities

In this section, system level evaluation assumption and potential issues on CSI reporting enhancement for high/medium UE velocities are discussed respectively.

Discussion on evaluation assumption for high/medium UE velocities

In order to justify CSI enhancement for high/medium UE velocities, performance evaluation is necessary. There are two bullets for the first objective of WID to be studied, i.e., CSI reporting enhancement for high/medium UE velocities are based on Rel-16/17 Type II codebook and Doppler shift measured by TRS measurement.

For the first bullet, Rel-16/17 Type II codebook is refined to enhance CSI reporting. Hence, the system-level simulation assumption of Rel-16/17 Type II codebook should be a starting point. According to identified evaluation assumption for the two codebook types, 80% users with 3 Km/h are located the indoor and 20% users with 30 Km/h are located the outdoor. Since CSI enhancement based on Type II codebook is targeting for high/medium UE, all users should be located the outdoor. According to the generated channel model in TR 38.901, there are two options to generate Doppler shift.

- Option1: The generated Doppler shift depends on the arrival angles (AOA, ZOA), and UE velocity vector with speed v, travel azimuth angle ϕv, elevation angle θv and is given by

vn,m=rrx,n,mT.vλ0,where v=v.sinθvcosφv sinθvsinφv cosθvT.            (1)

- Option2: The generated Doppler shift generally depends on the time evolution of the channel and is given by

vn,m= t0trrx,n,mT(t)⋅vtλ0dt,                                                           (2)

where, rrx,n,m(t) is the normalized vector that points into the direction of the incoming wave as seen from the Rx at time t. vt denotes the velocity vector of the Rx at time t, while t0 denotes a reference point in time that defines the initial phase, e.g. t0=0.

Usually, the arrival angle, azimuth angle ϕv and elevation angle θv are not changed during simulation. If the speed of user is a constant value, the Doppler shift is invariable according to Option 1. However, according to Option 2, the Doppler shift will be variable as the arrival angle, azimuth angle and elevation angle change on the time evolution. Our intention is that the precoder or channel can be predicted by using time domain correlation or Doppler-domain information. If system-level performance can be improved at the same overhead by using the enhanced codebook compared with the codebook supported in current specification, it can justify that it is necessary to enhance CSI for high/medium UE, no matter the Doppler shift is changed or not during simulation. Thus, variable Doppler shift for simulation evaluation is not needed. In addition, the simulation effort of Option 1 is less than that of Option 2. Thus, Option 1 is preferred. Based on above discussion, we give the following proposals.

Proposal 1: The evaluation assumptions of both Rel-16 Type II codebook and/or Rel-17 Type II port selection codebook can be a starting point, and all users are located the outdoor.

Proposal 2: The generated Doppler shift does not depend on the time evolution of the channel for performance valuation.

According to (1), we can see that Doppler shift is in proportion to the UE’s speed when AOA, ZOA, ϕv, and θv keep constant. The larger Doppler shift leads to significant channel variation in time domain.  If the UE’s speed is not fixed, the simulation results of different companies cannot be aligned. In order to address the issue, the UE’s speed should be fixed with some representative values, e.g., 30 Km/h, 60 Km/h, 120 Km/h, to evaluate the performance gain.

Proposal 3: Some representative values, e.g., 30 Km/h, 60 Km/h, 120 Km/h are adopted as the UE’s speed.

Discussion on CSI reporting enhancement for high/medium UE velocities

According to the first objective of WID [1], CSI reporting enhancement through Rel-16/17 Type II codebook refinement and the Doppler shift measured by TRS is to be studied. In this subsection, the two schemes of CSI enhancement are respectively discussed.

CSI reporting enhancement based on Rel-16/17 Type II codebook

Assume that gNB is equipped with a uniform line array (ULA) of M antennas. For a user severed by it, its multipath channel can be expressed as

ht=k=1KgkDLaθke-j2πfktδt-τk,                                              (3)

where

- K is the number of propagation paths from gNB to the user.

- gkDL is downlink complex gain of the k-th propagation path.

- aθk=1,e-j2πdλsinθk,⋯,e-j2πM-1dλsinθkT∈CM×1 is the steering vector of UPA for the k-th propagation path with θk, where λ is carrier wavelength.

- fk=vλcosϕkt is the Doppler shift of the k-th propagation path, where ϕkt is angle between the direction of user mobility and the direction of received signal for the UE at time t for the k-th propagation path.

- τk is the delay of the k-th propagation path.

- δ (·) is the Dirac impulse function.

According to (3), we can see each propagation path corresponds a Doppler shift.

Observation 1: Each propagation path has its own Doppler shift.

Compared with Rel-16 Type II codebook, Rel-17 Type II PS codebook can improve system performance or reduce feedback overhead by exploring the angle and delay reciprocities of uplink and downlink channel. In fact, Doppler shift of uplink and downlink channel is also reciprocal [2] since the user’s velocity or the scatters’ velocity is independent of the carrier frequency. This implies that Doppler shift is not only obtained through downlink channel, but also through uplink channel. Different methods to obtain Doppler shift leads to different codebook designs and different processing procedures. According to the methods of obtained Doppler shift, potential codebook design and processing procedure are discussed as follows.

Doppler component is obtained at UE side

For Rel-16/17 Type II codebook, the propagation path’s angle and delay of downlink channel can be characterized by a spatial domain basis and a frequency domain basis, and they are calculated according to uplink spatial-frequency channel matrix. Similarly, the propagation path’s Doppler shift can also be characterized by time domain basis. But the time domain basis should be calculated according to spatial-frequency-time domain channel. This implies that multiple downlink pilot reference signal should be transmitted at different instants. E.g., CSI-RS is transmitted to UE in T adjacent slots, i.e., CSI-RS burst can be designed to obtain the Doppler component of downlink channel.

The spatial and/or frequency domain basis which characters the angle and/or delay of path is reported though CSI feedback for Rel-16 Type II codebook and Rel-16 Type II PS codebook. For Rel-17 Type II codebook, only reporting beam combination coefficients can also work since the spatial and frequency domain basis have been obtained through uplink channel. According to the WID, the spatial and frequency domain bases are not modified. In order to refine Rel-16/17 Type II codebook by exploring Doppler information, time domain basis should be reported to gNB. One question is how to explore time domain basis to design the enhanced codebook. In our view, the following two alternatives can be considered to refine Rel-16/17 Type II codebook.

- Alt1: W=W1W2Wd⨂WfH

- Alt2: W=Wf*⨂W1W2WdH.

where W1 is a spatial domain basis matrix or port selection matrix, W2 is a combination coefficients matrix, Wf is a frequency domain basis matrix, Wd is a time domain basis matrix. From the mathematic perspective, Alt 1 is same to Alt 2 after some mathematic operation. However, both of them corresponds to different reported quantity or different processing procedure. For Alt 1, each combination coefficient corresponds to a spatial domain basis and a time-frequency domain basis pair. Whereas each combination coefficients corresponds to spatial-frequency domain basis pair and a time domain basis for Alt 2. This leads that the CSI feedback may be different, which has impact on system performance and feedback overhead. Therefore, the two codebook structures should be studied.

Proposal 4: The codebook structure W=W1W2Wd⨂WfHor W=Wf*⨂W1W2WdH can be considered to enhance Rel-16/17 Type II codebook, where  W1 is a spatial domain basis matrix or port selection matrix, W2 is a combination coefficients matrix, Wf is a frequency domain basis matrix, Wd is a time domain basis matrix.

Doppler component is obtained at gNB

In order to calculate Doppler shift at gNB side, uplink spatial-frequency-time channel should be estimated at different instants. This implies uplink pilot signal should be sent by UE at multiple instants. E.g., SRS are transmitted with time bundling at multiple slots. Then, the time domain basis can be calculated according to the estimated uplink channel. Similar to Rel-17 Type II PS codebook, in order to obtain combination coefficients, beamformed CSI-RS is required to send to UE. The combination coefficients are calculated according to the estimated effective channel through beamformed CSI-RS.  One remained issue is how to design the beams of CSI-RS. The straightforward way is that each beam of CSI-RS includes the Doppler shift. E.g., the beam is designed as ejφtf⨂s at time t, where φt denotes the phase shift due to Doppler shift, s and f denote spatial domain basis and frequency domain basis, respectively, and ⨂ denotes the Kronecker operation. Assume the reciprocity of angle, delay and Doppler shift are ideal reciprocity, the enhancement codebook can be defined as W=W1W2, where W1 and W2 are the port selection matrix and combination coefficients matrix, respectively. Compared with Alt 1 and Alt2, the time domain basis is not to be reported, which can save feedback overhead.

Proposal 5: The codebook structure  W=W1W2 can be considered to enhance Rel-17 Type II codebook when Doppler component is calculated by gNB, where W1 is a port selection matrix and W2 is a combination coefficients matrix.

CSI reporting enhancement based on the Doppler shift measured by TRS

The high-resolution Doppler shift and delay can be directly measured through TRS due to its special design of time-frequency density. The Doppler shifts of multiple propagation paths can be obtained through power delay profile estimation. Assume the Doppler shift is constant during the period  TTRS of TRS. For TDD systems, gNB estimates the uplink channel of time t0 through received SRS in the period. Then, gNB can predict the downlink channel during TTRS-t0 according to the estimated uplink channel and the reported Doppler shift by UE. The remained issue is how to report the Doppler shift to gNB. In our view, there are two options to solve it.

- Option 1: The Doppler shift is directly reported.

- Option 2: The phase shift corresponding to the Doppler shift is reported, where the variation of phase shift is restricted in the range of 0 to 2π.

Assume the Doppler shift for a path is 100 Hz, at least 7 bits are required to quantized by using Option 1. However, for Option 2, if the phase shift corresponding to the path is restricted in the range of 0 to 2π, only 3 or 4 bits are required to quantized it. Compared with Option1, Option2 can save feedback overhead, especially when there are multiple propagation paths corresponding to multiple Doppler shifts to be reported. According to expression (3), Dopplers shift is calculated as fk=vλcosϕkt. Hence, the maximum Doppler shift is vλ . Let  ∆t=λv,  then the phase shift φ=2πfk∆t. Since fk∆t≤1, thus the phase shift φ can be restricted in the range of 0 to 2π.

Proposal 6: The phase shift corresponding to the Doppler shift is reported to gNB.

CSI acquisition enhancement for coherent joint transmission (CJT)

Use cases

Refer to [3], intra-site, inter-site, intra-cell M-TRP and inter-cell M-TRP for CJT are considered. From our point of view, the experienced downlink channel of the user from the collaboration TRPs is different for these use cases. This leads to different schemes of codebook refinement to be required. Thus, the use case should be firstly confirmed from intra-site, inter-site, intra-cell M-TRP and inter-cell M-TRP. In addition, if inter-cell M-TRP is assumed, it requires that further discussion on whether intra-frequency, intra-RAT or intra-band CA is assumed or not.

Figure 1: Use case of CJT

Proposal 7: The potential use cases, e.g., intra-site, inter-site, intra-cell M-TRP and inter-cell M-TRP should be firstly confirmed.

Rel-16/17 codebook refinement

Figure 2: Example of 3-inter-site TRP CJT

According to the WID, enhancements of CSI acquisition for CJT targeting FR1 and up to 4 TRPs will be considered. In Figure 2, we give an example of inter-site TRP CJT with 3 TRPs.  And the channel of 3 TRPs are H1, H2 and H3, respectively. Here we assume that same time/frequency resource is applied to all TRPs for codebook design. Considering the experienced channel condition of UE, the following four alternatives are considered to refine Rel-16/17 Type II codebook.

- Alt1: SD basis per TRP, FD basis per TRP and combinatorial coefficients per TRP

W=W1,1W2,1Wf,1H⋮W1,NW2,NWf,NH， W1,i∈CNt×2Li，W2,i∈C2Li×Mv,i，Wf,i∈CN3,i×Mv,i

- N is the number of TRPs for CJT.

- Nt  is the number of transmit antenna ports or CSI-RS ports.

- Li is the number of SD basis for the ith TRP.

- Mv,i is the number of FD basis for the ith TRP with rank=v.

- N3,i is the number of subbands for the ith TRP. Here we assume the number of subband is same for each TRP.

- W1,i, W2,i and Wf,i are the spatial domain basis matrix, combination coefficients matrix and frequency domain basis matrix, respectively, for the ith TRP.

For Alt1, we prefer to consider it as baseline since it is a straightforward method with less specification impact. This codebook structure can be applied to all use cases.

- Alt2: SD basis per TRP, both FD basis and combinatorial coefficients across TRPs.

W=W1,1⋱W1,NW2WfH， W1,i∈CNt×2Li，W2∈Ci=1N2Li×Mv，Wf∈CN3×Mv

For example, for intra-site TRPs, the angle of arrive from different TRP is different. However, the propagation delay is almost same to UE, since the distance from UE to those TRPs is almost same. Therefore, a single FD basis set can be enough to calculate the precoder for CJT. In addition, feedback overhead of Alt2 is less than that of Alt1.

- Alt3: SD basis per TRP group, FD basis per TRP group and combinatorial coefficients per TRP group

W=W1,1W2,1Wf,1H⋮W1,GW2,GWf,GH，

- W1,g is the SD basis for the gth TRP group，Wf,gis the FD basis for the gth TRP group， W2,gis the combinatorial coefficients for the gth TRP group.

For example, there are 4 TRPs. Wherein TRP#1 and TRP#2 are two intra-site TRPs, and TRP#3 and TRP#4 are the other two intra-site TRPs. Thus TRP#1 and TRP#2 can be considered as the 1st TRP group, and TRP#3 and TRP#4 can be considered as the 2nd TRP group.

- Alt4: SD basis across TRPs, FD basis across TRPs and combinatorial coefficients across TRPs

W=W1W2WfH，W1∈CNt×2L or W1∈CN*Nt×2L，W2∈C2L×Mv，Wf∈CN3×Mv

where W1 is a spatial domain matrix or port selection matrix, W2 is a combination coefficients matrix and Wf is a frequency domain basis matrix. For Alt 4, there are two sub alternatives. For example, Alt 4-1 targets the scenario that all TRPs are co-located and belong to one cell, i.e., intra-site intra-cell M-TRPs. In such case, both FD basis and the SD basis are same across mTRPs as like the SD basis design for Type I-Multi-panel codebook which assumes that the SD basis is same across different TRPs. While Alt 4-2 means that all downlink channel from different TRPs are considered together to derive the spatial domain matrix or port selection matrix, combination coefficients matrix and frequency domain basis matrix, respectively.

Proposal 8: Consider codebook refinement from following alternatives and take Alt 1 as baseline:

- Alt1: SD basis per TRP, FD basis per TRP and combinatorial coefficients per TRP.

- Alt2: SD basis per TRP, both FD basis and combinatorial coefficients across TRPs.

- Alt3: SD basis per TRP group, FD basis per TRP group and combinatorial coefficients per TRP group.

- Alt4: SD basis, FD basis and combinatorial coefficients are all across TRPs.

CSI reporting for CJT

As for Multi-TRP NCJT, a single CSI reporting setting is assumed and different resource groups in one resource set are mapped to different TRPs. For a CSI report associated with Multi-TRP NCJT, two options are supported in RAN1-104 e-meeting.

Agreement

For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, support following two options:

- Option 1: the UE can be configured to report X CSIs associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis

  - X = 0, 1, 2

    - If X=2, two CSIs are associated with two different single-TRP measurement hypotheses with CMRs from different CMR groups

    - Support of X=1,2 is UE optional for the UE supporting option 1

  - FFS omission of CSI associated with NCJT measurement hypothesis

- Option 2: the UE can be configured to report one CSI associated with the best one among NCJT and single-TRP measurement hypotheses

  - FFS how to report recommended measurement hypothesis associated with that CSI report

For CJT, we think similar CSI feedback framework can be considered. But there are multiple measurement hypotheses for 4 TRPs. For single-TRP, there are up to 4 measurement hypotheses. For 2 TRPs, there are up to 6 measurement hypotheses. For 3 TRPs, there are up to 4 measurement hypotheses. For 4 TRPs, there is 1 measurement hypothesis. Thus, for CSI reporting up to 4 TRPs CJT, the UE can be configured to report X CSIs associated with single-TRP measurement hypotheses, report Y CSIs associated with two-TRP measurement hypotheses, report Z CSIs associated with three-TRP measurement hypotheses, and M CSIs associated with four-TRP measurement hypothesis. And the candidate value can be given as follows:

- X = 0, 1, 2, 3, 4.

- Y = 0, 1, 2, 3, 4, 5, 6.

- Z = 0, 1, 2, 3, 4.

- M = 0, 1.

Different options for their combination should be considered with different values of X, Y, Z and M. For example, X = 0,1, 2, 3, 4, Y=1, Z=1 and M=1, or only one of X, Y, Z, M equals to 1, others are 0. The potential combinations of X, Y, Z and M can be studied.

Proposal 9: For CSI report for CJT up to 4 TRPs, the UE can be configured to report X CSIs associated with single-TRP measurement hypotheses, report Y CSIs associated with two-TRP measurement hypotheses, report Z CSIs associated with three-TRP measurement hypotheses, and M CSIs associated with four-TRP measurement hypothesis. The potential combinations of X, Y, Z and M can be studied.

Conclusion

In this contribution, we discuss about the CSI enhancement for high/medium UE velocities and mTRP CJT, The following proposals and observations on CSI enhancement are provided.

Observations

Observation 1: Each propagation path has its own Doppler shift.

Proposals

Proposal 1: The evaluation assumptions of both Rel-16 Type II codebook and Rel-17 Type II port selection codebook can be a starting point, and all users are located outdoor .

Proposal 2: The generated Doppler shift does not depend on the time evolution of the channel for performance valuation.

Proposal 3: Some representative values, e.g., 30 Km/h, 60 Km/h, 120 Km/h are adopted as the UE’s speed.

Proposal 4: The codebook structure W=W1W2Wd⨂WfHor W=Wf*⨂W1W2WdH can be considered to enhance Rel-16/17 Type II codebook, where  W1 is a spatial domain basis matrix or port selection matrix, W2 is a combination coefficients matrix, Wf is a frequency domain basis matrix, Wd is a time domain basis matrix.

Proposal 5: The codebook structure  W=W1W2 can be considered to enhance Rel-17 Type II codebook when Doppler component is calculated by gNB, where W1 is a port selection matrix and W2 is a combination coefficients matrix.

Proposal 6: The phase shift corresponding to the Doppler shift is reported to gNB.

Proposal 7: The potential use cases, e.g., intra-site, inter-site, intra-cell M-TRP and inter-cell M-TRP should be firstly confirmed.

Proposal 8: Consider codebook refinement from following alternatives and take Alt 1 as baseline:

- Alt1: SD basis per TRP, FD basis per TRP and combinatorial coefficients per TRP.

- Alt2: SD basis per TRP, both FD basis and combinatorial coefficients across TRPs.

- Alt3: SD basis per TRP group, FD basis per TRP group and combinatorial coefficients per TRP group.

- Alt4: SD basis, FD basis and combinatorial coefficients are all across TRPs.

Proposal 9: For CSI report for CJT up to 4 TRPs, the UE can be configured to report X CSIs associated with single-TRP measurement hypotheses, report Y CSIs associated with two-TRP measurement hypotheses, report Z CSIs associated with three-TRP measurement hypotheses, and M CSIs associated with four-TRP measurement hypothesis. The potential combinations of X, Y, Z and M can be studied.

References

- RP-213598, Samsung, “New WID: MIMO Evolution for Downlink and Uplink,” RAN#94e, Dec 2021.

- Wang M ,  Gao F ,  Jin S , et al. An Overview of Enhanced Massive MIMO with Array Signal Processing Techniques[J]. Selected Topics in Signal Processing, IEEE Journal of, 2019, 13(5):886-901.

- RP-212150, Huawei, “Updated views on R18 downlink MIMO enhancements”, RAN#94e, Dec 2021.