3GPP TSG RAN WG1 #109-e			R1-2203801

e-Meeting, May 9th – 20th, 2022

Source: 	     Xiaomi

Title:		     Discussion on multi-carrier UL Tx switching scheme

Agenda item:    9.10.2

Document for:  Discussion

Introduction

In RAN#94 e-meeting, new WID on Multi-carrier enhancements was officially approved. One of the objectives is to study the potential enhancement for multi-carrier UL operation, i.e. UL Tx switching across up to 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs. The detailed objective is excerpted as below: [1].

2. Study and if necessary specify following enhancements for multi-carrier UL operation [RAN1, RAN2, RAN4] - UL Tx switching schemes across up to 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs, including mechanisms to enable more configured UL bands than its simultaneous transmission capability and to support dynamic Tx carrier switching across the configured bands for both single TAG and multiple TAGs configurations (RAN1, RAN4) - UE capability and RRC configuration related signalling (RAN2) - Note: strive for RAN1/2 design agnostic with the number of bands, i.e., common design between 3 and 4 bands - Note: no additional TAG is introduced for UL transmission on a carrier without corresponding DL carrier - Note: this objective does not target to extend the SUL framework to support more than 1 SUL for 1 NUL - Switching time and other RF aspects, and RRM requirements for above UL Tx switching schemes across up to 3 or 4 bands (RAN4) - Note: Prioritize UL Tx switching across up to 3 bands is to be addressed first and then that for up to 4 bands can also be addressed

In this contribution, we provide our analyses and views on UL Tx switching across more than 2 bands.

Discussion

Currently, a UE can be configured with up to 2 UL bands for UL Tx switching. The 2 UL bands are configured via RRC signalling, i.e., BandCombination-UplinkTxSwitch. In other words, gNB has to update or re-configure the band combination list for UL Tx switching if the current band list is not the optimal choice any more. There are several potential reasonings for a UE to request a new band or band combination for uplink transmission:

- Reasoning#1: The traffic on the current band is quite heavy and there is limited opportunity for uplink transmission.

- Reasoning#2: UE needs a UL dominated TDD UL-DL configuration while the current band is DL dominated and needs to be maintained as DL dominated. Dynamic SFI is an alternative solution with configuring more UL slots or symbols within a TDD UL-DL configuration period. However, it will impacts the downlink transmission as less DL resources are perceived.

- Reasoning#3: The configured UL band has a relative small bandwidth which cannot satisfy the requirement from a higher throughput, e.g. n13 and n14 for which the UL band is 10 MHz.

- Reasoning#4: The channel condition on the configured UL band somehow becomes pessimistic which would lead to low QoS, e.g. sporadic interference from interference source.

RRC-based solutions for uplink band switching would bring significant latency which may lead to jeopardized uplink performance. On the other hand, if UE can switch UL Tx chain among more than two bands, the issues listed above can be resolved or at least be alleviated. gNB can indicate a UE to switch to one or two uplink bands more suitable compared to the current band(s) the Tx chain associated with. The UL Tx switching can be finished within tens of or hundreds of micro-seconds depending on UE capability.  Subsequently, higher uplink throughput, spectrum utilization and UL capacity can be achieved.

Observation 1: UL Tx switching across up to 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs is beneficial to the flexibility and robustness of uplink transmission.

In Rel-16/17, UL Tx switching can be applied to EN-DC, inter-band CA and SUL, as shown in Figure1-3. Furthermore, in order to avoid the ambiguity during UL Tx switching between two 1-Tx UL bands, two options, i.e. switchedUL and dualUL, for UL Tx switching are configurable. Once UL Tx switching happens, a switching period between two adjacent UL transmission is needed, during which no transmission is expected.

Figure 1: Illustration of UL Tx switching for EN-DC

Figure 2: Illustration of UL Tx switching for Inter-band CA

Figure 3: Illustration of UL Tx switching for SUL

In summary, the following table shows the relationships between scenarios and UL Tx switching schemes.

Table 1: supported UL Tx switching schemes in Rel-17

| Scenarios | switchedUL | dualUL |
| --- | --- | --- |
| EN-DC | 1 Tx for each UL band and simultaneous transmission with single port on both bands is not available. | 1 Tx for each UL band and simultaneous transmission with single port on both bands is available. |
| Inter-band CA | 1 Tx for each UL band and simultaneous transmission each with single port on two bands is not available. | 1 Tx for each UL band and simultaneous transmission each with single port on two bands is available. |
| SUL | Uplink transmission occurs either on NUL or SUL. |  |

For Rel-18 multi-carrier enhancement, although up to 3 or 4 UL bands can be configured for UL Tx switching, UE can only transmit uplink channel or signal on up 2 UL bands due to the restriction of 2 Tx.  From perspective of the two bands on which UL Tx switching occurs, the same switching scheme as current specification, e.g. 1Tx-to-1Tx, 1Tx-to-2Tx, 2Tx-to-1Tx, can be observed. Similarly, we need to exploit different UL Tx switching schemes in order to satisfy the diverse requirement in different scenarios. Based on the above analyses, we have the following proposal:

Proposal 1: For Rel-18 multi-carrier UL Tx switching, both ‘switchedUL’ and ‘dualUL’ should be supported.

For EN-DC, enhanced UL Tx switching should not impact LTE system. On the other hand, the motivation of supporting multi-carrier UL Txswitching is to improve NR performance, including UL throughput, UL capacity and spectrum utilization. It is straightforward to configure more than one UL bands for SCG while keep only one UL band for MCG. For inter-band CA, the current band combination can be easily extended to inter-band CA with a band combination larger than 2 relevant to UL Tx switching. For SUL, 1 SUL can only be associated with 1 NUL for now. Multiple NULs associated with 1 SUL in Rel-18 can be considered.  Still, UE can only transmit uplink channel or signal on either SUL or NUL(s).

Figure 4: Scenarios for Rel-18 UL Tx switching

Proposal 2: For Rel-18 multi-carrier UL Tx switching, switching schemes should be discussed case by case  for EN-DC, inter-band CA and SUL respectively.

Regarding to the mechanism of indicating UL Tx switching, UE can get the message of UL Tx switching via UL grant or the pre-configured RRC signalling.  To be specific, gNB and UE make the decision via the following mechanisms for dynamic scheduling and configured grant respectively:

- For DG, based on CIF and antenna ports, gNB and UE determine whether UL Tx switching happens across UL bands between two adjacent UL transmissions.

- For CG, based on the RRC configuration, gNB and UE determine whether UL Tx switching happens or not across UL bands between two adjacent UL transmissions.

Figure 5: Indication for UL Tx switching

Even up to 4 UL bands for UL Tx switching is supported, the current mechanism on indicating UL Tx switching is sufficient. No additional specification change for indicating the target UL band is needed.

Observation 2: For Rel-18 multi-carrier UL Tx switching, current mechanism, i.e. via UL grant or RRC signalling, is sufficient to indicate UL Tx switching between two adjacent uplink transmissions.

With regard to the maximum number of UL bands for UL Tx switching, 4 UL bands can provide more flexibility to scheduling while the standard impacts are similar with 3 UL bands. Hence we prefer to support up to 4 UL bands.

Proposal 3: UL Tx switching across up to 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs can be supported.

In Rel-17, the minimum granularity of UL Tx switching in time domain is a slot determined by the maximum SCS between the two UL bands for switching. The relevant descriptions in TS38.214 are excerpted as below.

The UE does not expect to perform more than one uplink switching in a slot with µUL = max(µUL, 1, µUL, 2), where the µUL, 1 corresponds to the subcarrier spacing of the active UL BWP of one uplink carrier before the switching gap and the µUL, 2 corresponds to the subcarrier spacing of the active UL BWP of the other uplink carrier after the switching gap.

Considering more than two UL bands can be supported for UL Tx switching, e.g. 4 UL bands, if the minimum granularity of UL Tx switching is still determined by the two UL bands on which switching happens, it may be impossible for UL switching on other band pair. One example is shown in Figure 6, assuming there are 4 UL bands are configured for UL Tx switching, i.e. {band#1, band#2, band#3, band#4}. The SCS is {30kHz, 15kHz, 15kHz, 15kHz} respectively. Assuming switching granularity is determined only by two UL bands on which UL Tx switching is happening, the following results can be observed:

- UL Tx switching from band#2 to band#3 becomes impossible. UE only expects one uplink switching in a slot with µUL = max(µband#2, µband#3)=15kHz. However, there is already one switching within the slot, i.e. uplink switching from band#1 to band#2. It brings additional restriction for UL Tx switching and results in more delay.

Figure 6: One example on switching granularity

In order to address the above issue, the switching granularity can be determined by the largest SCS among four UL bands for UL Tx switching.

Proposal 4:  The UE does not expect to perform more than one uplink switching in a slot with µUL, wherein µUL is the largest SCS among all bands configured for UL Tx switching.

As mentioned aforementioned, both switchedUL and dualUL should be supported for Rel-18 UL Tx switching. For switchedUL, UE can only transmit uplink channel/signal with one antenna port on a UL band at same time. There is no ambiguity on whether a switching period is needed between two adjacent uplink transmissions, no matter which band pair is used for UL Tx switching. For dualUL, it becomes more complicated as the antenna port used for uplink transmission for different Tx chain status may be same. The following table summarize all combination of UL transmission assuming 4 UL bands are configured for UL Tx switching. Generally speaking, the UL switching scheme across four UL bands can be categorized as below:

- Type 1: switching between 2 UL bands with single port and the other 2 UL bands with single port, e.g. case#1(1P+1P+0P+0P) and case#6(0P+0P+1P+1P)

- Type 2: switching between 2 UL bands with single port and another UL band with single port, e.g. case#1(1P+1P+0P+0P) and case#2(0P+0P+1P+0P)

- Type 3: switching between 1 UL band with single port and another UL band with single port not belonging to the same band pair, e.g. case#1(1P+0P+0P+0P) and case#2(0P+0P+1P+0P).

- Type 4: switching between 1 UL band with single port and another UL band with two ports not belonging to the same band pair, e.g. case#1(1P+0P+0P+0P) and case#10(0P+0P+0P+2P).

- Type 5: switching between 1 UL band with single port and the same UL band with two ports, e.g. case#1(1P+0P+0P+0P) and case#7(2P+0P+0P+0P).

- Type 6: switching between 1 UL band with single port and the same UL band with single ports, e.g. case#1(1P+0P+0P+0P) and case#2(1P+0P+0P+0P).

Table 2: Combinations of UL transmission

|  | Band#1 | Band#2 | Band#3 | Band#4 | Combination of UL transmission( P-> port) |
| --- | --- | --- | --- | --- | --- |
| Case#1 | 1 | 1 | 0 | 0 | 1P+0P+0P+0P; 0P+1P+0P+0P; 1P+1P+0P+0P |
| Case#2 | 1 | 0 | 1 | 0 | 1P+0P+0P+0P; 0P+0P+1P+0P; 1P+0P+1P+0P |
| Case#3 | 1 | 0 | 0 | 1 | 1P+0P+0P+0P; 0P+0P+0P+1P; 1P+0P+0P+1P |
| Case#4 | 0 | 1 | 1 | 0 | 0P+1P+0P+0P; 0P+0P+1P+0P; 0P+1P+1P+0P |
| Case#5 | 0 | 1 | 0 | 1 | 0P+1P+0P+0P; 0P+0P+0P+1P; 0P+1P+0P+1P |
| Case#6 | 0 | 0 | 1 | 1 | 0P+0P+1P+0P; 0P+0P+0P+1P; 0P+0P+1P+1P |
| Case#7 | 2 | 0 | 0 | 0 | 2P+0P+0P+0P; 1P+0P+0P+0P; |
| Case#8 | 0 | 2 | 0 | 0 | 0P+2P+0P+0P; 0P+1P+0P+0P; |
| Case#9 | 0 | 0 | 2 | 0 | 0P+0P+2P+0P; 0P+0P+1P+0P; |
| Case#10 | 0 | 0 | 0 | 2 | 0P+0P+0P+2P; 0P+0P+0P+1P; |

For Type 1, Type 2 and Type 4, there is no ambiguity on the switching period as the status between previous transmission and pending transmission can be easily differentiated. For Type 5, the current mechanism specified in TS38.214 is sufficient. For Type 3 and Type 6, it may have some issues as the same antenna port status and Tx chain state can be associated with different cases listed above. Take (1P+0P+0P+0P) and (0P+0P+1P+0P) as example, it may be associated with the following two different Tx chain states:

- (1P+0P+0P+0P) and (0P+0P+1P+0P) associates with case#2, i.e. 1 Tx chain associates with band#1 and the other associates with band#3. In this case, no switching period is needed between two adjacent uplink transmissions.

- (1P+0P+0P+0P) associates with case#1 and (0P+0P+1P+0P) associates with case#2 respectively. In this case, switching period is needed between two adjacent uplink transmissions.

Observation 3:  There may be ambiguity on determine the Tx chain state between two adjacent uplink transmissions in some cases, which need further study case by case.

Conclusion

In this contribution, we discuss possible issues on Rel-18 multi-carrier UL Tx switching, we have the following observations:

Observation 1: UL Tx switching across up to 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs is beneficial to the flexibility and robustness of uplink transmission.

Observation 2: For Rel-18 multi-carrier UL Tx switching, current mechanism, i.e. via UL grant or RRC signalling, is sufficient to indicate UL Tx switching between two adjacent uplink transmissions.

Observation 3:  There may be ambiguity on determine the Tx chain state between two adjacent uplink transmissions in some cases, which need further study case by case.

Furthermore, we have the following proposals according to the analyses in section 2:

Proposal 1: For Rel-18 multi-carrier UL Tx switching, both ‘switchedUL’ and ‘dualUL’ should be supported.

Proposal 2: For Rel-18 multi-carrier UL Tx switching, switching schemes should be discussed case by case for EN-DC, inter-band CA and SUL respectively.

Proposal 3: UL Tx switching across up to 4 bands with restriction of up to 2 Tx simultaneous transmissions for FR1 UEs can be supported.

Proposal 4:  The UE does not expect to perform more than one uplink switching in a slot with µUL, wherein µUL is the largest SCS among all bands configured for UL Tx switching.

Reference

  - RP-213577, New WID on Multi-carrier enhancements, NTT DOCOMO, INC.