3GPP TSG RAN WG1 #109-e		R1-2203804

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.12.1

Source:	          Xiaomi

Title:	Discussion on coverage enhancement for NR-NTN

Document for:	Decision

- Introduction

In RAN#95e Meeting [1], a new WID [2] was approved for study on enhancements for NG-RAN based Non-Terrestrial Networks, which in order to offer optimized performance especially when addressing handset terminals (including smartphones with more realistic assumptions on antenna gains instead of 0 dBi antenna gain with the specific realistic antenna gain assumption to be determined at the working group level) w.r.t. coverage considering the NTN characteristics such as large propagation delay and satellite movement. The detailed objectives of coverage enhancement are as follows:

4.1.1 Coverage enhancement The Rel-18 NTN objectives are focused on the applicability of the solutions developed by general NR coverage enhancement to NTN, and identifying potential issues and enhancements if necessary, considering the NTN characteristics including large propagation delay and satellite movement. Only NTN-specific characteristics are to be included in this coverage enhancement work, otherwise it should be part of another WI (e.g., UL enhancement of coverage). The work needs to cover the use case of voice and low-data rate services using commercial smartphones with more realistic assumptions on antenna gains instead of 0dBi currently assumed for link budget analysis for non-terrestrial networks. The specific realistic antenna gain assumption will be determined at the working group level. The evaluation should also take into account any related regulatory requirements, e.g., ITU limitation of power flux density. Have a 1-TU 6-month study phase focusing on the following (to derive clear & limited scope): - Evaluate the coverage performance and identify the candidate physical radio channels that have coverage issues specific to NTN with following target services taking into account the studies in TR38.830 where appropriate, as well as general coverage enhancement techniques specified in Rel-18 [RAN1,RAN2,RAN4] - VoIP and low-data rate services for commercial handset terminals The following items are shown as examples of areas to consider in the next step of the study. The actual items for study will be based on the evaluation of coverage issues specific to NTN identified above. - NTN-specific repetitions enhancements beyond techniques covered in Rel-17 CovEnh WI for the relevant channels - NTN-specific techniques for improved diversity and/or reduced polarization loss - Improved performance of low-rate codecs in link budget limited situation including reducing RAN protocol overhead for VoNR - NOTE: Intent is to optimize the NTN-based NG-RAN to work with the lowest rate codec currently available and will not introduce a new codec. RAN to determine by RAN#97 (for RAN1 items) and RAN#98 (for RAN2 items) whether the study phase has identified any need for NTN-specific coverage enhancements in Rel-18. If needed, the set of NTN-specific work item objectives will be updated.

In this contribution, we present our views on the evaluation methodology of NTN coverage performance and initial simulation results are provided.

- Discussion

2.1 Evaluation methodology

The basic evaluation methodology is based on link-level simulation and link budget calculation in 2 steps.

Step 1 aims to obtain the required SNR for the physical channels in target scenarios with target service requirement. Based on the WID, the target service is VoIP and low-data rate services for commercial handset terminals.

- The VoIP and low-data rate service have different assumptions on the data rate requirement. The performance requirements for satellite access is listed in table 1 in TS 22.261[4], the low-data rate for DL and UL is assumed as 1Mbps and 100kbps respectively. A packet size of 320 bits with 20ms data arriving intervals is assumed for VoIP based on TR 38.830 [5]. Companies are encouraged to provide TB size assumed in evaluation.

- For the channel model assumption, the NTN-CDL/NTN-TDL channel models in TR 38.811 [6] can be considered for evaluation. Companies are encouraged to provide detailed parameters such as delay spread of channel model assumed in evaluation.

- For the antenna configuration, 1Tx/Rx is assumed for satellite antenna configuration and (1,1,2) with omni-directional antenna element is assumed for UE antenna configuration based on TR 38.821 [7].

- For the channel-specific parameters for channels such as PUSCH, PDSCH, PUCCH, PDCCH and PRACH, taking the parameter assumptions in Annex <A> of TR 38.830 [3] as baseline. Companies are encouraged to provide the detailed parameters if different channel-specific parameters assumption are assumed in evaluation.

Table 1: Performance requirements for satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | Activity factor | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pedestrian (note 2) | [1] Mbit/s | [100] kbit/s | 1,5 Mbit/s/km2 | 150 kbit/s/km2 | [100]/km2 | [1,5] % | Pedestrian | Handheld |
| Public safety | [3,5] Mbit/ss | [3,5] Mbit/s | TBD | TBD | TBD | N/A | 100 km/h | Handheld |
| Vehicular connectivity (note 3) | 50 Mbit/s | 25 Mbit/s | TBD | TBD | TBD | 50 % | Up to 250 km/h | Vehicle mounted |
| Airplanes connectivity (note 4) | 360 Mbit/s/ plane | 180 Mbit/s/ plane | TBD | TBD | TBD | N/A | Up to 1000 km/h | Airplane mounted |
| Stationary | 50 Mbit/s | 25 Mbit/s | TBD | TBD | TBD | N/A | Stationary | Building mounted |
| Video surveillance (note 4a) | [0,5] Mbit/s | [3] Mbit/s | TBD | TBD | TBD | N/A | Up to 120km/h or stationary (note 4b) | Vehicle mounted or fixed installation |
| Narrowband IoT connectivity | [2] kbit/s | [10] kbit/s | 8 kbit/s/km2 | 40 kbit/s/km2 | [400]/km2 | [1] % | [Up to 100 km/h] | IoT |

Step 2 aims to obtain the link budget results in different scenarios, and identify the bottleneck channels by comparing the link budget results with SNR requirement of different channels.

- The following scenarios listed in Table 2 can be considered for link budget calculation.

- Take into account the realistic antenna setting, the antenna gain is assumed to be -6dBi for smart phone in the link budget calculation.

- The polarization loss need to be considered for link budget calculation assuming UE is linear polarization while the satellite is circular polarization. Based on TR 38.821, for downlink transmission, if the UE is dual-polarized with both horizontal and vertical polarization, a combination of the two Rx branches allows to prevent depolarization. While if the UE is single-polarization antenna, a 3dB depolarization loss need to be considered. For uplink transmission, a 3dB depolarization loss should be taken into account.

- Considering NTN focus on FDD and there is no FDD bands in Ka-band, we assumed the frequency band as S-band.

Table 2: NTN scenarios for link budget calculation

| Case | Satellite orbit | Satellite parameter set | Terminal | Frequency Band |
| --- | --- | --- | --- | --- |
| 1 | GEO | Set 1 | Handheld | S-band |
| 2 | LEO-600 | Set 1 | Handheld | S-band |
| 3 | LEO-1200 | Set 1 | Handheld | S-band |
| 4 | GEO | Set 2 | Handheld | S-band |
| 5 | LEO-600 | Set 2 | Handheld | S-band |
| 6 | LEO-1200 | Set 2 | Handheld | S-band |

Proposal 1: The evaluation methodology listed below is taken as the baseline for the evaluation on the NTN coverage enhancement.

- The basic evaluation methodology is based on link-level simulation and link budget calculation in 2 steps.

- The low-data rate for DL and UL is assumed as 1Mbps and 100kbps respectively. A packet size of 320 bits with 20ms data arriving intervals is assumed for VoIP, companies are encouraged to provide TB size assumed in evaluation if other data rate assumed.

- Use NTN-CDL/NTN-TDL channel models for evaluation. Companies are encouraged to provide detailed parameters such as delay spread of channel model assumed in evaluation

- S-band is assumed for the frequency band

- -6dBi is assumed for antenna gain of smart phone

2.2 Evaluation results

Based on the discussion in section 2.1, we present our initial BLER vs SNR evaluation results and link budget calculation results in this section. The simulation assumptions for step 1 and step 2 can be found in Annex. The BLER vs SNR results and the link budget calculation results are provided in table 3 and table 4 respectively. The channels that have coverage issues are identified by comparing the gap between required SNR and CNR in Table 5. Based on the evaluation results, we have the following observations:

- The UL channel is more challenging than DL channel, the UL coverage is limited in all the scenarios while the DL coverage is limited in GEO Set2 scenario

- The GEO scenario is more challenging than LEO scenario, the average UL gap for GEO is 20 dB for Set 1 and 25 dB for Set 2, while the average UL gap is 5dB for LEO-600km Set1 and 10dB for LEO-1200km Set1.

- For GEO Set2, all the channels have the coverage issues.

Observations:

- The UL channel is more challenging than DL channel, the UL coverage is limited in all the scenarios while the DL coverage is limited in GEO Set2 scenario

- The GEO scenario is more challenging than LEO scenario, the average UL gap for GEO is 20 dB for Set 1 and 25 dB for Set 2, while the average UL gap is 5dB for LEO-600km Set1 and 10dB for LEO-1200km Set1.

- For GEO Set 2, all the channels have the coverage issues.

The solutions to resolve the coverage issues for UL channels are specified in Rel-17 and further enhancements will be done in Rel-18 CE WI. The existing coverage enhancement mechanisms can be set as baseline and further identify whether the UL channel is bottle neck. If identified, NTN-specific UL coverage enhancement. For DL data channel, the gap is around 4 dB and can be compensated by repetition, e.g., aggregation factor set to 8. However, there is no existing mechanisms specified for improving the DL control channel and it is suggested to study the solutions to improve the  PDCCH coverage in Rel-18 NTN. Another aspect is polarization loss, in TN, both the gNB and UE use linear polarization, there is no polarization loss. While in NTN, most of the satellite antenna use circular polarization, while most of the smart phones use linear polarization, there is polarization loss due to polarization mismatch. Therefore, it is suggested to study mechanisms to compensate polarization loss.

Proposal 2: The mechanism to improve the PDCCH coverage can be further studied in Rel-18 NTN.

Proposal 3: The mechanism to compensate the polarization loss can be further studied in Rel-18 NTN.

Table 3: BLER vs SNR results

| Channel | SNR/dB(BLER=0.1 for data channel; BLER=0.01 for control channel) |
| --- | --- |
| PUSCH | -0.2 |
| PUCCH format 3 (4bit) | -4 |
| PUCCH format 3 (11 bit) | 0 |
| PUCCH format 3 (22 bit) | 3.45 |
| PDSCH | -6.5 |
| PDCCH | -6.6 |

Table 4: Link budget calculation results

| Case | Case 1 |  | Case 2 |  | Case 3 |  | Case 4 |  | Case 5 |  | Case 6 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Trans mode | DL | UL | DL | UL | DL | UL | DL | UL | DL | UL | DL | UL |
| Frequency [GHz] | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 |
| TX: EIRP [dBm] | 95.99 | 17.01 | 70.99 | 17.01 | 76.99 | 17.01 | 90.49 | 17.01 | 64.99 | 17.01 | 70.99 | 17.01 |
| RX: G/T [dB/T] | -37.6 | 19.00 | -37.6 | 1.10 | -37.6 | 1.10 | -37.6 | 14.00 | -37.6 | -4.90 | -37.6 | -4.90 |
| Bandwidth [MHz] | 5.00 | 0.54 | 5.00 | 0.54 | 5.00 | 0.54 | 5.00 | 0.54 | 5.00 | 0.54 | 5.00 | 0.54 |
| Free space path loss [dB] | 190 | 190 | 156 | 156 | 162 | 162 | 190 | 190 | 156 | 156 | 162 | 162 |
| Atmospheric loss [dB] | 0.20 | 0.20 | 0.10 | 0.10 | 0.10 | 0.10 | 0.20 | 0.20 | 0.10 | 0.10 | 0.10 | 0.10 |
| Shadow fading margin [dB] | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 |
| Scintillation Loss [dB] | 2.20 | 2.20 | 2.20 | 2.20 | 2.20 | 2.20 | 2.20 | 2.20 | 2.20 | 2.20 | 2.20 | 2.20 |
| Polarization loss [dB] | 0.00 | 3.00 | 0.00 | 3.00 | 0.00 | 3.00 | 0.00 | 3.00 | 0.00 | 3.00 | 0.00 | 3.00 |
| Additional losses [dB] | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| CNR [dB] | -5.28 | -20.9 | 3.58 | -5.01 | 3.76 | -10.8 | -10.8 | -26 | -2.42 | -11.01 | -2.24 | -16.83 |

Table 5: Link budget calculation results

|  | Gap = Required SNR-CNR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channels required SNR | case1 | case 3 | case 5 | case 7 | case 9 | case 11 |
| PUSCH = -0.2 | 20.77 | 4.81 | 10.63 | 25.77 | 10.81 | 16.63 |
| PUCCH 4 bit = -4 | 16.97 | 1.01 | 6.83 | 21.97 | 7.01 | 12.83 |
| PUCCH 11 bit = 0 | 20.97 | 5.01 | 10.83 | 25.97 | 11.01 | 16.83 |
| PUCCH 22 bit = 3.45 | 24.42 | 8.46 | 14.28 | 29.42 | 14.46 | 20.28 |
| PDSCH = -6.5 | -1.22 | -10.0 | -10.26 | 4.28 | -4.08 | -4.26 |
| PDCCH = -6.6 | -1.32 | -10.2 | -10.36 | 4.18 | -4.18 | -4.36 |

- Conclusions

In this contribution, we present the discussion on the evaluation methodology for the NTN coverage performance and provide some initial results on the coverage performance for DL/UL channels. Based on our evaluation and analysis, we have the following observations and proposals.

Observation 1:

- The UL channel is more challenging than DL channel, the UL coverage is limited in all the scenarios while the DL coverage is limited in GEO Set2 scenario

- The GEO scenario is more challenging than LEO scenario, the average UL gap for GEO is 20 dB for Set 1 and 25 dB for Set 2, while the average UL gap is 5dB for LEO-600km Set1 and 10dB for LEO-1200km Set1.

- For GEO Set 2, all the channels have the coverage issues.

Proposal 1: The evaluation methodology listed below is taken as the baseline for the evaluation on the NTN coverage enhancement.

- The basic evaluation methodology is based on link-level simulation and link budget calculation in 2 steps.

- The low-data rate for DL and UL is assumed as 1Mbps and 100kbps respectively. A packet size of 320 bits with 20ms data arriving intervals is assumed for VoIP, companies are encouraged to provide TB size assumed in evaluation if other data rate assumed.

- Use NTN-CDL/NTN-TDL channel models for evaluation. Companies are encouraged to provide detailed parameters such as delay spread of channel model assumed in evaluation

- S-band is assumed for the frequency band

- -6dBi is assumed for antenna gain of smart phone

Proposal 2: The mechanism to improve the PDCCH coverage can be further studied in Rel-18 NTN.

Proposal 3: The mechanism to compensate the polarization loss can be further studied in Rel-18 NTN.

Annex

Table 6, General parameters assumption of LLS

| Parameters | Assumptions |  |
| --- | --- | --- |
| Carrier frequency | 2 GHz |  |
| Subcarrier spacing | 15 kHz |  |
| Channel model | NTN-TDL-D for LOS, 200ns, 50 degree |  |
| BW(PRB)/MCS | UL: 100kbps, MCS0, QPSK RB=3, TBS=112 | DL: 1Mbps, MCS0, QPSK RB=30, TBS=1112 |
| System bandwidth | 30MHz |  |
| Satellite antenna configuration | 1Rx/1Tx |  |
| UE antenna configuration | (1, 1, 2) with omni-directional antenna element |  |
| UE Speed | 3km/h |  |

Table 7, Satellite parameters Set 1 (Ref：TR38.821 Table 6.1.1.1-1）

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band | 22 m | 2 m | 2 m |
| Satellite EIRP density | (i.e. 2 GHz) | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter (Note 2) |  | 250 km | 90 km | 50 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band | 22 m | 2 m | 2 m |
| G/T | (i.e. 2 GHz) | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi | 30 dBi |

Table 8, Satellite parameters Set 2 (Ref：TR38.821 Table 6.1.1.1-2）

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band | 12 m | 1 m | 1 m |
| Satellite EIRP density | (i.e. 2 GHz) | 53.5 dBW/MHz | 34 dBW/MHz | 28 dBW/MHz |
| Satellite Tx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi |
| 3dB beamwidth |  | 0.7353 deg | 8.8320 deg | 8.8320 deg |
| Satellite beam diameter (Note 2) |  | 450 km | 190 km | 90 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band | 12 m | 1 m | 1 m |
| G/T | (i.e. 2 GHz) | 14 dB K-1 | -4.9 dB K-1 | -4.9 dB K-1 |
| Satellite Rx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi |

Table 9, Satellite parameters Set 2 (Ref：TR38.821 Table 6.1.1.1-3）

| Characteristics | Handheld |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | (1, 1, 2) with omni-directional antenna element |
| Polarization | Linear: +/-45°X-p ol |
| Rx Antenna gain | -6dBi |
| Antenna temperature | 290 K |
| Noise figure | 7 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | -6dBi |

Reference

[1] RAN#94e Meeting Reports

[2] 3GPP TSG RAN Meeting #95e, March 17 - 23, 2022, RP-220953, NR NTN (Non-Terrestrial Networks) enhancements, Thales

[3] 3GPP TR 38.830 V17.0.0 (2020-12)

[4] 3GPP TS 22.261 V18.6.0 (2022-03)

[5] 3GPP TR 38.830 V17.0.0 (2020-12)

[6] 3GPP TR 38.811 V15.4.0 (2020-09)

[7] 3GPP TR 38.821 V16.1.0 (2021-05)