3GPP TSG RAN WG1 #109-e			R1-2203806

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.12.3

Source:		Xiaomi

Title:	Discussion on the improved GNSS operation for IoT NTN

Document for:	Discussion

Introduction

In RAN#94e meeting, a new WID on IoT NTN enhancements was approved in [1] with the following RAN1 related objectives:

The work item aims to specify further enhancements for E-UTRA (LTE-RAN) based NTN (non-terrestrial networks) according to the following assumptions:

-	GEO and NGSO (LEO and MEO).

-	Earth fixed Tracking area. Earth fixed & Earth moving cells for NGSO

-	FDD mode

-	UEs with GNSS capabilities

The detailed objectives are to specify enhanced NB-IoT NTN and eMTC NTN radio interfaces and E-UTRAN/NG-RAN as follows:

4.1.1	IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17

This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below:

-	Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2]

-	Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption [RAN1]

In this contribution, we present the discussion on the improved GNSS operation in IoT NTN scenario.

Discussion

In Rel-17 IoT NTN design, 	GNSS capability in the UE is taken as a working assumption for both NB-IoT and eMTC devices. With this assumption, UE can estimate and pre-compensate timing and frequency offset with sufficient accuracy for UL transmission. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed.

Only sporadic short transmission is supported in Rel-17. For sporadic short transmission, UE in RRC_CONNECTED should go back to idle mode and re-acquire a GNSS position fix if GNSS becomes outdated which means UE is not required to perform GNSS measurement during the connected mode. However, this would bring significant signal overhead and delay due to the GNSS measurement to re-acquire a GNSS position fix. A straightforward solution is to keep the IoT device stays in the connected mode when re-acquiring GNSS position fix. However, this may impose additional cost increase for some IoT device. The IoT device is cost-sensitive typically. Whether the GNSS module and cellular module can work simultaneously may dependent on the scenario, and thus it is suggested to report the capability that indicating whether simultaneously GNSS and cellular operation is supported or not. The eNB can make proper decision based on UE’s reporting.

Proposal 1: The IoT UE reports the capability that indicating whether simultaneously GNSS and cellular operation is supported or not

For the IoT device that is capable of simultaneous GNSS and cellular operation, the measurement gap can be configured. Upon the expire of the GNSS position fix, the UE can re-acquire the GNSS position fix during the configured measurement window.

Proposal 2: The IoT UE that is capable of simultaneously GNSS and cellular operation can be configured with measurement gap to perform GNSS measurement.

Another issue related to the long transmission is that assistant information acquisition. In Rel-17 design, the assistant information including common TA parameters and satellite ephemeris can be carried by SIBx and can be updated by dedicated signalling if needed in connected mode as IoT UE is not required to acquire SIB information in legacy design. In Rel-18 if the long transmission is supported, the assistant information may be updated. Much signalling overhead is expected if the assistant information is updated by RRC dedicated signalling. Thus, it is proposed to support the SIB reading during the connected mode.

Proposal 3: The IoT UE can acquire the assistant information from SIBx in connected mode.

Conclusions

In this contribution, we discuss the GNSS operation and SIB reading related to support long transmission. Based on our analysis, we have the following proposals:

Proposal 1: The IoT UE reports the capability that indicating whether simultaneously GNSS and cellular operation is supported or not

Proposal 2: The IoT UE that is capable of simultaneously GNSS and cellular operation can be configured with measurement gap to perform GNSS measurement.

Proposal 3: The IoT UE can acquire the assistant information from SIBx in connected mode.

References

- 3GPP RAN1 108-e chairman-notes, 3GPP TSG RAN WG1 #108-e meeting, Mar.2022.

- 3GPP TS 36.331, Radio Resource Control (RRC); Protocol specification, V16.7.0 (2021-12)