3GPP TSG RAN WG1 #109-e                                       	       R1-2203810

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.2.3.1

Source:	Xiaomi

Title:                     Evaluation on AI/ML for beam management

Document for:	Discussion and Decision

Introduction

In RAN-94 e-meeting, the SI of Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1] was approved. The objective can be seen as below:

Use cases to focus on:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project.

AI/ML model, terminology and description to identify common and specific characteristics for framework investigations:

- Characterize the defining stages of AI/ML related algorithms and associated complexity:

  - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable

  - Inference operation, e.g., input/output, pre-/post-process, as applicable

- Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g.,

  - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes]

  - Various levels of UE/gNB collaboration targeting at separate or joint ML operation.

- Characterize lifecycle management of AI/ML model: e.g.,  model training, model deployment , model inference, model monitoring, model updating

- Dataset(s) for training, validation, testing, and inference

- Identify common notation and terminology for AI/ML related functions, procedures and interfaces

- Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate

For the use cases under consideration:

- Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set:

  - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations.

    - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed.

    - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study.

    - Need for common assumptions in dataset construction for training, validation and test for the selected use cases.

    - Consider adequate model training strategy, collaboration levels and associated implications

    - Consider agreed-upon base AI model(s) for calibration

    - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes

  - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases.

    - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline

    - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered.

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

  - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1

    - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference),  and management of data and AI/ML model, per RAN1 input

    - Collaboration level specific specification impact per use case

  - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2

    - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable

    - Consider the need and implications for AI/ML processing capabilities definition

Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved.

Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

This contribution provides our evaluation assumptions and results of AI/ML for beam management.

Motivation

According to 38.802 [2], the following DL L1/L2 beam management procedures are supported within one or multiple TRPs:

-	P-1: is used to enable UE measurement on different TRP Tx beams to support selection of TRP Tx beams/UE Rx beam(s)

-	For beamforming at TRP, it typically includes a intra/inter-TRP Tx beam sweep from a set of different beams. For beamforming at UE, it typically includes a UE Rx beam sweep from a set of different beams.

-	P-2: is used to enable UE measurement on different TRP Tx beams to possibly change inter/intra-TRP Tx beam(s)

-	From a possibly smaller set of beams for beam refinement than in P-1. Note that P-2 can be a special case of P-1.

-	P-3: is used to enable UE measurement on the same TRP Tx beam to change UE Rx beam in the case UE uses beamforming

During P-1, there will be a large number of TRP Tx beam sweeping and a large number of UE Rx beam sweeping. And the number of TxRx beam pair will be M*N, where M is the number of TRP Tx beam and N is the number of UE Rx beam. It means UE need to measure M*N beam pairs at most to find the best TRP Tx beam and the best UE Rx beam. It will introduce high reference signaling overhead, high UE measurement complexity and long measurement latency. In order to solve this problem, a new SI on AI/ML for NR Air Interface was approved. And in this contribution, we will discuss the AI/ML for beam management.

AI/ML for beam management

Use case

In the SID, the initial set of use cases for beam management includes beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement. While in this contribution, we will focus on the beam prediction in spatial domain for overhead, complexity and latency reduction.

The principle can be seen in Figure 1, there are total M*N beam pairs. Based on an AI model, only the measured quality of a few beam pairs will be input into the AI model, and the AI model will output the quality of the total M*N beam pairs.

Figure 1, Principle of AI based beam prediction in spatial domain

Evaluation methodologies

KPI

For beam management, the purpose is to obtain the Tx beam ID and the L1-RSRP of the best N Tx beams, which can be obtained from the output of the AI model. Thus the KPI should include the first one that whether the predicted Tx beam ID is right or not. The predicted Tx beam ID is right means that it is same as the real Tx beam ID with best L1-RSRP. In addition, the KPI should include the second one that the L1-RSRP gap between the real L1-RSRP of predicted best Tx beam ID and the real L1-RSRP of the real best Tx beam ID should be small. In this way, UE can report the best N Tx beam ID and the corresponding predicted L1-RSRP for beam selection at gNB side. Thus we suggest the following two KPIs for AI based beam management:

- The probability of right best Tx beam predicted by AI;

- The L1-RSRP gap between the predicted best Tx beam and the real best Tx beam.

AI/ML model

As noted in the section of KPI, the purpose is to obtain the Tx beam ID and the L1-RSRP of the best N Tx beams. Thus the output of the AI model should include the Tx beam ID and the L1-RSRP of the best N Tx beams. In order to realize it, we use the AI model based on Fully Connected Neural Network with 5 layer in Figure 2 and the parameters can be seen in Table 1. Here B is the number of input beam pairs, the L1-RSRP or L1-RSRP+beam ID will be input. And 256 is the number of total beam pairs with 32 TRP Tx beam and 8 UE Rx beam. The L1-RSRP of 256 beam pairs will be output with the ascending order of beam ID.

As for the data set, we use about 50000 samples for model training (95%) and testing. And the data are obtained from system level simulation in urban macro. The number of input beam pairs’ RSRP is 16, the number of output beam pairs’ RSRP is 256. The model is trained based on the MSE as loss function.

Figure 2, AI network

Table 1, Parameters of AI model

| Layer | Output shape | Parameters |
| --- | --- | --- |
| Input layer | (None, B) / (None, 3B) | 0 |
| Dense_1(hidden unit=512) | (None, 512) | 4608 / 16896 12800 / 49664 |
| Activation(ReLU) | (None, 512) |  |
| Dense_2(hidden unit=512) | (None, 512) | 262656 |
| Activation(ReLU) | (None, 512) |  |
| Dense_3(hidden unit=256) | (None, 256) | 131328 |
| Activation(ReLU) | (None, 256) |  |
| Dense_4(hidden unit=256) | (None, 256) | 65792 |
| Activation(ReLU) | (None, 256) |  |
| Dense_5(hidden unit=256) | (None, 256) | 65792 |

Simulation assumption and parameters

This section provides the simulation assumptions and parameters in Figure 3 and Table 2. Based on these assumptions and parameters, we evaluate 4 schemes listed below:

- Scheme 0: Non AI scheme

  - Select 16 beam pairs randomly and choose the Tx beam with the highest L1-RSRP as the best Tx beam.

- Scheme 1: Random selection of beam pairs for L1-RSRP input;

  - Select 16 beam pairs randomly and input their L1-RSRP to AI model for beam prediction.

- Scheme 2: Random selection of beam pairs for L1-RSRP and beam ID input;

  - Select 16 beam pairs randomly and input their L1-RSRP and beam ID to AI model for beam prediction.

- Scheme 3: Always select 16 beam pairs with same beam pair IDs for L1-RSRP input;

  - Select 16 beam pairs with same beam pair IDs and input their L1-RSRP to AI model for beam prediction.

Figure 3, Network deployment

Table 2, Simulation parameters

| Parameters | Value |
| --- | --- |
| Scenarios (carrier frequency) | Urban Macro, 30 GHz |
| Number of cells | 7 cell with 3 sectors per cell |
| UE number/ per sector | 40 |
| Bandwidth | 80 MHz |
| SCS | 60 KHz |
| Mode | SU-MIMO |
| BS antenna configurations |  |
| BS Tx beam pattern |  |
| BS Tx beam number | 32 |
| UE antenna configurations |  |
| UE Rx beam pattern |  |
| UE Rx beam number | 8 |
| Beam measurement quality | L1-RSRP |

Simulation results

Based on the simulation assumptions and parameters listed above, we obtained two KPIs of these three schemes.

- KPI#1: The probability of right best Tx beam predicted by AI;

- KPI#2: The L1-RSRP gap between the predicted best Tx beam and the real best Tx beam.

In order to align with the beam measurement report, we consider two cases. In case 1, the assumption is that 2 best Tx beams are reported. And if any one best Tx beam is the real best Tx beam, it is considered as the best Tx beam is predicted correctly. And the L1-RSRP gap is the minimum gap of 2 gaps. In case 2, the assumption is that 4 best Tx beams are reported. And if any one best Tx beam is the real best Tx beam, it is considered as the best Tx beam is predicted correctly. And the L1-RSRP gap is the minimum gap of 4 gaps.

The KPIs of case 1 and case 2 can be seen in Table 3 and Table 4 respectively. And the cdf of L1-RSRP of case 1 and case can be seen in Figure 4 and Figure 5 respectively. From the simulation results, we can see that AI based beam prediction provide large performance gain compared to non-AI based beam management scheme. In addition, we can also find that scheme 2 and scheme 3 can further improve the performance by input beam ID or by input L1-RSRP of same beam pairs. Thus we propose an observation and proposal as below.

Table 3, Probability and L1-RSRP gap for case 1

|  | KPI#1: Probability | KPI#2: L1-RSRP gap | gain |
| --- | --- | --- | --- |
| Scheme 0 | 6.15% | 3.9539 | - |
| Scheme 1 | 48.06% | 1.6723 | 681.46% / 57.71% |
| Scheme 2 | 60.08% | 1.1233 | 876.91% / 71.59% |
| Scheme 3 | 80.60% | 0.4730 | 1210.57% / 88.04% |

Table 4, Probability and L1-RSRP gap for case 2

|  | KPI#1: Probability | KPI#2: L1-RSRP gap | gain |
| --- | --- | --- | --- |
| Scheme 0 | 6.15% | 3.9539 | - |
| Scheme 1 | 67.26% | 1.0062 | 993.66% / 74.55% |
| Scheme 2 | 80.04% | 0.7978 | 1201.46% / 79.82% |
| Scheme 3 | 90.60% | 0.3385 | 1373.17% / 91.44% |

Figure 4, CDF of L1-RSRP for best Tx beam in case 1

Figure 5, CDF of L1-RSRP for best Tx beam in case 2

Observation 1: AI based beam prediction in spatial domain can provide good performance. And the performance can be further improved by inputting corresponding beam pair ID in addition to measured L1-RSRP or by inputting L1-RSRP of same beam pair IDs.

Proposal 1: Adopt the evaluation methodologies listed below:

- Scenario: Urban Macro

- Data set: separate data set for each scenario

- AI model:

  - Input:

    - Scheme 1: L1-RSRP of beam pairs selected randomly

    - Scheme 2: L1-RSRP of beam pairs selected randomly and corresponding beam pair IDs

    - Scheme 3: L1-RSRP of beam pairs with same beam pair IDs

  - Output

    - L1-RSRP of all beam pairs with ascending order of beam pair ID

- KPI:

  - KPI#1: The probability of right best Tx beam predicted by AI.

  - KPI#2: The L1-RSRP gap between the predicted best Tx beam and the real best Tx beam.

Conclusion

In this contribution, we provides our evaluation assumptions and results of AI/ML for beam management. Based on above analysis, we provide the following observations and proposals.

Observation 1: AI based beam prediction in spatial domain can provide good performance. And the performance can be further improved by inputting corresponding beam pair ID in addition to measured L1-RSRP or by inputting L1-RSRP of same beam pair IDs.

Proposal 1: Adopt the evaluation methodologies listed below:

- Scenario: Urban Macro

- Data set: separate data set for each scenario

- AI model:

  - Input:

    - Scheme 1: L1-RSRP of beam pairs selected randomly

    - Scheme 2: L1-RSRP of beam pairs selected randomly and corresponding beam pair IDs

    - Scheme 3: L1-RSRP of beam pairs with same beam pair IDs

  - Output

    - L1-RSRP of all beam pairs with ascending order of beam pair ID

- KPI:

  - KPI#1: The probability of right best Tx beam predicted by AI.

  - KPI#2: The L1-RSRP gap between the predicted best Tx beam and the real best Tx beam.

References

- RP-213599 New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- TR 38.802 V14.2.0