3GPP TSG RAN WG1 #109-e			R1-2203814

e-Meeting, May 9th – 20th, 2022

Source: 	     Xiaomi

Title:		     Discussion on evaluation on NR duplex

Agenda item:    9.3.1

Document for:  Discussion

Introduction

In RAN#94 e-meeting, new SID on evolution of NR duplex operation was approved. One of the objectives is to study the evaluation on NR duplex, including study the deployment scenarios and evaluation methodology. The detailed objective is excerpted as below: [1]

The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In this contribution, we provide our views on evaluation on NR duplex evolution.

Discussion

Deployment scenarios for duplex enhancement

Different from legacy TDD operation, Rel-18 duplex gNB can transmit and receive on the same OFDM symbols. Although the DL and UL on the same OFDM symbols are FDMed, the interference between transmitter and receiver is much more complex. One example is shown in Figure 1.

Figure 1: Example on the interference types in duplex scenario

Overall, self-interference, gNB-to-gNB CLI and UE-to-UE are introduced by duplex operation on the same OFDM symbols. Especially, gNB-to-gNB cross link interference and self-interference need more attention due to the following reasons:

- gNB has much higher transmission power than UE

- Typically there is light of sight propagation among gNBs

- Self-interference is the interference from Tx to Rx belonging to the same gNB

There are two directions to alleviate the impacts on the victim channels from the above interference, i.e. gNB transmission power in the applicable scenarios should be as low as possible and/or good isolation among cells is available.  In Rel-16 CLI and RIM, urban macro, dense urban and indoor hotspot were selected for evaluation. For subband-based duplex, the same scenarios can be considered as the starting point of applicable and relevant deployment scenarios.

Figure2: Illustration of urban Macro, Dense urban and Indoor/hotspot

Proposal 1: For subband non-overlapping full duplex, performance evaluation should be done at least in the indoor/hotspot, dense urban and macro urban scenarios.

Duplex evolution is a long term evolution. It has high possibility that legacy gNB and duplex gNB co-exists in the same network.  It would introduce new interference type to legacy UE/gNB. On the other hand, the interference to duplex UE from full DL slot is also different from that of a duplex slot.  Therefore, we should consider the co-existence between legacy gNB and duplex gNB.

Figure 3: Interference between legacy gNB and duplex gNB

Proposal 2: For evaluation of subband non-overlapping full duplex, co-existence between legacy gNB and duplex gNB in the same network should be considered.

In Rel-16 CLI, we study the CLI introduced by dynamic TDD, wherein the cross link interference comes from adjacent channel. In Rel-18 duplex operation, we need to further exploit the possibility and the potential enhancement of dynamic TDD in co-channel case. After comprehensive evaluation, it was observed that even if the aggressor gNB and victim gNB are located on adjacent channel, it is still not recommended to apply dynamic TDD because of the performance degradation due to severe gNB-to-gNB interference. [2] The recommendation from previous work should be taken into account. For co-channel dynamic TDD, the gNB-to-gNB interference would be much more serious as it comes from the same channel.  Based on the knowledge we learned from Rel-16 CLI, scenarios with single layout should not be considered for co-channel dynamic TDD evaluation, unless there is sufficient isolation among gNBs. Accordingly, HetNet scenario with both macro layout and indoor/hotspot layout can be considered. For macro layout, semi-static TDD UL-DL configuration can be assumed as the statistical UL DL traffic ratio is perceived. For indoor/hotspot layout, dynamic TDD can be considered to improve the performance, e.g. DL or UL throughput, as the UL/DL traffic load is more fluctuating. Considering there are walls between macro layout and indoor/hotspot layout, the cross link interference between these two layouts may be acceptable. Furthermore, the low transmission power of indoor/hotspot TRP is friendly for the application of dynamic TDD on indoor/hotspot layout.

Figure 4: Illustration of HetNet consist of urban Macro layout and indoor/hotspot layout

Proposal 3: For flexible/dynamic TDD, evaluate and study the performance in HetNet scenario.

- 6.3.1 FR1 - 6.3.1.1 Macro-to-Macro scenario - Performance degradation was observed from the BS-to-BS interference for macro-macro scenario, which suggests that dynamic TDD should not be operated in such scenarios. - 6.3.1.2 Indoor scenarios (Indoor-to-Macro and Indoor-to-Indoor) - Performance degradations were not observed from operating dynamic TDD between an indoor network and a macro network and vice versa if there is sufficient isolation between them. No significant impact from operating dynamic TDD for the indoor scenario was observed as long as the BS and UE powers are similar and the operators co-ordinate so that base station positions are offset. If higher BS power is assumed, some throughput degradation in the indoor scenario was observed due to BS to BS interference. The observations imply that dynamic TDD can be used in indoors as long as care is taken. - 6.3.2 FR2 - 6.3.2.1 Macro-to-Macro scenario - Some performance degradation was observed from the BS-to-BS interference for macro-macro scenario. The differences in the simulation results imply that operating dynamic TDD in this scenario without impact to neighbor network may be deployment dependent and requires at least careful planning and collaboration between operators to avoid performance impact. - 6.3.2.2 Indoor scenarios (Indoor-to-Macro and Indoor-to-Indoor) - Performance degradations were not observed from operating dynamic TDD between an indoor network and a macro network if there is sufficient isolation between them. Results suggested that to avoid degradation, careful layout and parameterization are necessary for indoor to indoor scenario. Overall, the observations imply that dynamic TDD can be used indoors as long as care is taken. - 6.3.2.3 Micro-to-Micro scenario - For micro to micro, the differences in the simulation results imply that to avoid BS to BS interference, operators may need to consider the proximity of micro BS in the same area. Overall, the observations imply that dynamic TDD can be used in certain micro deployments as long as care is taken.

Simulation cases for SBFD

It is well known that the TDD UL-DL configuration can be changed in a semi-static manner or in a dynamic manner.  Duplex operation can be regarded as an improvement on the performance of UL transmission on top of current TDD operation. Furthermore, duplex operation on the same OFDM symbols introduces self-interference and cross link interference compared to aligned TDD configuration. Hence it is straightforward to take aligned TDD configuration without CLI as the baseline. Regarding to the subband configuration across gNBs, it can be same or different, which leads to different set of interference.  For example, if aligned UL subband configuration is applied to gNBs, we only need to consider inter-subband CLI. However, if different UL subbands can be configured semi-statically or indicated dynamically, the interference status are further complicated. As analysed above, we need to simulate all the interference type, i.e. 1)-5). Additionally, the motivation of unaligned UL subband, especially for dynamic UL subband indication, is not justified yet. It may be better to evaluate this case after such mechanism is identified and justified in section 9.3.2.

Observation: Scenarios wherein different UL subband are configured across gNBs can be deferred until we have progress in section 9.3.2.

Figure 5: Illustration on subband configuration among cells

As mentioned above, the interference situation is much more complicated due to the introduction of duplex operation. From UE perspective, there is only UL transmission or DL reception on an OFDM symbol. However, from gNB perspective, it not only schedules duplex UE on the UL subband, but also schedules legacy UE on the DL subband. To be specific, the inference suffered by legacy UE and duplex UE can be summarized as below, wherein assuming aligned UL subband is configured across gNBs:

- Uplink direction: Self-interference, Inter-subband CLI, UE-to-gNB interference

- Downlink direction: Inter-subband CLI, gNB-to-UE interference

Based on the above analyses, we propose to at least simulate the following cases:

- Indoor hotspot(UL, DL)

  - Baseline: DL dominated TDD w/o CLI

  - Duplex case: common UL subband configuration

- Dense urban(UL, DL)

  - Baseline: DL dominated TDD w/o CLI

  - Duplex case: common UL subband configuration

- Urban Macro(UL, DL)

  - Baseline: DL dominated TDD w/o CLI

  - Duplex case: common UL subband configuration

- Co-channel co-existence of duplex gNB and legacy gNB(UL, DL)

  - Baseline: DL dominated TDD w/o CLI

  - Duplex case: common UL subband configuration + legacy TDD

Proposal 4: For evaluation of SBFD, the following two cases should be considered for each identified scenarios to evaluate the uplink performance:

- Indoor hotspot

  - Baseline: DL dominated TDD w/o CLI

  - Duplex case: common UL subband configuration

- Dense urban

  - Baseline: DL dominated TDD w/o CLI

  - Duplex case: common UL subband configuration

- Urban Macro

  - Baseline: DL dominated TDD w/o CLI

  - Duplex case: common UL subband configuration

- Co-channel co-existence of duplex gNB and legacy gNB

  - Baseline: DL dominated TDD w/o CLI

  - Duplex case: common UL subband configuration + legacy TDD

Proposal 5: For evaluation of SBFD, the following two cases should be considered for each identified scenarios to evaluate the downlink performance:

- Indoor hotspot

  - Baseline: DL dominated TDD w/o CLI

  - Duplex case: common UL subband configuration

- Dense urban

  - Baseline: DL dominated TDD w/o CLI

  - Duplex case: common UL subband configuration

- Urban Macro

  - Baseline: DL dominated TDD w/o CLI

  - Duplex case: common UL subband configuration

- Co-channel co-existence of duplex gNB and legacy gNB

  - Baseline: DL dominated TDD w/o CLI

  - Duplex case: common UL subband configuration + legacy TDD

Simulation metric for SBFD and dynamic TDD

As mentioned in the justification part in the SID of Rel-18 duplex enhancement, the motivation of duplex evolution is to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operations in unpaired spectrum.

Generally speaking, the UL coverage, reduced latency and improved system capacity can bring high UPT. In order to evaluate the benefits for latency, the delay between packet arrival and transmission can be also simulated.

Besides, the introduction of UL subband in DL slot would impacts the performance of downlink transmission. Accordingly, the DL UPT should also be simulated.

Proposal 6: At least the following metrics should be considered for performance evaluation:

- UL/DL UPT

- Latency of uplink packet transmission

Interference modelling for SBFD

Generally speaking, the interference type can be summarized as below:

- 1) Self-interference. It is the interference from transmitter to receiver of the same gNB within the same channel bandwidth.

- 2) gNB-to-gNB cross link interference. It is the interference from neighbour cell’s downlink transmission to victim cell’s uplink reception. More specifically, there are two potential gNB-to-gNB CLI depending on the deployment or configuration:

  - 2-1) Inter-subband CLI

  - 2-2) Intra-subband CLI

- 3) UE-to-UE cross link interference. It is the interference from aggressor UE’s uplink transmission to victim UE’s downlink reception. More specifically, there are two potential gNB-to-gNB CLI depending on the deployment or configuration:

  - 3-1) Inter-subband CLI

  - 3-2) Intra-subband CLI

- 4) gNB-to-UE interference. It is the interference from aggressor cell’s downlink transmission to victim UE’s downlink reception.

- 5) UE-to-gNB interference. It is the interference from aggressor UE’s uplink transmission to victim gNB’s uplink reception.

As mentioned in the previous sections, the following scenarios can be the starting point for SBFD simulation wherein the interference type was also summarized:

- Urban Macro:

For DL reception, the interference including 3-1) and 4).

For UL transmission, the interference including 1), 2-1) and 5).

Figure 6: Illustration on interference for Urban Macro scenario with aligned UL subband configuration

- Dense Urban:

For DL reception, the interference including 3-1) and 4).

For UL transmission, the interference including 1), 2-1) and 5).

Figure 7: Illustration on interference for Dense Urban scenario with aligned UL subband configuration

- Indoor hotspot

For DL reception, the interference including 3-1) and 4).

For UL transmission, the interference including 1), 2-1) and 5).

Figure 8: Illustration on interference for Indoor hotspot scenario with aligned UL subband configuration

- Urban Macro with duplex gNB and legacy gNB co-existence

For DL reception, the interference including 3-1), 3-2) and 4).

For UL transmission, the interference including 1), 2-1), 2-2) and 5).

Figure 8: Illustration on interference legacy gNB and duplex gNB co-existence scenario

For cross link interference, the first step should determine the channel model.  For gNB-to-gNB and gNB-to-UE, the channel model in TR38.901 could be used as the starting point, which is defined for NR physical evaluation. For UE-to-UE channel model, it is absent in TR38.901. There are several sources providing different UE-to-UE channel modelling, which are summarized in the following table.

Table 1: Comparison on UE-to-UE channel modelling defined in TR38.802 and TR38.828

| Specification number | TR38.802 | TR38.828 |
| --- | --- | --- |
| Comments | It defines channel model for physical-layer aspects of new radio access technologies. Also, precise UE-to-UE channel model is defined. | It defines channel models for studying the RF and co-existence aspects for cross link interference. The UE-to-UE model reuse TR36.828, which is defined for eIMTA |

Considering TR38.802 defines channel model specially for new radio access technology and the channel is more precise than that defined in TR38.828, we slightly prefer to use the channel model captured in TR38.802 for UE-to-UE link.

Proposal 7: Study and determine the channel model for different links. The following assumption can be the starting point:

- gNB-to-gNB, reuse the channel model defined in TR38.901

- gNB-to-UE, reuse the channel model defined in TR38.901

- UE-to-UE, reuse the channel model defined in TR38.802

As mentioned in the start of the section, there are two kinds of CLI when UL subband is configured in a DL slot, i.e. intra-subband CLI and inter-subband CLI. The former one is same as co-channel CLI which can be simulated without considering ACIR. For legacy adjacent channel, the ACLR and ACS is defined as below respectively:

- ACLR: the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent NR channel frequency at nominal channel spacing.

- ACS: the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

However, the DL subband and UL subband are within the same channel bandwidth. In the other words, the current ACIR values defined for modeling the separation between adjacent channels may be not suitable.  RAN4 should be involved as early as possible and provide the proper values. Hence we have the following proposal:

Proposal 8: Send an LS to RAN4 to check the views on the following two issue:

- How to model the inter-subband interference from a DL subband to a UL subband?

- How to model the inter-subband interference from a UL subband to a UL subband?

Self-interference plays an important role for evaluation on NR duplex evolution. It is the interference from a Tx chain to a Rx chain belong to the same gNB. Generally speaking, self-interference can be modeled as below:

Iself=PTx+ Gantenna-Sup

Wherein PTx is the transmit power, Gantenna is the antenna gain and Sup is the capability of suppressing self-interference. The self-interference suppression capability can be considered at least from perspective of spatial isolation, RF isolation, frequency isolation and digital filtering. One way is up to RAN4 to provide an overall value or value sets for Sup.

Proposal 9: Self-interference can be modeled by the following formula and the capability on self-interference suppression is up to RAN4.

Iself=PTx+ Gantenna-Sup

Conclusion

In this contribution, we provide our views on evaluation on NR duplex evolution. We have the following observation:

Observation: Scenarios wherein different UL subband are configured across gNBs can be deferred until we have progress in section 9.3.2.

Furthermore, we have the following proposals:

Proposal 1: For subband non-overlapping full duplex, performance evaluation should be done at least in the indoor/hotspot, dense urban and macro urban scenarios.

Proposal 2: For evaluation of subband non-overlapping full duplex, co-existence between legacy gNB and duplex gNB in the same network should be considered.

Proposal 3: For flexible/dynamic TDD, evaluate and study the performance in HetNet scenario.

Proposal 4: For evaluation of SBFD, the following two cases should be considered for each identified scenarios to evaluate the uplink performance:

- Indoor hotspot

  - Baseline: DL dominated TDD w/o CLI

  - Duplex case: common UL subband configuration

- Dense urban

  - Baseline: DL dominated TDD w/o CLI

  - Duplex case: common UL subband configuration

- Urban Macro

  - Baseline: DL dominated TDD w/o CLI

  - Duplex case: common UL subband configuration

- Co-channel co-existence of duplex gNB and legacy gNB

  - Baseline: DL dominated TDD w/o CLI

  - Duplex case: common UL subband configuration + legacy TDD

Proposal 5: For evaluation of SBFD, the following two cases should be considered for each identified scenarios to evaluate the downlink performance:

- Indoor hotspot

  - Baseline: DL dominated TDD w/o CLI

  - Duplex case: common UL subband configuration

- Dense urban

  - Baseline: DL dominated TDD w/o CLI

  - Duplex case: common UL subband configuration

- Urban Macro

  - Baseline: DL dominated TDD w/o CLI

  - Duplex case: common UL subband configuration

- Co-channel co-existence of duplex gNB and legacy gNB

  - Baseline: DL dominated TDD w/o CLI

  - Duplex case: common UL subband configuration + legacy TDD

Proposal 6: At least the following metrics should be considered for performance evaluation:

- UL/DL UPT

- Latency of uplink packet transmission

Proposal 7: Study and determine the channel model for different links. The following assumption can be the starting point:

- gNB-to-gNB, reuse the channel model defined in TR38.901

- gNB-to-UE, reuse the channel model defined in TR38.901

- UE-to-UE, reuse the channel model defined in TR38.802

Proposal 8: Send an LS to RAN4 to check the views on the following two issue:

- How to model the inter-subband interference from a DL subband to a UL subband?

- How to model the inter-subband interference from a UL subband to a UL subband?

Proposal 9: Self-interference can be modeled by the following formula and the capability on self-interference suppression is up to RAN4.

Iself=PTx+ Gantenna-Sup

Reference

  - RP-213591, New SI: Study on evolution of NR duplex operation, CMCC

  - TR38.828, Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR, v16.1.0