3GPP TSG RAN WG1 #109-e			R1-2203816

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.3.3

Source:		Xiaomi

Title:	Discussion on potential enhancements on dynamic TDD

Document for:	Discussion

Introduction

In RAN#94e meeting [1], a new SID “Study on evolution of NR duplex operation” was approved in [2] with the following objectives:

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, we present the discussion on the aspects related to the possible enhancements on dynamic/flexible TDD.

Discussion

Scenario

The NR TDD specifications allow the dynamic/flexible allocation of downlink and uplink in time. During the Rel-14 study on flexible duplex [3], considerable system performance gain has been shown to support the flexible TDD based on the evaluation results for indoor hotspot, dense urban and urban macro scenario. In Rel-18, further evaluation on the two-layer layout scenario should be prioritized, where the macro layer operates with semi-static TDD and the small cell layer operates with dynamic TDD.

Proposal 1: Further evaluation on the two-layer layout scenario should be prioritized in Rel-18, where the macro layer operates with semi-static TDD and the small cell layer operates with dynamic TDD.

Potential issues for Rel-18 dynamic/flexible TDD

The main challenge to support the flexible/dynamic TDD is how to handle the cross link interference which is expected to be very serious in some co-channel deployment scenario.

- Inter-gNB CLI                                                                     (b) Inter-UE CLI

The Rel-16 Work Item Cross Link Interference (CLI) handling to support flexible resource adaptation for unpaired NR cells achieves the following objectives:[4]

- CLI measurements and reporting at a UE (i.e., CLI-RSSI and SRS-RSRP), and network coordination mechanism(s) (i.e., exchange of intended DL/UL configuration) are developed.

  - Layer 3 filtering can be applied to the measurement result for both CLI-RSSI measurement and SRS-RSRP measurement.

  - Both event triggered and periodic reporting are supported for CLI-RSSI measurement and SRS-RSRP measurement.

- Perform coexistence study to identify conditions of coexistence among different operators in adjacent channels is accomplished.

However, if the TDD pattern can be adapted dynamically, e.g, with a slot level among interfering cells, the L3 measurement and periodic reporting may not be enough to accommodate the dynamic changes of the interference level. Meanwhile, Rel-16 CLI only specify the solutions on the inter-UE CLI. The inter-gNB CLI is expected to be more severe due to higher transmit power and LoS propagation, which needs to be studied in Rel-18.

Proposal 2: The Rel-18 study on dynamic/flexible TDD should focus on the following aspects:

- Inter-gNB CLI

- Further enhancement on the inter-UE CLI

Candidate solution for Rel-18 dynamic/flexible TDD

CLI measurement

Accurate CLI measurement is necessary to enable CLI handling. CLI measurement and reporting by the UE had been specified in Rel-16 which was a long term measurement. gNB can make proper scheduling decisions based on the long term measurement. However, for the UE-UE CLI measurement, if the TDD pattern from the interfering cells can be dynamically changed, the interference level may be varying frequently. In this sense, dynamic CLI measurement can be studied. UE can competes for the transmission opportunity based on the instantaneous measurement similar to the LBT-like mechanism when operating on unlicensed spectrum. The instantaneous measurement can be either based on energy detection, or signal detection.

Proposal 3: Dynamic UE CLI measurement can be further studied to acquire instantaneous interference level.

For the gNB-gNB CLI measurement, similar to the UE-UE CLI measurement, the gNB measures the interference level from interfering cell’s DL transmission. The interference can be the total received interference from multiple interfering cells. Meanwhile, the interference can also be the received interference from each interfering cell. In order to identify the interfering source, the interference measurement can be based on the common reference signal such as SSB or other reference signals. Network listening based interference measurement can be further studied.

Proposal 4: Network listening based interference measurement can be further studied.

CLI handling

During the study on flexible duplex in Rel-14, many solutions had been proposed to resolve the CLI issues [3]. Generally, the solutions can be divided into three categories:

| IM solutions | Categories | Enabler | Usage |
| --- | --- | --- | --- |
| Interference suppression based solutions | eMMSE-IRC; IS/IC receiver; Reduced complexity maximum likelihood (RML) receiver | - Information exchange on Xn or OTA - RS orthogonality | TRP-TRP UE-UE |
| Coordination based solutions | Transmission adjustment based on the interference measurements | - Long term interference measurement - Adaptation on the transmission parameters including power domain, time or frequency resource, beam domain. | TRP-TRP UE-UE |
| Sensing based solutions | Transmission adjustment based on the channel sensing results | - Short term interference measurement by LBT-like mechanism - Adaptation on the transmission parameters including power domain, time or frequency resource, beam domain. | TRP-TRP UE-UE |

The interference suppression based solution is applied at the receiver side to suppress/cancel interference. Some assistance information related to the interference may be needed at the receiver side to perform advanced receiver.

The coordination based solutions is based on the long term measurement results, the gNB can make proper scheduling decisions including adaptation on the transmission power, time/frequency resource for data transmission or transmission beam to avoid severe interference.

The sensing based solution is more like a dynamic interference avoidance solution. The transmitter can adjust its transmission based on instantaneous measurement results.

Proposal 5: The candidate solutions for CLI handling studied in Rel-14 duplex can be the baseline for further investigation in Rel-18.

Conclusions

In this contribution, we provide the potential scenario and candidate solutions for the dynamic/flexible TDD, the following proposals are made based on our analysis:

Proposal 1: Further evaluation on the two-layer layout scenario should be prioritized in Rel-18, where the macro layer operates with semi-static TDD and the small cell layer operates with dynamic TDD.

Proposal 2: The Rel-18 study on dynamic/flexible TDD should focus on the following aspects:

- Inter-gNB CLI

- Further enhancement on the inter-UE CLI

Proposal 3: Dynamic UE CLI measurement can be further studied to acquire instantaneous interference level.

Proposal 4: Network listening based interference measurement can be further studied.

Proposal 5: The candidate solutions for CLI handling studied in Rel-14 duplex can be the baseline for further investigation in Rel-18.

References

- 3GPP RAN#94e meeting chairman-notes, 3GPP TSG RAN#94-e electronic Meeting, December 6 – 17, 2021.

- RP-213591, New SI: Study on evolution of NR duplex operation, CMCC, 3GPP TSG RAN#94-e electronic Meeting, December 6 – 17, 2021.

- 3GPP TR 38.802, Study on New Radio Access Technology, Physical Layer Aspects (Rel-14)

- RP-201280, Work Item Summary for Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR, LG Electronics, 3GPP TSG RAN meeting #88e Electronic Meeting, June 29 - July 3, 2020