3GPP TSG RAN WG1 #109-e			R1-2203817

e-Meeting, May 9th – 20th, 2022

Source: 	     Xiaomi

Title:		     Other issues on NR duplex evolution

Agenda item:    9.3.4

Document for:  Discussion

Introduction

In RAN#94 e-meeting, new SID on evolution of NR duplex operation was approved. One of the objectives is to study the evaluation on NR duplex, including study the deployment scenarios and evaluation methodology. The detailed objective is excerpted as below: [1]

The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In this contribution, we provide our views on evaluation on NR duplex evolution.

Discussion

Preliminary simulation assumptions for SBFD

As discussed in our companion contribution, we should focus on indoor hotspot, dense urban and urban Macro to evaluate the performance of duplex UE and legacy UE. Generally speaking, the interference type can be summarized as below:

- 1) Self-interference. It is the interference from transmitter to receiver of the same gNB on the same channel bandwidth.

- 2) gNB-to-gNB cross link interference. It is the interference from neighbour cell’s downlink transmission to victim cell’s uplink reception. More specifically, there are two potential gNB-to-gNB CLI depending on the deployment or configuration:

  - 2-1) Inter-subband CLI

  - 2-2) Intra-subband CLI

- 3) UE-to-UE cross link interference. It is the interference from aggressor UE’s uplink transmission to victim UE’s downlink reception. More specifically, there are two potential gNB-to-gNB CLI depending on the deployment or configuration:

  - 3-1) Inter-subband CLI

  - 3-2) Intra-subband CLI

- 4) gNB-to-UE interference. It is the interference from aggressor cell’s downlink transmission to victim UE’s downlink reception.

- 5) UE-to-gNB interference. It is the interference from aggressor UE’s uplink transmission to victim gNB’s uplink reception.

It can be observed the major challenge is cross link interference and self-interference. Hence, it is reasonable to reuse the simulation parameters defined in TR38.828 as much as possible.  For the parameters not captured in TR38.828, the ones defined in TR38.802 could be used as the starting point.

For SBFD, we propose to use the following simulation parameters as the starting point.

Table 1: Simulation parameters for evaluation on SBFD

| Parameters | Indoor hotspot | Dense Urban | Urban Macro |
| --- | --- | --- | --- |
| Layout | Single layer with 6 BSs per 120 m x 50 m | Single layer with 7 hexagonal cell with wrap around | Single layer with 7 hexagonal cell with wrap around |
| Minimum BS-UE (2D) distance | Indoor-to-Indoor: 0 m: TR38.828 | 35 m: TR38.828 |  |
| Minimum UE-UE (2D) distance | Indoor-to-Indoor: 1 m : TR38.828 | 3 m: TR38.828 |  |
| Inter-BS distance | 20 m | 200 m | 500 m |
| Carrier frequency | 4 GHz for FR1, 30 GHz for FR2 |  |  |
| BS Tx power | 24 dBm: TR38.802 | 44dBm: TR38.802 | 49 dBm: TR38.802 |
| UE Tx power | 23 dBm: TR38.802 |  |  |
| Path-loss model | - Indoor (Aggressor) → Indoor (Victim) - BS-to-BS: TR38.901 - BS-to-UE: TR38.901 - UE-to-UE: TR38.802 | -Macro(Aggressor) → Macro(Victim) - BS-to-BS: TR38.901 - BS-to-UE: TR38.901 - UE-to-UE: TR38.802 | -Macro(Aggressor) → Macro(Victim) BS-to-BS: TR38.901 - BS-to-UE: TR38.901 - UE-to-UE: TR38.802 |
| BS antenna configurations | (Mg,Ng,M,N,P)=(1,1,4,4,2) (dH,dV)=(0.5,0.5) λ | (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.5) λ: TR38.828 | (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.5) λ: TR38.828 |
| BS antenna height | 3 m: TR38.802 | 25 m: TR38.802 | 25 m: TR38.802 |
| BS antenna element gain + connector loss | 3.5 dBi: TR38.828 | 5 dBi : TR38.828 | 5 dBi : TR38.828 |
| BS receiver noise figure | 5 dB: TR38.802 | 5 dB: TR38.802 | 5 dB: TR38.802 |
| UE antenna configuration | Omni: TR38.802 | Omni: TR38.802 | Omni: TR38.802 |
| UE antenna height | 1.5 m: TR38.828 | hUT=3(nfl-1)+1.5 nfl for outdoor UEs: 1 nfl for indoor UEs: nfl~uniform(1,Nfl) where Nfl = 1: TR38.802 | hUT=3(nfl-1)+1.5 nfl for outdoor UEs: 1 nfl for indoor UEs: nfl~uniform(1,Nfl) where Nfl = 1: TR38.802 |
| UE antenna gain | 0: TR38.828 | 0 dBi: TR38.828 | 0 dBi: TR38.828 |
| UE receiver noise figure | 9 dB: TR38.802 | 9 dB: TR38.802 | 9 dB: TR38.802 |
| UE power control | Power control as defined in TS38.213 section 7.1, (p0=-100dBm, alpha=0.8) |  |  |
| Cell selection criteria | Cell selection is based on RSRP |  |  |
| Traffic model | FTP model 3 |  |  |

Proposal 1: The simulation parameters included in Table 1 is used for SBFD simulation as the starting point.

Preliminary simulation assumptions for dynamic TDD

As discussed in our companion contribution [1], HetNet scenario should be used for evaluation of co-channel dynamic TDD. We propose to use the following simulation parameters as the starting point.

Table 2: Simulation parameters for evaluation on SBFD

| Parameters | Hetnet |
| --- | --- |
| Layout | Two layer Macro layer: Hex. Grid Micro layer: Random drop -3 micro BSs per macro BS (38.802) |
| Minimum BS-UE (2D) distance | 35 m: TR38.828 |
| Minimum UE-UE (2D) distance | 3 m: TR38.828 |
| Inter-BS distance | Macro-macro: 200 m: TR38.802 |
| Carrier frequency | 4 GHz for FR1, 30 GHz for FR2: TR38.802 |
| BS Tx power | Macro layer (TR 38.802): 4GHz: 44 dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 44 dBm 30GHz: 40 dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 40 dBm Micro layer (TR 38.802): 4 GHz: 33dBm for 20MHz system bandwidth Above 6GHz: 33 dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 33 dBm. |
| UE Tx power | 23 dBm: TR38.802 |
| Channel mode | TR 38.802 |
| TDD configuration | Macro layer: DDDSU Micro layer: DSUUU/Flexible TDD |
| Traffic model | FTP model 3 |

Proposal 2: The simulation parameters included in Table 2 is used for co-channel dynamic TDD simulation as the starting point.

Conclusion

In this contribution, we provide our views on the preliminary simulation assumptions for SBFD and dynamic TDD evaluation. We have the following proposals:

Proposal 1: The simulation parameters included in Table 1 is used for SBFD simulation as the starting point.

Proposal 2: The simulation parameters included in Table 2 is used for co-channel dynamic TDD simulation as the starting point.

Reference

  - R1-2203814, Discussion on evaluation on NR duplex evolution, Xiaomi