3GPP TSG RAN WG1 #109-e                                       	       R1-2203824

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.5.2.2

Source:	Xiaomi

Title:                     Improved accuracy based on NR carrier phase measurement

Document for:	Discussion and Decision

Introduction

In RAN- 94 e-meeting, the SI of Rel-18 positioning [1] is approved, and the objectives on improved accuracy, integrity, and power efficiency can be seen as below:

- Improved accuracy, integrity, and power efficiency:

  - Study solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]:

    - Identify the error sources, [RAN1, RAN2].

    - Study methodologies, procedures, signalling, etc for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2]

    - Focus on reuse of concepts and principles being developed for RAT-Independent GNSS positioning integrity, where possible.

  - Study solutions for accuracy improvement based on PRS/SRS bandwidth aggregation for intra-band carriers considering e.g. timing errors, phase coherency, frequency errors, power imbalance, etc [RAN4]:

  - Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4]

    - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1]

    - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary

  - Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1]

    - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study.

    - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state

In this contribution, we focus on the discussion on solutions for accuracy improvement based on NR carrier phase measurements.

Carrier phase

Carrier phase principle

The principle of carrier phase based positioning can be seen in Figure 1. Take DL for example, TRPs transmit PRS and UE measures carrier phase of each PRS from each TRP. As for the measurement of carrier phase, UE need to obtain the phase of each PRS by PLL. But it cannot obtain the number of integer multiples of the carrier wavelength, which is called as integer ambiguity. It is similar as time based positioning method, UE cannot obtain the absolute transmission time between TRP and UE by one trip. It can only obtain the time difference of each TRP unless RTT is used. Thus for absolute carrier phase measurement, it is necessary to solve the problem of integer ambiguity and obtain the number of integer multiples of the carrier wavelength. Else, the other way is to use the relative carrier phase measurement between TRPs or different times.

Figure 1: carrier phased based positioning

Absolute carrier phase measurement

Integer ambiguity

In order to solve the problem of integer ambiguity and obtain the number of integer multiples of the carrier wavelength, refer to [2], multiple carrier frequencies can be used. With this method, first the distance between TRP and UE need to be estimated based on, e.g., time based positioning such as TDoA. Then the number of integer multiples of the virtual carrier wavelength can be estimated with small search space. After that the accurate positioning can be calculated based on the integer multiples and the measured carrier phase of the virtual carrier wavelength. But the accuracy depends on the virtual carrier wavelength which is large than that of f1 and f2. In order to further improve the accuracy, it need to obtain the number of integer multiples of f1 or f2. Here we assume the number of integer multiples of f1 is N1 , the number of integer multiples of f2 is  N2, and the number of integer multiples of fv is  Nv.

Nv=-λvPv-Tλv-[wT-wvλv]#1-1

N1=-λ1P1+λvPv+λvNvλ1-wv-wP1λ1#1-2

And the search space of Nv depends on wTλv. While wT is the measurement error of TDoA, λv is the virtual carrier wavelength which depends on the gap between f1 and f2. The smaller the gap between  f1 and f2, the smaller the search space of Nv. But The smaller the gap between  f1 and f2, the larger the search space of N1.

With introducing virtual carrier wavelength based on multiple carrier frequencies, there are many issues need to be studied. First the impacts on UE RF structure and requirements. For example, it is not sure the gap between f1 and f2, or the bandwidth of f1 and f2. If the NR resource grid with SCS 15KHz need to be considered, the smallest gap between f1 and f2 should be not less than 15KHz without enhancement of the resource grid of UE receiver. And if PRS is reused for DL positioning, two carrier frequencies can only be supported simultaneously by UE with capability of carrier aggregation. Else the positioning accuracy based on multiple carrier frequencies is not clear.

- Proposal 1: Study the potential solution for integer ambiguity and considering the impacts on UE RF with reusing of PRS/SRS and other specification impacts.

Timing error

In order to solve the problem of integer ambiguity and as well as the clock error, double differential operation is a basic approach which can be seen in Figure 2. The requirement of double differential operation is two receivers, so the PRU introduced in R17 positioning can be used to be as the other receiver. In this case, based on the measured phase difference between TRP#1 and TRP#2 at the target UE side, and the measured phase difference between TRP#1 and TRP#2 at the PRU side, the timing error can be removed. After obtain the integer ambiguity of the double differential operation based on TRP#1 and TRP#2, TRP#1 and TRP#3, and TRP#2 and TRP#3, the location of the target UE can be estimated.

In addition to the timing error, there is also some phase error at UE and TRP side, which means the phase will be changed between baseband unit and RF unit. And the phase error may be solved by similar mechanism, i.e., based on phase error group and PRU.

Figure 2, Double differential operation

- Proposal 2: Study PRU assisted carrier phase measurement for timing error and phase error mitigation.

Relative carrier phase measurement

Definition of relative carrier phase

In this section, we will discuss the relative carrier phase measurement. It includes relative carrier phase between different TRPs or relative carrier phase between different times.

For relative carrier phase between different TRPs, it is similar as time difference of arrival and can be seen in Figure 3. And the relative carrier phase means the carrier phase difference between PRS#1 from TRP#1 and PRS#2 from TRP#2. So the reference TRP need to be defined for relative carrier phase measurement. In addition, it is better that phase synchronization can be achieved between TRPs. Or a fixed phase shift between TRPs can be assumed.

Figure 3, Relative carrier phase between TRPs

For relative carrier phase between different times, it means the relative carrier phase at different time and different location based one PRS. The suitable use case is for mobile UE, which can be seen in Figure 4. It means that UE measures the phase#1 at time t1 at point A and move to point B at t2, and measures the phase#2 at time t2 at point B. The relative carrier phase is the phase between t1 and t2, which including the number of integer multiples of the carrier wavelength and the part less than one carrier wavelength. And the distance between point A and B can be estimated by UE. Based on this information, the distance between TRP#1 and point A can be estimated. Then based on the distance between UE and multiple TRPs, the location of UE can be estimated.

Figure 4, Relative carrier phase between different times

- Proposal 3: Study relative carrier phase measurement between different TRPs or different times.

The cycle slips

The problem of relative carrier phase measurement is the detection and repair of the cycle slips. There are also many potential solutions for cycle slips detection and repair including multiple carrier frequencies, differential operation, pseudo range combined with carrier phase measurement, and so on. As same as integer ambiguity, potential solutions should be studied with considering impacts on RF at UE side and specifications.

- Proposal 4: Study the potential solution for cycle slips detection and repair and considering the impacts on UE RF and other specification impacts.

Conclusion

In this contribution, we discussed about accuracy improvement based on NR carrier phase measurements. Based on above discussion, we provide the following proposals.

- Proposal 1: Study the potential solution for integer ambiguity and considering the impacts on UE RF with reusing of PRS/SRS and other specification impacts.

- Proposal 2: Study PRU assisted carrier phase measurement for timing error and phase error mitigation.

- Proposal 3: Study relative carrier phase measurement between different TRPs or different times.

- Proposal 4: Study the potential solution for cycle slips detection and repair and considering the impacts on UE RF and other specification impacts.

References

- RP-213588 Revised SID on Study on expanded and improved NR positioning

- R1-1901980, NR RAT-dependent DL Positioning, CATT.