3GPP TSG RAN WG1 Meeting #109-e 	R1-2203834

E-meeting, May 9th – 20th, 2022

Source: 	    Xiaomi

Title:		      Discussion on NR PDCCH reception in symbols with LTE CRS REs

Agenda item:   9.9.1

Document for: Discussion

Introduction

In RAN#94 meeting, a new work item on Rel-18 DSS (Dynamic spectrum sharing) was approved [1], details are as follows:

The following objectives shall be included for improvement of NR spectrum efficiency for LTE-NR co-existence (RAN1): - Study and if needed specify NR PDCCH reception in symbols with LTE CRS REs. [RAN1] - Investigate enabling LTE CRS to puncture NR PDCCH, including the impact to NR PDCCH DMRS if there is the performance gain from the additional PDCCH resources. - Allow a UE to support, and be configured with, two overlapping CRS rate matching patterns regardless of support or configuration of multi-TRP [RAN1]

In this contribution, we provide our views about NR PDCCH reception on OFDM symbols with LTE CRS REs. Simulation results of different scenarios are provided as well.

Discussion

If NR PDCCH can be transmitted on the OFDM symbols containing LTE CRS, there are two issues we need to pay attention: 1) With enabling LTE CRS to puncture NR PDCCH, PDCCH transmission on symbols with LTE REs can improve PDCCH capacity. However, the performance of NR PDCCH would be jeopardized due to information loss. 2) With enabling LTE CRS to puncture NR PDCCH, there is possibility that NR PDCCH DMRS is punctured by LTE CRS as well. Considering DMRS is used for channel estimation, the performance of NR PDCCH may be degraded significantly.

In order to find out the impacts from information bits loss and DMRS loss, we evaluate following schemes:

- Baseline (2sym, no punc): CORESET (Control Resource Set) occupies symbol#1 and symbol#2, on which there is no CRS.

- Scheme 1 (2sym, punc): CORESET occupies symbol#1 and symbol#2 while LTE CRS locates on symbol#0 and symbol#1 (4-port CRS). NR PDCCH overlaps with LTE CRS on symbol#1, as shown in Fig 2-1. If NR PDCCH and corresponding DMRS conflict with LTE CRS, both NR PDCCH and DMRS will be punctured. UE will not receive NR PDCCH and DMRS on conflicting REs. The channel estimation is performed only with the received DMRS on non-conflicting REs.

- Scheme 2 (2sym, punc, cover): Same as Scheme 1, CORESET occupies symbol#1 and symbol#2 while LTE CRS locates on symbol#0 and symbol#1. NR PDCCH overlaps with LTE CRS on symbol#1. If NR PDCCH conflicts with LTE CRS, NR PDCCH will be punctured. If NR PDCCH DMRS conflicts with LTE CRS, the DMRS will be shifted in frequency domain. In the other words, NR PDCCH DMRS is guaranteed and none of them are punctured or dropped.

Fig 2-1 Scenario of NR PDCCH overlapping with 4-port LTE CRS

Fig 2-2 provides the simulation results for NR PDCCH performance of above three schemes. Simulation results for AL2, AL4, AL8 and AL16 for different schemes are provided respectively.  Additionally, 100ns delay spread and 3 km/h UE speed are assumed. The same bandwidth is assumed for both CORESET and CRS, e.g. 96 RBs. The detailed simulation assumptions for the link level simulation can be found in Appendix.

Figure 2-2 The performance of NR PDCCH with different schemes

Based on the simulation results, we have the following observations:

- Scheme 1: With AL2, there is about 1.39 dB performance loss to achieve 1% BLER compared with Baseline. LTE CRS REs puncture NR PDCCH will result in poorer PDCCH decoding performance. On the other hand, the performance loss because of puncturing can be alleviate for large aggregation level, e.g. there is only 0.73dB performance loss for AL16. The reason is that smaller aggregation level has higher effective coding rate which is much more sensitive to information loss.

- Scheme 2: With AL2, there is about 1.92 dB performance loss to achieve 1% BLER compared with Baseline. LTE CRS REs puncture NR PDCCH will result in worse PDCCH decoding performance. On the other hand, the SNR loss reduces to 0.75dB with AL16.

- Scheme 1&Scheme 2: Compared with Scheme 1, there is about 0.02dB (AL16) ~0.53dB (AL2) performance loss for Scheme 2. The impact of channel estimation loss due to DMRS puncturing of Scheme 1 is comparable to that of the higher effective coding rate of Scheme 2.

Observation 1: If enabling LTE CRS to puncture NR PDCCH and DMRS, limited SNR performance loss is observed compared with Baseline.

Observation 2: If NR PDCCH DMRS conflicts with LTE CRS, directly puncturing the DMRS outperforms the scheme of shifting the DMRS in frequency domain.

Compared with Rel-16 scheme that the NR PDCCH can’t be transmitted on the symbols with LTE CRS, the number of available REs for NR PDCCH increases. Even if we consider the case wherein 4-port CRS is configured, more REs can be exploited on LTE CRS OFDM symbols. Depending on the realistic configuration of CORESET, i.e. on which OFDM symbols it can be configured, gNB can have more resources to transmit NR PDCCH compared Rel-15/16/17 mechanism. An exemplary table is shown below. Consequently, the NR PDCCH capacity increases along with more available resources.

Table 1 The number of available REs per RB occupied by PDCCH and DMRS

| Symbols occupied by PDCCH | symbol #2 | symbol #1&symbol#2 | symbol#0&symbol#1&symbol #2 |
| --- | --- | --- | --- |
| Additional available REs per RB | 12 | 20 | 28 |

Based on the above analyses, we have the following proposal:

Propose 1: Support NR PDCCH reception in symbols with LTE CRS REs and enable LTE CRS to puncture NR PDCCH.

Regarding to the DMRS operation, we have the following proposal:

Propose 2: NR PDCCH DMRS can be directly punctured by LTE CRS RE once collision happens.

Conclusion

According to the above discussions, we have the following observation and proposals:

Observation 1: If enabling LTE CRS to puncture NR PDCCH and DMRS, limited SNR performance loss is achieved compared with Baseline.

Observation 2: If NR PDCCH DMRS conflicts with LTE CRS, directly puncturing the DMRS outperforms the scheme of shifting the DMRS in frequency domain.

Propose 1: Support NR PDCCH reception in symbols with LTE CRS REs and enable LTE CRS to puncture NR PDCCH.

Propose 2: NR PDCCH DMRS can be directly punctured by LTE CRS RE once collision happens.

References

- RP-213559, “New WI: Enhancement of NR Dynamic spectrum sharing (DSS)”

Appendix

Table 5-1 Link-level simulation parameters for NR PDCCH

| Parameters | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| Subcarrier spacing | 15 kHz |
| BW | 20MHz (106 RBs) |
| Channel model | TDL-A |
| Delay spread | 100ns |
| UE Speed | 3km/h |
| BS antenna configuration | 4Tx |
| UE antenna configuration | 2Rx |
| Channel estimation | Practical |
| DCI payload (excluding CRC) | 60 bits |
| Interleaving | Interleaved |
| REG bundle size | 6 PRBs |
| CORESET | 96 RBs |
| AL | 2,4,8,16 |