3GPP TSG RAN WG1 Meeting #109-e 	R1-2203836

E-meeting, May 9th – 20th, 2022

Source:	 Xiaomi

Title:	 Other issues on enhancement of NR Dynamic Spectrum Sharing

Agenda Item:	  9.9.3

Document for:	Discussion and Decision

Introduction

During RAN#94 meeting, a new work item on DSS (Dynamic spectrum sharing) was approved for Rel-18 [1] with the following details:

The following objectives shall be included for improvement of NR spectrum efficiency for LTE-NR co-existence (RAN1): - Study and if needed specify NR PDCCH reception in symbols with LTE CRS REs. [RAN1] - Investigate enabling LTE CRS to puncture NR PDCCH, including the impact to NR PDCCH DMRS if there is the performance gain from the additional PDCCH resources. - Allow a UE to support, and be configured with, two overlapping CRS rate matching patterns regardless of support or configuration of multi-TRP [RAN1]

In this contribution, we present the discussion on the impact on the NR PDCCH reception due to the LTE PHICH and PCFICH transmission.

Discussion

The legacy LTE signals PHCH and PCFICH are transparent to NR UEs. However, the transmission of PHICH and PCFICH may introduce severe interference to the NR PDCCH as illustrated in Fig 2-1. NIDcell =0, NRBDL=25, mi=1 , Ng=2, PCFICH and extended PHICH overlap with 4 port LTE CRS in TDD scheme are assumed.

Fig 2-1 Scenario of PHICH and PCFICH overlap with 4-port CRS

In an extended PHICH configuration shown in Fig 2-1, the PHICH may be transmitted on the first three symbols (symbol#0, symbol#1, symbol#2) with continuous REs in frequency domain. If the NR PDCCH is transmitted on colliding REs, the NR PDCCH may suffer severe interference.

Observation 1: PCFICH and PHICH may also cause severe interference to NR PDCCH.

In current specification, if the UE knows some interference resource-related parameters, it can handle interference by operating rate matching, or interference cancellation, or receiving the punctured data performed by the gNB. In order to reduce the interference of PCFICH and PHICH, some parameters related to the positions of PCFICH and PHICH may be notified to the UE.

Proposal 1: Some parameters related to the positions of PCFICH and PHICH may be notified to the UE to reduce their interference.

Conclusion

According to the above discussions, we have the following observation and proposals:

Observation 1: PCFICH and PHICH may also cause severe interference to NR PDCCH.

Proposal 1: Some parameters related to the positions of PCFICH and PHICH may be notified to the UE to reduce their interference.

References

- RP-213559, “New WI: Enhancement of NR Dynamic spectrum sharing (DSS)”