3GPP TSG RAN WG1 #109-e	R1-2203837

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.8.1

Source: 	Langbo

Title: 	Remaining issues on TB processing over multi-slot PUSCH

Document for:	Discussion and Decision

# 1	Introduction

In this document, remaining issues on TB processing over multi-slot PUSCH are addressed. Corresponding text proposals are provided also.

# 2        HARQ-ACK information interpretation in DFI

For a PUSCH transmission of TB processing over multiple slots, TBS determination is based on a number of slots N indicated by numberOfSlotsTBoMS, where each repetition of the PUSCH transmission occupies N slots. For the PUSCH transmission of TB processing over multiple slots, considerable amount of coded bits of the TB may not be able to be received after a last symbol of the PUSCH transmission in a first slot from the multiple slots. As a result, ACK is quite unlikely to be generated until the first repetition has been received. Similarly to the case where numberOfSlotsTBoMS is not provided, it is natural to use the last symbol of the first repetition instead of the last symbol of the first slot from the multiple slots as the reference time for the HARQ-ACK information validation when numberOfSlotsTBoMS is provided. Correspondingly, the following text proposal can be considered.

************************************* Start of TP for TS 38.213 **************************************

< Unchanged parts are omitted >

## 10.5	HARQ-ACK information for PUSCH transmissions

< Unchanged parts are omitted >

For a PUSCH transmission scheduled by a DCI format, HARQ-ACK information for a transport block of a corresponding HARQ process number is valid if a first symbol of the PDCCH reception is after a last symbol of the PUSCH transmission by a number of symbols provided by cg-minDFI-Delay or, if the PUSCH transmission is over multiple slots,

-	after a last symbol of the PUSCH transmission in a first slot from the multiple slots by a number of symbols provided by cg-minDFI-Delay, if a value of the HARQ-ACK information is ACK and numberOfSlotsTBoMS is not present in the resource allocation table.

-	after a last symbol of the PUSCH transmission, or of a first repetition of the PUSCH transmission, by a number of symbols provided by cg-minDFI-Delay, if a value of the HARQ-ACK information is ACK and numberOfSlotsTBoMS is present in the resource allocation table.

-	after a last symbol of the PUSCH transmission in a last slot from the multiple slots by a number of symbols provided by cg-minDFI-Delay, if a value of the HARQ-ACK information is NACK.

< Unchanged parts are omitted >

*************************************** End of TP for TS 38.213  *************************************

# 3        Conclusions

In this contribution, remaining issues on TB processing over multi-slot PUSCH for Rel-17 NR coverage enhancement are discussed and the corresponding text changes are proposed.