3GPP TSG RAN WG1 #109-e	R1-2203838

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.12.1

Source: 	Langbo

Title: 	Remaining issues on mechanisms to improve reliability for RRC_CONNECTED UEs

Document for:	Discussion and Decision

# 1	Introduction

In this document, remaining issues on mechanisms to improve reliability for RRC_CONNECTED UEs are addressed. Corresponding text proposals are provided also.

# 2        Overlapping resolving for NACK-only feedback mode

For NACK-only feedback mode, when a number of HARQ-ACK information bits is 2, 3, or 4, the UE can be configured to select one from a set of PUCCH resources for the PUCCH transmission based on the values of HARQ-ACK information bits. It is still open how to resolve overlapping among PUCCH resources in the set of PUCCH resources and other PUCCHs or PUSCHs. If PUCCH resources in the set of PUCCH resources are considered independently, i.e., only the overlapping among the selected PUCCH resource and other PUCCHs or PUSCHs is resolved, the multiplexing result depends on the value of the HARQ-ACK information bits. This may lead to chick-and-egg situation at gNB side as gNB may not be able to know the multiplexing result prior to successfully decoding the HARQ-ACK information bits. Note that blind decoding by gNB may not work effectively especially when the HARQ-ACK bits are multiplexed on a non-sequence based PUCCH or PUSCH. A simple way to address this problem is to consider the PUCCH resources in the set of PUCCH resources as a whole when multiplexing with other PUCCHs or PUSCHs.

The following text changes are proposed.

************************************* Start of TP for TS 38.213 **************************************

< Unchanged parts are omitted >

### 18	Multicast Broadcast Services

< Unchanged parts are omitted >

If a UE would multiplex HARQ-ACK information that the UE would provide according to the second HARQ-ACK reporting mode with other UCI in a first PUCCH, or in a PUSCH, as described in clauses 9 and 9.2.5, the UE provides the HARQ-ACK information according to the first HARQ-ACK reporting mode. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH, the UE considers that the UE would transmit the second PUCCH when all values of the HARQ-ACK information are ‘ACK’.

If a UE is indicated by moreThanOneNackOnly-Mode to provide the HARQ-ACK information bits by selecting a resource from a set of resources for the PUCCH transmission based on the values of the HARQ-ACK information bits and the UE would transmit a first PUCCH with a selected resource in the set of resources and there exists a resource in the set of resources overlaps with a second PUCCH or a PUSCH, the UE assumes the first PUCCH overlaps with the second PUCCH or the PUSCH.

< Unchanged parts are omitted >

*************************************** End of TP for TS 38.213  *************************************

# 3      HARQ-ACK bits determination for Type-3 HARQ-ACK codebook with NACK-only feedback mode

For a type-3 HARQ-ACK codebook, if NDI values are not included, a UE determines the value of a HARQ-ACK information bit according to whether HARQ-ACK information for a TB of corresponding HARQ process number has been reported or not. When NACK-only reporting mode adopted, whether a PUCCH not being transmitted due to including only HARQ-ACK information with ACK values is considered as having been reported or having not been reported need be clarified. Considering gNB has obtained the ACK values by confirming that PUCCH is not detected as well as to avoid the HARQ-ACK information misalignment between UE and gNB, it is reasonable to assume that the HARQ-ACK information with ACK values has been reported, and the corresponding HARQ-ACK bits in a type-3 HARQ-ACK codebook should be set to default values of NACK.

Correspondingly, the following text proposal can be considered.

************************************* Start of TP for TS 38.213***************************************

< Unchanged parts are omitted >

### 9.1.4	Type-3 HARQ-ACK codebook determination

< Unchanged parts are omitted >

If a UE multiplexes HARQ-ACK information in a PUSCH transmission, the UE generates the HARQ-ACK codebook as described in this clause except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH.

For generating a Type-3 HARQ-ACK codebook with NDIHARQ=1, if  the UE does not transmit a PUCCH that would include only HARQ-ACK information with ACK values according to the second HARQ-ACK reporting mode as described in clause 18, the UE assumes that corresponding  HARQ-ACK information with ACK values is reported.

< Unchanged parts are omitted >

*************************************** End of TP for TS 38.213  *************************************

# 4        Conclusions

In this contribution, the remaining issues on mechanisms to improve reliability for RRC_CONNECTED UEs for Rel-17 NR Multicast and Broadcast Services are discussed and the corresponding text changes are proposed.