3GPP TSG RAN WG1 #109 	R1-2203841

e-Meeting, May 9th – 20th, 2022

Agenda item:		9.12.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Enhancements for long connections in NB-IoT/eMTC over NTN

Document for:		Discussion and Decision

# Introduction

The release 18 work item [1] on IoT support of non-terrestrial networks includes the following objective

Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1]

In release 17, the work was limited to short and sporadic connection and therefore it was feasible that a UE upon GNSS validity timer expiry could move to RRC Idle mode.

However, the release 18 work will address the broader use case of long connection time. Therefore, it is important to consider how the UE can obtain new GNSS-based position fix during the long connection. It is also good to note that the simultaneous use of GNSS and IoT operation is not assumed as was also the case in release 17.

In this contribution we discuss the relevant assumptions for release 18, the need for a GNSS measurement gap, the use of the GNSS validity timer, how to configure the gap and related UE actions. Furthermore, the impact of long connection times on the uplink transmission segment procedure and potential cell reselection are discussed.

# Discussion

## Assumptions

With regards to the objective of long connection times, the release 18 work item has the same assumptions as the release 17 work item:

- GEO and LEO satellites, the latter having either earth fixed or earth moving cells.

- Non-simultaneous operation of GNSS and IoT NTN

- FDD mode, which for NB-IoT is half-duplex only

Furthermore, in release 17 the following agreements were made regarding use of GNSS:

RAN1 #106-e

- For sporadic short transmission, UE in RRC_CONNECTED should go back to idle mode and re-acquire a GNSS position fix if GNSS becomes outdated.

- Note: The duration of the short transmission is not longer than the “validity timer for UL synchronization” referred to in the WID objective (but which still needs further discussion for specifying further details)

RAN1 #1 107-e

- The UE autonomously determines its GNSS validity duration X and reports information associated with this valid duration to the network via RRC signalling.​

  - X = {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity}

RAN2 #116bis-e

- UE need to have a valid GNSS fix before going to connected. RAN2 assumes that the UE may need to re-aquire the GNSS fix right before establishing the connection (regardless if previously valid or not), if needed to avoid interruption during the connection.

- When the GNSS fix becomes outdated in RRC_CONNECTED mode, the UE goes to IDLE mode.

The RAN1/2 agreements defining the UE moves to RRC Idle when the GNSS fix becomes outdated was acceptable in release 17 where the connection, according to RAN1, was “short”. On the contrary, the release 18 objective noted above assumes “long” connections. If the UE having a “long” connection moves to RRC Idle multiple times during a data transfer it will increase device energy consumption, signaling overhead, and latency.

Thus, the 3GPP shall investigate methods to support acquiring a new GNSS position fix during the long connection as outlined by the release 18 objective.

Observation 1: the release 18 assumption on “long” connection require the UE can obtain a new GNSS position fix during the connection to minimize the impact on device energy consumption, signaling overhead and latency.

## GNSS re-acquisition

According to [2] the GNSS position fix duration for the hot state is 1-2 s, which would most likely be the scenario if the UE acquires a new fix shortly before the RRC Connection is set up. If the UE is in good GNSS channel conditions the position fix may be achieved even faster. Thus, the position fix may complete faster than the time required to perform initial access and set up of the RRC Connection after the UE has moved to RRC Idle (in accordance with the release 17 agreement).

From this PoV, release 18 UE should not go back to RRC Idle mode but remain RRC Connected. Therefore, a simple new GNSS measurement will be the best choice for IoT UE in NTN scenario with a long connection.

Furthermore, the need for a new GNSS position may occur quite frequently for a moving UE in order to comply with timing requirements. According to TS 36.133, the UE initial transmission timing error shall be less than or equal to ±Te where the timing error limit value is 80*Ts (2.6 µs) for NB-IoT and 24*Ts (0.78 µs) for eMTC CE Mode A. As illustrated in Figure 1 below, in the worst-case scenario (10⁰ elevation angle), the timing accuracy requirement corresponds to 126 meter and 38 meter UE location error respectively for NB-IoT (Te = 80 Ts) and eMTC (Te = 24 Ts). For a moving UE, it can easily move 126 m or 38 m in 10 seconds (the minimum GNSS validity duration according to RAN1 107-e agreement noted above). For example, if the UE is on a truck moving with 80 km/h (about 22 m/s) it would take less than 6 seconds to violate the requirement for NB-IoT and less than 2 seconds for eMTC.

Figure 1 TA error caused by UE movement

Thus from this PoV, also a new GNSS measurement is needed in RRC Connected mode.

Proposal 1: GNSS measurement window in CONNECTED mode should be specified for a new GNSS measurement when GNSS position is about to outdated.

Considering “Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed”, the GNSS measurement will impact the achievable IoT throughput performance. The reason is that the UE is not able to receive/transmit data during the GNSS measurement window (about 1 s). A larger and/or more frequent GNSS measurement window will impact the IoT throughput performance more and thus careful configuration of such a measurement window is needed. It is of particular importance that the UE and network share a common understanding of when the UE is performing the GNSS measurement (i.e. time-wise location of the GNSS measurement window), because the UE is unable to receive/transmit IoT data during the measurement.

Observation 2: Common understanding on GNSS measurement window in time domain between UE and network is needed.

Like all other measurements, the time requested for UE's GNSS measurement will depend on UE capability and GNSS channel status. Obviously a better GNSS channel status will provide a better GNSS receiving quality, which will require less time to receive the GNSS signal from multiple GNSS satellite and less time to achieve a accurate GNSS information for UE.

While for UE capability, two aspects should be considered for GNSS measurement:

- A UE with more receiving antenna will have more receiving gain, which will provide a higher GNSS receiving quality.

- A UE with more processing capability can provide a faster processing on same size of packets with same receiving signal qualtiy.

Obviously, NB-IoT UE may have less number or receiving antenna and less processing capability than eMTC UE, while both of them will have less capability than normal UE. A UE with higher UE capability on GNSS processing will request less time for GNSS measurement.

Observation 3: Multiple IoT UE with different capability and channel status may request different GNSS measurement window.

Two alternatives can be considered for determining the measurement window duration and periodicity: network selection and UE selection.

When network has to select the GNSS measurement window, UE assistance information is needed. Although UE capability for GNSS measurement is fixed, the GNSS channel status may change especially for moving UE, resulting in the requested GNSS measurement window size and periodicity to be changed accordingly.

As IoT UL transmission may last for a long time due to use of repetitions, it may take many resources if the GNSS channel status is reported too frequently. Meanwhile, the GNSS channel status may already have changed when UE reports it or when eNB configures the GNSS measurement window.

Therefore, it is better for eNB to control the GNSS measurement window based on UE’s suggestion of GNSS measuerement window. In this solution, UE can initially select the candidate effective accurate GNSS measurement window according to its GNSS related capability and GNSS channel status. Then UE can report the parameters of candidate GNSS measuerement window to network and network to finally decide the GNSS meausurement window and configure to UE.

Proposal 2: Overhead reduction should be considered for selection of GNSS measurement window and coordination between UE and eNB.

Proposal 3: UE report the GNSS measurement gap should be the specified, to keep a low overhead.

As indicated in Figure 1 even a small UE location error may violate the timing requirements. According to the release 18 assumptions, the UE is unable to use the GNSS to detect such movement and thus the UE will in principle not be aware that it has moved and that the uplink timing of an ongoing transmission is drifting and potentially violating the requirements.

Observation 4: UE may not be aware that it is moving during a long connection (uplink repetitions), because it cannot use the GNSS simultaneously. The UE movement will result in misaligned transmission timing.

Proposal 4: GNSS error because of UE-unawared movement should be studied and solved.

## Validity timer expiry

As noted in the RAN1 #1 107-e agreement, the UE has to reports the GNSS validity duration to the network. This is useful information for the network to decide, when to schedule a GNSS measurement gap for the UE. For example, network should take care to ensure the GNSS validity does not expire during a long transmission independently of whether it is uplink or downlink, because UE would not be allowed to transmit data/feedback if the GNSS position is invalid. Alternatively, for downlink data the network could ensure the UE has time to perform a GNSS position fix update before the UE has to provide feedback in uplink.

Even if there is remaing GNSS validity time the UE may be in need of a new position because of UE movement. As noted in Figure 1 even movement of a few 10s to 100s of meters can result in violation of the timing requirements. However, since the UE is not able to utilize GNSS simultaneously with the IoT communication it may be challenging for the UE to determine that is actually moving, because it would have to rely on other means.

In the rare scenario, where the GNSS validity timer has expired, but further data is pending, it will be beneficial to avoid the UE moves to RRC Idle. As noted previously, the signaling and latency of the random access procedure and RRC Connection setup is to be avoided if possible. One alternative approach can therefore be that UE performs the GNSS position fix and afterwards notifies the network, e.g. through the random access procedure, but using a pre-configured preamble (i.e. contention-free random access).

Proposal 5: To save power consumption and latency, keeping RRC connection and new UL synchronization after re-acquiring GNSS should be considered for long term connnection, instead of going back to IDLE mode.

Additionally, for IoT UE, the repetitions may last for long time, e.g. several seconds or even tens of seconds. For each HARQ process, the LTE NB-IoT transmission time is “slot duration * number of repetitions * number of resources units * number of slots in resource unit”. When considering the largest number of repetitions, number of resources units (RU), number of slot in RU, the maximum transmission time could be 0.5 ms * 128 * 10 * 16 = 10240 ms for 15kHz SCS or 2 ms * 128 * 10 * 16 = 40960 ms for 3.75kHz SCS. But the validity timer may already expire when the repetition complete. How to solve this issue should be studied.

Proposal 6: How to solve the issue as GNSS expire during the long repetition for IoT should be studied.

## PUSCH/PRACH transmission

The release 18 objective on “improved GNSS operations for a new position fix for UE pre-compensation during long connection times” entails the UE is receiving and/or transmitting data for long periods of time. This has implications for some features, which were defined in release 17 including the use of segment durations for uplink and also for the potential continuation of data transfer after a cell reselection, which was not considered in release 17 due to the assumption of short connection time.

### Configuration of segment for long uplink transmission

In release 17 the use of transmission segments within a transmission block of 256 ms was specified to perform TA error compensation due to time drift caused by the satellite movement.

However, the case where multiple blocks of 256 ms are used for long uplink was not discussed in detail. During the transmission period of the multiple blocks, the TA drift rate will dramatically change due to the wide range of elevation angle of the satellite as the satellite moves along the orbit. Therefore, it can be needed to update the segment configuration every block of 256 ms. That will however increase signaling overhead with additional delay and some risk of link failure during a long UL transmission. Also, to avoid such segment configuration update, a single segment can be set to the shortest segment duration that is commonly applicable to UE in any elevation angle. However, it will also result in more frequent TA adjustments than necessary, thus adding to the UE processing complexity. Moreover, half-duplex UE transmitting in uplink cannot receive the downlink data indicating a segment change. Therefore, a half-duplex UE needs to complete all current uplink repetitions before changing uplink segment size according to a downlink indication.

Observation 5: Over a long uplink transmission the elevation angle change will cause large variation of TA drift rate.

Observation 6: Different segment sizes may be needed depending on the TA drift rate.

One approach to handle the issue of need for different segment sizes during a long uplink transmission is to configure multiple segment sizes, such that UE can switch from one segment size to the next during a single transmission. This will provide for a better tradeoff between segment duration and segment configuration signaling overhead.

Proposal 7: RAN1 to discuss how to configure multiple segment sizes for an uplink transmission.

### TA error in segment

RAN1 has agreed TA processing before each segment and that the TA will be not changed in the segment. Instead the TA is updated between segment according with the satellite movement. However, as the TA is not changed in the segment, the used TA can only be aligned with one subframe of the segment while there will still be a TA error for the other subframe(s) of the segment.

In RAN1 it is not discussed which subframe/repetition of the segment the TA is adjusted for. A general way is like UE processing in TN, where the TA error for the first subframe (of the segment) will be 0 us. Thus the TA error for the following subframes will increase with time. For the last subframe m the TA error may be determined as ∆TA * (m-1), where ∆TA is the TA drifting in time of one subframe. As the maximum time error of transmission can not be larger than RAN4 requirement, the value of ∆TA * (m-1) also directly limit the size of the segment.

It is good to note that the TA error will impact the decoding performance of the repetitions. The smaller the TA error, the better performance. Therefore, RAN shall consider and discuss how the TA error for repetitions can be reduced to ensure good performance..

Proposal 8: How to reduce the TA error for repetitions in the segment should be considered and discussed.

## Expiry of ephemeris validity timer

An additional issue, which needs to be considered for the scenario of long connections is that the validity timer for ephemeris (and common TA) may expire during a transmission relying on repetitions. The reason is that the validity timer is based on the Epoch time of the information, while the UE’s transmission can start at any time.

Observation 7: Ephemeris validity timer may expire during a UE’s uplink transmission using repetitions.

When the validity timer has expired the UE would need to reacquire the ephemeris, before it can continue the transmission of the repetitions. Additionally, when new ephemeris used, the TA for remaining repetitions may be different from the previous repetitions. How to require ephemeris and update TA during the long repetition should be studied.

Proposal 9: How to require ephemeris and update TA during the long repetition should be studied.

## Continuation of repetitions between two NTN cells

The IoT technologies rely on, among others, repetitions to improve the link budget.

The transmission times of 10 s and 40 s, as mentioned above, can be larger than the time the UE is served by a single cell in the LEO scenarios. For example, the maximum coverage time of one cell may be 50 km / 7.56 km/s =6.6 s based on the assumption of 50km satellite beam diameter for set 1, or 234 km /7.56 km/s = 31s for set 3 with 234km satellite beam diameter.

Thus, the issue is that the NB-IoT NTN UE in many cases can not complete the scheduled number of repetitions before performing a cell reselection. Even if the number of repetitions for example only lead to a total transmission time, which is half of the coverage time (i.e. 3.3 s for the 50 km cell) this will result in the network only being able to schedule such UEs in the first half of the coverage time. This will limit the spectral efficiency and increase the latency.

A potential solution could be to enable repetition continuation for the HARQ process from one cell to another. When considering the transparent scenario, at least intra-satellite mobility would entail the two cells most likely originate from the same eNB and thus transfer of received bits would be an eNB-internal process. The target would be to enable the data transfer to continue after a cell reselection instead of restarting. Likewise, the procedure could be for handovers in eMTC and should support both uplink and downlink data transfes.

If such functionality is not supported it will limit the number of repetitions, which the network can schedule in uplink and downlink, and therefore also the coverage, because the link budget depends on the number of repetitions.

Thus, it is suggested that RAN1 can consider if such continiuation is feasible on the PHY layer in terms of keeping soft bits/repetition data, while solutions on higher layer RLC can also be envisioned. This is required to support the long connection times, which are envisioned by the objective in release 18

Proposal 10: RAN1 should discuss the issue of repetition continuation between two NTN cells.

# Conclusion

In this contribution, we discussed time and frequency synchronization for NB-IoT/eMTC over NTN, our observations and proposals are presented as following:

Observation 1: the release 18 assumption on “long” connection require the UE can obtain a new GNSS position fix during the connection to minimize the impact on device energy consumption, signaling overhead and latency.

Observation 2: Common understanding on GNSS measurement window in time domain between UE and network is needed.

Observation 3: Multiple IoT UE with different capability and channel status may request different GNSS measurement window.

Observation 4: UE may not be aware that it is moving during a long connection (uplink repetitions), because it cannot use the GNSS simultaneously. The UE movement will result in misaligned transmission timing.

Observation 5: Over a long uplink transmission the elevation angle change will cause large variation of TA drift rate.

Observation 6: Different segment sizes may be needed depending on the TA drift rate.

Observation 7: Ephemeris validity timer may expire during a UE’s uplink transmission using repetitions.

Proposal 1: GNSS measurement window in CONNECTED mode should be specified for a new GNSS measurement when GNSS position is about to outdated.

Proposal 2: Overhead reduction should be considered for selection of GNSS measurement window and coordination between UE and eNB.

Proposal 3: UE report the GNSS measurement gap should be the specified, to keep a low overhead.

Proposal 4: GNSS error because of UE-unawared movement should be studied and solved.

Proposal 5: To save power consumption and latency, keeping RRC connection and new UL synchronization after re-acquiring GNSS should be considered for long term connnection, instead of going back to IDLE mode.

Proposal 6: How to solve the issue as GNSS expire during the long repetition for IoT should be studied.

Proposal 7: RAN1 to discuss how to configure multiple segment sizes for an uplink transmission.

Proposal 8: How to reduce the TA error for repetitions in the segment should be considered and discussed.

Proposal 9: How to require ephemeris and update TA during the long repetition should be studied.

Proposal 10: RAN1 should discuss the issue of repetition continuation between two NTN cells.

# References

- RP-220979, Revised WID on IoT NTN enhancements, 3GPP TSG RAN Meeting #95e, Electronic Meeting, March 17 - 23, 2022.

- 3GPP TR 36.763, V17.0.0, "Study on NB-IoT/eMTC support for non-terrestrial networks".