3GPP TSG RAN WG1 #109	R1-2203845

e-Meeting, 09 – 20 May, 2022

Agenda item:		9.12.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Other aspects related to NTN operation for Rel-18

Document for:		Discussion and Decision

# Introduction

In this contribution we discuss some of the aspects that does not naturally fall under the main agenda items under this topic. The items discussed here will be related to polarization aspects for NTN operation for Rel-18 as well as a discussion on which channels and priorities should be targeted based on analysis of TR38.830. We also discussed the need for further enhancement to discontinuous coverage of IoT NTN, considering the potential aspects from RAN1.

# Discussion

## Polarization aspects for NR over NTN for Rel-18

At the previous RAN1 e-meetings as part of the Rel-17 work, there were continued discussions related to the aspect of potentially providing an indication of the polarization configuration on a per-SSB level, which would allow the gNB to utilize a potential orthogonalization between different beams which may be served by left-hand and right-hand circular polarizations respectively.

From system level perspective, we however see such an approach as highly unattractive. There are several reasons for this, where the main obstacles are:

- Increased system overhead for indication of the polarization for each beam

- Challenges in performing intra-slot UE multiplexing

- Need for mandatory UE support for various polarization modes

The increased system overhead gives itself, while the two other points are caused by the physical properties of the polarization. As the polarization is directly coupled to the transmission node of the satellite antenna(s), it would by nature apply to the entire transmission as part of the system bandwidth, and switching between transmit antennas would normally not be possible or feasible between symbols. Hence, it would be expected that transmission using a given polarization mode would apply to at least one slot at a time. Following this, the gNB would need to perform ”grouping” of UEs for scheduling according to the different UEs and their respective capabilities in the system. Further, in order for the gNB to utilize such polarization orthogonalization between multiple UEs it is important that UEs will be able to indicate their support for such polarization operation, and preferably, all UEs should mandatorily support at least two polarization modes.

Observation 1: Polarization diversity will cause challenges for intra-slot UE multiplexing.

Proposal 1: For polarization diversity to be feasible in Rel-18 substantial gains should be shown, and UEs should be offering mandatory support for at least two polarization modes.

One additional aspect that need consideration for polarization diversity is the fact that UL and DL polarization diversity may see different challenges. For instance, in case the gNB has configured two neighboring cells with left-hand circular polarization and right-hand circular polarization for introducing orthogonalization between overlapping geographical areas. For such a scenario it is apparant that there may be a benefit of supporting polarization, but once more there are at least two main impacts here:

- No-coverage in DL between the two different circular polarization modes

- Cross-interference in UL between circular and linear polarized UEs

Due to the orthogonal nature of the two circular polarization modes, it should be noted that a UE supporting only RHCP would not be able to receive any LHCP signals that are transmitted from the gNB (or rather from the satellite). Hence, it would not be feasible to operate any cells under this kind of mode. And UE using linear polarization for its reception would naturally be able to receive the DL signals, but with a 3 dB loss compared to the UE having the correct polarization mode implemented in the first place. Further, there is the aspect of cross-interference in the UL where for instance a UE transmitting with a linear polarization transmitting towards a satellite with circular polarization would be (a) having a 3 dB loss by not having a matched polarization, and (b) causing interference to any satellite beam covering the same area and cause collisions for e.g. the Random Access procedure.

It should be noted that under current NR over NTN for Rel-17, the support for different polarization modes is voluntary for the UEs, the only support that is provided is for the gNB to indicate through optional signaling which DL and UL polarization mode that is used within a cell.

Proposal 2: All UEs in the cells are mandatorily supporting at least LHCP and RHCP for Rel-18 coverage enhancements.

Proposal 3: Any evaluations of polarization based coverage enhancements shall take support for Rel-17 UEs into account.

## Potential additional coverage enhancements for NTN support in Rel-18

From the description provided for this work, it is stated that coverage enhancement candidates from TR38.830 [1] should be considered. Some of the existing elements from existing Rel-17 coverage enhancements are discussed in our contribution on AI 9.12.1 [3]. However, this contribution does not go too much in details with respect to the evaluations as presented in TR38.830. To have a good starting point for the discussions, our starting point would be to use Table 5.1.1.5-3 from [1] since it represents the needed values for maximum path loss for the 2 GHz Rural case with FDD operation (which is the baseline assumption for NR over NTN for Rel-17). This table is copied for reference in the following.

Table 5.1.1.5-3: Representative values of MPL for Rural 2GHz FDD NLOS O2I scenario

| Channels (and Frame format) | Number of Samples | Representative value | Standard Deviation (w/o outlier) | Gap from Deployment dependent target 1732m ISD=125.6dB |
| --- | --- | --- | --- | --- |
| PUSCH for eMBB UUUUU | 9 | 127.16 | 2.40 | 1.61 |
| PUSCH for VoIP UUUUU | 12 | 129.20 | 1.97 | 3.65 |
| PUSCH with SIP invite UUUUU | 1 | 123.46 | 0.00 | -2.09 |
| PUSCH for CSI 11bit UUUUU | 0 |  |  |  |
| PUSCH for CSI 22bit UUUUU | 0 |  |  |  |
| PUCCH Format 1 | 9 | 130.02 | 2.17 | 4.47 |
| PUCCH Format 3 11bit | 7 | 127.24 | 2.24 | 1.69 |
| PUCCH Format 3 22bit | 6 | 125.76 | 0.91 | 0.21 |
| SSB | 3 | 136.97 | 6.12 | 11.42 |
| PRACH Format 0 | 2 | 128.42 | 3.18 | 2.87 |
| PRACH Format B4 | 4 | 122.45 | 0.96 | -3.10 |
| PRACH Format C2 | 0 | 0.00 | 0.00 |  |
| Broadcast PDCCH(PDCCH of Msg.2) | 2 | 131.80 | 0.91 | 6.24 |
| PDSCH for Msg.2 | 1 | 134.93 | 0.00 | 9.38 |
| PUSCH of Msg.3 | 7 | 128.86 | 3.92 | 3.31 |
| PDSCH of Msg.4 | 2 | 135.01 | 0.66 | 9.45 |
| PUCCH w/ HARQ-ACK for Msg.4 | 2 | 126.11 | 5.70 | 0.55 |
| Unicast PDCCH | 8 | 140.37 | 1.76 | 14.81 |
| PDSCH for eMBB UUUUU | 8 | 140.58 | 1.86 | 15.02 |

As can be seen from the table, there is a general trend that UL channels (PRACH, PUCCH, PUSCH) are having a lower value for the MPL compared to the DL channels (SSB, PDCCH, PDSCH), meaning that it might be beneficial to focus the efforts for coverage enhancements on the UL coverage. From the table the following observations may be made:

Observation 2: PRACH format B4 is having substantially lower coverage than PRACH format 0.

Observation 3: PUCCH formats with lower payload will have better coverage than PUCCH formats with higher payloads.

Some may see such observations as obvious, but these may provide guidance to how to approach the NTN coverage enhancements for Rel-18. As is observed in the above the lost coverage for PRACH formats with a short preamble would easily be addressed by configuring a PRACH format with a longer duration, which in turn would ensure that there is additional time for the gNB to collect energy from the UE’s preamble transmission. In a similar manner, it is observed that network configurations with high payloads will suffer from less coverage, and again the solution would be natural – namely to decrease the payload for e.g. the UL control signaling.

Proposal 4: NTN coverage enhancements for Rel-18 should primarily focus on uplink channels.

Proposal 5: Improvements for any uplink channel should only be considered for cases where there is no alternative existing solutions for reaching similar improvement.

Proposal 6: RAN1 to consider approaches of introducing mechanisms for reducing the needed UL payload sizes.

## IoT NTN discontinuous coverage

Discontinuous coverage may be an important scenario for NTN, especially at early stage when small number of satellites are in the orbits. It will be an issue how to handle UE’s transmission/repetition when there is discontinuous coverage.

Based on recent agreement in RAN2 #116bis-e, the ephemeris for discontinuous coverage scenario will be the same as the serving cell ephemeris for UL pre-compensation:

The contents of the ephemeris / assistance info for non-continuous coverage: Confirm that we Reuse the satellite ephemeris orbital parameters, already agreed for UL pre-compensation, for multiple satellites (Ref L1 params from R1).

There are also other open issues related to paging and UE sleeping discussed by RAN2.

Therefore, it is not likely that the release 17 support for discontinuous coverage will result in efficient system operation, e.g. impacting UE battery life and network signalling.

However, the release 18 WID (RP-220979) contains a possible objective to continue the work:

4.1.3	Further enhancement to discontinuous coverage

This objective will be revisited at RAN#96e / June 2022, subject to Rel-17 IoT-NTN status on discontinuous coverage at RAN#96e, SA#94e, CT#95e.

Important aspects to consider from RAN1 perspective could be:

- Enhanced ephemeris for discontinuous coverage with low signaling overhead allowing for reasonable satellite position estimation (i.e. lower accuracy than ephemeris for UL pre-compensation)

- Data transfer continuation in discontinuous coverage, because the use of repetitions may result in a transfer not completing in time. This will also apply to the release 18 objective on enhanced mobility, because it is not limited to discontinuous coverage.

Observation 4: Further enhancements to discontinuous coverage for IoT-NTN are likely needed in release 18.

# Conclusion

In this contribution we have presented our observations and proposals which are as follows:

Observation 1: Polarization diversity will cause challenges for intra-slot UE multiplexing.

Observation 2: PRACH format B4 is having substantially lower coverage than PRACH format 0.

Observation 3: PUCCH formats with lower payload will have better coverage than PUCCH formats with higher payloads.

Observation 4: Further enhancements to discontinuous coverage for IoT-NTN are likely needed in release 18.

Proposal 1: For polarization diversity to be feasible in Rel-18 substantial gains should be shown, and UEs should be offering mandatory support for at least two polarization modes.

Proposal 2: All UEs in the cells are mandatorily supporting at least LHCP and RHCP for Rel-18 coverage enhancements.

Proposal 3: Any evaluations of polarization based coverage enhancements shall take support for Rel-17 UEs into account.

Proposal 4: NTN coverage enhancements for Rel-18 should primarily focus on uplink channels.

Proposal 5: Improvements for any uplink channel should only be considered for cases where there is no alternative existing solutions for reaching similar improvement.

Proposal 6: RAN1 to consider approaches of introducing mechanisms for reducing the needed UL payload sizes.

# References

- TR 38.380, ”Technical Specification Group Radio Access Network; Study on NR coverage enhancements (Release 17)”, v. 17.0.0, 3GPP

- RP-220953, “NR NTN (Non-Terrestrial Networks) enhancements”, Thales

- R1-2203844, “Solutions for coverage enhancements in NR over NTN”, Nokia, Nokia Shanghai Bell