3GPP TSG RAN WG1 #109-e		R1-2203849

e-Meeting, May 9th – 20th, 2022

Title: 	Discussion on RAN2 LS on PDCCH Blind Detection in CA

Source: 	Samsung

Agenda item:	5

Document for:	Discussion and decision

# Introduction

In RAN1 #100b-e and RAN1 #101-e the following agreements were reached to define UE behavior for scaling PDCCH monitoring capability for slot-based (Rel-15) and span-based (Rel-16) monitoring in CA and DC framework.

Agreements: For the case with Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cells (i.e. case 3), UE will report one or more combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16) as UE capability.  If UE reports more than one combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16), - gNB configure which combination for UE to use for scaling PDCCH monitoring capability if the number of CCs configured is larger than the reported capability Agreement For NR-DC operation with at least one downlink cell using Rel-16 PDCCH monitoring capability and at least one downlink cell using Rel-15 PDCCH monitoring capability, and if a UE reports the capability of CC limits for NR-DC operation separately from CA operation, - UE reports one or more combination of (pdcch-BlindDetectionMCG-UE-r15, pdcch-BlindDetectionSCG-UE-r15, pdcch-BlindDetectionMCG-UE-r16, pdcch-BlindDetectionSCG-UE-r16) - One combination of (pdcch-BlindDetectionMCG-UE-r15, pdcch-BlindDetectionSCG-UE-r15, pdcch-BlindDetectionMCG-UE-r16, pdcch-BlindDetectionSCG-UE-r16) corresponds to one combination of (pdcch-BlindDetectionCA-r15, pdcch-BlindDetectionCA-r16) reported by a UE for CA operation - pdcch-BlindDetectionMCG-UE-r16 + pdcch-BlindDetectionSCG-UE-r16 >= pdcch-BlindDetectionCA-r16 - pdcch-BlindDetectionMCG-UE-r15 + pdcch-BlindDetectionSCG-UE-r15>= pdcch-BlindDetectionCA-r15 - 3<= The minimum of pdcch-BlindDetectionMCG-UE-r16 + pdcch-BlindDetectionSCG-UE-r16 + pdcch-BlindDetectionMCG-UE-r15 + pdcch-BlindDetectionSCG-UE-r15 <=16

In the incoming LS R1-2203045(R2-2204055), RAN2 raised questions on reporting these capabilities based on the latest Rel-16 UE feature table.

# Discussion on PDCCH blind detection in CA

RAN2 has defined capabilities for UE to report the combinations mentioned in the aforementioned agreements. Capabilities pdcch-BlindDetectionMCG-UE-Mixed-r16 and pdcch-BlindDetectionSCG-UE-Mixed are defined for reporting (pdcch-BlindDetectionMCG-UE-r15, pdcch-BlindDetectionSCG-UE-r15, pdcch-BlindDetectionMCG-UE-r16, pdcch-BlindDetectionSCG-UE-r16). gNB provides UE with (pdcch-BlindDetection3 for MCG, pdcch-BlindDetection3 for SCG, pdcch-BlindDetection2 for MCG, pdcch-BlindDetection2 for SCG ) to determine the number of cells for scaling the BD/CCE limits for Rel-15 and Rel-16 PDCCH monitoring on MCG and SCG. The provided 4-tuple by gNB needs to satisfy the corresponding maximum values reported by UE as shown below.

TS 38.213 V16.9.0 “When a UE is configured for NR-DC operation and is provided monitoringCapabilityConfig = r15monitoringcapability for at least one downlink cell and monitoringCapabilityConfig = r16monitoringcapability for at least one downlink cell where the UE monitors PDCCH, the UE may indicate, through pdcch-BlindDetectionMCG-UE1 and pdcch-BlindDetectionSCG-UE1, respective maximum values for pdcch-BlindDetection3 for the MCG and pdcch-BlindDetection3 for the SCG, and through pdcch-BlindDetectionMCG-UE2 and pdcch-BlindDetectionSCG-UE2 respective maximum values for pdcch-BlindDetection2 for the MCG and pdcch-BlindDetection2 for the SCG.”

The provided 4-tuple also needs to satisfy the reported (pdcch-BlindDetectionCA1, pdcch-BlindDetectionCA2) by UE.

TS 38.213 V16.9.0 “When a UE is configured for NR-DC operation with a total of NNR-DCDL,cells downlink cells on both the MCG and the SCG and the UE is provided monitoringCapabilityConfig = r15monitoringcapability for NNR-DC,r15DL,cells downlink cells and monitoringCapabilityConfig = r16monitoringcapability for NNR-DC,r16DL,cells downlink cells where the UE monitors PDCCH, the UE expects to be provided pdcch-BlindDetection3 and pdcch-BlindDetection2 for the MCG, and pdcch-BlindDetection3 and pdcch-BlindDetection2 for the SCG with values that satisfy - pdcch-BlindDetection3 for the MCG + pdcch-BlindDetection3 for the SCG <= pdcch-BlindDetectionCA1, if the UE reports pdcch-BlindDetectionCA1, or - pdcch-BlindDetection3 for the MCG + pdcch-BlindDetection3 for the SCG <= NNR-DC,r15DL,cells, if the UE does not report pdcch-BlindDetectionCA1 and - pdcch-BlindDetection2 for the MCG + pdcch-BlindDetection2 for the SCG <= pdcch-BlindDetectionCA2, if the UE reports pdcch-BlindDetectionCA2, or - pdcch-BlindDetection2 for the MCG + pdcch-BlindDetection2 for the SCG <= NNR-DC,r16DL,cells, if the UE does not report pdcch-BlindDetectionCA2 ”

According to the current RAN1 spec. UE may indicate the capabilities for MCG and SCG. If it reports for both CGs then the provided (pdcch-BlindDetection3 for MCG, pdcch-BlindDetection3 for SCG, pdcch-BlindDetection2 for MCG, pdcch-BlindDetection2 for SCG) satisfies the reported maximum numbers. However, it seems that UE doesn’t need to report the capability for both CGs. In that case, there is no explicit maximum supported values for the non-reported CG. For example if UE doesn’t report pdcch-BlindDetectionSCG-UE-Mixed, then there is no explicit limit for pdcch-BlindDetection3 for SCG and pdcch-BlindDetection2 for SCG, but they still need to satisfy the CA limits given in the above excerpt from TS 38.213.

Question 1. RAN2 defined two separate PDCCH blind detection capabilities for MCG (pdcch-BlindDetectionMCG-UE-Mixed-r16) and SCG (pdcch-BlindDetectionSCG-UE-Mixed-r16). In current RAN2 specification, there is no requirement for UE supporting NR-DC to report the capabilities for both MCG and SCG. RAN2 respectively asks RAN1 to clarify whether the wording above means that UE is required to report both capabilities for MCG and SCG? RAN2 concerns if adding such restriction leads to a non-backward compatible change for Release-16.

Answer 1: In our understanding UE is not required to report the capabilities for both MCG and SCG. If the capability is not reported for a CG, then there is no explicit maximum supported values of pdcch-BlindDetection3 or pdcch-BlindDetection2 for the CG.

Another question was raised by RAN2 on simultaneous reporting of FG 11-2c and 11-2g. The FGs are shown in the following table. As can be seen, there can be some cases where interpretation of the reporting can be ambiguous. For example if UE reports 11-2c with support of “aligned spans only” and it also reports 11-2g, there will be conflicting indication of supported alignments. Also if UE reports different values of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16) for 11-2c and 11-2g, e.g. (3,4) for 11-2c and (4,3) for 11-2g with supports of both aligned and non-aligned spans, which set of numbers should be used for scaling the BD/CCE limit is not clear. Although it is possible to define a general behaver based on a “higher” capability among the two, how to define a higher capability itself is not straightforward. Therefore unless RAN2 is willing to define a higher capability among the two, UE should be restricted to only report one of these two capabilities.

| 11-2c | Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 16 and Rel. 15 PDCCH monitoring capabilities on different carriers | - Supported combination(s) of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16) - Candidate values for pdcch-BlindDetectionCA-R15 is 1 to 15 - Candidate values for pdcch-BlindDetectionCA-R16 is 1 to 15 - Supported span arrangement for CA - Candidate value for the component: {aligned spans only, aligned spans and non-aligned spans} |
| --- | --- | --- |
| 11-2g | Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 16 and Rel. 15 PDCCH monitoring capabilities on different carriers with restriction for non-aligned span case | - Supported combination(s) of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16) - Candidate values for pdcch-BlindDetectionCA-R15 is 1 to 15 - Candidate values for pdcch-BlindDetectionCA-R16 is 1 to 15 - UE supports aligned span and non-aligned span In case of non-aligned span when the configured number of cells with Rel-16 PDCCH monitoring is larger than the UE reported value, PDCCH monitoring occasion(s) should be configured only on same symbol(s) every slot |

Question 2 RAN2 would also like to point out that regarding note 3 and note 4, in existing RAN2 specification there is no such restriction that only one of FG 11-2c and FG 11-2g is reported if supported, and no restriction that only one of FG 11-2a and FG 11-2f is reported if supported. To ensure backward compatibility, RAN2 respectively asks RAN1 to clarify how to interpret it if the UE reports both of FG 11-2c and FG 11-2g, or both of FG 11-2a and FG 11-2f?

Answer 2: If UE reports both FG 11-2c and 11-2g or both 11-2a and 11-2f, the report can be interpreted to be the higher capability. How to choose the higher capability between the two needs further discussion.

# Conclusion

In conclusion, we provide the following answers to the questioned asked in RAN2 LS.

Question 1. RAN2 defined two separate PDCCH blind detection capabilities for MCG (pdcch-BlindDetectionMCG-UE-Mixed-r16) and SCG (pdcch-BlindDetectionSCG-UE-Mixed-r16). In current RAN2 specification, there is no requirement for UE supporting NR-DC to report the capabilities for both MCG and SCG. RAN2 respectively asks RAN1 to clarify whether the wording above means that UE is required to report both capabilities for MCG and SCG? RAN2 concerns if adding such restriction leads to a non-backward compatible change for Release-16.

Answer 1: In our understanding UE is not required to report the capabilities for both MCG and SCG. If the capability is not reported for a CG, then there is no explicit maximum supported values of pdcch-BlindDetection3 or pdcch-BlindDetection2 for the CG.

Question 2 RAN2 would also like to point out that regarding note 3 and note 4, in existing RAN2 specification there is no such restriction that only one of FG 11-2c and FG 11-2g is reported if supported, and no restriction that only one of FG 11-2a and FG 11-2f is reported if supported. To ensure backward compatibility, RAN2 respectively asks RAN1 to clarify how to interpret it if the UE reports both of FG 11-2c and FG 11-2g, or both of FG 11-2a and FG 11-2f?

Answer 2: If UE reports both FG 11-2c and 11-2g or both 11-2a and 11-2f, the report can be interpreted to be the higher capability. How to choose the higher capability between the two needs further discussion.

# References

R1-2203045(R2-2204055), Reply LS to RAN1 on Blind Detection in CA, RAN2.