3GPP TSG RAN WG1 #109-e		R1-2203851

e-Meeting, May 9th – 20th, 2022

Agenda Item:		7.2.5

Source:		Samsung

Title:		Remaining issues on intra-UE multiplexing/prioritization

Document for:		Discussion and decision

Introduction

In RAN1#108-e, intra UE multiplexing/prioritization for more than 2 overlapping PUCCHs including repetitions and/or PUSCHs was discussed under Rel-17 URLLC. It seems companies have different understanding on UE behaviour of the same priority. This contribution discusses intra UE multiplexing/prioritization for more than 2 overlapping PUCCHs including repetitions and/or PUSCHs.

Discussion

More than two overlapping PUCCHs with repetitions of the same priority

In TS 38.213 9.2.6, resolving two overlapping PUCCHs including at least one PUCCH with repetitions was defined, however, for more than 2 overlapping PUCCHs including at least one PUCCH with repetitions, UE behaviour is not clear.

For example, as shown in Figure 1, a SR PUCCH repetition overlaps with both HARQ-ACK PUCCH and CSI PUCCH where the HARQ-ACK PUCCH and CSI PUCCH are not overlapping. There can be two UE behaviours.

Behaviour 1:  UE first resolves the collision of SR and CSI and UE drops CSI and then UE resolves the collision of SR and HARQ-ACK, the SR will be dropped. The UE will transmit the HARQ-ACK PUCCH.

Behaviour 2: UE first resolves the collision of SR and HARQ-ACK, the SR will be dropped and UE can transmit both HARQ-ACK and CSI.

Figure 1

There are similar cases that result in unclear UE behaviour, a unified solution is preferred. A simple solution can be that UE chooses up to 2 PUCCHs according to selection order defined in the pseudo-code in TS 38.213 9.2.5 and then UE determines a single PUCCH according to the rules defined in clause 9.2.6 if there is repetition or clause 9.2.5, otherwise.

Proposal 1: For resolving more than two overlapping PUCCHs of the same priority, reuse the pseudo-code in TS 38.213 9.2.5 with the restriction of selecting up to 2 PUCCH resources and apply the rules defined in clause 9.2.6 if there is repetition or clause 9.2.5, otherwise. Adopt the following TP for section 9.2.5 and 9.2.6 of TS 38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9.2.5 UE procedure for reporting multiple UCI types

<Unchanged text omitted>

Set  to the cardinality of

Set to be the first symbol of resource  in the slot

Set  to be the number of symbols of resource  in the slot

Set  - index of first resource in set

Set  - counter of overlapped resources

while

if  and resource  overlaps with resource  and the resources in set Q does not include a PUCCH with repetitions, or

j<CQ-1 and resource Q(j-o) overlaps with resource Q(j+1), o=0, and the resources in set Q includes at least a PUCCH with repetitions

else

if

determine a single resource for multiplexing UCI associated with resources  as described in clauses 9.2.5.0, 9.2.5.1, and 9.2.5.2 and 9.2.6

set the index of the single resource to

% start from the beginning after reordering unmerged resources at next step

% function that re-orders resources in current set

Set  to the cardinality of

else

end if

end if

end while

<Unchanged text omitted>

9.2.6	PUCCH repetition procedure

<Unchanged text omitted>

A UE does not multiplex different UCI types in a PUCCH transmission with repetitions over NPUCCHrepeat>1 slots. If a UE would transmit a first PUCCH over more than one slot and at least a second PUCCH over one or more slots, and the transmissions of the first PUCCH and the second PUCCH would overlap in a number of slots then, for each slot of the number of slots and with UCI type priority of HARQ-ACK > SR > CSI with higher priority > CSI with lower priority

-	the UE does not expect the first PUCCH and any of the second PUCCHs to start at a same slot and include a UCI type with same priority

-	if the first PUCCH and any of the second PUCCHs include a UCI type with same priority, the UE transmits the PUCCH starting at an earlier slot and does not transmit the PUCCH starting at a later slot

-	if the first PUCCH and any of the second PUCCHs do not include a UCI type with same priority, the UE transmits the PUCCH that includes the UCI type with higher priority and does not transmit the PUCCH that include the UCI type with lower priority

If a UE would transmit more than two overlapping PUCCH transmissions where at least one PUCCH transmission is with repetitions over NPUCCHrepeat>1 slots, the UE resolves the overlapping for PUCCH transmissions within the slot using the pseudo-code in clause 9.2.5.

<Unchanged text omitted>

----------------------------------------------------- End of text proposal ------------------------------------------------------

More than two overlapping PUCCHs with repetitions and PUSCH(s) of the same priority

For the case of overlapping PUCCH and PUSCH, an example is given in Figure 2, in this case UE behaviour is not clear either. There can be two UE behaviours.

Behaviour 1:  UE first resolves the collision of SR and PUSCH and UE drops the PUSCH. The UE will transmit the HARQ-ACK and SR.

Behaviour 2: UE first resolves the collision of HARQ-ACK and PUSCH, the HARQ-ACK will be multiplexed in the PUSCH and then UE resolves the collision of SR and PUSCH. In this case, the PUSCH with HARQ-ACK will be dropped. The UE will transmit the SR PUCCH.

Figure 2

Similar issue was discussed in Rel-18 URLLC and we made the following agreement.

Agreement For resolving collision of overlapping PUCCHs and PUSCHs of different priorities in Step 2.2, - If a LP PUSCH overlaps with a HP PUCCH with repetitions, the LP PUSCH is dropped before multiplexing with HP HARQ-ACK if any. - If a HP PUSCH overlaps with a LP PUCCH with repetitions, the LP PUCCH is dropped.

Clearly, the Behaviour 1 has better performance and should be adopted.

Proposal 2: For resolving overlapping PUCCHs and PUSCH(s) of the same priority, UE first drops a PUSCH before multiplexing the UCI in the PUSCH if the PUSCH overlaps with a PUCCH with repetitions.

Proposal 3：Adopt the following TP for section 9 of TS38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9 UE procedure for reporting control information

<Unchanged text omitted>

If a UE has overlapping resources for PUCCH transmissions in a slot and at least one of the PUCCH transmissions is with repetitions over multiple slots, the UE first follows the procedures described in clause 9.2.6 for resolving the overlapping among the resources for the PUCCH transmissions.

If a UE has overlapping resources for PUCCH and PUSCH transmissions in a slot and at least one of the PUCCH transmissions is with repetitions over multiple slots, the UE first drops PUSCH transmissions that would overlap with a PUCCH transmission with repetitions as described in clauses 9.2.6, and then the UE resolves the overlapping for PUCCH transmissions without repetitions and for PUSCH transmissions, if any, described in this clause

<Unchanged text omitted>

----------------------------------------------------- End of text proposal ------------------------------------------------------

Overlapping PUCCHs with repetitions and/or PUSCH(s) of the different priorities

We have the following agreement in RAN1#102-e.

Agreement Confirm the following working assumption and remove the brackets as follows: - A UE behavior of handling intra-UE prioritization/multiplexing for overlapping UL transmissions on semi-static flexible symbols is not affected by UL cancellation due to dynamic SFI or [DL grant] - Note: The UE performs prioritization/multiplexing first and once done applies dynamic SFI

During the discussion, PUCCH repetitions were not considered. The case including PUCCH repetitions should be captured in the spec as well.

Proposal 4: Adopt the following TP for sections 11.1 and 11.1.1 of TS38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

11.1	Slot configuration

<Unchanged text omitted>

For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then

-	If the UE does not indicate the capability of [partialCancellation], the UE does not expect to cancel the transmission of the PUCCH or PUSCH or PRACH in the set of symbols if the first symbol in the set occurs within Tproc,2  relative to a last symbol of a CORESET where the UE detects the DCI format; otherwise, the UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS38.214], determined from clauses 9, and 9.2.5 and 9.2.6 or clause 6.1 of [6, TS38.214], or the PRACH transmission in the set of symbols.

-	If the UE indicates the capability of [partialCancellation], the UE does not expect to cancel the transmission of the PUCCH or PUSCH or PRACH in symbols from the set of symbols that occur within Tproc,2 relative to a last symbol of a CORESET where the UE detects the DCI format. The UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], determined from clauses 9, and 9.2.5 and 9.2.6 or clause 6.1 of [6, TS 38.214], or the PRACH transmission in remaining symbols from the set of symbols.

<Unchanged text omitted>

11.1.1	UE procedure for determining slot format

<Unchanged text omitted>

If a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as downlink or flexible, or the UE detects a DCI format indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then

-	If the UE does not indicate the capability of [partialCancellation], the UE does not expect to cancel the transmission of the PUCCH or PUSCH or PRACH in the set of symbols if the first symbol in the set occurs within Tproc,2 relative to a last symbol of a CORESET where the UE detects the DCI format; otherwise, the UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS38.214], determined from clauses 9, and 9.2.5 and 9.2.6 or clause 6.1 of [6, TS38.214], or the PRACH transmission in the set of symbols.

-	If the UE indicates the capability of [partialCancellation], the UE does not expect to cancel the transmission of the PUCCH or PUSCH or PRACH in symbols from the set of symbols that occur within Tproc,2 relative to a last symbol of a CORESET where the UE detects the DCI format. The UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], determined from clauses 9, and 9.2.5 and 9.2.6 or clause 6.1 of [6, TS 38.214], or the PRACH transmission in remaining symbols from the set of symbols.

<Unchanged text omitted>

-	if the UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in the set of symbols of the slot and the UE is not provided enableConfiguredUL, then

-	if the UE does not indicate the capability of [partialCancellation], the UE does not expect to cancel the transmission of the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], as determined in clauses 9, and 9.2.5 and 9.2.6 or in clause 6.1 of [6. TS 38.214], or the PRACH in the slot if the first symbol of the PUCCH or the PUSCH or actual repetition of the PUSCH or the PRACH in the slot occurs within Tproc,2 relative to a last symbol of a CORESET where the UE is configured to monitor PDCCH for DCI format 2_0; otherwise, the UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], as determined in clauses 9, and 9.2.5 and 9.2.6 or in clause 6.1 of [6. TS 38.214], or the PRACH in the slot;

-	if the UE indicates the capability of [partialCancellation], the UE does not expect to cancel the transmission of the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], as determined in clauses 9, and 9.2.5 and 9.2.6 or in clause 6.1 of [6. TS 38.214], or the PRACH in symbols from the set of symbols that occur within Tproc,2 relative to a last symbol of a CORESET where the UE is configured to monitor PDCCH for DCI format 2_0. The UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], as determined in clauses 9, and 9.2.5 and 9.2.6 or in clause 6.1 of [6. TS 38.214], or the PRACH transmission in remaining symbols from the set of symbols;

<Unchanged text omitted>

----------------------------------------------------- End of text proposal ------------------------------------------------------

Conclusions

This contribution discusses the remaining issues related to intra-UE multiplexing and prioritization of traffic with different priorities. The proposals are summarized below.

Proposal 1: For resolving more than two overlapping PUCCHs of the same priority, reuse the pseudo-code in TS 38.213 9.2.5 with the restriction of selecting up to 2 PUCCH resources and apply the rules defined in clause 9.2.6 if there is repetition or clause 9.2.5, otherwise. Adopt the following TP for section 9.2.5 and 9.2.6 of TS38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9.2.5 UE procedure for reporting multiple UCI types

<Unchanged text omitted>

Set  to the cardinality of

Set to be the first symbol of resource  in the slot

Set  to be the number of symbols of resource  in the slot

Set  - index of first resource in set

Set  - counter of overlapped resources

while

if  and resource  overlaps with resource  and the resources in set Q does not include a PUCCH with repetitions, or

j<CQ-1 and resource Q(j-o) overlaps with resource Q(j+1), o=0, and the resources in set Q includes at least a PUCCH with repetitions

else

if

determine a single resource for multiplexing UCI associated with resources  as described in clauses 9.2.5.0, 9.2.5.1, and 9.2.5.2 and 9.2.6

set the index of the single resource to

% start from the beginning after reordering unmerged resources at next step

% function that re-orders resources in current set

Set  to the cardinality of

else

end if

end if

end while

<Unchanged text omitted>

9.2.6	PUCCH repetition procedure

<Unchanged text omitted>

A UE does not multiplex different UCI types in a PUCCH transmission with repetitions over NPUCCHrepeat>1 slots. If a UE would transmit a first PUCCH over more than one slot and at least a second PUCCH over one or more slots, and the transmissions of the first PUCCH and the second PUCCH would overlap in a number of slots then, for each slot of the number of slots and with UCI type priority of HARQ-ACK > SR > CSI with higher priority > CSI with lower priority

-	the UE does not expect the first PUCCH and any of the second PUCCHs to start at a same slot and include a UCI type with same priority

-	if the first PUCCH and any of the second PUCCHs include a UCI type with same priority, the UE transmits the PUCCH starting at an earlier slot and does not transmit the PUCCH starting at a later slot

-	if the first PUCCH and any of the second PUCCHs do not include a UCI type with same priority, the UE transmits the PUCCH that includes the UCI type with higher priority and does not transmit the PUCCH that include the UCI type with lower priority

If a UE would transmit more than two overlapping PUCCH transmissions where at least one PUCCH transmission is with repetitions over NPUCCHrepeat>1 slots, the UE resolves the overlapping for PUCCH transmissions within the slot using the pseudo-code in clause 9.2.5.

<Unchanged text omitted>

----------------------------------------------------- End of text proposal ------------------------------------------------------

Proposal 2: For resolving overlapping PUCCHs and PUSCH(s) of the same priority, UE first drops a PUSCH before multiplexing the UCI in the PUSCH if the PUSCH overlaps with a PUCCH with repetitions.

Proposal 3：Adopt the following TP for section 9 of TS38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

9 UE procedure for reporting control information

<Unchanged text omitted>

If a UE has overlapping resources for PUCCH transmissions in a slot and at least one of the PUCCH transmissions is with repetitions over multiple slots, the UE first follows the procedures described in clause 9.2.6 for resolving the overlapping among the resources for the PUCCH transmissions.

If a UE has overlapping resources for PUCCH and PUSCH transmissions in a slot and at least one of the PUCCH transmissions is with repetitions over multiple slots, the UE first drops PUSCH transmissions that would overlap with a PUCCH transmission with repetitions as described in clauses 9.2.6, and then the UE resolves the overlapping for PUCCH transmissions without repetitions and for PUSCH transmissions, if any, described in this clause

<Unchanged text omitted>

----------------------------------------------------- End of text proposal ------------------------------------------------------

Proposal 4: Adopt the following TP for sections 11.1 and 11.1.1 of TS38.213.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

11.1	Slot configuration

<Unchanged text omitted>

For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then

-	If the UE does not indicate the capability of [partialCancellation], the UE does not expect to cancel the transmission of the PUCCH or PUSCH or PRACH in the set of symbols if the first symbol in the set occurs within Tproc,2  relative to a last symbol of a CORESET where the UE detects the DCI format; otherwise, the UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS38.214], determined from clauses 9, and 9.2.5 and 9.2.6 or clause 6.1 of [6, TS38.214], or the PRACH transmission in the set of symbols.

-	If the UE indicates the capability of [partialCancellation], the UE does not expect to cancel the transmission of the PUCCH or PUSCH or PRACH in symbols from the set of symbols that occur within Tproc,2 relative to a last symbol of a CORESET where the UE detects the DCI format. The UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], determined from clauses 9, and 9.2.5 and 9.2.6 or clause 6.1 of [6, TS 38.214], or the PRACH transmission in remaining symbols from the set of symbols.

<Unchanged text omitted>

11.1.1	UE procedure for determining slot format

<Unchanged text omitted>

If a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as downlink or flexible, or the UE detects a DCI format indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then

-	If the UE does not indicate the capability of [partialCancellation], the UE does not expect to cancel the transmission of the PUCCH or PUSCH or PRACH in the set of symbols if the first symbol in the set occurs within Tproc,2 relative to a last symbol of a CORESET where the UE detects the DCI format; otherwise, the UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS38.214], determined from clauses 9, and 9.2.5 and 9.2.6 or clause 6.1 of [6, TS38.214], or the PRACH transmission in the set of symbols.

-	If the UE indicates the capability of [partialCancellation], the UE does not expect to cancel the transmission of the PUCCH or PUSCH or PRACH in symbols from the set of symbols that occur within Tproc,2 relative to a last symbol of a CORESET where the UE detects the DCI format. The UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], determined from clauses 9, and 9.2.5 and 9.2.6 or clause 6.1 of [6, TS 38.214], or the PRACH transmission in remaining symbols from the set of symbols.

<Unchanged text omitted>

-	if the UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in the set of symbols of the slot and the UE is not provided enableConfiguredUL, then

-	if the UE does not indicate the capability of [partialCancellation], the UE does not expect to cancel the transmission of the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], as determined in clauses 9, and 9.2.5 and 9.2.6 or in clause 6.1 of [6. TS 38.214], or the PRACH in the slot if the first symbol of the PUCCH or the PUSCH or actual repetition of the PUSCH or the PRACH in the slot occurs within Tproc,2 relative to a last symbol of a CORESET where the UE is configured to monitor PDCCH for DCI format 2_0; otherwise, the UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], as determined in clauses 9, and 9.2.5 and 9.2.6 or in clause 6.1 of [6. TS 38.214], or the PRACH in the slot;

-	if the UE indicates the capability of [partialCancellation], the UE does not expect to cancel the transmission of the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], as determined in clauses 9, and 9.2.5 and 9.2.6 or in clause 6.1 of [6. TS 38.214], or the PRACH in symbols from the set of symbols that occur within Tproc,2 relative to a last symbol of a CORESET where the UE is configured to monitor PDCCH for DCI format 2_0. The UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], as determined in clauses 9, and 9.2.5 and 9.2.6 or in clause 6.1 of [6. TS 38.214], or the PRACH transmission in remaining symbols from the set of symbols;

<Unchanged text omitted>

----------------------------------------------------- End of text proposal ------------------------------------------------------

- Reference

[1] R1- 2112889, Summary of email discussion [107-e-NR-L1enh-URLLC-05] on UCI multiplexing and prioritization in Rel-16, Moderator (Apple), RAN1#107-e, November. 2021.

[2] R1- 2112712, Summary #2 of email thread [107-e-NR-R17-IIoT-URLLC-03], Moderator (CATT), RAN1#107-e, November. 2021.