3GPP TSG RAN WG1 #109-e			R1-2203857

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.1.3

Source: 	Samsung

Title: 	Maintenance on Rel-17 SRS

Document for:	Discussion and Decision

Introduction

In Rel.17 NR FeMIMO WID, enhancements on SRS are given as follows [1]:

- Enhancement on SRS, targeting both FR1 and FR2: - Identify and specify enhancements on aperiodic SRS triggering to facilitate more flexible triggering and/or DCI overhead/usage reduction - Specify SRS switching for up to 8 antennas (e.g., xTyR, x = {1, 2, 4} and y = {6, 8}) - Evaluate and, if needed, specify the following mechanism(s) to enhance SRS capacity and/or coverage: SRS time bundling, increased SRS repetition, partial sounding across frequency

This contribution provides Samsung’s view on the highlighted topic above.

SRS enhancements

  - Discussion on inter-set guard period

In RAN1#108-e [2], it has been discussed that how to handle the case where the interval between SRS resource sets is larger than Y where Y is the time gap between SRS resources which is already defined in current specification. There are two points to discuss as follows.

- 1) How long the interval is needed

- 2) Given the interval, whether UL/DL signal is allowed or not (i.e., the existence of scheduling restriction).

Regarding 1), since RAN4 informs that the transient period between SRS resources is 15us, if UL transmission exists the interval between SRS resource sets, then the first Y symbols of the interval (from the end of the last OFDM symbol of SRS resource set as shown by orange GP in the following Figure 1) and the last Y symbols of the interval (before the start of the first OFDM symbol of SRS resource set as shown by yellow GP in the following Figure 1) should be needed. It is also described in Figure 6.3.3.7-1 and 6.3.3.7-2 in TS38.101-1, that is, the required locations of a transient time (15us) is defined before and after SRS transmission, which means that the transient time should be required between SRS transmission and PUSCH/PUCCH transmission, and also between PUSCH/PUCCH transmission and SRS transmission. We think that this is the same case which PUSCH/PUCCH transmission is located in the interval between two SRS resource sets. Hence, we support that guard period (for transient period) should be the first and the last Y symbols of the interval between SRS resource sets.

Figure 1. Required inter-set guard period between SRS resource sets

Regarding 2), based on RAN4 LS [3], we can figure out that RAN4 also thinks that restriction on transmitting PUSCH and PUCCH in the interval between SRS resources belonging to different sets where usage is set to antenna switching is inefficient. Hence, given the interval, there is no need to make a scheduling restriction.

If the change is not approved, then the consequence is that it is ambiguous how to handle the case where the interval between SRS resource sets is larger than Y, e.g., whether UL/DL signal can be allowed or not. Hence, we support to adopt the following TP.

Proposal 1: Adopt the following TP in TS 38.214.

- 6.2.1.2 UE sounding procedure for DL CSI acquisition <Unchanged text omitted> The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set. For two SRS resource sets of an antenna switching located in two consecutive slots, if UE is capable of transmitting SRS in all symbols in one slot, a guard period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot. For the inter-set guard period, the UE does not transmit any other signal on any symbols of the interval if the interval between SRS resource sets is Y symbols. - When both the SRS resource on all of the corresponding symbols prior to the gap and the SRS resource on all of the corresponding symbols after the gap are dropped due to collision handling, the gap period is also dropped with same priority and can be used for UL transmission. For the inter-set guard period, if the interval between two SRS resource sets for antenna switching is larger than 2Y symbols, the first Y symbols and the last Y symbols of the interval are reserved for scheduling restriction. <Unchanged text omitted>

Conclusion

In this contribution, the following observations and proposals are given:

Proposal 1: Adopt the following TP in TS 38.214.

- 6.2.1.2 UE sounding procedure for DL CSI acquisition <Unchanged text omitted> The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set. For two SRS resource sets of an antenna switching located in two consecutive slots, if UE is capable of transmitting SRS in all symbols in one slot, a guard period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot. For the inter-set guard period, the UE does not transmit any other signal on any symbols of the interval if the interval between SRS resource sets is Y symbols. - When both the SRS resource on all of the corresponding symbols prior to the gap and the SRS resource on all of the corresponding symbols after the gap are dropped due to collision handling, the gap period is also dropped with same priority and can be used for UL transmission. For the inter-set guard period, if the interval between two SRS resource sets for antenna switching is larger than 2Y symbols, the first Y symbols and the last Y symbols of the interval are reserved for scheduling restriction. <Unchanged text omitted>

References

- 3GPP RAN, RP-193133, Samsung, New WID: further enhancements on MIMO for NR

- 3GPP RAN1, RAN1#108-e Chair’s note

- R1-2200895, LS on Rel-17 FeMIMO SRS related impact.