3GPP TSG RAN WG1 #109-e	R1-2203859

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.2.2

Source:	Samsung

Title:	Maintenance on PDCCH monitoring enhancements for NR from 52.6 GHz to 71 GHz

Document for:	Discussion and Decision

# Introduction

This contribution discusses the remaining issues for the PDCCH monitoring enhancements for NR 52.6 to 71 GHz, including the following aspects, and the TPs, if needed, associated with the proposals are listed in the appendix:

- Remaining issues for SSSG switching in multi-slot PDCCH monitoring

# Remaining issues for SSSG switching in multi-slot PDCCH monitoring

In RAN1 #108, SSSG switching has been agreed to be supported, and it was also agreed that the boundary of SSSG switching is always aligned with the boundary of a slot group. However, there is still a remaining issue left that whether the switching is allowed for different combinations of Xs, Ys or only allowed for same combination of Xs, Ys. Actually, this discussion is related to the definition of the combination of Xs, Ys, and one of the following options need to be clarified:

- Option 1: Combination Xs, Ys is defined for all SS sets within the active SSSG of an active BWP.

- Option 2: Combination Xs, Ys is defined for all SS sets within all the SSSGs of an active BWP.

In Option 1, only the SS sets within the active SSSG contribute to the determination of the combination Xs, Ys, and the configuration of SS sets in the two SSSGs can be relatively independent. For example, the gNB can configure the SS sets in the first SSSG according to a first UE reported capability on combination Xs, Ys, and configured the SS sets in the second SSSG according to a second UE reported capability on combination Xs, Ys. A UE determines the combination Xs, Ys for each SSSG separately, and the CCE and BD budget are also counted separately. For Option 1, there could be the case that the time difference between two MOs (e.g. in Group (1) SS sets) or the starting of Ys in SSSG switching (e.g. one corresponding to the combination Xs, Ys before switching, and the other one corresponding to the combination Xs, Ys after switching) is not larger than Xs, or even back-to-back slot MOs can happen. To resolve this issue, either introducing limitation to SS set configuration, or skipping MOs before/after switching can be considered.

In Option 2, all the SS sets within all the SSSG contribute to the determination of the combination Xs, Ys, and the configuration of SS sets in the two SSSGs should be considered jointly. For example, the gNB needs to jointly consider the configuration of SS sets in the two SSSGs, such that the UE can determine a joint combination Xs, Ys for all the SS sets. The benefit of this option is no need to consider the issue of time difference of neighboring Ys smaller than Xs (since this has been handled by gNB’s configuration already), but it introduces restriction on the SS set configuration, and limit the BD/CCE budget in some cases.

Here are some detailed examples for 960 kHz SCS, comparing Option 1 and Option 2, wherein each SSSG is assumed to have one SS set and only Group (1) SS sets are considered for simplicity (e.g. the first SSSG has SS set configuration #1, and the second SSSG has SS set configuration #2).

| Example # | First SSSG SS set #1 bitmap | Second SSSG SS set #2 bitmap | Option | UE’s determined combination Xs, Ys | (BD budget, CCE budget) | Distance between starting of Ys when switching |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1000 | 0100 | Option 1 | (4, 1) | (10, 16) | 5 |
|  |  |  | Option 2 | (4, 2) | (10, 16) | 4 |
| 2 | 0100 | 1000 | Option 1 | (4, 1) | (10, 16) | 3 |
|  |  |  | Option 2 | (4, 2) | (10, 16) | 4 |
| 3 | 1000 | 0010 | Option 1 | (4, 1) | (10, 16) | 6 |
|  |  |  | Option 2 | Cannot support |  |  |
| 4 | 1000 | 10000000 | Option 1 | (4, 1) or (8, 1) | (10, 16) or (20, 32) | 4 |
|  |  |  | Option 2 | (4, 1) | (10, 16) | 4 |
| 5 | 1000 | 00001000 | Option 1 | (4, 1) or (8, 1) | (10, 16) or (20, 32) | 4 |
|  |  |  | Option 2 | Cannot support |  |  |

From above analysis, it can be observed that although Option 2 is simpler, it has too much restriction in SS set configuration for the SSSGs, which is violating the intention to support SSSG switching, and may not efficiently allow large number of BD/CCE budget, so Option 1 is more preferable, and further discussion on how to mitigate/eliminate the issue with close MO in switching can be further discussed.

Proposal 1: For SSSG switching in multi-slot based PDCCH monitoring, support combination Xs, Ys being defined for all SS sets within the active SSSG of an active BWP, and further study the issue of close MOs in SSSG switching.

- Adopt TP#1 for TS 38.213.

If Option 1 is adopted, then correspondingly, SSSG switching between different combinations Xs, Ys should be supported, and one more issue to clarify is, the slot group boundary that SSSG switching happens should corresponds to the slot group after SSSG switching.

Proposal 2: For SSSG switching in multi-slot based PDCCH monitoring, the slot group boundary that SSSG switching happens corresponds to the slot group after SSSG switching.

- Adopt TP#2 for TS 38.213.

# Conclusion

The proposals made in this contribution are summarized below:

Proposal 1: For SSSG switching in multi-slot based PDCCH monitoring, support combination Xs, Ys being defined for all SS sets within the active SSSG of an active BWP, and further study the issue of close MOs in SSSG switching.

- Adopt TP#1 for TS 38.213.

Proposal 2: For SSSG switching in multi-slot based PDCCH monitoring, the slot group boundary that SSSG switching happens corresponds to the slot group after SSSG switching.

- Adopt TP#2 for TS 38.213.

# Appendix

## TP#1 for TS 38.213

============================== Start of TP #1 for TS 38.213 ==================================

10 	UE procedure for receiving control information

=============================== Unchanged Text Omitted ===================================

For SCS configuration μ=5 or μ=6, a UE can indicate a capability to monitor PDCCH according to one or more combinations Xs,Ys, where Xs and Ys are numbers of consecutive slots. Groups of Xs slots are consecutive and non-overlapping and the Ys slots are within the Xs slots. The first group of Xs slots starts from the beginning of a subframe. The start of two consecutive groups of Ys slots is separated by Xs slots.

If a UE monitors PDCCH on a cell according to combination Xs,Ys, the UE can monitor PDCCH for Type1-PDCCH CSS set provided by dedicated higher layer signalling, Type3-PDCCH CSS sets, and USS sets in any slot of the Ys slots, and the UE can monitor PDCCH for Type0/0A/2-PDCCH CSS set and Type1-PDCCH CSS set provided in SIB1 in any slot of the Xs slots, wherein the search space sets are associated with the active SSSG. The UE determines the number of monitored PDCCH candidates and the number of non-overlapped CCEs for combination Xs,Ys based on all search space sets associated with the active SSSG within the Xs slots, as applicable according to the search space set configurations, and maximum corresponding values are provided in Table 10.1-2B and Table 10.1-3B, respectively.

For μ=6, if the UE indicates a capability to monitor PDCCH according to multiple combinations Xs,Ys and a configuration of search space sets to the UE for PDCCH monitoring on a serving cell results to a separation of every two consecutive groups of Ys slots that is not smaller than Xs for more than one combinations Xs,Ys, of the multiple combinations Xs,Ys, the UE monitors PDCCH on the cell according to the combination Xs,Ys, from the more than one combinations Xs,Ys, that is associated with the largest maximum number of MPDCCHmax,Xs,μ and CPDCCHmax,Xs,μ defined in Table 10.1-2B and Table 10.1-3B.

=============================== Unchanged Text Omitted ===================================

============================== End of TP #1 for TS 38.213 ==================================

## TP#2 for TS 38.213

============================== Start of TP #2 for TS 38.213 ==================================

10.4	Search space set group switching and skipping of PDCCH monitoring

=============================== Unchanged Text Omitted ===================================

If a UE is provided by SearchSpaceSwitchTrigger a location of a search space set group switching flag field in a DCI format 2_0, as described in clause 11.1.1, for a serving cell where the UE has active DL BWP with SCS configuration μ

-	if the UE detects a DCI format 2_0 and a value of the search space set group switching flag field in the DCI format 2_0 is 0, the UE starts monitoring PDCCH according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, for the serving cell

-	at the beginning of the first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0 when μ∈0, 1, 2, 3

-	at the beginning of the first slot, of a group of Xs slots, that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0 when μ∈5, 6, wherein Xs corresponds to the value in combination (Xs,Ys) after switching

-	if the UE detects a DCI format 2_0 and a value of the search space set group switching flag field in the DCI format 2_0 is 1, the UE starts monitoring PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, for the serving cell

-	at the beginning of the first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0, when μ∈0, 1, 2, 3

-	at the beginning of the first slot, of a group of Xs slots, that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0 when μ∈5, 6, wherein Xs corresponds to the value in combination (Xs,Ys) after switching

and the UE sets the timer value to the value provided by searchSpaceSwitchTimer

-	if the UE monitors PDCCH for a serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH for the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, for the serving cell

-	at the beginning of the first slot that is at least Pswitch symbols after a slot where the timer expires or after a last symbol of a remaining channel occupancy duration for the serving cell if indicated by DCI format 2_0 when μ∈0, 1, 2, 3

-	at the beginning of the first slot, of a group of Xs slots, that is at least Pswitch symbols after a slot where the timer expires or after a last symbol of a remaining channel occupancy duration for the serving cell if indicated by DCI format 2_0 when μ∈5, 6, wherein Xs corresponds to the value in combination (Xs,Ys) after switching

If a UE is not provided SearchSpaceSwitchTrigger for a serving cell,

-	if the UE detects a DCI format by monitoring PDCCH according to a search space set with group index 0, the UE starts monitoring PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, for the serving cell

-	at the beginning of the first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format when μ∈0, 1, 2, 3,

-	at the beginning of the first slot, of a group of Xs slots, that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format when μ∈5, 6, wherein Xs corresponds to the value in combination (Xs,Ys) after switching

the UE sets the timer value to the value provided by searchSpaceSwitchTimer if the UE detects a DCI format by monitoring PDCCH in any search space set

-	if the UE monitors PDCCH for a serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH for the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, for the serving cell

-	at the beginning of the first slot that is at least Pswitch symbols after a slot where the timer expires or, if the UE is provided a search space set to monitor PDCCH for detecting a DCI format 2_0, after a last symbol of a remaining channel occupancy duration for the serving cell if indicated by DCI format 2_0 when μ∈0, 1, 2, 3

-	at the beginning of the first slot, of a group of Xs slots, that is at least Pswitch symbols after a slot where the timer expires or, if the UE is provided a search space set to monitor PDCCH for detecting a DCI format 2_0, after a last symbol of a remaining channel occupancy duration for the serving cell if indicated by DCI format 2_0 when μ∈5, 6, wherein Xs corresponds to the value in combination (Xs,Ys) after switching

=============================== Unchanged Text Omitted ===================================

============================== End of TP #2 for TS 38.213 ==================================