3GPP TSG RAN WG1 #109-e			R1-2203860

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.2.3

Source:	Samsung

Title:	Maintenance on PDSCH/PUSCH enhancements for NR from 52.6 GHz to 71 GHz

Document for:	Discussion and Decision

- Introduction

This contribution discusses the remaining issues for multi-PDSCH/PUSCH scheduling by a single DCI and the TPs, if needed, associated with the proposals are listed in Appendix.

- Multi-PDSCH/PUSCH scheduling and HARQ

3.1 Multi-PDSCH/PUSCH scheduling

3.1.1 Restriction on the number of PDSCH receptions/PUSCH transmissions in a slot

We have the following agreements for 480/960 kHz SCS scenario.

Agreement: (RAN1#106-e) - For single TRP operation, for 480/960 kHz SCS, - A UE does not expect to be scheduled with more than one unicast PDSCH in a slot, by a single DCI or multiple DCIs. - A UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs. Agreement (RAN1#107-e) - For multi-TRP operation, for 480/960 kHz SCS, - A UE does not expect to be scheduled with more than one unicast PDSCH in a slot, by a single DCI or multiple DCIs, from the same TRP. - A UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs, from the same TRP. - Note: This does not preclude a UE being scheduled with two PDSCHs (or two PUSCHs) in the same slot from two different TRPs for multi-DCI based multi-TRP mechanism.

If there is SPS PDSCH/CG PUSCH in the slot, UE behaviour is not clear. For example, whether UE can receive a unicast PDSCH and a SPS PDSCH in a slot? The intention of the agreement is to restrict that there is up to 1 PDSCH/PUSCH per slot. Same rule should apply to SPS PDSCH/CG PUSCH. In addition, whether the scheduled PDSCH/PUSCH is a valid one should be clarified.

Proposal 1: For single TRP or multi-TRP operation, for 480/960 kHz SCS,

  - A UE does not expect to receive more than one unicast PDSCH in a slot on a serving cell from the same TRP.

  - A UE does not expect to transmit more than one PUSCH in a slot on a serving cell from the same TRP.

3.1.2 Remaining issues on PxSCH repetitions

In the last RAN1 meeting, RAN1 discusses whether to support PDSCH (PUSCH) repetition when a TDRA row with one SLIV entry is indicated and the DCI format is scrambled with CS-RNTI. Some companies requested to clarify the DCI format 1_1 includes CS-RNTI (or DCI format 0_1 includes CS-RNTI) in the following agreements.

Agreement (RAN1#107-e) - If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PDSCH for DCI format 1_1, the UE does not expect to be configured with repetitionNumber for the TDRA table, and if pdsch-AggregationFactor is configured in PDSCH-config, it does not apply to DCI format 1_1. - Note: repetitionNumber cannot be configured with pdsch-TimeDomainAllocationListDCI-1-2 as in Rel-16. - Note: Under agenda item 8.2.4, in RAN1#106-bis, it was already agreed that within the TDRA table for multi-PDSCH scheduling, the UE does not expect to be configured with the higher layer parameter repetitionNumber. - Note: These does not preclude pdsch-AggregationFactor can be configured and applies to DCI format 1_2 - If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PUSCH for DCI format 0_1, the UE does not expect to be configured with numberOfRepetitions for the TDRA table, and if pusch-AggregationFactor is configured in PUSCH-config, it does not apply to DCI format 0_1. - Note: These does not preclude numberOfRepetitions is configured for TDRA table corresponding to DCI format 0_2 - Note: These does not preclude pusch-AggregationFactor can be configured and applies to DCI format 0_2

Since the agreements does not differentiate C-RNTI and CS-RNTI, our understanding is the pdsch-AggregationFactor is not applied to DCI format 1_1 with any RNTIs (including C-RNTI and CS-RNTI). Also, it is unclear a potential use case where PDSCH repetition is allowed to DCI format 1_1 with CS-RNTI and not with C-RNTI. Anyway, gNB would configure DCI format 1_2 with pdsch-AggregationFactor to enable PDSCH repetition. So that gNB can indicate DCI format 1_2 with CS-RNTI to indicate SPS PDSCH repetition.

Proposal 2. RAN1 clarifies that

  - if pdsch-AggregationFactor is configured in SPS-config, it does not apply to DCI format 1_1 with any RNTIs, and

  - if repK is configured in ConfiguredGrantConfig, it does not apply to DCI format 0_1 with any RNTIs

3.1.3 Remaining issues on DCI fields when two mapping types are indicated

In the RAN1#104b-e meeting, RAN1 agreed to support a TDRA row includes separate SLIV and mapping type.

Agreement: (RAN1#104b-e) - For a DCI that can schedule multiple PUSCHs, - TDRA: Alt 2 (TDRA table is extended such that each row indicates up to 8 multiple PUSCHs (that can be non-continuous in time-domain). Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is implicitly indicated by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI.), as per agreement made in RAN1#104-e - FFS: signaling details - Note: Alt 2 does not preclude continuous resource allocation in time-domain. - For a DCI that can schedule multiple PDSCHs, - TDRA: TDRA table is extended such that each row indicates up to 8 multiple PDSCHs (that can be non-continuous in time-domain). Each PDSCH has a separate SLIV and mapping type. The number of scheduled PDSCHs is implicitly indicated by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI. - FFS: signaling details - Note: This does not preclude continuous resource allocation in time-domain. - Note: Multi-PDSCH scheduling for the case of 120 kHz SCS is still FFS as per prior agreement. This case can be addressed after this FFS has been decided.

For example, as shown in figure 1, a DCI format can schedule 3 PDSCHs, where the first and last PDSCHs have mapping type B while the second PDSCH has mapping type A. Such a different mapping type is necessary for the case where a SLIV within a TDRA row cannot include a symbol for the first DMRS symbol of mapping type A but other SLIVs within the TDRA row can include a symbol for the first DMRS symbol of mapping type A. If the same mapping type for SLIVs within a TDRA row is only allowed, gNB cannot align the first DMRS position.

Figure 1. Different mapping types for SLIVs within a TDRA row

According to the RRC parameter, PDSCH-Config, copied from TS38.331, mapping type A and mapping type B can have separate DMRS configurations. Note that even though two mapping types have separate DMRS configurations, only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength in DMRS-DownlinkConfig may be set differently for mapping type A and B.

#### – PDSCH-Config The PDSCH-Config IE is used to configure the UE specific PDSCH parameters. PDSCH-Config information element -- ASN1START -- TAG-PDSCH-CONFIG-START PDSCH-Config ::= SEQUENCE { dataScramblingIdentityPDSCH INTEGER (0..1023) OPTIONAL, -- Need S dmrs-DownlinkForPDSCH-MappingTypeA SetupRelease { DMRS-DownlinkConfig } OPTIONAL, -- Need M dmrs-DownlinkForPDSCH-MappingTypeB SetupRelease { DMRS-DownlinkConfig } OPTIONAL, -- Need M dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 DMRS configuration for PDSCH transmissions using PDSCH mapping type A (chosen dynamically via PDSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B. The field dmrs-DownlinkForPDSCH-MappingTypeA applies to DCI format 1_1 and the field dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 applies to DCI format 1_2 (see TS 38.212 [17], clause 7.3.1).. dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 DMRS configuration for PDSCH transmissions using PDSCH mapping type B (chosen dynamically via PDSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B. The field dmrs-DownlinkForPDSCH-MappingTypeB applies to DCI format 1_1 and the field dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 applies to DCI format 1_2 (see TS 38.212 [17], clause 7.3.1)..

Observation 1: Mapping type A and mapping type B can have separate DMRS configurations, where the fields dmrs-Type, dmrs-AdditionalPosition and maxLength in DMRS-DownlinkConfig may be set differently.

- Issue on Antenna port field

Consider the case where mapping type A and B have the different DMRS configurations resulting different antenna port tables, For example, the following configurations can be considered:

For Mapping type A, the followings are configured by dmrs-DownlinkForPDSCH-MappingTypeA

- dmrs-Type – type2, maxLength – len1

For Mapping type B, the followings are configured by dmrs-DownlinkForPDSCH-MappingTypeA

- dmrs-Type – type2, maxLength – len2

Note that here the only difference between mapping type A and B is maxLength, and dmrs-AdditionalPosition is not explicitly mentioned since dmrs-AdditionalPosition does not affect any antenna port table determination. According to dmrs-Type and maxLength configurations, the antenna port table is determined. That is, Table 7.3.1.2.2-3 or Table 7.3.1.2.2-4 in TS38.212 is used to indicate DMRS port(s) and the number of DMRS symbols for mapping type A or mapping type B, respectively. Both tables are copied below for reference.

Table 7.3.1.2.2-3: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, maxLength=1

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 3 | 0-4 |
| 1 | 1 | 1 | 1 | 3 | 0-5 |
| 2 | 1 | 0,1 | 2-31 | reserved | reserved |
| 3 | 2 | 0 |  |  |  |
| 4 | 2 | 1 |  |  |  |
| 5 | 2 | 2 |  |  |  |
| 6 | 2 | 3 |  |  |  |
| 7 | 2 | 0,1 |  |  |  |
| 8 | 2 | 2,3 |  |  |  |
| 9 | 2 | 0-2 |  |  |  |
| 10 | 2 | 0-3 |  |  |  |
| 11 | 3 | 0 |  |  |  |
| 12 | 3 | 1 |  |  |  |
| 13 | 3 | 2 |  |  |  |
| 14 | 3 | 3 |  |  |  |
| 15 | 3 | 4 |  |  |  |
| 16 | 3 | 5 |  |  |  |
| 17 | 3 | 0,1 |  |  |  |
| 18 | 3 | 2,3 |  |  |  |
| 19 | 3 | 4,5 |  |  |  |
| 20 | 3 | 0-2 |  |  |  |
| 21 | 3 | 3-5 |  |  |  |
| 22 | 3 | 0-3 |  |  |  |
| 23 | 2 | 0,2 |  |  |  |
| 24-31 | Reserved | Reserved |  |  |  |

Table 7.3.1.2.2-4: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, maxLength=2

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 3 | 0-4 | 1 |
| 1 | 1 | 1 | 1 | 1 | 3 | 0-5 | 1 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,6 | 2 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0,1,2,3,6,8 | 2 |
| 4 | 2 | 1 | 1 | 4 | 2 | 0,1,2,3,6,7,8 | 2 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 6 | 2 | 3 | 1 | 6-63 | Reserved | Reserved | Reserved |
| 7 | 2 | 0,1 | 1 |  |  |  |  |
| 8 | 2 | 2,3 | 1 |  |  |  |  |
| 9 | 2 | 0-2 | 1 |  |  |  |  |
| 10 | 2 | 0-3 | 1 |  |  |  |  |
| 11 | 3 | 0 | 1 |  |  |  |  |
| 12 | 3 | 1 | 1 |  |  |  |  |
| 13 | 3 | 2 | 1 |  |  |  |  |
| 14 | 3 | 3 | 1 |  |  |  |  |
| 15 | 3 | 4 | 1 |  |  |  |  |
| 16 | 3 | 5 | 1 |  |  |  |  |
| 17 | 3 | 0,1 | 1 |  |  |  |  |
| 18 | 3 | 2,3 | 1 |  |  |  |  |
| 19 | 3 | 4,5 | 1 |  |  |  |  |
| 20 | 3 | 0-2 | 1 |  |  |  |  |
| 21 | 3 | 3-5 | 1 |  |  |  |  |
| 22 | 3 | 0-3 | 1 |  |  |  |  |
| 23 | 2 | 0,2 | 1 |  |  |  |  |
| 24 | 3 | 0 | 2 |  |  |  |  |
| 25 | 3 | 1 | 2 |  |  |  |  |
| 26 | 3 | 2 | 2 |  |  |  |  |
| 27 | 3 | 3 | 2 |  |  |  |  |
| 28 | 3 | 4 | 2 |  |  |  |  |
| 29 | 3 | 5 | 2 |  |  |  |  |
| 30 | 3 | 6 | 2 |  |  |  |  |
| 31 | 3 | 7 | 2 |  |  |  |  |
| 32 | 3 | 8 | 2 |  |  |  |  |
| 33 | 3 | 9 | 2 |  |  |  |  |
| 34 | 3 | 10 | 2 |  |  |  |  |
| 35 | 3 | 11 | 2 |  |  |  |  |
| 36 | 3 | 0,1 | 2 |  |  |  |  |
| 37 | 3 | 2,3 | 2 |  |  |  |  |
| 38 | 3 | 4,5 | 2 |  |  |  |  |
| 39 | 3 | 6,7 | 2 |  |  |  |  |
| 40 | 3 | 8,9 | 2 |  |  |  |  |
| 41 | 3 | 10,11 | 2 |  |  |  |  |
| 42 | 3 | 0,1,6 | 2 |  |  |  |  |
| 43 | 3 | 2,3,8 | 2 |  |  |  |  |
| 44 | 3 | 4,5,10 | 2 |  |  |  |  |
| 45 | 3 | 0,1,6,7 | 2 |  |  |  |  |
| 46 | 3 | 2,3,8,9 | 2 |  |  |  |  |
| 47 | 3 | 4,5,10,11 | 2 |  |  |  |  |
| 48 | 1 | 0 | 2 |  |  |  |  |
| 49 | 1 | 1 | 2 |  |  |  |  |
| 50 | 1 | 6 | 2 |  |  |  |  |
| 51 | 1 | 7 | 2 |  |  |  |  |
| 52 | 1 | 0,1 | 2 |  |  |  |  |
| 53 | 1 | 6,7 | 2 |  |  |  |  |
| 54 | 2 | 0,1 | 2 |  |  |  |  |
| 55 | 2 | 2,3 | 2 |  |  |  |  |
| 56 | 2 | 6,7 | 2 |  |  |  |  |
| 57 | 2 | 8,9 | 2 |  |  |  |  |
| 58-63 | Reserved | Reserved | Reserved |  |  |  |  |

Based on the current specification of TS38.212, 5 bits or 6 bits are required to indicate one row in Table 7.3.1.2.2-3 or Table 7.3.1.2.2-4, respectively. Hence, the DCI format scheduling PDSCH includes 6 bits (maximum of 5 bits and 6 bits) for antenna port field. If the DCI format includes a TDRA row scheduling mapping type A, the UE needs 5 bits to indicate one row in Table 7.3.1.2.2-3 so that the UE regards MSB 1 bit is zero-padded and 6 bits is used to indicate DMRS port(s) and the number of DMRS symbols. Note that this is equal to use LSB 5 bits for antenna port indication.  Similarly, if the DCI format includes a TDRA row scheduling mapping type B, all 6 bits in antenna port field are used to indicate one row in Table 7.3.1.2.2-4.

Now, consider the DCI format includes a TDRA row scheduling both mapping types. Again, based on the current specification of TS38.212, the UE would interpret the antenna port field with the legacy rule, which means that the value indicated by the antenna port field applies to both mapping types (both antenna port tables associated with both mapping types). It results in unnecessary connection on entries with the same index of both antenna port tables. For example, the values 24~57 (yellow colored rows) cannot be indicated since the DMRS port(s) for mapping type A (reserved in Table 7.3.1.2.2-3) is not defined in case of single codeword is enabled. Also, the values 2-5 (green colored rows) cannot be indicated since the DMRS port(s) for mapping type A (reserved in Table 7.3.1.2.2-3) is not defined in case of two codewords are enabled. It means that 2 DMRS symbols cannot be indicated for mapping type B even if maxLength = len2 is configured.

Therefore, if a DCI format indicate TDRA row scheduling multiple PDSCHs with two different mapping types, the DCI format needs to indicate each row from two Antenna port tables (one for mapping type A and another for mapping type B).

Proposal 3. Support that antenna port field indicates each entry of two Antenna Port tables if a TDRA row with both mapping type A and mapping type B is indicated and the corresponding antenna port tables are different.

  - Adopt the TP#1 in Appendix for TS38.212

- Issue on PTRS-DMRS Association field

Rel-15 introduced PTRS-DMRS association field to indicate the best antenna port for UL PTRS transmission among DMRS antenna ports. In Rel-17, the field is further enhanced, i.e., in order to support PUSCH transmission with two mTRPs, NR supports to introduce second PTRS-DMRS association field. For FR2-2, PTRS is a key component to support higher order modulation since phased noise effect severely degrades UL performance. Therefore, it is necessary for gNB to indicate the best antenna port for UL PTRS transmission among DMRS antenna port(s) in FR2-2.

As we discussed above, a TDRA row can include two mapping types and each mapping type has separate DMRS configuration. As a result, a UE can be indicated with different DMRS port(s) for different mapping types. For example, assuming that

- mapping type A – Table 7.3.1.1.2-14 (dmrs-Type=1, maxLength=2, rank=3),

- mapping type B – Table 7.3.1.1.2-22 (dmrs-Type=2, maxLength=2, rank=3)

and the 3th row (corresponding to value = 2, cyan colored) of both Antenna port tables, Table 7.3.1.1.2-14 and Table 7.3.1.1.2-22, is indicated. Both tables are copied below for reference.

Table 7.3.1.1.2-14: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 2 | 0,1,4 | 2 |
| 2 | 2 | 2,3,6 | 2 |
| 3-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-18: Antenna port(s), transform precoder is disabled, dmrs-Type=2, maxLength=1, rank =3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1 | 3 | 0-2 |
| 2 | 3 | 3-5 |
| 3-15 | Reserved | Reserved |

For a PUSCH with mapping type A, DMRS ports of index 2, 3, and 6 are used and for a PUSCH with mapping type B, DMRS ports of index 3, 4, and 5 are used. Suppose that the DMRS antenna port of index 3 provides the largest SNR, i.e., the best antenna port for UL PTRS transmission. To indicate the DMRS antenna port of index 3 as UL PTRS transmission for the PUSCH with mapping type A, the second row (corresponding to value 1) in table 7.3.1.1.2-25 should be indicated. However, it results in DMRS antenna port 4 is indicated for the PUSCH with mapping type B. Therefore, it is impossible to indicate best PTRS antenna port for both mapping types.

Hence, to indicate each PTRS antenna port for each mapping type, the DCI format 0_1 needs to have two entries, one for mapping type A and another for mapping type B if the indicated TDRA row includes both mapping types.

Proposal 4. Support that PTRS-DMRS association field indicates each DMRS port as PTRS ports for mapping type A and mapping type B.

  - Adopt the TP#2 in Appendix for TS38.212

3.2 HARQ-ACK feedback for Multi-PDSCH scheduling

3.2.1 Remaining issues on Type-1 HARQ-ACK codebook

Support PDSCH repetitions when time-domain bundling is configured

In the last RAN1#107-e meeting, RAN1 agreed the following agreements.

Agreement - If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PDSCH for DCI format 1_1, the UE does not expect to be configured with repetitionNumber for the TDRA table, and if pdsch-AggregationFactor is configued in PDSCH-config, it does not apply to DCI format 1_1. - Note: repetitionNumber cannot be configured with pdsch-TimeDomainAllocationListDCI-1-2 as in Rel-16. - Note: Under agenda item 8.2.4, in RAN1#106-bis, it was already agreed that within the TDRA table for multi-PDSCH scheduling, the UE does not expect to be configured with the higher layer parameter repetitionNumber. - Note: These does not preclude pdsch-AggregationFactor can be configured and applies to DCI format 1_2 - If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PUSCH for DCI format 0_1, the UE does not expect to be configured with numberOfRepetitions for the TDRA table, and if pusch-AggregationFactor is configued in PUSCH-config, it does not apply to DCI format 0_1. - Note: These does not preclude numberOfRepetitions is configured for TDRA table corresponding to DCI format 0_2 - Note: These does not preclude pusch-AggregationFactor can be configured and applies to DCI format 0_2

Based on the following agreements, we can observe that 1) a UE can be provided pdsch-TimeDomainAllocationListForMultiPDSCH-r17 and pdsch-AggregationFactor at the same time and if the UE are provided the both, then DCI format 1_1 may schedule more than one PDSCHs by pdsch-TimeDomainAllocationListForMultiPDSCH-r17 and DCI format 1_2 may schedule PDSCH repetition by pdsch-AggregationFactor. Therefore, if the UE should make type-1 HARQ-ACK CB for multi-PDSCH scheduling and PDSCH repetitions.

In Clause 9.1.2.1 of TS38.213 v17.0.0, the pseudo-code was introduced to support multi-PDSCH scheduling with/without time domain bundling.

If time domain bundling is not configured (see the green highlight below), PDSCH occasions are generated by taking into account multi-PDSCH scheduling and PDSCH repetition. Note that for multi-PDSCH scheduling, the row r of the set R contains single SLIVs decomposing multiple SLIVs in a TDRA table (i.e., SLIV decomposition) and for PDSCH repetition, the single SLIV is assumed to be repeated over NPDSCHrepeat,max slots. Therefore, some of rows associated with DCI format 1_1 does not need to be repeated but all of rows are assumed to be repeated over NPDSCHrepeat,max slots, which make some overhead in type-1 HARQ-ACK CB. However, it can be acceptable since it does not bring any scheduling restrictions.

If time domain bundling is configured (see the yellow highlight below), PDSCH occasions are generated by taking into account multi-PDSCH scheduling only, but not PDSCH repetition. As a results, the type-1 HARQ-ACK CB with time domain bundling does not includes PDSCH occasions for PDSCH repetition. Therefore, gNB may not schedule PDSCH repetitions by using DCI format 1_2.

while r<CR if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if HARQ-ACK information for PDSCH time resource derived by row r in slot nU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU R=R\r; elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot nU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of the PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r. R=R\r; R'=R'\r; else r=r+1; end if end while

Observation 2: If time domain bundling is configured, Type-1 HARQ-ACK CB does not cover PDSCH repetitions scheduled by DCI format 1_2.

To address this issue, we suggest to consider the following two options.

- Option 1. A row r is removed when two conditions are met

  - Condition 1 for multi-PDSCH scheduling: each SLIVs of the TDRA row r overlapped with a semi-static UL symbol

  - Condition 2 for PDSCH repetition: the last SLIV of the TDRA row r over K slots overlapped with a semi-static UL symbol.

    - Note that the last SLIV is used for PDSCH repetition since the TDRA row r may include more than one SLIVs. Note that this may result in some overhead because the TDRA rows only for multi-PDSCH scheduling are also used for PDSCH repetition.

- Option 2. Treat it as an error case. i.e., a UE does not expect to be configured with multi-PDSCH scheduling with time domain bundling and pdsch-AggregationFactor at the same time.

Since type-1 HARQ-ACK CB is mainly used for a coverage limited scenario where a PDCCH scheduling PDSCH may be missed often, support of PDSCH repetitions scheduled by a DCI format 1_2 would be beneficial. Therefore, we prefer to support that type-1 HARQ-ACK CB covers both multi-PDSCH scheduling and PDSCH repetition.

Proposal 5: To support multi-PDSCH scheduling by DCI format 1_1 and PDSCH repetition by DCI format 1_2 in type-1 HARQ-ACK CB, a row r in the set R’ and the set R is removed when the both conditions are met

  - Condition 1 for multi-PDSCH scheduling) each SLIVs of the TDRA row r overlapped with a semi-static UL symbol

  - Condition 2 for PDSCH repetition) the last SLIV of the TDRA row r over K slots overlapped with a semi-static UL symbol.

  - Adopt TP#3 in Appendix for TS38.213

- Conclusion

The observations and proposals made in this contribution are summarized below:

Observation 1: Mapping type A and mapping type B can have separate DMRS configurations, where the fields dmrs-Type, dmrs-AdditionalPosition and maxLength in DMRS-DownlinkConfig may be set differently.

Observation 2: If time domain bundling is configured, Type-1 HARQ-ACK CB does not cover PDSCH repetitions scheduled by DCI format 1_2.

Proposal 1: For single TRP or multi-TRP operation, for 480/960 kHz SCS,

  - A UE does not expect to receive more than one unicast PDSCH in a slot on a serving cell from the same TRP.

  - A UE does not expect to transmit more than one PUSCH in a slot on a serving cell from the same TRP.

Proposal 2. RAN1 clarifies that

  - if pdsch-AggregationFactor is configured in SPS-config, it does not apply to DCI format 1_1 with any RNTIs, and

  - if repK is configured in ConfiguredGrantConfig, it does not apply to DCI format 0_1 with any RNTIs

Proposal 3. Support that antenna port field indicates each entry of two Antenna Port tables if a TDRA row with both mapping type A and mapping type B is indicated and the corresponding antenna port tables are different.

  - Adopt the TP#1 in Appendix for TS38.212

Proposal 4. Support that PTRS-DMRS association field indicates each DMRS port as PTRS ports for mapping type A and mapping type B.

  - Adopt the TP#2 in Appendix for TS38.212

Proposal 5: To support multi-PDSCH scheduling by DCI format 1_1 and PDSCH repetition by DCI format 1_2 in type-1 HARQ-ACK CB, a row r in the set R’ and the set R is removed when the both conditions are met

  - Condition 1 for multi-PDSCH scheduling) each SLIVs of the TDRA row r overlapped with a semi-static UL symbol

  - Condition 2 for PDSCH repetition) the last SLIV of the TDRA row r over K slots overlapped with a semi-static UL symbol.

  - Adopt TP#3 in Appendix for TS38.213

- Appendix

TP#1 for TS 38.212

============================== Start of TP #1 for TS 38.212 ==================================

7.3.1.1.2	Format 0_1

=============================== Unchanged Text Omitted ===================================

-	Antenna ports – number of bits determined by the following

If the row indicated by the Time domain resource assignment field includes single mapping type or dmrs- UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB have same dmrs-Type and maxLength

-	2 bits as defined by Tables 7.3.1.1.2-6, if transform precoder is enabled, dmrs-Type=1, and maxLength=1, except that dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured and π/2 BPSK modulation is used;

-	2 bits as defined by Tables 7.3.1.1.2-6A, if transform precoder is enabled and dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured, π/2 BPSK modulation is used, dmrs-Type=1, and maxLength=1, where nSCID is the scrambling identity for antenna ports defined in [Clause 6.4.1.1.1.2, TS38.211];

-	4 bits as defined by Tables 7.3.1.1.2-7, if transform precoder is enabled, dmrs-Type=1, and maxLength=2, except that dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured and π/2 BPSK modulation is used;

-	4 bits as defined by Tables 7.3.1.1.2-7A, if transform precoder is enabled and dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured, π/2 BPSK modulation is used, dmrs-Type=1, and maxLength=2, where nSCID is the scrambling identity for antenna ports defined in [Clause 6.4.1.1.1.2, TS38.211];

-	3 bits as defined by Tables 7.3.1.1.2-8/9/10/11, if transform precoder is disabled, dmrs-Type=1, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	4 bits as defined by Tables 7.3.1.1.2-12/13/14/15, if transform precoder is disabled, dmrs-Type=1, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	4 bits as defined by Tables 7.3.1.1.2-16/17/18/19, if transform precoder is disabled, dmrs-Type=2, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	5 bits as defined by Tables 7.3.1.1.2-20/21/22/23, if transform precoder is disabled, dmrs-Type=2, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook.

If the row indicated by the Time domain resource assignment field includes two mapping types and dmrs- UplinkForPUSCH -MappingTypeA and dmrs- UplinkForPUSCH –MappingTypeB have different dmrs-Type or maxLength

-	xA+xB bits where the MBS xA bits is the "Antenna ports" bitwidth derived according to dmrs- UplinkForPUSCH -MappingTypeA and the LSB xB bits is the "Antenna ports" bitwidth derived according to dmrs- UplinkForPUSCH -MappingTypeB.

where the number of CDM groups without data of values 1, 2, and 3 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively.

If a UE is configured with both dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB, the bitwidth of this field equals , where  is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeA and  is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeB. A number of  zeros are padded in the MSB of this field, if the mapping type of the PUSCH corresponds to the smaller value of  and .

=============================== Unchanged Text Omitted ===================================

7.3.1.2.2	Format 1_1

=============================== Unchanged Text Omitted ===================================

-	Antenna port(s) – 4, 5, or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4 and Tables 7.3.1.2.2-1A/2A/3A/4A if the row indicated by the Time domain resource assignment field includes single mapping type or dmrs- DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB have same dmrs-Type and maxLength; xA+xB bits where the MBS xA bits is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeA and the LSB xB bits is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeB if the row indicated by the Time domain resource assignment field includes two mapping types and dmrs- DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB have different dmrs-Type or maxLength, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports  shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4 or Tables 7.3.1.2.2-1A/2A/3A/4A. When a UE receives an activation command that maps at least one codepoint of DCI field 'Transmission Configuration Indication' to two TCI states, the UE shall use Table 7.3.1.2.2-1A/2A/3A/4A; otherwise, it shall use Tables 7.3.1.2.2-1/2/3/4. The UE can receive an entry with DMRS ports equals to 1000, 1002, 1003 when two TCI states are indicated in a codepoint of DCI field 'Transmission Configuration Indication'.

If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB and if the row indicated by the Time domain resource assignment field have single mapping type or dmrs- DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB have same dmrs-Type and maxLength, the bitwidth of this field equals , where  is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeA and  is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeB. A number of  zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of  and .

============================== End of TP #1 for TS 38.212 ==================================

TP#2 for TS 38.212

============================== Start of TP #1 for TS 38.212 ==================================

7.3.1.1.2	Format 0_1

=============================== Unchanged Text Omitted ===================================

-	PTRS-DMRS association – number of bits determined as follows

-	0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRank=1;

If the row indicated by the Time domain resource assignment field includes single mapping type or dmrs- UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB have same dmrs-Type and maxLength,

-	2 bits otherwise, where Table 7.3.1.1.2-25/7.3.1.1.2-25A and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field. When the SRS resource set indicator field is present and maxRank>2, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26. When the SRS resource set indicator field is present and equals "10" and "11" and maxRank=2, the MSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A.

If the row indicated by the Time domain resource assignment field includes two mapping types and or dmrs- UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB have different dmrs-Type or maxLength,

-	4 bits, where MSB 2 bits are for dmrs-UplinkForPUSCH-MappingTypeA and LSB 2bits are for dmrs-UplinkForPUSCH-MappingTypeB. For each 2 bits, Table 7.3.1.1.2-25/7.3.1.1.2-25A and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field. When the SRS resource set indicator field is present and maxRank>2, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26. When the SRS resource set indicator field is present and equals "10" and "11" and maxRank=2, the MSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A.

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part.

-	Second PTRS-DMRS association – 2 bits if PTRS-DMRS association field and SRS resource set indicator field are present and maxRank>2 and if the row indicated by the Time domain resource assignment field includes single mapping type or dmrs- UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB have same dmrs-Type and maxLength; 4 bits if PTRS-DMRS association field and SRS resource set indicator field are present and maxRank>2 and if the row indicated by the Time domain resource assignment field includes two mapping types and dmrs- UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB have different dmrs-Type or maxLength, where the MSB 2 bits are for dmrs-UplinkForPUSCH-MappingTypeA and LSB 2bits are for dmrs-UplinkForPUSCH-MappingTypeB; 0 bit otherwise. For each 2 bits, Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second precoding information field when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field.

============================== End of TP #2 for TS 38.212 ==================================

TP#3 for TS 38.213

============================== Start of TP #3 for TS 38.213 ==================================

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

=============================== Unchanged Text Omitted ===================================

while r<CR

if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if PDSCH-TimeDomainResourceAllocationListForMultiPDSCH is provided and HARQ-ACK information for PDSCH time resource derived by row r in slot n0,knU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU

R=R\r;

elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot n0,knU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of each PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r of set R is configured as UL

R=R\r;

R'=R'\r;

else

r=r+1;

end if

end while

============================== End of TP #3 for TS 38.213 ==================================