3GPP TSG RAN WG1 #109-e	                                                         R1-2203865

e-Meeting, May 9th – 20th, 2022

Agenda Item:		8.5.2

Source:				Samsung

Title:					Maintenance on latency and efficiency improvement related enhancement

Document for:	Discussion

# Introduction

In the R17 WI for positioning enhancement [1], one important direction is to improve positioning latency and efficiency of the Rel-16 NR positioning methods. This contribution discusses the maintenance on latency and efficiency improvement related enhancement.

# PRS processing window

In RAN1 #107-e meeting, the following agreement was made for latency reduction:

Agreement The following options are supported subject to UE capability for priority handling of PRS when PRS measurement is outside MG. - Option 1: UE may indicates support of two priority states. - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 2: UE may indicate support of three priority states - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than PDCCH and URLLC PDSCH and higher priority than other PDSCH/CSI-RS - Note: The URLLC channel corresponds a dynamically scheduled PDSCH whose PUCCH resource for carrying ACK/NAK is marked as high-priority. - State 3: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 3: UE may indicate support of single priority state - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - Note: SSB is a separate issue. Agreement The priority of PRS for UE supporting two priority states and three priority states can at least be indicated in RRC. Agreement For capability 1A as per working assumption made in RAN1#106-e, the DL signalings/channels in a per UE fashion (i.e. both across NR & LTE) inside the PRS processing window are dropped if the DL PRS is determined to be higher priority. For capability 1B as per working assumption made in RAN1#106-e, only the DL signalings/channels from a certain band inside the PRS processing window are dropped if the DL PRS is determined to be higher priority. Working assumption: Subject to UE capability, support PRS measurement outside the MG, within a PRS processing window, and UE measurement inside the active DL BWP with PRS having the same numerology as the active DL BWP. - Inside the PRS processing window, subject to the UE determining that DL PRS to be higher priority, support the following UE capabilities: - Capability 1: PRS prioritization over all other DL signals/channels in all symbols inside the window. - Cap. 1A: The DL signals/channels from all DL CCs (per UE) are affected. - Cap. 1B: Only the DL signals/channels from a certain band/CC are affected. - FFS: band or CC - Capability 2: PRS prioritization over other DL signals/channels only in the PRS symbols inside the window - A UE shall be able to declare a PRS processing capability outside MG. - FFS: Details of capability signalling (e.g., per UE or per band, etc.)

In RAN1 #108-e meeting, the following agreement was made for latency reduction:

Conclusion RAN1 understand that the priority between SSB and PRS is up to RAN4 to define. Agreement UE may indicate support of more than one processing types and corresponding capability on a band on which it supports PRS processing outside the MG inside the PRS processing window. Agreement For capability 2 as per working assumption made in RAN1#106-e - For FR1, only the DL signals/channels from a certain CC inside the PRS processing window, which overlap with DL PRS symbols in time, are dropped if the DL PRS is determined to be higher priority - For FR2, only the DL signals/channels from a certain band inside the PRS processing window, which overlap with DL PRS symbols in time, are dropped if the DL PRS is determined to be higher priority For the DL signals/channels from a different FR2 band than the FR2 band of the DL PRS for capability 1B and 2, subject to dropping due to the same Rx beam across multiple FR2 bands if the DL PRS is determined to be higher priority, it is up to RAN4 to define. - Send an LS to RAN4

In RAN4 #102-e meeting, the following agreement was made for latency reduction:

Agreements: - Priority between PRS and SSB when PRS and SSB collide within PPW is under discussion in RAN1. - Postpone the discussion on impact on PRS measurement requirements until RAN1 concludes on the above issue. Agreements: - CSSF design for PRS measurements without gaps is related to priority between PRS and SSB when PRS and SSB collide within PPW is under discussion in RAN1 (related issue 1-2-1I). Postpone the discussion on CSSF outside MG until RAN1 concludes on the above issue.

## Priority with SSB

Based on the agreement in RAN1 #108-e and RAN4 #102-e meeting, the priority between PRS and SSB within the PPW needs to be further discussed at RAN1 meeting as it is critical for CSSF design in RAN4 as well as the collision handling between SSB vs PRS within PPW.

One basic fact is that SSB is semi-statically configured, and not all the SSB indicated to be transmitted has to be received/measured by a UE.  For example, considering the Type-D QCL relationship between PRS resource and SSB, so in case of SSB 0 has associated with PRS 0, if UE is able to monitor SSB 0 before monitor PRS 0, UE could do that and achieve latest path loss for SSB thus improve the measurement accuracy of PRS0. If UE is unable to do that, or SSB 0 has no relation with PRS 0, then UE could ignore SSB 0 and measure PRS 0. So our view is that, the priority of PRS resource and SSB can be equal inside the PRS processing window subject to UE capability. SSB doesn’t have to be always higher or lower priority than PRS. According to RAN4 decision, there could be a case that UE support parallel processing. Thus, the priority of PRS resource and SSB can be equal subject to UE capability, and which one(s) to receive could be up to UE implementation.

Figure 1 – illustration of SSB vs PRS.

Proposal 1: a priority indication is introduced to indicate the priority state between PRS resource and SSB, the candidate value can be high, low and equal inside the PRS processing window subject to UE capability.

Proposal 2: when the priority state of PRS resource and SSB is equal inside the PRS processing window, the reception of PRS and/or SSB is up to UE implementation.  Inform RAN4 about this decision.

## Priority between PRS vs other DL/UL signals

Although the priority states for PRS with PDCCH/PDSCH/CSI-RS can be indicated by NW, there was a discussion on the PRS collision detection timeline considering the cancelation time and decoding time for DCI/MAC CE scheduling PDSCH/PRS/other signals. At meantime, the capability for PRS processing window (1A, 1B, 2).

PRS is higher priority

Usually, when PRS is higher priority, PRS should be received as always. However, there is a special case when it happens in the first activated PPW. Considering that if the UE determines it expects to receive the other DL signals/channels before N time units earlier than the start of the first PRS in the first activated PPW, such N time units should cover the time needed to know the PPW is activated, e.g., N2+3ms. Thus, the UE could have already proceeded to receive the DL signals/channels without knowing the PPW would be activated. Because the PPW is activated after other DL signals/channels expects to be received. Thus, in this case, the DL signals may be received subject to UE capability and the PRS is expected to be received; Otherwise, the DL signals are dropped and the PRS is still expected to be received.

Observation 1: the other DL signals/channels could be received when UE determines they are expected to be received before N time units earlier than the start of the first PRS in the first activated PPW.

PRS is lower priority

When PRS has a lower priority and when PRS collide with other DL signals/channels, UE should receive PDCCH/PDSCH/CSI-RS other than PRS . one arguable case was that if the DL signals/channels are coming late, whether PRS can be received and should the DL signals/channels should be dropped if PRS is received?

In our view, in this case,  PRS can be received subject UE capability without affecting the reception of other DL signals/channels, since it can reduce the latency for positioning. In general, the PRS collision detection timeline under this condition can be described as if there are any DL signals or DCI/MAC CE scheduling/activating DL signals expected to be received in the PPW determined by UE at the time earlier than N time units before the start of PRS, UE will not receive PRS and receive normal PDCCH/PDSCH/CSI-RS; Otherwise, UE expects to receive DL signals in the PPW and may receive PRS subject UE capability.

Observation2： PRS can be received subject UE capability without affecting the reception of other DL signals/channels

Proposal 3: Adopt following collision handling between PRS and other DL signals/channels .

| PRS priority state | Collision handling vs other DL signals in the window (type 1A/1B) and overlapped part (type 2) | X factor |
| --- | --- | --- |
| high priority | - For all non-first activated PPW, PRS is received over all semi-static other DL signals except SSB; - For the first activated PPW, - if UE could determine that it expects to receive other DL signals before a N time units earlier than the start of the first activated PPW, UE expects to receive PRS and may receive the other DL signals subject to its capability - if UE could NOT determine that it expects to receive other DL signals before a N time units earlier than the start of the first activated PPW, UE expects to receive PRS and drop the other DL signals; | The PPW is activated by MAC CE (which is carried by DCI/scheduled PDSCH); so if UE cannot determine the PRS has to be received before it starts its procedure for receiving the other DL signal, then such (partial) reception of other DL signals should be allowed even the PRS reception should always be prioritized; |
| Low priority | - For an activated PPW, - If UE could determine that it expects to receive other DL signals before a N time units earlier than the start of the first PRS, UE expects to receive other DL signals and drop the PRS; - If UE could NOT determine that it expects to receive other DL signals before a N time units earlier than the start of the first PRS, UE expects to receive other DL signals and may receive the PRS subject to its capability; | When PRS is low priority, the first thing is the DL signals are always prioritized; and the PRS could be allowed to be received if UE cannot determine it should do the DL signal reception early enough, then the PRS reception could be allowed. |

PRS vs UL signals

It should be noticed that the priority of PRS and UL signals/channels need to be further clarified because the collision between PRS and UL signals/channels is inevitable. At this maintenance stage, we think the most straightforward way is to specify the priority of PRS and the UL signals/channels (at least for LP UL signals/channels) can follow the PRS priority determination (priority state) with that for other DL signals/channels.

Considering the importance of HP UL signals, e.g, HP PUSCH/PUCCH and PRACH, it might be reasonable to consider the HP UL signals have higher priority than PRS. In other words, if there are any HP UL signals expected/determined to be transmitted in the PPW determined by UE at the time earlier than N time units before the start of PRS, UE will not receive PRS and transmit the HP UL signals; Otherwise, UE expects to transmit HP UL signals in the PPW and may receive PRS subject to UE capability.

Proposal 4: The priority of PRS and LP UL signals/channels follow determined PRS priority state with that for other DL signals/channels. HP UL signals/channels have higher priority than PRS.

After specify the priority of PRS and UL signals/channels, the PRS collision detection timeline should be defined for PRS and UL signals/channels due to the switching time between DL reception and UL transmission. It should be noticed that the UE behavior can be different considering the legacy priority of UL signals in our opinion. In order to follow the existing priority rules, UE should receive PRS other than transmit PUSCH/PUCCH/SRS when the priority of the PRS is higher than that of UL signal, and UE should transmit PUSCH/PUCCH/SRS other than receive PRS when the priority of the PRS is lower than that of UL signal.

More specifically, when the priority of the PRS is higher than UL signal, if there are any PUSCH/PUCCH/SRS expected/determined to be transmitted in the PPW by UE at the time earlier than N time units before the start of PRS, the PRS is expected to be received and may transmit the PUSCH/PUCCH/SRS subject to UE capability; Otherwise, the PUSCH/PUCCH/SRS are dropped and the PRS is expected to be received. This is based on the similar reason of allowing other DL signals/channels to be received in case of PRS is higher priority. And when the priority of the PRS is lower than UL signal, if there are any PUSCH/PUCCH/SRS expected/determined to be transmitted in the PPW by UE at the time earlier than N time units before the start of PRS, UE will not receive PRS and transmit PUSCH/PUCCH/SRS; Otherwise, UE expects to transmit LP PUSCH/PUCCH/SRS in the PPW and may receive PRS subject to UE capability. These operations can reduce the latency of PRS measurement or UL transmission.

Proposal 5: Adopt following collision handling between PRS and UL signals/channels .

| PRS priority state | Collision handling vs UL signals in the window (type 1A/1B) and overlapped part (type 2) |
| --- | --- |
| high priority | - For all non-first activated PPW, PRS is received over all semi-static other UL signals/channels transmission; - For the first activated PPW, - if UE could determine that it expects to transmit the UL signals before a N time units earlier than the start of the PRS in the first activated PPW, UE expects to receive PRS and may transmit the UL signals subject to its capability; - if UE could NOT determine that it expects to receive UL signals before a N time units earlier than the start of the first activated PPW, UE expects to receive PRS and drop the UL signals tx; |
| Low priority | - For an activated PPW, - If UE could determine that it expects to transmit the UL signals before a N time units earlier than the start of the first PRS, UE expects to transmit the UL signals and drop the PRS; - If UE could NOT determine that it expects to receive other DL signals before a N time units earlier than the start of the first PRS, UE expects to transmit UL signals and may receive the PRS subject to its capability; |

## Low latency PRS processing capability

There was lot of discussion in last meeting on whether to introduce a new UE operation mode that UE only measurement first N ms PRS and also finish the processing of these PRS within the rest part of the PPW. In order to do that, proponents proposed that the PPW length should cover the time period for N and the end of the PRS to the end of the PPW should be larger than T-N.

From our understanding, the benefit on introducing such particular mode suppose to reduce the latency of positioning procedure. But as we discussed heavily in last meeting, limiting the buffering the first N, and processing the N within the rest T-N is contributing only one of the multiple aspects for the actual latency experience.   And such contributing is only in potential level, which means it can only serve it’s purpose in a very particular situation, e.g., the measured N ms PRS are all good enough, the reporting resource is right after the N-T ms or the end of PPW. Thus, we are not supportive to introduce such operation mode.

Proposal 6: no new PRS processing capability or mode is introduced.

## PRS duration K

Since there exist a closed mathematical expression to calculate the measurement period for PRS RSTD/RSRP measurement in positioning frequency layer i according to the time duration of available PRS in the positioning frequency layer i   Lavailable_PRS,i , and   Lavailable_PRS,i is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 , it should be clarified in RAN1 which potential PRS resources should be counted when PRS processing window is configured considering the different priority of PRS with other DL/UL signals.

Proposal 7: Text proposal for TS38.214  5.1.6.5	 PRS reception procedure

*** Unchanged text is omitted ***

-	For Type 1, Tsstart, Tsend is the smallest interval in msec within slot s corresponding to an integer number of OFDM symbols based on the numerology of the DL PRS of a serving cell that covers the union of the potential PRS symbols and determines the PRS symbol occupancy within slot s, where the interval Tsstart, Tsend considers the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS resource sets (target and reference).

-	For Type 2, μ is the numerology of the DL PRS, and S is the cardinality of the set S.

For the case when PRS processing window is configured, the potential DL PRS resources referred in the calculation of the duration K msec of DL PRS symbols are the DL PRS resources not collides with other higher priority DL/UL signals.

*** Unchanged text is omitted ***

# Positioning in Inactive state and on-demand PRS

In RAN1 #107-e meeting, the following agreement was made for latency reduction:

Agreement - The following options are supported for SRS for positioning transmission by RRC_INACTIVE UEs: - Option 1: - Subject to UE capability (which is a prerequisite for option 2), a UE may be configured with an SRS for Positioning associated with the initial UL BWP and transmitted, during the RRC_INACTIVE state, inside the initial UL BWP with the same CP and SCS as configured for initial UL BWP. - Option 2: - Subject to UE capability, a UE may be configured with an SRS for Positioning where the following parameters are additionally configured for the transmission of the SRS for Positioning during the RRC_INACTIVE state: frequency location and bandwidth, SCS, CP length. The UE shall not transmit the SRS for Positioning when it is expected to perform UL transmissions in the initial UL BWP in RRC_INACTIVE state. Agreement - From RAN1 perspective, for LMF-initiated request of on-demand DL PRS, the following group of on-demand DL PRS parameters is defined and signaled - per resource set per positioning frequency layer per FR - DL PRS Periodicity - DL PRS Resource Bandwidth - DL PRS Resource Repetition Factor - Number of DL PRS Resource Symbols per DL PRS Resource - DL-PRS CombSizeN Two options for indication of DL PRS QCL-Info, either - Option 1: per resource set per positioning frequency layer per FR - LMF recommends a list of QCL sources - Option 2: per resource set per positioning frequency layer per FR - LMF requests to provide the QCL information in the assistance data in NRPPa - per FR - Number of DL PRS frequency layers - either per resource set per positioning frequency layer or per UE - Start/end time of DL PRS transmission - either per resource, or per resource set, or per UE - ON/OFF indicator (for LMF initiated request only) Agreement - From RAN1 perspective, for UE-initiated request of on-demand DL PRS, the following group of on-demand DL PRS parameters is defined and signalled - per positioning frequency layer per FR - DL PRS Periodicity - DL PRS Resource Bandwidth - DL PRS Resource Repetition Factor - Number of DL PRS Resource Symbols per DL PRS Resource - DL-PRS CombSizeN - per FR - Number of DL PRS frequency layers - per UE - Start/end time of DL PRS transmission Two options for indication of DL PRS QCL-Info, either - Option 1: per resource set per positioning frequency layer per FR - UE recommends a list of QCL sources - Option 2: per resource set per positioning frequency layer per FR UE requests to provide the QCL information in the assistance data

## Positioning in Inactive state

Working assumption

For Option 2 of SRS for positioning transmission in RRC_INACTIVE, a UE capability for switching time between SRS Tx and other Tx in initial UL BWP or Rx in initial DL BWP is introduced

- The capability is reported per band

  - The capability applies at least to TDD

  - FFS: FDD

- The switching time value(s) are left up to RAN4 discussion

- If the transmission of SRS for positioning with the switching time overlaps/collides in time domain with other DL reception or UL transmission at least for TDD, the SRS for positioning transmission is dropped in the symbol(s) where the overlap/collision occurs

  - Note: Transmission of SRS for positioning with the switching time covers the following example TDD cases:

    - “switching after SRS” (i.e., transmission of SRS + switching time)

    - “switching before SRS” (i.e., switching time + transmission of SRS)

Since the there could be SDT procedure in inactive state, which means the regular scheduled DL reception or UL transmission could happen, as well as non-scheduling transmission, e.g, RACH. So the collision handling between SRS and these signals should be considered as well.

SRS vs scheduled UL/DL signals

Following the discussion like above, we think if there are a DL signal expected to be received (or a UL signal expected to be transmitted) in the time span of SRS transmission determined by UE at a time earlier than N time unit before the start of SRS transmission with a gap, UE expects to receive DL signals (or transmit the UL signals) and drop SRS; Otherwise, UE expects to receive the DL (or transmit the UL signals) and may transmit SRS for positioning

proposal 8: if there are a DL signal expected to be received (or a UL signal expected to be transmitted) in the time span of SRS transmission with a gap determined by UE at a time earlier than N time unit before the start of SRS transmission, UE expects to receive DL signals (or transmit the UL signals) and drop SRS; Otherwise, UE expects to receive the DL (or transmit the UL signals) and may transmit SRS for positioning

SRS vs valid RO

Different from the collision handing of the SRS vs UL Tx/DL Rx, there could be case that if the SRS for pos transmission (with a gap) overlaps with a RACH resource (e.g., valid RO). In this case, such SRS should be dropped as well even this UE did not transmit RACH in this valid RO. Because RACH resource is in cell-specific manner and other UEs could transmit in this valid RO. Thus if the SRS for pos has been transmitted, it will create interference to the RACH reception at gNB side. Thus we have following proposal.

Proposal 9: if the SRS transmission (with a gap) collides with a valid RO, the SRS transmission is dropped.

DRX cycle determination

In last RAN4 meeting, the drx cycle is used to calculate the measurement period for the inactive state positioning. However, the DRX cycle is not always activated for a UE in inactive state, especially for Rel16 power saving UE. A PEI (paging early indication) has been introduced to let UE know how many DRX cycles it needs to wake for monitoring paging. Thus, we think only these indicated DRX cycles by PEI should be considered in the positioning in inactive state.

Proposal 10: only the PEI indicated DRX cycle could be used for positioning in inactive state.

## On-demand PRS

By further design for OD-PRS, a UE could be aware of whether the OD-PRS configuration is satisfied or not. To further satisfied the positioning requirement, on-demand PRS is requested to improve the location accuracy. Based on the agreements in previous meetings, UE can start measuring on-demand PRS at the start time of DL PRS transmission and will stop measuring on-demand PRS at the end time of DL PRS transmission for LMF-initiated/ UE-initiated request of on-demand DL PRS. After UE receiving the end time of DL PRS transmission, the network will stop transmitting on-demand PRS and the PRS configuration will fallback to normal PRS to perform subsequent PRS measurements. It should be noticed that the subsequent PRS measurements could be implemented in the MG or PRS processing window within current PRS measurement period.

Proposal 11: After UE receiving the end time of DL PRS transmission, the UE will stop measuring on-demand PRS and the PRS configuration will fallback to normal PRS to perform subsequent PRS measurements.

# Conclusion

This contribution discusses the latency reduction for measurement/report part for the positioning methods. Observations and proposals are summarized as follows:

Proposal 1: a priority indication is introduced to indicate the priority state between PRS resource and SSB, the candidate value can be high, low and equal inside the PRS processing window subject to UE capability.

Proposal 2: when the priority state of PRS resource and SSB is equal inside the PRS processing window, the reception of PRS and/or SSB is up to UE implementation.  Inform RAN4 about this decision.

Observation 1: the other DL signals/channels could be received when UE determines they are expected to be received before N time units earlier than the start of the first PRS in the first activated PPW.

Observation2： PRS can be received subject UE capability without affecting the reception of other DL signals/channels

Proposal 3: Adopt following collision handling between PRS and other DL signals/channels .

| PRS priority state | Collision handling vs other DL signals in the window (type 1A/1B) and overlapped part (type 2) | X factor |
| --- | --- | --- |
| high priority | - For all non-first activated PPW, PRS is received over all semi-static other DL signals except SSB; - For the first activated PPW, - if UE could determine that it expects to receive other DL signals before a N time units earlier than the start of the first activated PPW, UE expects to receive PRS and may receive the other DL signals subject to its capability - if UE could NOT determine that it expects to receive other DL signals before a N time units earlier than the start of the first activated PPW, UE expects to receive PRS and drop the other DL signals; | The PPW is activated by MAC CE (which is carried by DCI/scheduled PDSCH); so if UE cannot determine the PRS has to be received before it starts its procedure for receiving the other DL signal, then such (partial) reception of other DL signals should be allowed even the PRS reception should always be prioritized; |
| Low priority | - For an activated PPW, - If UE could determine that it expects to receive other DL signals before a N time units earlier than the start of the first PRS, UE expects to receive other DL signals and drop the PRS; - If UE could NOT determine that it expects to receive other DL signals before a N time units earlier than the start of the first PRS, UE expects to receive other DL signals and may receive the PRS subject to its capability; | When PRS is low priority, the first thing is the DL signals are always prioritized; and the PRS could be allowed to be received if UE cannot determine it should do the DL signal reception early enough, then the PRS reception could be allowed. |

Proposal 4: The priority of PRS and LP UL signals/channels follow determined PRS priority state with that for other DL signals/channels. HP UL signals/channels have higher priority than PRS.

Proposal 5: Adopt following collision handling between PRS and UL signals/channels .

| PRS priority state | Collision handling vs UL signals in the window (type 1A/1B) and overlapped part (type 2) |
| --- | --- |
| high priority | - For all non-first activated PPW, PRS is received over all semi-static other UL signals/channels transmission; - For the first activated PPW, - if UE could determine that it expects to transmit the UL signals before a N time units earlier than the start of the PRS in the first activated PPW, UE expects to receive PRS and may transmit the UL signals subject to its capability; - if UE could NOT determine that it expects to receive UL signals before a N time units earlier than the start of the first activated PPW, UE expects to receive PRS and drop the UL signals tx; |
| Low priority | - For an activated PPW, - If UE could determine that it expects to transmit the UL signals before a N time units earlier than the start of the first PRS, UE expects to transmit the UL signals and drop the PRS; - If UE could NOT determine that it expects to receive other DL signals before a N time units earlier than the start of the first PRS, UE expects to transmit UL signals and may receive the PRS subject to its capability; |

Proposal 6: no new PRS processing capability or mode is introduced.

Proposal 7: Text proposal for TS38.214  5.1.6.5	 PRS reception procedure

*** Unchanged text is omitted ***

-	For Type 1, Tsstart, Tsend is the smallest interval in msec within slot s corresponding to an integer number of OFDM symbols based on the numerology of the DL PRS of a serving cell that covers the union of the potential PRS symbols and determines the PRS symbol occupancy within slot s, where the interval Tsstart, Tsend considers the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS resource sets (target and reference).

-	For Type 2, μ is the numerology of the DL PRS, and S is the cardinality of the set S.

For the case when PRS processing window is configured, the potential DL PRS resources referred in the calculation of the duration K msec of DL PRS symbols are the DL PRS resources not collides with other higher priority DL/UL signals.

*** Unchanged text is omitted ***

Proposal 8: if there are a DL signal expected to be received (or a UL signal expected to be transmitted) in the time span of SRS transmission with a gap determined by UE at a time earlier than N time unit before the start of SRS transmission, UE expects to receive DL signals (or transmit the UL signals) and drop SRS; Otherwise, UE expects to receive the DL (or transmit the UL signals) and may transmit SRS for positioning

Proposal 9: if the SRS transmission (with a gap) collides with a valid RO, the SRS transmission is dropped.

Proposal 10: only the PEI indicated DRX cycle could be used for positioning in inactive state.

Proposal 11: After UE receiving the end time of DL PRS transmission, the UE will stop measuring on-demand PRS and the PRS configuration will fallback to normal PRS to perform subsequent PRS measurements.

# References

- RP-210903, New WID on NR Positioning Enhancements, RAN#91-e, Mar, 2021.