3GPP TSG RAN WG1 #109-e	        		     R1-2203867

e-Meeting, May 9th – 20th, 2022

Agenda item:		8.7.1

Source:			Samsung

Title:			Maintenance on enhancements for idle/inactive-mode UE power saving

Document for:		Discussion and decision

# Introduction

NR Rel-17 support the following two PS schemes for idle/inactive mode UEs,

- Paging enhancements, and

- TRS for idle/inactive UEs

For paging enhancement, we think the scheme is completed without any remaining issues. In this contribution, we discuss remaining issue to support TRS for idle/inactive UEs.

# Remaining issue

## Clarification on reference point for TRS validity duration

In RAN1#108-e meeting [1], the following agreement was made for the remaining issue about clarification on reference point for TRS validity duration.

Agreement Down-select one of the TPs to clarify the current default DRX cycle used for determining the reference point for start of validity duration for an availability indication - Alt1: Adopt TP#1 - Alt2: Adopt TP#2 - Note: TRS availability indication transmitted in all PDCCH monitoring occasions of the same PEI-O or PO is same. - Note: Reference point for TRS availability indication in a given DCI is identical for all UEs regardless of UE specific DRX value. ============================= start of TP#1========================================== 10.4B Indication of TRS resources - <Unchanged text omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame within the DRX cycle that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. - <Unchanged text omitted> ============================= end of TP#1========================================== ============================= start of TP#2========================================== 10.4B Indication of TRS resources - <Unchanged text omitted> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame of the DRX cycle associated with the first PDCCH MO of the PEI-O or PO that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. - <Unchanged text omitted> ============================= end of TP#2==========================================

According the FL summary [2], no different between Alt1 and Alt2 in most of cases when a PO/PEI-O (consists of multiple PDCCH MOs) is within a single DRX cycle. However, for the corner case, when a PO/PEI-O is configured across the boundary of a DRX cycle, the effective validity duration can be different for Alt1 and Alt2.  The behaviours on gNB and UE side for the two alternatives can be clarified as follows.

- For Alt1:

  - If UE receives TRS availability indication in a PDCCH MO (e.g. the first PDCCH MO associated QCLed with SSB#1) in DRX cycle n, the UE assumes validity duration starts in DRX cycle n, …, n+ validityDuration-1.

  - If UE receives TRS availability indication in a PDCCH MO(e.g. the last PDCCH MO) in DRX cycle n + 1 , the UE assumes validity duration starts in DRX cycle n + 1, …, n+ validityDuration.

  - Since gNB has to send the same DCI payload in all PDCCH MOs, gNB needs to make sure indicated available TRS resources are available in DRX cycle n, n +1, .., n + validityDuration.

- For Alt2:

  - If UE receives TRS availability indication in a PDCCH MO(e.g. the first PDCCH MO associated QCLed with SSB#1) in DRX cycle n, the UE assumes validity duration starts in DRX cycle n, …, n+ validityDuration-1.

  - If UE receives TRS availability indication in a PDCCH MO(e.g. the last PDCCH MO) in DRX cycle n + 1 , the UE assumes validity duration starts in DRX cycle n, …, n+ validityDuration-1.

  - gNB needs to make sure indicated available TRS resources are available in DRX cycle n, n +1, .., n + validityDuration -1.

Give above, there is no ambiguity between gNB and UE for any alternative. We prefer Alt1 for the benefit of simplicity and less spec change.

Proposal 1: Down-select Alt1/TP#1 to clarify the current default DRX cycle used for determining the reference point for start of validity duration for an availability indication.

# Conclusion

This contribution discussed remaining issue to support TRS for idle/inactive UEs. The following proposal were made:

Proposal 1: Down-select Alt1/TP#1 to clarify the current default DRX cycle used for determining the reference point for start of validity duration for an availability indication.

# Reference

[1] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #108-e

[2] R1-2202651	Moderator summary#3 on TRS/CSI-RS occasion(s) for idle/inactive UEs	Moderator (Samsung)