3GPP TSG RAN WG1 #109-e	        		               R1-2203868

e-Meeting, May 9th – 20th, 2022

Agenda item:		8.7.2

Source:			Samsung

Title:			Maintenance on DCI-based power saving techniques

Document for:		Discussion and decision

# Introduction

In this contribution, we discuss remaining issues to support DCI-based PDCCH monitoring adaptation including both SSSG switching and PDCCH skipping.

# Applicable search space sets

According to the FL summary for RAN1#108-e [1], the views on applicable search space sets for adaptation of PDCCH skipping is still controversial. A remaining issue is whether or not PDCCH skipping can impact monitoring of Type0/0A/1/2 CSS.

We are not convinced to support PDCCH skipping on search space sets or RNTIs other than USS sets and Type3-PDCCH CSS sets.  The additional power saving gain by applying PDCCH skipping to C-RNTI, MCS-C-RNTI, or CS-RNTI in Type0/0A/1/2 CSS is negligible, because UE may still monitor other RNTIs in the Type0/0A/1/2 CSS and be wake-up during the configured PDCCH MOs. More importantly, PDCCH scrambled by C-RNTI, MCS-C-RNTI or CS-RNTI in Type 0/1/1A/2 CSS are necessary for BFR or radio link failure recovery in RRC_CONNECTED state. PDCCH monitoring adaptation for power saving gain shouldn’t impact UE procedure for maintain the link quality.

Conclusion #1:  Monitoring of PDCCH candidates in Type0/0A/1 or 2 PDCCH CSS is not affected by PDCCH monitoring adaptation.

- Adopt proposed TP #1 for TS 38.213

Proposed TP #1 for TS 38.213

## 10.4 Search space set group switching and skipping of PDCCH monitoring ============================= Unchanged Text Omitted =============================== A UE can be provided a set of durations by PDCCHSkippingDurationList for PDCCH monitoring in Type3-PDCCH CSS set or USS set on a serving cell and, if the UE is not provided searchSpaceGroupIdList-r17, a DCI format 0_1, and/or DCI format 1_1, and/or DCI format 0_2, and/or DCI format 1_2 that schedules a PUSCH transmission or a PDSCH reception can include a PDCCH monitoring adaptation field of 1 bit or of 2 bits. ============================= Unchanged Text Omitted ===============================

# Impact of PDCCH skipping on SR and RA procedure

In RAN1#108-e [2], the following answer was provided to RAN2 regarding how to capture the above RAN2 agreements in RAN1 specification.

Agreement Answer 2 for R1-2200884 (R2-2201960): RAN1 would discuss and conclude how to capture the above RAN2 agreements in RAN1 specification. o It is RAN1 understanding PDCCH skipping is not applied to perform PDCCH monitoring during RAR window/MsgB window/contention resolution timer or when SR is pending. - FFS: If the UE is skipping PDCCH monitoring for a duration and at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, the UE stops PDCCH skipping (i.e., PDCCH skipping is not activated).

In order to support ignoring PDCCH skipping for SR, a UE can start monitoring PDCCH at least for DCI formats scheduling a PUSCH transmission in corresponding USS set(s). Also, UE can monitor PDCCH for DCI formats scheduling a PDSCH reception when the DCI formats have a same size as DCI format scheduling a PUSCH transmission. Alternatively, UE can perform full PDCCH monitoring for all DCI formats after the UE transmits a positive SR. For the benefit of simplicity, we are OK with the solution in the FFS bullet from the above agreement, i.e. PDCCH skipping is cancelled after transmission of a positive SR.

Proposal #1: UE cancels PDCCH skipping adaptation after transmission of a positive SR.

For the impact on RAR reception, the following UE behaviors are supported for RAR/MsgB reception in NR:

- A) UE monitors PDCCH addressed to RA-RNTI/MsgB-RNTI in ra-SearchSpace for RAR/MsgB

- B) UE monitors PDCCH addressed to C-RNTI in a search space set associated with recoverySearchSpaceId for RAR/MsgB triggered for BFR

According to the existing agreements, PDCCH skipping is applicable to USS and Type3-PDCCH CSS. So, A) is not impacted by PDCCH skipping. However, B) will be impacted as recoverySearchSpaceId can be configured for a USS set. Additional RAN1 specification work is needed to support UE ignores PDCCH skipping for RAR reception during BFR process per RAN2 agreement.

While contention resolution timer is running, for contention resolution:

- UE monitors PDCCH addressed to TC-RNTI

- UE monitors PDCCH addressed to C-RNTI, if C-RNTI was included in Msg3.

Therefore, additional RAN1 specification work is needed to support UE ignores PDCCH skipping for contention resolution addressed to C-RNTI in USS per RAN2 agreement.

The same principle for PDCCH cancellation related to SR can be reused for PDCCH cancellation during RAR window/MsgB window/ contention resolution timer. If the UE is skipping PDCCH monitoring for a duration and at the first slot after the last OFDM symbol of a PRACH transmission, the UE stops PDCCH skipping (i.e., PDCCH skipping is not activated).

Proposal #2: UE cancels PDCCH skipping adaptation after transmission of PRACH.

# Application delay

In RAN1#108-e, the following working assumption and agreement were made regarding application delay for PDCCH monitoring adaptation.

Working Assumption - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B), - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - Note: Pswitch is defined in Table 10.4-1 in TS38.213 Agreement - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating PDCCH skipping (i.e., Beh 1A), select one of the following schemes - Alt 1a: the UE applies Beh 1A on an active BWP of the serving cell at the first slot after the last OFDM symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - FFS: whether the UE will monitor PDCCH when DRX Retransmission timer is running Working assumptions: - When the UE receives DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication' field in slot n, the UE does not expect to receive DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay. - If SSSG timer expires in slot n, the UE does not expect to receive DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

If a minimum applicable scheduling offset is configured in the BWP, a larger application delay is needed due to relaxed PDCCH processing when cross-slot scheduling is enabled. The issue exists for both SSSG switching and PDCCH skipping. For SSSG switching, the critical issue is that UE may miss receiving a new PDCCH according to the new SSSG if the application delay is too small. For example, when gNB transmits a triggering PDCCH carries an adaptation indication of SSSG switching in slot n, and gNB transmits a scheduling PDCCH before slot, n + minK0, according to the new SSSG. UE will miss the scheduling PDCCH due to relaxed PDCCH processing of the triggering PDCCH. For PDCCH skipping, UE may not be able to track the remaining PDCCH skipping duration correctly if application delay is too small. When cross-slot scheduling is enabled if a minimum applicable scheduling offset is configured, a UE is not able to finish decoding a PDCCH in a first slot after the PDCCH reception. When the PDCCH carrying a PDCCH skipping duration, the UE can’t start counting remaining PDCCH skipping duration till UE finishes decoding the PDCCH.

To resolve the issues, it’s necessary to extend application delay for PDCCH monitoring adaptation based on applied minimum K0 if a minimum applicable scheduling offset is configured.

Proposal #3: Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating PDCCH monitoring adaptation, the UE applies PDCCH monitoring adaptation on an active BWP of the serving cell at the first slot that is at least minK0 slots after the PDCCH reception if a minimum applicable scheduling offset is configured

- minK0 is applied minimum applicable scheduling offset for K0

Also, UE should be able to receive scheduling PDCCH within an application delay. To avoid any ambiguity, UE doesn’t expect to receive different PDCCH adaptation indication during the application delay. Therefore, we support the working assumption about PDCCH monitoring behavior within application delay.

Proposal #4: Confirm the following working assumption

Working assumptions:

- When the UE receives DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication' field in slot n, the UE does not expect to receive DCI format  0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

- If SSSG timer expires in slot n, the UE does not expect to receive DCI format  0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

# Conclusion

This contribution discussed remaining issue for supporting PDCCH monitoring adaptation within DRX active time. The following conclusion and proposals were made:

Conclusion #1:  Monitoring of PDCCH candidates in Type0/0A/1 or 2 PDCCH CSS is not affected by PDCCH monitoring adaptation.

- Adopt proposed TP #1 for TS 38.213

Proposal #1: UE cancels PDCCH skipping adaptation after transmission of a postive SR.

Proposal #2: UE cancels PDCCH skipping adaptation after transmission of PRACH.

Proposal #3: Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating PDCCH monitoring adaptation, the UE applies PDCCH monitoring adaptation on an active BWP of the serving cell at the first slot that is at least minK0 slots after the PDCCH reception if a minimum applicable scheduling offset is configured

- minK0 is applied minimum applicable scheduling offset for K0

Proposal #4: Confirm the following working assumption

Working assumptions:

- When the UE receives DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication' field in slot n, the UE does not expect to receive DCI format  0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

- If SSSG timer expires in slot n, the UE does not expect to receive DCI format  0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

# References

[1] R1-2202894, “FL summary#5 of DCI-based power saving adaptation”, Moderator (vivo)

- [2] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #108-e