3GPP TSG RAN WG1 #109-e	R1-2203873

# e-Meeting, May 9th – 20th, 2022

Agenda item:	8.11.2

Source: 	Samsung

Title: 	Maintenance on Inter-UE Coordination for Mode2 Enhancements

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss about key remaining issues for inter-UE coordination and provide our views with text proposals.

- Remaining issues on inter-UE coordination

Open issue #1 from the status report [RP-220945]

In RAN#95-e meeting [1], it was agreed to capture the following issue in the status report [2] for Rel-17 sidelink enhancement and to be handled as part of RAN1 maintenance work as:

- Finalization of UE-B’s behavior when it receives both preferred resource set and non-preferred resource set from the same UE-A or different UE-As

We provide the agreements from RAN1#108-e meeting [3] and specifically, the FFS (marked in yellow) part requires further RAN1 discussion to finalize the issues.

Agreement - For UE-B’s behavior when UE-B receives multiple preferred resource sets from the same UE-A - It is up to UE-B implementation to use one or multiple of them in its resource (re)selection - Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same UE-A - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection Agreement - For UE-B’s behavior when UE-B receives multiple preferred resource sets from the different UE-As, - UE-B uses each received preferred resource set for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set. - Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the different UE-As. - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e (except for the processing timeline) - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As, - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

For the FFS issues, we can specify for UE-B to use both the preferred and non-preferred resources. Specifically, at first UE-B can use the intersection of preferred resource and its own sensing. As a next step, if that is not enough, UE-B can use own sensing results by excluding the non-preferred resource. However, this is an optimization issue and we can left this as UE-B’s implementation.

Proposal 1: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection when UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A or from the different UE-As.

TP is not necessary with Proposal 1.

Open issue #2 from the status report [RP-220945]

In RAN#95-e meeting [1], it was agreed to capture the following issue in the status report [2] for Rel-17 sidelink enhancement and to be handled as part of RAN1 maintenance work as:

- Finalization of relationship between start/end slots of resource selection window used for sidelink transmission carrying inter-UE coordination information and start/end slots of resource selection window for determining the set of resource

We provide the agreement from RAN1#108-e meeting [3] and specifically, the FFS (marked in yellow) part requires further RAN1 discussion to finalize the issues.

Agreement - Notations: - (n+T_1) – Start slot of resource selection window for determining the set of resources - For inter-UE coordination information triggered by UE-B’s explicit request, this value of (n+T_1) is provided by UE-B’s request as per the existing agreement - For inter-UE coordination information triggered by a condition other than explicit request reception, this value of (n+T_1) is determined by UE-A’s implementation as per the existing agreement - (n+T_2) – End slot of resource selection window for determining the set of resources - For inter-UE coordination information triggered by UE-B’s explicit request, this value of (n+T_2) is provided by UE-B’s request as per the existing agreement - For inter-UE coordination information triggered by a condition other than explicit request reception, this value of (n+T_2) is determined by UE-A’s implementation as per the existing agreement - (n’+T’_1) – Start slot of resource selection window used for sidelink transmission carrying inter-UE coordination information - (n’+T’_2) – End slot of resource selection window used for sidelink transmission carrying inter-UE coordination information - n' is the slot where UE procedure of determining TX resources of sidelink transmission carrying inter-UE coordination information is triggered - For inter-UE coordination information triggered by UE-B’s explicit request - Alt 1-1: - X1 ≤ (n’+T’_1) - (n’+T’_2) ≤ X2 - For inter-UE coordination information triggered by a condition other than explicit request reception, - Alt 2-2: - (n’+T’_2) < X3 - FFS: Values for X1, X2, X3

The purpose for defining the values for X1, X2, X3 in the above agreement is for UE-A to process the resource selection for coordination information with proper timeline with the resource selection for UE-A’s own transmission. However, the selection of resource selection window (i.e. T1 and T2) is originally up to UE implementation. Therefore, the timeline between two selection windows also can be left as UE implementation.

Proposal 2: The values for X1, X2, X3 are selected by UE-A’s implementation.

The following table provides the corresponding text proposal for Proposal 2:

Text proposal for Proposal 2: In TS 38.214 [4], Section 8.1.4A UE procedure for determining a set of preferred or non-preferred resources for another UE's transmission When this procedure is triggered, the following parameters are provided by the higher layer: - the resource pool from which the preferred or non-preferred resources are to be determined; - the resource selection window [n+T1,n+T2] within which the preferred or non-preferred resources are to be determined. It is up to UE implementation to determine timeline with the resource selection window described in clause 8.1.4; - the resource set type (either preferred or non-preferred resource set); <omitted parts>

Open issue #3

When the UE transmits IUC message or IUC request using SCI Format 2-C and MAC CE possibly with other SL data, the outcome of reception at UE-B can be one of:

- UE-B misses SCI Format 2-C, in this case UE-B doesn’t transmit PSFCH. UE-A can re-transmit the SL transmission including SCI Format 2-C and MAC CE

- UE-B successfully decodes SCI Format 2-C, but fails to decode the SL data included in the corresponding PSSCH. UE-B sends a NACK on PSFCH to UE-A. In this case, if PSSCH only includes MAC-CE, the UE-A doesn’t need to re-transmit as the information has already been received in the corresponding SCI Format 2-C. If the PSSCH includes other SL data, UE-A can re-transmit the PSSCH (MAC CE + SL data). However, re-transmitting SCI-Format 2-C is not needed as the information has already been received by UE-B. Therefore, the re-transmission of SL data can be done with SCI Format 2-A or 2-B.

- UE-B successfully decodes SCI Format 2-C, and the associated SL data. UE-B sends an ACK on PSFCH to UE-A. In this case, no re-transmission is needed.

When UE-B calculates the TBS for PSSCH, it uses the payload size of the corresponding second stage SCI Format. If the second stage SCI changes between the initial transmission and the re-transmission, and correspondingly the payload size of the SCI format changes, hence the TBS can also change. To avoid this, the TBS for a re-transmission scheduled by SCI Format 2-A or 2-B that was originally scheduled by SCI Format 2-C should use the payload size of SCI Format 2-C when calculating the TBS. A one-bit flag can be included in SCI Format 1-A to indicated whether to use the payload of SCI Format 2-A/2-B or the payload size of SCI Format 2-C when calculating the TBS.

Proposal 3: UE-A can re-transmit a SL transmission originally scheduled by SCI Format 2-C, using SCI Format 2-A or 2-B

- SCI Format 1-A, includes a one-bit flag to indicated whether to use the payload of SCI Format 2-A/2-B or the payload size of SCI Format 2-C when calculating the corresponding TBS

The following table provides the corresponding text proposal for Proposal 3:

Text proposal for Proposal 3: In TS 38.212 [5], Section 8.3.1.1 SCI format 1-A <omitted parts> Reserved – a number of bits as determined by the following: - Nreserved bits as configured by higher layer parameter sl-NumReservedBits, with value set to zero, if higher layer parameter indicationUEBScheme2 is not configured, or if higher layer parameter indicationUEBScheme2 is configured to 'Disabled', and higher layer parameter containerScheme1CoordInfo is configured to ‘Disabled’; - (Nreserved-1) bits if only one of the following is true: - higher layer parameter indicationUEBScheme2 is configured, and if higher layer parameter indicationUEBScheme2 is configured to 'Enabled' - higher layer parameter containerScheme1CoordInfo is configured to ‘Enabled’. - (Nreserved-2) bits otherwise, with value set to zero. - Conflict information receiver flag – 0 or 1 bit - 1 bit if higher layer parameter indicationUEBScheme2 is configured to 'Enabled', where the bit value of 0 indicates that the UE cannot be a UE to receive conflict information and the bit value of 1 indicates that the UE can be a UE to receive conflict information as defined in Clause 16.3.0 of [5, TS 38.213]; - 0 bit otherwise. - SCI Format 2-C re-transmission using SCI Format 2-A or 2-B flag – 0 or 1 bit - 1 bit if higher layer parameter containerScheme1CoordInfo is configured to ‘Enabled’, wherein bit value of 1 indicates that the corresponding SCI Format 1-A or SCI Format 1-B corresponds to a re-transmission of a SL transmission originally scheduled by SCI Format 2-C, bit value of 0 indicates otherwise. - 0 bit otherwise. In TS 38.214 [4], Section 8.1 UE procedure for transmitting the physical sidelink shared channel <omitted parts> The UE shall set the contents of the SCI format 2-A as follows: <omitted parts> - the UE shall set value of the 'CSI request' field as indicated by higher layers. - the UE shall set ‘SCI Format 2-C re-transmission using SCI Format 2-A or 2-B’ to - ‘1’ if the SL transmission is a re-transmission of a SL transmission originally scheduled by SCI Format 2-C. - ‘0’ otherwise The UE shall set the contents of the SCI formats 2-B as follows: <omitted parts> - the UE shall set the 'Communication range requirement' field as indicated by higher layers. - the UE shall set ‘SCI Format 2-C re-transmission using SCI Format 2-A or 2-B’ to - ‘1’ if the SL transmission is a re-transmission of a SL transmission originally scheduled by SCI Format 2-C. - ‘0’ otherwise The UE shall set the contents of the SCI format 2-C as follows: <omitted parts> - the UE shall set value of the ['Reference slot location'] as indicated by higher layers - the UE shall set ‘SCI Format 2-C re-transmission using SCI Format 2-A or 2-B’ to ‘0’ <omitted parts> In TS 38.214 [4], Section 8.1.3.2 Transport block size determination <omitted parts> - NRESCI,2 is the number of coded modulation symbols generated for 2nd-stage SCI transmission (prior to duplication for the 2nd layer, if present) according to Clause 8.4.4 of [5, TS 38.212], with the assumption of γ=0. If ‘SCI Format 2-C re-transmission using SCI Format 2-A or 2-B’ is absent or set to ‘0’, the corresponding SCI Format indicated by ‘2nd-stage SCI format’ is assumed for calculating NRESCI,2, otherwise, SCI Format 2-C is assumed for calculating NRESCI,2

Open issue #4

We provide the agreement from RAN1#107bis-e meeting [6] as follows.

Agreement For inter-UE coordination triggered by a condition rather than request reception in Scheme 1, - A resource pool level (pre-)configuration can enable one of the following alternatives: - Alt 1: it is up to UE-A’s implementation whether or not to trigger the inter-UE coordination information generation. - Alt 2: the inter-UE coordination information generation can be triggered only when UE-A has data to be transmitted together with the inter-UE coordination information to UE-B - Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the inter-UE coordination information triggered by a condition.

Since the container of inter-UE coordination information includes both MAC CE and 2nd SCI, PHY layer is needed to generate 2nd SCI format carrying inter-UE coordination information when it being triggered by a condition. Whether the condition being satisfied can be determined by higher layer, but we think the PHY layer specification should also explicitly capture the UE behaviour of when the generation and transmission of inter-UE coordination information being triggered.

In addition, for the case of inter-UE coordination triggered by explicit request, the container of explicit request includes both MAC CE and 2nd SCI as agreed in RAN1#107bis-e meeting [6] as follows:

Agreement For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives: - (Working Assumption) Alt1: MAC CE and 2nd SCI are used as the container of an explicit request transmission from UE-B to UE-A - A single format SCI 2-C is used for inter-UE coordination information and request - 1 bit in format 2-C is used to indicate whether the SCI is used for request to coordination information or for conveying coordination information - SCI 2-C is UE RX optional - It is up to UE implementation to additionally use 2nd SCI (for UE-B). - Alt2: MAC CE is used as the container of an explicit request transmission from UE-B to UE-A

Since SCI format 2-C was agreed to be a container, PHY layer itself can recognize the explicit request by SCI decoding. Correspondingly, the PHY layer specification should also explicitly capture the UE behaviour of when the generation and transmission of inter-UE coordination information being triggered by decoding SCI conveying the explicit request. Therefore, the following is proposed:

Proposal 4: Specify the procedure of UE being triggered to determine inter-UE coordination information with the following conditions:

- Triggered by a condition that UE-A has data to be transmitted together with the inter-UE coordination information to UE-B, if (pre)configured by resource pool, which can be specified as the trigger parameters being provided by higher layer.

- Triggered by an explicit request at least when a MAC CE conveying explicit request to inter-UE coordination information is received, which can be specified as the trigger parameters being provided by higher layer.

- Triggered by an explicit request at least when a SCI format 2-C conveying explicit request to inter-UE coordination information is received.

The following table provides the corresponding text proposal for Proposal 4:

Text proposal for Proposal 4: 8.1.4A UE procedure for determining a set of preferred or non-preferred resources for another UE's transmission The higher layer can request the UE to determine a set of preferred or non-preferred resources for another UE’s transmission. The UE can determine a set of preferred or non-preferred resource for another UE’s transmission when a SCI format 2-C is received with 'Providing/Requesting indicator' field set to 1. When this procedure is triggered, the following parameters are provided by the higher layer and/or in the SCI format 2-C: - the resource pool from which the preferred or non-preferred resources are to be determined; - the resource selection window [n+T1,n+T2] within which the preferred or non-preferred resources are to be determined; - the resource set type (either preferred or non-preferred resource set); - if the resource set type indicates preferred set, then the higher layer additionally provides the following parameters: - L1 priority, prioTX; - the number of sub-channels to be used for the PSSCH/PSCCH transmission in a slot, LsubCH; - the resource reservation period, Prsvp_TX, if present. The value of Cresel is determined by the UE according to clause 8.1.5.

- Conclusions

This contribution discusses on issues for maintenance of inter-UE coordination in Rel-17 NR sidelink enhancement and proposes the followings depending on the discussion:

Proposal 1: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection when UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A or from the different UE-As.

Proposal 2: The values for X1, X2, X3 are selected by UE-A’s implementation.

Proposal 3: UE-A can re-transmit a SL transmission originally scheduled by SCI Format 2-C, using SCI Format 2-A or 2-B

- SCI Format 1-A, includes a one-bit flag to indicated whether to use the payload of SCI Format 2-A/2-B or the payload size of SCI Format 2-C when calculating the corresponding TBS

Proposal 4: Specify the procedure of UE being triggered to determine inter-UE coordination information with the following conditions:

- Triggered by a condition that UE-A has data to be transmitted together with the inter-UE coordination information to UE-B, if (pre)configured by resource pool, which can be specified as the trigger parameters being provided by higher layer.

- Triggered by an explicit request at least when a MAC CE conveying explicit request to inter-UE coordination information is received, which can be specified as the trigger parameters being provided by higher layer.

- Triggered by an explicit request at least when a SCI format 2-C conveying explicit request to inter-UE coordination information is received.

References

3GPP TSG RAN#95-e, Final Report of 3GPP TSG RAN.

RP-220945, “Status report to TSG: NR Sidelink enhancement,” RAN#95-e, Mar., 2022.

3GPP TSG RAN1#108-e, Final Report of 3GPP TSG RAN.

TS 38.214, “NR; Physical layer procedures for data,” v17.1.0.

TS 38.212, “NR; Multiplexing and channel coding,” v17.1.0.

3GPP TSG RAN1#107bis-e, Final Report of 3GPP TSG RAN.