3GPP TSG RAN WG1 #109-e			R1-2203877

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.16.1

Source: 	Samsung

Title: 	Views on UE features for Rel-17 NR FeMIMO

Document for:	Discussion and Decision

Introduction

This contribution provides Samsung’s views on Rel-17 UE features for FeNR-MIMO [1].

Enhancements on Multi-beam Operation

Based on the discussion in RAN1#108-e [2], the latest version of FGs (23-1-1, 23-1-1b ~ 23-1-1j, 23-1-2, 23-1-3, 23-1-4) have been agreed.

Regarding the name of FG 23-1-1 and FG 23-1-1b which should be basic features that enables Rel-17 unified TCI framework for both intra-cell and inter-cell beam management, we support to remove bracket for “[and inter-cell]” in the name of FG to confirm that this FG can enable basic functionalities for both intra- and inter-cell beam management for Unified TCI.

Proposal 1: Remove the bracket for “[and inter-cell]” in the name of FG to support functionalities for both intra- and inter-cell beam management on FG 23-1-1 and FG 23-1-1b.

Regarding the description of FG 23-1-1g, we do not support to include beam alignment related parameters/proposals for PLRS until we have a RAN1 agreement.

Proposal 2: Do not support FG 23-1-1g without the corresponding RAN1 agreement.

2. [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS] 4. [The minimum time gap between the beam indication PDCCH and first slot where beam is applied]

Regarding Component 2 for FG 23-1-1b as above, we support the component for minimum beam activation latency (Y symbols). The candidate values for the minimum Y per SCS can be discussed further. Note that Y is configurable by the NW. Regarding Component 4 for FG 23-1-1b as above, since there is no corresponding agreement in RAN1, we don’t see any necessity.

Proposal 3: Support Component 2 for FG 23-1-1b, i.e., “[The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS]”, and the candidate values per SCS can be discussed further.

Proposal 4: Delete Component 4 for FG 23-1-1b, i.e., “[The minimum time gap between the beam indication PDCCH and first slot where beam is applied]”.

- TCI state indication [mode]: update and activation [in case of updates] b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - The maximum number of MAC-CE activated joint TCI states across per CC [in a band] [in a band combination]

Regarding the Components 1 and 3 for FG 23-1-1b as above, a UE that supports Rel-17 unified TCI framework should support at least 4 activated TCI states by MAC-CE+DCI-based TCI indication with and without DL assignment for both intra-cell and inter-cell beam management (as a basic feature).

Proposal 5: On TCI state indication and maximum number of MAC-CE activated joint TCI states:

- As a Rel-17 TCI basic feature, support 4 activated TCI states by MAC-CE+DCI-based TCI indication with and without DL assignment for both intra-cell and inter-cell beam management.

Regarding the Components 2 and 5 for FG 23-1-1 as below,

- WA: The maximum number of configured joint TCI states [per BWP per CC] [in a band] [in a band combination] - The maximum number of MAC-CE activated joint TCI states across all CC(s) in a band

For FG 23-1-1, in order to avoid duplicated functionality for the “counting” features as above components 2 and 5., e.g., if UE supports N TCI states for intra-cell, it should support N for inter-cell as well.

Proposal 6: Support same value for “counting” features for both intra-cell and inter-cell beam management.

Regarding FG 23-1-2, this feature should cover both inter-cell beam measurement and reporting. Since the components which are relevant to inter-cell beam management are already included in FG 23-1-1, we would like to rename this feature to support of inter-cell beam measurement and reporting.

Proposal 7: Rename FG 23-1-2 as “Inter-cell beam measurement and reporting”.

Also, FG 23-1-1, FG 23-1-1b, and FG 23-1-2 should be basic features for Rel-17 TCI framework. For such features, we would like to propose the signalling granularity as per band based on two reasons. First reason is that based on working assumption in RAN1#107-e, the UE is not expected to be configured with Rel-15/Rel-16 TCI/SpatialRelationInfo if the UE is configured with Rel-17 TCI in any CC in a band. Second reason is that according to RAN2 guidance, the UE features which have “per-band combination” or “per-band per band combination” should be minimized. Hence, “per band” granularity is quite natural starting point to consider for those basic features.

Proposal 8: Support FG 23-1-1, FG 23-1-1b, and FG 23-1-2 as basic features and per-band granularity.

Regarding Component in FG 23-1-4 as below,

1. Supported UE capability value [sets] and corresponding max number of SRS ports for each UE capability value [set]

Since RAN1 can only agree on one UE capability for this feature (i.e. max # SRS ports, and concluded no consensus on the support of other types), it is still not clear why “value set” is needed. It should be replaced with “value” in the above description, so we would like to remove [sets] in the above description.

Proposal 9: Remove [sets] in the Component description in FG 23-1-4.

Enhancements for Multi-TRP Deployment

  - Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH

PDCCH enhancement

Based on the discussion in RAN1#108-e, the latest version of FGs (23-2-1, 23-2-1a, 23-2-2, 23-2-4) have been agreed.

Regarding the description of Component 1 for FG 23-2-1 as follows,

1. Support of intra-slot PDCCH repetition based on two linked SS sets associated with corresponding CORESETs [with [non-SFN scheme] TDM and FDM (except FDM based PDCCH repetition for FR2)] including PDCCH repetition for Type 3 CSS

- We are fine the wording only “with TDM and FDM (except FR2)” since it seems “non-SFN scheme” and “TDM and FDM” are duplication, and since FDM in FR2 is supported only when FG 23-2-2 is supported, the clarification on FR2 is fine for us.

Proposal 10: Support “with TDM and FDM (except FR2)”, “including PDCCH repetition for Type 3 CSS” in the Component 1 and the above Note in FG 23-2-1.

Regarding FG 23-2-4 supporting simultaneous configuration of PDCCH repetition and multi-DCI based multi-TRP, since it was agreed that two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values as in RAN1#107-e, it would be better to capture the above sentence as a Note.

Proposal 11: In FG 23-2-4, add a Note “Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values”.

In RAN1#107-e, there was an agreement to handle UE complexity / memory requirements for linked PDCCH candidates as follows.

Agreement To handle UE complexity / memory requirements for linked PDCCH candidates, address the issue by UE capability, where UE indicates a limit (X) associated with the total number of linked candidates of which the first candidate is received and the second one has not been received at any given span. - The limit X is indicated as a total count assuming count 1 for AL=1; 2 for AL=2; 4 for AL=4 or 8 or 16. - The limit X is indicated per CC and also across all CCs - Note: “received” and “not been received” is wrt the end of the corresponding span of PDCCH candidate. - Above is applicable at least for the inter-span case (FFS: intra-span case)

Since it has not been discussed during UE feature discussion for Rel-17 FeMIMO, we propose to support a new FG (e.g., FG 23-2-5) including the appropriate contents of the above agreement.

- Name: UE complexity / Memory requirements for PDCCH repetition

- Component 1: A limit (X1) associated with the total number of linked candidates of which the first candidate is received and the second one has not been received at any given span per CC.

- Component 2: A limit (X2) associated with the total number of linked candidates of which the first candidate is received and the second one has not been received at any given span across all CCs.

- Pre-requisite: FG 23-2-1

- Reporting granularity: per band and per BC

- Candidate values for each component: FFS

- Note: For component 1 and 2, X1 and X2 is indicated as a total count assuming count 1 for AL=1; 2 for AL=2; 4 for AL=4 or 8 or 16. This is applied for the inter-span case.

Proposal 12: Support FG 23-2-5 as a new FG handling UE complexity / memory requirements for PDCCH repetition.

PUCCH/PUSCH enhancement

In RAN1#108-e, RAN1 clarified the below FG for inter-slot repetition as an agreement:

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown 23. NR_FeMIMO; 23-3-2; Multi-TRP PUCCH repetition scheme 1 (inter-slot); 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] [2. Support of up to two PUCCH power control parameter sets/spatial relation info per PUCCH resource]; FFS; Yes; Multi-TRP PUCCH repetition scheme 1 (inter-slot) is not supported; Per band; n/a; n/a; n/a; Note: power control parameter sets only apply to FR1 Note: spatial relation info only applies to FR2; Optional with capability signalling.

To support intra-slot mTRP PUCCH repetition (scheme 3), two sets of power control parameter per PUCCH resource in FR1 or two spatial relation infos per PUCCH resource in FR2 should be supported. Therefore, the component 2 in FG 23-3-2 is needed also for FG 23-3-3. We suggest to add component 2 of FG 23-3-2 as basic feature of FG 23-3-3 and also same note as follow:

| 23. NR_FeMIMO | 23-3-3 | Multi-TRP PUCCH repetition-intra-slot | 1. Support of PUCCH repetition scheme 3 (intra-slot repetition) - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of up to two PUCCH power control parameter sets/spatial relation info per PUCCH resource | FFS | Yes |  | PUCCH repetition scheme 3 (intra-slot repetition) is not supported | Per FS | n/a | n/a | n/a | Note: power control parameter sets only apply to FR1 Note: spatial relation info only applies to FR2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 13: Support to add component 2 and note in FG 23-3-3 as highlighted part:

| 23. NR_FeMIMO | 23-3-3 | Multi-TRP PUCCH repetition-intra-slot | 1. Support of PUCCH repetition scheme 3 (intra-slot repetition) - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of up to two PUCCH power control parameter sets/spatial relation info per PUCCH resource | FFS | Yes |  | PUCCH repetition scheme 3 (intra-slot repetition) is not supported | Per FS | n/a | n/a | n/a | Note: power control parameter sets only apply to FR1 Note: spatial relation info only applies to FR2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Enhancements on Multi-TRP inter-cell operation

For component 23-4, we do not think it is necessary to report 0 for X1 because 1 as the minimum capability for X1 should be sufficient. Support to confirm 4, 5 and 6 as candidate values for X1 and X2.

Proposal 14: Do not support candidate value 0 for X1, support to confirm candidate values 4, 5 and 6 for X1 and X2.

  - Enhancements on beam management for multi-TRP

For FG 23-5-1, we suggest candidate values for component 2 as {1, 2, 3, 4} and component 3 as {8, 16, 32, 64}.

For FG 23-5-2a and FG 23-5-2b, we do not see the need to add the note: a UE that supports FG 23-5-2 must indicate this FG is supported with at least component candidate value 1.

Proposal 15: Support components 2 and 3 in FG 23-5-1 with candidate values {1, 2, 3, 4} and {8, 16, 32, 64} respectively.

Proposal 16: Delete the note “a UE that supports FG 23-5-2 must indicate this FG is supported with at least component candidate value 1” in FG 23-5-2a and FG 23-5-2b, which is unnecessary.

  - Enhancements on HST-SFN deployment

Based on the discussion in RAN1#108-e, the latest version of FGs (23-6-1, 23-6-1-1, 23-6-1a, 23-6-1b, 23-6-2, 23-6-2b, 23-6-3, 23-6-4, 23-6-4a) have been agreed for HST-SFN.

In order to identify two QCL-TypeD properties to receive SFNed PDCCH, since FG 23-2-2 is used for PDCCH repetition, a separate FG supporting to identify two QCL-TypeD properties for SFNed PDCCH which two TCIs have been activated on a CORESET should be needed.

Proposal 17: Support separate FG as identifying two QCL-TypeD properties for SFNed PDCCH.

CSI enhancements: MTRP and FR1 FDD reciprocity

Multi-TRP CSI

Before Rel-17, when we only consider single-TRP CSI measurement hypothesis, there are UE capability signalling FG 2-35 including Component 8 meaning that UE can process Y CSI report(s) simultaneously in a CC and Component 9 meaning that UE can process X CSI report(s) simultaneously across all CCs. Component 8 of FG 2-35 can have a candidate value from 1 to 8, and Component 9 of FG 2-35 can have a candidate value from 5 to 32.

In RAN1#108-e, FG 23-7-1 has been agreed with 6 Components including the extended version of Components 8 and 9 in FG 2-35 considering NCJT CSI as well. Among those, based on Component 5, UE can report a list of (Y1, Y2) meaning that UE can process Y1 NCJT CSI and Y2 single-TRP CSI measurement hypothesis simultaneously in a CC. The possible list of candidate values can be 16 pairs of (Y1,Y2) where Y1 can be from 1 to 4 and Y2 can be from 1 to 8. Also, based on Component 6, UE can report a list of (X1, X2) meaning that UE can process X1 NCJT CSI and X2 single-TRP CSI measurement hypothesis simultaneously across all CCs. The possible list of candidate values can be 16 pairs of (X1,X2) where X1 can be from 1 to 16 and X2 can be from 1 to 32.

Therefore, comparing Component 8 of FG 2-35 with Component 5 of FG 23-7-1, if a Rel-17 UE reports (Y1,Y2) as (4,8) which is the combination of the maximum values for both Y1 and Y2, then the maximum capability is increased from 8 to 12. Similarly, comparing Component 9 of FG 2-35 with Component 6 of FG 23-7-1, if a Rel-17 UE reports (X1,X2) as (16,32) which is the combination of the maximum values for both X1 and X2, then the maximum capability is increased from 32 to 48. These increased maximum values would be significant burden for UE to calculate both single-TRP and NCJT CSI measurement hypothesis. Hence, we would like to propose the limitation of the value Y1+Y2 as 8 and X1+X2 as 32, respectively, which is aligned with the maximum capability of current UE.

Proposal 18: Regarding Component 5 and 6 in FG 23-7-1, support Y1+Y2≤8 and X1+X2≤32.

In RAN1#107b-e there was a discussion on how to incorporate UE capability reporting on the supported mixed codebook types pertinent to NCJT CSI reporting. In order to keep the structure of the legacy UE capability reporting for mixed codebook types in FG 16-8 while limiting the new capability reporting to CSI reporting for NCJT, we support a separate mixed codebook types capability reporting for NCJT as FG 23-7-1b.

Proposal 19: Support FG 23-7-1b for mixed codebook reporting for NCJT with reporting structure {Codebook 1, Codebook 2, Codebook 3}

- NCJT + Type 1 SP (for sTRP) as a candidate value for Codebook 1.

- Support a candidate value {Codebook 2, Codebook 3}={Null, Null}.

Conclusion

In this contribution, the following proposals are given:

Proposal 1: Remove the bracket for “[and inter-cell]” in the name of FG to support functionalities for both intra- and inter-cell beam management on FG 23-1-1 and FG 23-1-1b.

Proposal 2: Do not support FG 23-1-1g without the corresponding RAN1 agreement.

Proposal 3: Support Component 2 for FG 23-1-1b, i.e., “[The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS]”, and the candidate values per SCS can be discussed further.

Proposal 4: Delete Component 4 for FG 23-1-1b, i.e., “[The minimum time gap between the beam indication PDCCH and first slot where beam is applied]”.

Proposal 5: On TCI state indication and maximum number of MAC-CE activated joint TCI states:

- As a Rel-17 TCI basic feature, support 4 activated TCI states by MAC-CE+DCI-based TCI indication with and without DL assignment for both intra-cell and inter-cell beam management.

Proposal 6: Support same value for “counting” features for both intra-cell and inter-cell beam management.

Proposal 7: Rename FG 23-1-2 as “Inter-cell beam measurement and reporting”.

Proposal 8: Support FG 23-1-1, FG 23-1-1b, and FG 23-1-2 as basic features and per-band granularity.

Proposal 9: Remove [sets] in the Component description in FG 23-1-4.

Proposal 10: Support “with TDM and FDM (except FR2)”, “including PDCCH repetition for Type 3 CSS” in the Component 1 and the above Note in FG 23-2-1.

Proposal 11: In FG 23-2-4, add a Note “Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values”.

Proposal 12: Support FG 23-2-5 as a new FG handling UE complexity / memory requirements for PDCCH repetition.

Proposal 13: Support to add component 2 and note in FG 23-3-3 as highlighted part:

| 23. NR_FeMIMO | 23-3-3 | Multi-TRP PUCCH repetition-intra-slot | 1. Support of PUCCH repetition scheme 3 (intra-slot repetition) - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of up to two PUCCH power control parameter sets/spatial relation info per PUCCH resource | FFS | Yes |  | PUCCH repetition scheme 3 (intra-slot repetition) is not supported | Per FS | n/a | n/a | n/a | Note: power control parameter sets only apply to FR1 Note: spatial relation info only applies to FR2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 14: Do not support candidate value 0 for X1, support to confirm candidate values 4, 5 and 6 for X1 and X2.

Proposal 15: Support components 2 and 3 in FG 23-5-1 with candidate values {1, 2, 3, 4} and {8, 16, 32, 64} respectively.

Proposal 16: Delete the note “a UE that supports FG 23-5-2 must indicate this FG is supported with at least component candidate value 1” in FG 23-5-2a and FG 23-5-2b, which is unnecessary.

Proposal 17: Support separate FG as identifying two QCL-TypeD properties for SFNed PDCCH.

Proposal 18: Regarding Component 5 and 6 in FG 23-7-1, support Y1+Y2≤8 and X1+X2≤32.

Proposal 19: Support FG 23-7-1b for mixed codebook reporting for NCJT with reporting structure {Codebook 1, Codebook 2, Codebook 3}

- NCJT + Type 1 SP (for sTRP) as a candidate value for Codebook 1.

- Support a candidate value {Codebook 2, Codebook 3}={Null, Null}.

References

- 3GPP RAN, RP-193133, Samsung, New WID: further enhancements on MIMO for NR

- R1-2202929, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e”, AT&T, NTT DOCOMO, INC.