3GPP TSG RAN WG1 #109-e					R1-2203878

e-Meeting, May 10th – 20th, 2022

Agenda item:	8.16.2

Source:	Samsung

Title:	UE features for supporting NR from 52.6 GHz to 71 GHz

Document for:	Discussion and Decision

# Introduction

This contribution includes remaining issues for Rel-17 UE features for supporting NR from 52.6 GHz to 71 GHz, wherein the latest set of UE features are provided in [1].

# Remaining Issues for FG 24-8 and FG 24-9

In the last RAN1 meeting, the FG groups related to 32 HARQ processes in DL (FG 24-8) and UL (FG 24-9) have been discussed, and the remaining issue focuses on the type of the FGs.

| 24. NR_ext_to_71GHz | 24-8 | 32 DL HARQ processes for FR 2-2 | Support 32 HARQ processes in DL for 120/480/960 kHz | 24-1 | Yes | N/A | 32 DL HARQ processes for FR 2-2 is not supported | FFS | N/A | N/A | N/A | A UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL in FR2-2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-9 | 32 UL HARQ processes for FR 2-2 | Support 32 HARQ processes in UL for 120/480/960 kHz | 24-1 | Yes | N/A | 32 DL HARQ processes for FR 2-2 is not supported | FFS | N/A | N/A | N/A | A UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL in FR2-2 | Optional with capability signalling |

32 HARQ processes in DL/UL is not a considered as mandatorily needed in implementation but an optimization. In this sense, enforcing the FG of 32 HARQ processes in DL/UL as per UE or per band would be too inefficient in terms of UE over-designing or under-reporting, especially if the UE is in general wants to include FR2-2 as an add-on to other CA combinations. Setting the type as per UE or per band will enforce the UE to potentially drop the support of certain CA combination. Such concern could happen in current release (up to RAN4 design of CA combo), or happen in later releases when new CA combo may be introduced, and we should not restrict ourselves in the implementation from the very beginning.

For example, assume band A as a FR2-2 band for which a UE considers the support of 32 HARQ, and further assume band B and band C as non-FR2-2 bands which can be a part of CA combo with band A. Then, for (A, B) CA combo, there are total 32+16=48 HARQ when band A has 32 HARQ. For (A, B, C) CA combo, there are total 32+16+16=64 HARQ when band A has 32 HARQ. Hence, if a UE wants to support (A, B, C) and 32 HARQ on band A, then it forces a UE to support total 64 HARQ, which deprives a UE of considerable amount of control on its memory budget which it is supposed to be given. In this case, it is much more desirable to allow a UE to limit total HARQ to 48 while also supporting (A, B, C) CA combo. However, if 32 HARQ is per-band, there is no chance to support so. A UE needs to have 64 total HARQ unless it decides to drop support of either 32 HARQ on band A altogether or (A, B, C) combo itself.

The issue with above example can be avoided/mitigated by support the FGs per BC, but still it could be undesirable in a more complicated scenario. For example, if band A and B are FR2-2 bands while band C is a non-FR2-2 band, then 32 HARQ on both A and B would mean total 80 HARQ in (A, B, C) combo. If a UE has memory budget of 64 HARQ, then it could have supported (A, B, C) while supporting 32 HARQ only in either band A or B. To allow this, the FG as FSPC is needed.

Overall, if there is only single FG which needs to be FSPC in the entire Rel-17, it should be the one for 32 HARQ processes, since the number of HARQ processes has the most direct relevance and impact to the amount of memory in the modem.

Proposal: Revise FG 24-8 and FG 24-9 as follow:

| 24. NR_ext_to_71GHz | 24-8 | 32 DL HARQ processes for FR 2-2 | Support 32 HARQ processes in DL for 120/480/960 kHz | 24-1 | Yes | N/A | 32 DL HARQ processes for FR 2-2 is not supported | FFS FSPC | N/A | N/A | N/A | A UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL in FR2-2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-9 | 32 UL HARQ processes for FR 2-2 | Support 32 HARQ processes in UL for 120/480/960 kHz | 24-1 | Yes | N/A | 32 DL UL HARQ processes for FR 2-2 is not supported | FFS FSPC | N/A | N/A | N/A | A UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL UL shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL UL in FR2-2 | Optional with capability signalling |

# Conclusion

The proposal made in this contribution is summarized below:

Proposal: Revise FG 24-8 and FG 24-9 as follow:

| 24. NR_ext_to_71GHz | 24-8 | 32 DL HARQ processes for FR 2-2 | Support 32 HARQ processes in DL for 120/480/960 kHz | 24-1 | Yes | N/A | 32 DL HARQ processes for FR 2-2 is not supported | FFS FSPC | N/A | N/A | N/A | A UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL in FR2-2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-9 | 32 UL HARQ processes for FR 2-2 | Support 32 HARQ processes in UL for 120/480/960 kHz | 24-1 | Yes | N/A | 32 DL UL HARQ processes for FR 2-2 is not supported | FFS FSPC | N/A | N/A | N/A | A UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL UL shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL UL in FR2-2 | Optional with capability signalling |

# Reference

[1] R1-2202928, Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e, Moderators (AT&T, NTT DOCOMO, INC.), February, 2022.