3GPP TSG RAN WG1 #109-e			R1-2203880

e-Meeting, May 9th – 20th, 2022

Agenda Item:		8.16.5

Source:				Samsung

Title:					UE features for NR positioning enhancements

Document for:	Discussion

# Introduction

This contribution includes updated RAN1 UE features for Rel-17 NR provided from WI rapporteurs in [1]. This contribution discusses our views for some of the items.

# PRS measurement outside MG

## 27-3-3

Based the UE capability agreement in the RAN1#108 meeting, 27-3-3 is described as follows, with the yellow highlighted text not yet agreed:

| 27-3-3 | DL PRS Processing Capability outside MG - buffering capability | …… [2. Maximum duration of DL PRS symbols N in units of ms a UE can process in the first part of a PRS processing window assuming maximum DL PRS bandwidth in MHz, such that the UE is capable of reporting the measurements T-N ms after the last PRS symbol] …… | 27-3-2 | DL PRS measurement outside MG and in a PRS processing window is not supported | Per band | …… [Candidate 2 component values: a) N: {0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12} ms b) T: {N+4, N+5, N+6, N+8} ms] …… |
| --- | --- | --- | --- | --- | --- | --- |

In R16 UE capability, the component 3 in 13-1 Common DL PRS Processing Capability is described as follows:

- Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. - T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms - N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms

In our view, for low latency positioning, the PRS processing time need to be smaller than the values in R16. Thus for the set of values of T in component 2, we suggest to include some smaller value in T, e.g., {1,2,4}ms. And we find it’s not necessary to build fixed dependence on N for T, e.g., the N+5 etc, as it is sufficient that UE can choose to report the suitable value from the value sets.

Proposal 1: Support smaller numbers for T  in the existing UE PRS processing capability (N, T).

- T: {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms

| 27-3-3 | DL PRS Processing Capability outside MG - buffering capability | …… [2. Maximum duration of DL PRS symbols N in units of ms a UE can process in the first part of a PRS processing window assuming maximum DL PRS bandwidth in MHz, such that the UE is capable of reporting the measurements T-N ms after the last PRS symbol] …… | 27-3-2 | DL PRS measurement outside MG and in a PRS processing window is not supported | Per band | …… [Candidate 2 component values: a) N: {0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12} ms b) T: {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms] …… |
| --- | --- | --- | --- | --- | --- | --- |

# LOS/NLOS Indicator

For the UE capability regarding LOS/NLOS indication, the agreement in RAN1#108 is shown as follows with the yellow highlighted text not yet agreed:

| 27. NR_pos_enh | 27-12 | LOS/NLOS indicator for UE-based positioning assistance data | Support reception of the assistance data containing the LOS/NLOS indicator. 1.LOS/NLOS indicator type 2.LOS/NLOS indicator granularity |  |  |  | [Component 1 candidate values: {softValue, hardValue, both}] Component 2 candidate values: {resourceSpecific, trpSpecific[, both]} Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

According to the RAN1 #107 agreement, the LOS/NLOS indicators can be either soft values or hard values.

Agreement - Support the following two options of values for LoS/NLoS indicator reporting from UE/TRP: - Soft values: [0, 0.1, …, 0.9, 1] (in steps of 0.1) - Hard values: [0, 1] - The values correspond to the likelihood of LoS, with a value of 1 corresponding to LoS and a value of 0 corresponding to NLoS

A UE could support one of these options (either soft reporting or hard reporting). However, we did not find much motivation for supporting both (soft and hard reporting). For an advanced UE, if it’s capable of having soft values, it can choose to report hard or soft value, while the hard value is the only choice for a less advanced UE. But if “both” is indicated, which means UE gives the right to gNB to decide which type to be reported. While gNB or LMF did not have to choose since it’s the fact that soft value is more useful than hard value.  Thus, we support including either hard or soft values for component 1.

For component 2, “both” is useful because the two types are not exclusive. Thus, “both” is supported.

Proposal 2：Support the following statements regarding LOS/NLOS indicators for feature 27-4-1:

- Component 1 candidate values: {hard value, soft value}

- Component 2 candidate values: {trpSpecific, resourceSpecific, both}

| 27. NR_pos_enh | 27-12 | LOS/NLOS indicator for UE-based positioning assistance data | Support reception of the assistance data containing the LOS/NLOS indicator. 1.LOS/NLOS indicator type 2.LOS/NLOS indicator granularity |  |  |  | Component 1 candidate values:{softValue, hardValue} Component 2 candidate values: {resourceSpecific, trpSpecific, both} Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

# Conclusion

This contribution discusses the UE features for NR positioning enhancements. The proposals are summarized as follows:

Proposal 1: Support smaller numbers for T  in the existing UE PRS processing capability (N, T).

- T: {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms

| 27-3-3 | DL PRS Processing Capability outside MG - buffering capability | …… [2. Maximum duration of DL PRS symbols N in units of ms a UE can process in the first part of a PRS processing window assuming maximum DL PRS bandwidth in MHz, such that the UE is capable of reporting the measurements T-N ms after the last PRS symbol] …… | 27-3-2 | DL PRS measurement outside MG and in a PRS processing window is not supported | Per band | …… [Candidate 2 component values: a) N: {0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12} ms b) T: {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms] …… |
| --- | --- | --- | --- | --- | --- | --- |

Proposal 2：Support the following statements regarding LOS/NLOS indicators :

- Component 1 candidate values: {hard value, soft value}

- Component 2 candidate values: {trpSpecific, resourceSpecific, both}

| 27. NR_pos_enh | 27-12 | LOS/NLOS indicator for UE-based positioning assistance data | Support reception of the assistance data containing the LOS/NLOS indicator. 1.LOS/NLOS indicator type 2.LOS/NLOS indicator granularity |  |  |  | Component 1 candidate values:{softValue, hardValue} Component 2 candidate values: {resourceSpecific, trpSpecific, both} Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

# References

- R1-2202928, Updated RAN1 UE features list for Rel-17 NR after RAN1 108-e, Moderator, RAN1#108e.