3GPP TSG RAN WG1 #109-e             	                              								R1-2203881

e-Meeting, May 9th – 20th, 2022

Agenda Item:		8.16.6

Source:			Samsung

Title:			Discussion on UE features for RedCap

Document for:	             Discussion

- Introduction

In RAN1#108-e meeting the following agreements were made for RedCap.

Agreement Following capabilities are added as components in FG 28-1 - Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Separate initial DL BWP for RedCap UEs - It includes CSS/CORESET for random access - FFS: For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included Agreement - Component in FG 28-1 is updated as follows - Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources - Add a note in FG 28-1: It is up to RAN2 whether/how to capture the capabilities for early indication of RedCap UE in Msg 3 and Msg A Agreement - Delete FG 28-2 - (As conclusion) It is up to RAN4 whether/how to report UE Rx structure for FR2.

- Discussion on RedCap UE feature

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28. NR_redcap | 28-1 | RedCap UE | 1. Maximum FR1 RedCap UE bandwidth is 20 MHz. 2. Maximum FR2 RedCap UE bandwidth is 100 MHz. 3. Early indication of RedCap UE in Msg.1 for 4-step RACH 4. Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources 5. Separate initial DL BWP for RedCap Ues - It includes CSS/CORESET for random access - FFS: For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included FFS whether to add any other basic features for RedCap UE |  | Yes |  | [Network assumes the UE is not a RedCap UE] | Per UE | [No] | [No] |  | [RedCap UEs do not support carrier aggregation or dual connectivity.] It is up to RAN2 whether/how to capture the capabilities for early indication of RedCap UE in Msg 3 and Msg A | Optional with capability signaling RedCap UE must indicate this FG is supported |
| 28. NR_redcap | 28-3 | Half-duplex FDD operation type A for RedCap UE | 1. Half-duplex FDD operation (instead of full-duplex FDD operation) type A for RedCap UE | 28-1 | Yes |  | [UE is assumed to support FD-FDD in FDD bands] | Per band | [FDD only] | [FR1 only] |  |  | Optional with capability signaling |

The above table [1] summriazed the current status on RAN 1 UE feature for Redcap.

  - For FG 28-1

The following agreements were made in RAN 1 #108:

Agreement

A RedCap UE supports existing applicable mandatory feature(s) that are based on SSB using NCD-SSB (including NCD-SSB based measurements) as mandatory feature(s) in an RRC-configured DL BWP that does not include CD-SSB.

- NCD-SSB is ‘QCL’-ed with CD-SSB when the NCD-SSB and CD-SSB share the same SSB index.

- Note: RAN1 assumes that NCD-SSB is configured by higher layer

We suggest to add “UE supports existing applicable mandatory feature(s) that are based on SSB using NCD-SSB in an RRC-configured DL BWP that does not include CD-SSB” as description for Separate initial DL BWP for RedCap UEs.

Moreover, for the FFS for paging, we think the FFS can be removed.

The following was captured in TS 38.306 v12.0.0

supportOfRedCap-r17

Indicates that the UE is a RedCap UE with comprised of at least the following functional components:

- Maximum FR1 RedCap UE bandwidth is 20 MHz;

- Maximum FR2 RedCap UE bandwidth is 100 MHz;

- Support of RedCap early indication based on Msg1, MsgA and Msg3 for random access;

A RedCap UE shall always set the capability to “1”.

We suggest to align with RAN 2 spec on the description of early indication for Redcap.

Proposal #1: Update the component of FG 28-1 as following:

1. Maximum FR1 RedCap UE bandwidth is 20 MHz.

2. Maximum FR2 RedCap UE bandwidth is 100 MHz.

3. Early indication of RedCap UE in Msg.1 for 4-step RACH Msg1, MsgA and Msg3 for random access.

4. Separate initial UL BWP for RedCap UEs

- It includes the configuration(s) needed for RedCap UE to perform random access

- Enabling/disabling of frequency hopping for common PUCCH resources

5. Separate initial DL BWP for RedCap Ues

- It includes CSS/CORESET for random access

- FFS: For separate initial DL BWP used for paging, CD-SSB is included

- For separate initial DL BWP only used for RACH, SSB may or may not be included

- UE supports existing applicable mandatory feature(s) that are based on SSB using NCD-SSB in an RRC-configured DL BWP that does not include CD-SSB

FFS whether to add any other basic features for RedCap UE

- Conclusions

This contribution discusses on UE features for Redcap and proposes the followings depending on the discussion:

Proposal #1: Update the component of FG 28-1 as following:

1. Maximum FR1 RedCap UE bandwidth is 20 MHz.

2. Maximum FR2 RedCap UE bandwidth is 100 MHz.

3. Early indication of RedCap UE in Msg.1 for 4-step RACH Msg1, MsgA and Msg3 for random access.

4. Separate initial UL BWP for RedCap UEs

- It includes the configuration(s) needed for RedCap UE to perform random access

- Enabling/disabling of frequency hopping for common PUCCH resources

5. Separate initial DL BWP for RedCap Ues

- It includes CSS/CORESET for random access

- FFS: For separate initial DL BWP used for paging, CD-SSB is included

- For separate initial DL BWP only used for RACH, SSB may or may not be included

- UE supports existing applicable mandatory feature(s) that are based on SSB using NCD-SSB in an RRC-configured DL BWP that does not include CD-SSB

FFS whether to add any other basic features for RedCap UE

References

- R1-2202928	Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e	Moderators (AT&T, NTT DOCOMO, INC.)