3GPP TSG RAN WG1 #109-e	R1-2203884

# e-Meeting, May 9th – 20th, 2022

Agenda item:	8.16.13

Source:	Samsung

Title:	UE features for DSS

Document for:	Discussion and decision

- Introduction

This contribution discusses UE features for Rel-17 DSS WI based on the updated Rel-17 UE features [1].

- Disabling scaling factor α

According the following RAN1 agreement, FGs for disabling scaling factor α are needed.

Agreement - For a UE configured for CCS from sSCell to P(S)Cell, scaling factor α is not applied for PDCCH overbooking/BD/CCE limit computation when sSCell is deactivated, or when an activated sSCell is switched to dormant BWP; otherwise scaling factor α is applied - Timing for disabling scaling factor α when sSCell is deactivated follows sSCell deactivation timing in current specifications, i.e., no later than the minimum requirement defined in TS 38.133 as captured in 38.213 subclause 4.3 - Timing for disabling scaling factor α follows the non-dormant to dormant BWP switching delay in current specifications ( TS 38.133). - Introduce separate FG to indicate UE support for disabling scaling factor α when sSCell is deactivated - Introduce separate FG to indicate UE support for disabling scaling factor α when activated sSCell is switched to dormant BWP - Note: It is up to UE implementation whether/when to apply the scaling factor α during sSCell activation and during sSCell BWP switch

As per the agreement, introducing each separate FG would be fine while a single FG for both sSCell deactivation and dormant BWP is also acceptable for us. Like other FGs 34-1 and 34-2, the granularity can be ‘Per BC’.

Proposal 1: Add FGs to indicate UE support for disabling scaling factor α as following:

| Feature group | Components | Prerequisite feature groups | Type |
| --- | --- | --- | --- |
| Disabling scaling factor α for deactivated sSCell | Indicate UE support for disabling scaling factor α when sSCell is deactivated | 34-1 or 34-2 | Per BC |
| Disabling scaling factor α for dormant BWP | Indicate UE support for disabling scaling factor α when activated sSCell is switched to dormant BWP | 34-1 or 34-2 | Per BC |

- PDCCH monitoring occasion(s)

As for PDCCH monitoring occasion (MO), two candidate values have been discussed:

[Value 1: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is within the first 3 OFDM symbols of a PCell/PSCell slot. Value 2: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is not restricted to the first 3 OFDM symbols of a PCell/PSCell slot]

Current formulation is referring the MOs not only ‘on sSCell’ but also ‘on PCell/PSCell’. However, since the MOs for Type 0/0A/1/2 CSS do not have ‘the first 3 OFDM symbols’ restriction, it should be clarified to avoid unnecessary restriction. Further, it is understood that we are not targeting any symbol positions for ‘Value 2’. For example, in case of 15kHz PCell and 30kHz sSCell, the intended option would be either within in the first 3 OFDM symbols of the first sSCell slot or within the first 3 OFDM symbols of the second sSCell slot overlapping with the PCell slot.

Proposal 2: Update component 9) and 7) for FG 34-1 and 34-2, respectively, and corresponding candidate values as following:

Component: PDCCH monitoring occasion(s) on sSCell for cross-carrier scheduling to PCell/PSCell

Candidate values:

[Value 1: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is within the first 3 OFDM symbols of sSCell slot overlapping with the first 3 OFDM symbols of a PCell/PSCell slot.

Value 2: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is not restricted to within the first 3 OFDM symbols of any sSCell slot overlapping with a PCell/PSCell slot]

- Frame boundary alignment

Regarding unaligned frame boundary for sSCell, RAN1 has not yet addressed whether it is supported or not.

Conclusion (RAN1#106bis-e) - A UE configured for cross-carrier scheduling from SCell to P(S)Cell can also be configured with unaligned CA (i.e., using ca-SlotOffset ), and a non-zero value for ca-SlotOffset can be configured at least for SCells other than the sSCell - FFS: Whether case when sSCell is configured with non-zero ca-SlotOffset is supported and any associated capability signalling - Note: No additional L1 spec impact related to ca-SlotOffset had been identified

Since RAN1 discussion under UE feature so far can be assumed that it is supported, we need to first agree upon that point. Then, next question is whether to have a separate FG or not. Our view is that it is good enough for a UE to indicate Rel-16 FG 18-7 (CA with non-aligned frame boundaries) and FG34-1 (or 34-2) with corresponding band combination, i.e., no separate FG for unaligned frame boundary for sSCell.

Proposal 3: Support sSCell with non-zero ca-SlotOffset.

Proposal 4: Remove “FFS: frame boundary alignment between PCell/PSCell and sSCell” from FGs 34-1 and 34-2.

- Supported SCS combinations

In terms of the number of unicast DCI limits (component #4 for both FGs 34-1 and 34-2), the supported SCS combinations are still open. It is mainly whether or not to support PCell SCS beyond 15kHz:

- FFS: N is based on pair of (PCell/PSCell SCS, sSCell SCS): N=1 for(15,15), (30,30), (60,60) and N=2 for (15,30), (30,60) and N=4 for (15, 60)

We are open to discuss further in RAN1#109e. However, if RAN1 fails to converge in RAN1#109e, it would be better to remove (30, 30), (60, 60), and (30, 60) and close the discussion.

Proposal 5: Conclude the supported SCS combinations in RAN1#109e. Otherwise, remove (30, 30), (60, 60), and (30, 60).

- DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell for Type A UE

RAN1 has agreed the following for Type A UE (FG 34-1):

2) Search space restrictions: sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and at least following search space sets on PCell/PSCell can only be configured such that UE does not monitor them in overlapping slot of PCell/PSCell and sSCell - USS sets for DCI formats 0_1,1_1,0_2,1_2 - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI

Therefore, the following component #8 can be removed for consistency:

8) FFS: Support of monitoring DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s)

Proposal 6: Remove component 8) for FG 34-1.

- Conclusions

Based on above discussion, the followings are proposed:

Proposal 1: Add FGs to indicate UE support for disabling scaling factor α as following:

| Feature group | Components | Prerequisite feature groups | Type |
| --- | --- | --- | --- |
| Disabling scaling factor α for deactivated sSCell | Indicate UE support for disabling scaling factor α when sSCell is deactivated | 34-1 or 34-2 | Per BC |
| Disabling scaling factor α for dormant BWP | Indicate UE support for disabling scaling factor α when activated sSCell is switched to dormant BWP | 34-1 or 34-2 | Per BC |

Proposal 2: Update component 9) and 7) for FG 34-1 and 34-2, respectively, and corresponding candidate values as following:

Component: PDCCH monitoring occasion(s) on sSCell for cross-carrier scheduling to PCell/PSCell

Candidate values:

[Value 1: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is within the first 3 OFDM symbols of sSCell slot overlapping with the first 3 OFDM symbols of a PCell/PSCell slot.

Value 2: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is not restricted to within the first 3 OFDM symbols of any sSCell slot overlapping with a PCell/PSCell slot]

Proposal 3: Support sSCell with non-zero ca-SlotOffset.

Proposal 4: Remove “FFS: frame boundary alignment between PCell/PSCell and sSCell” from FGs 34-1 and 34-2.

Proposal 5: Conclude the supported SCS combinations in RAN1#109e. Otherwise, remove (30, 30), (60, 60), and (30, 60).

Proposal 6: Remove component 8) for FG 34-1.

Reference:

- R1-2202928, Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e, Moderators (AT&T, NTT DOCOMO, INC.)