3GPP TSG RAN WG1#109-e			R1-2203890

e-Meeting, May 9th – May 20th, 2022

Agenda item:	9.1.2

Source: 	Samsung

Title: 	Views on CSI enhancements

Document for:	Discussion and Decision

# Introduction

The Rel-18 WID [1] includes the following objectives regarding the CSI enhancements.

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking - Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

This contribution provides Samsung’s view regarding the Rel-18 CSI enhancements.

# CSI for Coherent-JT

## Scenarios

Figure 1: necessity of distributed MIMO at low bands

The study of coherent joint transmission (C-JT) from multiple TRPs (mTRPs) should target the following scenarios.

- Intra-cell mTRP for sub-1GHz bands (e.g. 700MHz): Recently, sub-1GHz frequency bands previously allocated for 4G deployments and digital TVs are being re-farmed for 5G NR. Sub-1GHz bands have several beneficial properties such as wide coverage thanks to the low path loss and less susceptible to Doppler effect. Due to the smaller available bandwidth, MIMO becomes even more crucial. At the same time, as shown in Figure 1, due to the large wavelength at the low frequency bands, employing an array with large number of antenna elements at a gNB is infeasible. This motivates utilizing geographically distributed remote radio heads (RRHs). Here, (N-1) RRHs are simple array extension of 1 array at gNB, i.e., 1 smart TRP and (N-1) dummy TRPs. In such a distributed antenna setup the number of ports per “TRP” tends to be small, for example, to 4 or 8.

- Inter-cell mTRP: some of the relevant inter-cell mTRP scenarios include inter-cell coordination (e.g. to improve coverage). The mTRP enhancements in Rel-16/17 focuses mainly on NC-JT transmission. For commercial deployments with high-performance backhaul and synchronization, system performance can be further improved with C-JT. Since such scenario is more relevant to typical PCS bands such as 2-3GHz bands, the number of ports per TRP in this scenario can be larger, e.g. 16.

Proposal 1: Consider at least the following scenarios for Type II C-JT codebook design

- Scenario 1: intra-cell, mTRP @ sub-1GHz bands (e.g. 700MHz) with 4 or 8 ports per TRP and {2, 3, 4} TRPs

- Scenario 2: inter-cell, mTRP @ 2GHz with 16 ports per TRP and 3 TRPs

## Potential enhancements

    - Measurements

There are two alternatives regarding the channel measurement (CMRs) for C-JT CSI.

- Alt1: the CMRs can be similar to CMRs in Rel-17 NCJT CSI. In particular, multiple NZP CSI-RS resources can be configured in a CSI-RS resource set, where a NZP CSI-RS resource corresponds to a TRP.

- Alt2: the CMRs can be similar to CMRs for CSI reporting based on Rel-15 Type I multi-panel codebook. That is, a NZP CSI-RS resource is configured in a CSI-RS resource set, where the NZP CSI-RS resource comprises multiple antenna port groups, and an antenna port group corresponds to a TRP.

These alternatives are similar to beam-formed (BF) CSI-RS resources with K=1 and K>1 supported in Rel-14 LTE. In particular, Alt1 is similar to BF CSI-RS resource with K>1 and Alt2 is similar to BF CSI-RS resource with K=1. In our view, both alternatives should be supported for C-JT CSI since they can be beneficial for different use cases and scenarios, and can also provide flexibility to the NW. For instance, Alt1 can be more suitable for a decoupled codebook which comprises separate intra-TRP components (per TRP precoder) and inter-TRP co-amplitude/phase, and Alt2 can be more suitable for a joint codebook which comprises at least one joint component (across TRPs).

Proposal 2: Support the following setups for C-JT CMR

- Alt1: >1 NZP CSI-RS resources, 1 NZP CSI-RS resource per TRP

- Alt2: 1 NZP CSI-RS resource comprising >1 antenna port groups, 1 antenna port group per TRP

    - Type II codebook refinement

Figure 2

Figure 2 illustrates two examples of Rel-16/17 Type II codebook refinements for Type II C-JT CSI. The first (top part of the figure) is an example of a decoupled codebook (referred to as CB1), where the codebook comprises the following components

- Intra-TRP components: N “parts” of per-TRP Rel-16/17 Type-II (P1 and P2 in the figure), where N is number of TRPs

- Inter-TRP components: co-amplitude/co-phase for each TRP (q1 and q2 in the figure)

This codebook structure is analogous that of Rel-15 Type-I multi-panel codebook where Type-II substitutes Type-I.

The second example (bottom part of the figure) is an example of a joint codebook (referred to as CB2), where comprises the following components

- N “parts” of SD basis, one for each TRP

- One joint Wf (FD basis) performing FD compression based on aggregated/composite channel across TRPs

- One joint W2 for the aggregated/component channel across TRPs

This codebook structures follow those of Rel-16 eType-II and Rel-17 FeType-II.

To ensure continuity with Rel-16/17 Type-II codebooks and manageable scope in Rel-18, maximum reuse of the Rel-16/17 detailed designs (e.g. SD/FD basis designs, UCI parameters) should be the goal. One potential area where some further optimization can be beneficial is the supported parameter combinations to ensure not only competitive UPT-overhead trade-off, but also to prevent excessive increase in PMI payload.

Extending the legacy concept of strongest coefficient to multi-TRP, introducing an indicator for the strongest TRP is also beneficial especially for overhead reduction. Similar to the SCI reporting in Rel-16/17, an indication about the strongest TRP may be needed if a component of the codebook (e.g. co-amplitude/co-phase) is normalized with respect to the strongest TRP.

Proposal 3: Support the following Type II codebook refinement for C-JT

- CB1 (decoupled): comprising following components

  - Intra-TRP: per TRP Rel-16/17 Type II codebook components (W1, W2, Wf)

  - Inter-TRP: newly introduced co-amplitude and co-phase

- CB2 (joint): comprising following components

  - Per TRP Rel-16/17 Type II SD basis W1

  - Single Rel-16/17 Type II FD basis Wf jointly applied across all TRPs

  - Single Rel-16/17 Type II linear combination coefficients (LCCs) W2 jointly applied across all TRPs

- Maximum reuse of the Rel-16/17 detailed designs (e.g. SD/FD basis designs, UCI parameters)

  - Possible optimization in the supported parameter combinations

  - Introduce strongest TRP indication for components that are reported relative to a reference (e.g. co-amplitude/co-phase)

## Simulation results

To analyse the user throughput (UPT) vs. the amount of PMI overhead (number of bits), the existing Rel-16 parameter combination (paraComb) table (Table 5.2.2.2.5-1 in TS.38.214) is utilized for CB1, CB2, and Rel-16 eType-II CB in the simulations. The detailed simulation assumptions for the SLS results are described in Table 1 of Appendix A in [2].

Figure 3 shows the average UPT gain vs. the amount of PMI overhead for CB1, CB2, and single-TRP (sTRP) with Rel-16 eType-II CB, with respect to paraComb=1,2,⋯,6, for the intra-cell scenario of N=4 and 8 ports per TRP. The avg. UPT gain is shown with respect to Rel-16 eType-II CB with paraComb=1 as reference.

It can be observed from Figure 3 that the proposed CB2 and CB1 yield 45% and 35% higher throughput than that of sTRP with Rel-16 eType-II CB, respectively. Such large gain comes from the joint precoding operation across TRPs which amounts to array, power, and beamforming gain from larger number of antenna ports. Between CB1 and CB2, the throughput of CB2 is higher than that of CB1, since CB2 exploits the composite delay profile across TRPs. Another observation is that the throughputs of CB2 and CB1 remains similar as the PMI overhead increases. As the overhead for both codebooks is high, this suggests that the set of parameter combinations can be refined for CB1/CB2 to further reduce the PMI overhead.

Observation 1: CB2 and CB1 yield higher throughput than that of Rel-16 Type-II CB, and CB2 yields the highest throughput gain.

Observation 2: The throughputs of CB2 and CB1 remains similar as PMI overhead increases. Since the PMI overhead for both codebooks is high, this suggests that the set of parameter combinations can be refined for CB1/CB2 to further reduce the PMI overhead.

Figure 3: Average UPT gain over sTRP w/ Rel-16 Type-II CB vs overhead for the intra-cell scenario with N=4 TRPs and 8 ports per TRP

Similar trends can be observed in the inter-cell scenario as well. The details simulation results for both intra- and inter-cell scenarios are provided in the companion contribution [2].

## Evaluation methodology

- Intra-cell scenario

- Inter-cell scenario

Figure 4

The evaluation methodology (EVM) can be based on Rel-16 EVM (for regular Type II) and Rel-17 EVM (for port selection Type II), with the following modifications.

- Number of TRPs (N=2,3,4)

- NW scenario: RMa or Dense Urban from 38.901 can be considered.

- Layout:

  - For the intra-cell CJT, the TRP layout (N=2,3,4 TRPs within a sector/cell) as shown in Figure 4(a) should be considered.

  - For inter-cell CJT, an example is given in Figure 3(b)

- Carrier frequency: 700MHz for intra-cell mTRP, and 2GHz for inter-cell mTRP

- Number of antenna ports per TRP = {4,8} (700MHz); {16} (2GHz)

- Per TRP power = normalized by N, i.e., total power is distributed across TRPs

The baseline should be single TRP using Rel-16/17 Type II codebook where the number of ports is the same as number of ports per TRP.

Proposal 4: For Type II C-JT codebook EVM, support Rel-16 and 17 EVMs as baseline, with the following modifications for mTRP (shown in red in Table 1 and Table 2 in Appendix)

- Number of TRPs (N) = 2,3,4

- NW scenario: RMa or dense urban (38.901)

- Layout: intra-cell CJT as shown in Figure 4(a), inter-cell in Figure 4(b)

- Carrier: 700MHz for intra-cell, 2GHz for inter-cell

- Number of antenna ports per TRP = {4,8} (700MHz); {16} (2GHz)

The baseline is single TRP using Rel-16/17 Type II codebook, where the number of ports is the same as number of ports per TRP.

CSI for high/medium UE velocities

According to the WID, the objective on CSI enhancements for high/medium UE velocities includes two items: augmenting 1) Rel-16/17 Type II with time-/Doppler-domain compression without modification to the spatial and frequency domain basis; 2) Reporting time-domain channel property (TDCP) possibly as a standalone or CSI-adjoint quantity.

## Doppler-domain compression

Figure 5

    - CSI-RS burst

For Doppler-domain (DD) compression, a UE can measure a burst of NZP CSI-RS resource(s), referred to as CSI-RS burst, in B>1 time slots (where B is configured such that the measurement “window” is well within the channel coherence time). There are two examples of a CSI-RS burst, as shown in Figure 5.

- Ex1: the B time slots are evenly/uniformly separated with an inter-slot spacing d.

- Ex2: the B time slots can be non-uniformly separated with inter-slot spacing e1=d1, e2=d2-d1, e3=d3-d2,…, so on, where ei≠ej for at least one pair (i,j) with i≠j.

The CSI-RS burst can be configured based on one SP CSI-RS resource, where the first and last slots of the CSI-RS burst are determined based on the MAC CE based activation and deactivation commands, respectively. This, however, can be too restrictive in some cases, e.g.:

- It can only be configured for uniformly separated CSI-RS bursts.

- According to the current specification, the minimum periodicity value of a SP CSI-RS resource is 4 slots, which may not be sufficient for high UE velocities.

The CSI-RS burst can also be configured based on a group of AP CSI-RS resources, which can be activated by a MAC CE based activation command (e.g. sub-selection step for AP CSI-RS resources in Rel-15). The slot offsets of the group of AP CSI-RS resources can determine the spacing (di) between AP CSI-RS resources. The AP CSI-RS resource based CSI-RS burst can be more flexible for CSI-RS burst configuration. It can address the two limitations with SP CSI-RS resource mentioned above. It can be used for both uniformly and non-uniformly separated CSI-RS bursts, and it can facilitate measurement in consecutive slots.

Observation 3: Regarding CSI-RS burst configuration

- SP CSI-RS resource can be restrictive due to (1) uniformly separated burst, and (2) the min periodicity = 4 slots.

- A group of AP CSI-RS resources can be used to address these limitations.

Based on this observation, we propose to support both SP CSI-RS resource and a group of AP CSI-RS resources for CSI-RS burst configuration for wider range of medium/high UE velocity.

Proposal 5: Support a CSI-RS burst configuration based on both SP CSI-RS resource and a group of AP CSI-RS resources

Figure 6

    - DD unit and DD basis vectors

The notion of FD unit for FD compression in Rel-16/17 Type II codebook can be extended to DD. In particular, a total of B CSI-RS measurement time instances: T0, T1,…,TB-1 can be partitioned into sub-time (ST) units ST0,ST1,…, each ST unit comprises NST measurement time instances. An example is shown in Figure 6 where NST=2.

The number of DD units (say N4, which is also the length of the DD basis vectors) can be determined based on B and NST. Similar to Rel-16/17, the number of DD basis vectors (say, D) for DD compression is configured.

Proposal 6: Extend the definition of FD unit from Rel-16/17 Type II codebook to DD,

- Introduce a ST unit comprising NST consecutive measurement time instances

- NST determines N4, i.e., the number of DD units or length of DD basis vectors

- Number of DD basis vectors (D) for DD compression is configured

As the baseline, the DD basis vectors for DD compression can be orthogonal DFT vectors. In this case, for instance, the Rel-16 Type-II codebook structure can be extended as follows:

W=1γW1W2Wf,dH

- W is a PCSIRS×N3N4 matrix whose columns are precoders for pairs of (FD,DD) units,

- W1 is a PCSIRS×2L SD basis matrix

- Wf is a N3×Mυ FD basis matrix

- Wd is a N4×D DD basis matrix

- Wf,d=kron(Wf,Wd), is (N3N4)×(MυD)

- W2 is a 2L×(MυD) coefficient matrix, and

- γ is a normalization factor such that W is normalized to power 1.

Some examples of more advanced basis design (to assess whether they can offer significant benefit over the baseline) are:

- Rotated or oversampled (non-orthogonal) DFT vectors.

- Non-DFT basis such as Slepian or discrete prolate spheroidal sequence basis [3], polynomial basis, DCT basis

Proposal 7: Regarding DD basis vectors,

- Orthogonal DFT basis vectors is the baseline

- Study whether the following more advanced basis can significantly outperform the baseline:

  - Rotated or oversampled DFT

  - Non-DFT basis, e.g. Slepian (DPSS), polynomial, DCT etc.

    - SD/FD basis and coefficients

According to the WID, the DD compression based on Rel-16/17 Type II codebook refinement can be considered without modifying SD/FD bases. Therefore, the SD/FD bases from Rel-16/17 codebooks are reused. There can be three candidates for SD/FD bases.

- Rel-16 enhanced (regular) Type II

- Rel-16 enhanced port selection Type II

- Rel-17 further enhanced port selection Type II

In our view, at least Rel-16 enhanced (regular) Type II based SD/FD bases should be supported.

Proposal 8: Regarding SD/FD basis vectors, reuse Rel-16 enhanced Type II SD/FD basis vectors

The coefficient matrix ( W2) comprises 2LMυD coefficients. The Rel-16/17 based W2 reporting mechanism (i.e., SCI, bitmap to indicate indices of NZ coefficients, and amplitude/phase of the NZ coefficients) should be reused. However, the total number of coefficients increases to D times when compared to Rel-16/17, which can be too large (for reporting via two-part UCI). Thus, mechanisms to limit the increase in W2 reporting payload may need to be considered, e.g. via parameter combination or bitmap.

Proposal 9: Regarding W2 reporting

- Rel-16/17 based W2 reporting mechanism is reused,

- Study the need for mechanisms to limit the payload increase of W2 reporting, e.g. via parameter combination or bitmap

## TDCP reporting based on TRS

The following aspects about the second item under this objective requires further discussion and clarification.

- Target use cases: Doppler reporting was already proposed (yet not supported) in Rel-17 HST for frequency offset pre-compensation. Therefore, to avoid unnecessary and repeated discussion as well as to facilitate evaluation, a more focused use case, e.g. for CSI prediction in a set of scenarios, should be decided. For instance, since TRS is used for measurement, does such TDCP target a single- or dual-port use case at higher speed than that for the Type-II DD compression?

- Also, whether the TDCP is a standalone report (e.g. a new reportQuantity) not a part of any CSI reporting, or can be multiplexed with other CSI parameters in a CSI report needs to be discussed and clarified. From our perspective, we prefer to study this as a standalone report so that we can focus on its benefits. Otherwise (if multiplexed with CSI), then the potential benefits can be difficult to separate out or/and quantity.

- The content of the TDCP report is open (not specified in WID). We need to list a few candidates for efficient discussion. In our view, the list of candidates can be included Doppler shift/spread, and time-domain correlation.

Proposal 10: For TRS-based TDCP reporting,

- Identify target use cases (e.g. CSI prediction for small number of ports and higher UE speed)

- Study a standalone report (not multiplexed with a CSI report)

- Study TDCP parameters, e.g. Doppler shift/spread, TD correlation

## Evaluation methodology

The EVM for the study on Doppler compression can be based on Rel-16 EVM (for regular Type II) and Rel-17 EVM (for port selection Type II), with the following modifications.

- Physical UE trajectory/mobility modeling (e.g. from Rel-17 multi-beam) should be considered since a UE moving with 30kmph can move 83.3m in 10sec (typical SLS simulation time), which is large.

- Doppler spectrum model

- UE speed, e.g. 10, 30 kmph for Type-II Doppler-domain extension; 60, 120 kmph for (stand-alone) TDCP report

- CSI feedback for Type-II Doppler-domain extension

  - Periodicity = W ms (e.g. W=100) – for simulation purposes

  - B measurement instances with separate d slots

    - d=5 (SP CSI-RS)

    - d=1 (group of AP CSI-RS resources)

    - B=W/5

  - Measurement window = (B-1)*d+1

- CSI-RS burst overhead should be considered in the throughput calculation

- Assumptions on UE-/NW-side extrapolation should be stated.

The baseline for this study should be based on Rel-16/17 Type II codebook with CSI feedback periodicity = 5ms.

Proposal 11: For the study of Doppler compression, support Rel-16/17 EVMs as baseline, with the following modifications (shown in red in Table 1 and Table 2 in Appendix)

- Physical UE trajectory/mobility modeling (e.g. from Rel-17 multi-beam)

- Doppler spectrum model

- UE speed, e.g. 10, 30 kmph for Type-II Doppler-domain extension; 60, 120 kmph for (stand-alone) TDCP report

- CSI feedback for Type-II Doppler-domain extension

  - Periodicity = W ms (e.g. W=100) – for simulation purposes

  - B measurement instances with separate d slots

    - d=5 (SP CSI-RS)

    - d=1 (group of AP CSI-RS resources)

    - B=W/5

  - Measurement window = (B-1)*d+1

- CSI-RS burst overhead

- Assumptions on UE-/NW-side extrapolation

The EVM for the study on TDCP reporting will depend on the target use cases and scenarios. Also, how (e.g. LLS or SLS) to assess benefits in what scenario(s) and use cases require further discussion.

Proposal 12: For the study of TDCP reporting, the EVM should be discussed based on the target use cases.

Conclusion

In this contribution, the following observations and proposals are made:

CSI for C-JT

Proposal 1: Consider at least the following scenarios for Type II C-JT codebook design

- Scenario 1: intra-cell, mTRP @ sub-1GHz bands (e.g. 700MHz) with 4 or 8 ports per TRP and {2, 3, 4} TRPs

- Scenario 2: inter-cell, mTRP @ 2GHz with 16 ports per TRP and 3 TRPs

Proposal 2: Support the following setups for C-JT CMR

- Alt1: >1 NZP CSI-RS resources, 1 NZP CSI-RS resource per TRP

- Alt2: 1 NZP CSI-RS resource comprising >1 antenna port groups, 1 antenna port group per TRP

Proposal 3: Support the following Type II codebook refinement for C-JT

- CB1 (decoupled): comprising following components

  - Intra-TRP: per TRP Rel-16/17 Type II codebook components (W1, W2, Wf)

  - Inter-TRP: newly introduced co-amplitude and co-phase

- CB2 (joint): comprising following components

  - Per TRP Rel-16/17 Type II SD basis W1

  - Single Rel-16/17 Type II FD basis Wf jointly applied across all TRPs

  - Single Rel-16/17 Type II linear combination coefficients (LCCs) W2 jointly applied across all TRPs

- Maximum reuse of the Rel-16/17 detailed designs (e.g. SD/FD basis designs, UCI parameters)

  - Possible optimization in the supported parameter combinations

  - Introduce strongest TRP indication for components that are reported relative to a reference (e.g. co-amplitude/co-phase)

Observation 1: CB2 and CB1 yield higher throughput than that of Rel-16 Type-II CB, and CB2 yields the highest throughput gain.

Observation 2: The throughputs of CB2 and CB1 remains similar as PMI overhead increases. Since the PMI overhead for both codebooks is high, this suggests that the set of parameter combinations can be refined for CB1/CB2 to further reduce the PMI overhead.

Proposal 4: For Type II C-JT codebook EVM, support Rel-16 and 17 EVMs as baseline, with the following modifications for mTRP (shown in red in Table 1 and Table 2 in Appendix)

- Number of TRPs (N) = 2,3,4

- NW scenario: RMa or dense urban (38.901)

- Layout: intra-cell CJT as shown in Figure 4(a), inter-cell in Figure 4(b)

- Carrier: 700MHz for intra-cell, 2GHz for inter-cell

- Number of antenna ports per TRP = {4,8} (700MHz); {16} (2GHz)

The baseline is single TRP using Rel-16/17 Type II codebook, where the number of ports is the same as number of ports per TRP.

CSI for high/medium UR velocities

Observation 3: Regarding CSI-RS burst configuration

- SP CSI-RS resource can be restrictive due to (1) uniformly separated burst, and (2) the min periodicity = 4 slots.

- A group of AP CSI-RS resources can be used to address these limitations.

Proposal 5: Support a CSI-RS burst configuration based on both SP CSI-RS resource and a group of AP CSI-RS resources

Proposal 6: Extend the definition of FD unit from Rel-16/17 Type II codebook to DD,

- Introduce a ST unit comprising NST consecutive measurement time instances

- NST determines N4, i.e., the number of DD units or length of DD basis vectors

- Number of DD basis vectors (D) for DD compression is configured

Proposal 7: Regarding DD basis vectors,

- Orthogonal DFT basis vectors is the baseline

- Study whether the following more advanced basis can significantly outperform the baseline:

  - Rotated or oversampled DFT

  - Non-DFT basis, e.g. Slepian (DPSS), polynomial, DCT etc.

Proposal 8: Regarding SD/FD basis vectors, reuse Rel-16 enhanced Type II SD/FD basis vectors

Proposal 9: Regarding W2 reporting

- Rel-16/17 based W2 reporting mechanism is reused,

- Study the need for mechanisms to limit the payload increase of W2 reporting, e.g. via parameter combination or bitmap

Proposal 10: For TRS-based TDCP reporting,

- Identify target use cases (e.g. CSI prediction for small number of ports and higher UE speed)

- Study a standalone report (not multiplexed with a CSI report)

- Study TDCP parameters, e.g. Doppler shift/spread, TD correlation

Proposal 11: For the study of Doppler compression, support Rel-16/17 EVMs as baseline, with the following modifications (shown in red in Table 1 and Table 2 in Appendix)

- Physical UE trajectory/mobility modeling (e.g. from Rel-17 multi-beam)

- Doppler spectrum model

- UE speed, e.g. 10, 30 kmph for Type-II Doppler-domain extension; 60, 120 kmph for (stand-alone) TDCP report

- CSI feedback for Type-II Doppler-domain extension

  - Periodicity = W ms (e.g. W=100) – for simulation purposes

  - B measurement instances with separate d slots

    - d=5 (SP CSI-RS)

    - d=1 (group of AP CSI-RS resources)

    - B=W/5

  - Measurement window = (B-1)*d+1

- CSI-RS burst overhead

- Assumptions on UE-/NW-side extrapolation

Proposal 12: For the study of TDCP reporting, the EVM should be discussed based on the target use cases.

# References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, RAN1#103-e

- R1-2203895, Initial SLS results on Type-II CSI enhancements for CJT, Samsung

- Thomas Zemen, “Time-variant channel estimation using discrete prolate spheroidal sequences,” IEEE Transactions Signal Processing, September 2005.

# Appendix A

Table 1: EVM for Rel-16 eType-II codebook refinement

| Parameter |  | Value (C-JT) | Value (Doppler) |
| --- | --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA | OFDMA |
| Scenario |  | RMa or Dense urban N_TRP (#TRPs): 2, 3, 4 Layout1: intra-cell (TRPs within each cell) Layout2: inter-cell (TRPs across cells) N_TRP (#TRPs): 3 | Dense Urban (Macro only) UE trajectory modelling (e.g. linear trajectory in Rel-17 multi-beam, RAN1#102-e) Doppler spectrum model? UE speed: 10, 30 kmph for Type-II Doppler-domain extension; 60, 120 kmph for (standalone) TDCP report |
| Frequency Range |  | FR1, 700Hz, 2GHz | FR1, 2GHz |
| Inter-BS distance |  | 1.7km (700MHz), 200m (2GHz) | 200m |
| Channel model |  | According to the TR 38.901 | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 700MHz - 4 ports per TRP: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.8)λ - 8 ports per TRP: (2,2,2,1,1,2,2), (dH,dV) = (0.5, 0.8)λ 2GHz: - 16 ports per TRP: (4,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Total #ports = N_TRP x {4,8,16} | For Type-II Doppler-domain extension, companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. For TDCP reporting, 1 or 2 ports |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | RMa (700 MHz) - 46 dBm in total, (normalized by N TRP, i.e., distributed across TRPs) Dense urban (2 GHz) - 41 dBm per TRP (not normalized) | 41 dBm |
| BS antenna height |  | 25m | 25m |
| UE antenna height & gain |  | Follow TR36.873 | Follow TR36.873 |
| UE receiver noise figure |  | 9dB | 9dB |
| Modulation |  | Up to 256QAM | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot | 14 OFDM symbol slot |
|  | SCS | 15kHz | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS | 52 for 15 kHz SCS |
| Simulation bandwidth |  | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. |
| Frame structure |  | Slot Format 0 (all downlink) for all slots | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation is a baseline for overhead reduction. For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed for higher rank extension. For medium/high RU, SU/MU-MIMO with rank adaptation is assumed for higher rank extension. | SU/MU-MIMO with rank adaptation is a baseline for overhead reduction. For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed for higher rank extension. For medium/high RU, SU/MU-MIMO with rank adaptation is assumed for higher rank extension. |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption - CSI feedback periodicity (full CSI feedback): 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms | CSI feedback periodicity: W ms (e.g. W = 100, note: W = 5 for baseline) CSI-RS burst: B measurement instances with separation d - 1 SP CSI-RS resource: d=4 (min possible) - B AP CSI-RS resources: d=1 - B = W/5 (e.g. 100/5 = 20 measurements) - measurement window (# slots): W=(B-1)*d+1 Companies should state any UE side extrapolation (future CSI) |
| Overhead |  | Companies shall provide the downlink overhead assumption | CSI-RS burst overhead |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 50/70 % for CSI overhead reduction 20/50% for high rank extension Companies are encouraged to report the MU-MIMO utilization. | 50/70 % for CSI overhead reduction 20/50% for high rank extension Companies are encouraged to report the MU-MIMO utilization. |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC | MMSE-IRC |
| Feedback assumption |  | Realistic | Realistic |
| Channel estimation |  | Realistic | Realistic |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | Single-TRP (N=1) with Rel-16 eType-II CB | Rel-16 eTypeII with CSI feedback periodicity: 5 ms W/ extrapolation @gNB |

Table 2: EVM for Rel-17 Type-II

| Parameter |  | Value (C-JT) | Value (Doppler) |
| --- | --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA | OFDMA |
| Scenario |  | RMa or Dense urban N_TRP (#TRPs): 2, 3, 4 Layout1: intra-cell (TRPs within each cell) Layout2: inter-cell/ (TRPs across cells) N_TRP (#TRPs): 3 | Dense Urban (Macro only) UE trajectory modelling (e.g. linear trajectory in Rel-17 multi-beam, RAN1#102-e) Doppler spectrum model? UE speed: 10, 30, 60 kmph |
| Frequency Range |  | FR1, 700Hz, 2GHz | FR1, 2GHz |
| Inter-BS distance |  | 1.7km (700MHz), 200m (2GHz) | 200m |
| Reciprocity model |  | For EVM for FDD CSI enhancement in Rel-17, use following Alt 1 as the baseline and Alt 2 as the optional - Alt 1: Based on Section 5.3 of TR 36.897, to generate FDD DL and UL channels. - Alt 2: Based on Section 7.6.5 of TR 38.901, to generate FDD DL and UL channels with following modifications: - Different per-cluster shadowing is generated for DL and UL, and DL (or UL) angles are generated based on DL (or UL) cluster powers. Then UL (or DL) uses the same angles and its own cluster powers to generate the channel matrix. - XPR is generated independently for DL and UL. | For EVM for FDD CSI enhancement in Rel-17, use following Alt 1 as the baseline and Alt 2 as the optional - Alt 1: Based on Section 5.3 of TR 36.897, to generate FDD DL and UL channels. - Alt 2: Based on Section 7.6.5 of TR 38.901, to generate FDD DL and UL channels with following modifications: - Different per-cluster shadowing is generated for DL and UL, and DL (or UL) angles are generated based on DL (or UL) cluster powers. Then UL (or DL) uses the same angles and its own cluster powers to generate the channel matrix. - XPR is generated independently for DL and UL. |
| Antenna setup and port layouts at gNB |  | 700MHz - 4 ports: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.8)λ - 8 ports: (2,2,2,1,1,2,2), (dH,dV) = (0.5, 0.8)λ 2GHz: - 16 ports: (4,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Total #ports = N_TRP x {4,8,16} | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | RMa (700 MHz) - 46 dBm in total, (normalized by N TRP, i.e., distributed across TRPs) Dense urban (2 GHz) - 41 dBm per TRP (not normalized) | 41 dBm |
| BS antenna height |  | 25m | 25m |
| UE antenna height & gain |  | Follow TR36.873 | Follow TR36.873 |
| UE receiver noise figure |  | 9dB | 9dB |
| Modulation |  | Up to 256QAM | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot | 14 OFDM symbol slot |
|  | SCS | 15kHz | 15kHz |
| Simulation bandwidth |  | 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered | 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered |
| Frame structure |  | Slot Format 0 (all downlink) for all slots | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | For low RU, SU-MIMO with rank adaptation are assumed For medium/high RU, SU/MU-MIMO with rank adaptation is assumed | For low RU, SU-MIMO with rank adaptation are assumed For medium/high RU, SU/MU-MIMO with rank adaptation is assumed |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms | CSI feedback periodicity: W ms (e.g. W = 100, note: W = 5 for baseline) CSI-RS burst: B measurement instances with separation d - 1 SP CSI-RS resource: d=4 (min possible) - B AP CSI-RS resources: d=1 - B = W/5 (e.g. 100/5 = 20 measurements) - measurement window (# slots): W=(B-1)*d+1 Companies should state any UE side extrapolation (future CSI) |
| Overhead |  | Companies shall provide the downlink overhead assumption | CSI-RS burst overhead |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes Other FTP model is not precluded. | FTP model 1 with packet size 0.5 Mbytes Other FTP model is not precluded. |
| Traffic load (Resource utilization) |  | - 70% for SU/MU-MIMO with rank adaptation - 20% for SU-MIMO with rank adaptation Companies are encouraged to report the MU-MIMO utilization. | - 70% for SU/MU-MIMO with rank adaptation - 20% for SU-MIMO with rank adaptation - Companies are encouraged to report the MU-MIMO utilization. |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic | Realistic |
| Channel estimation |  | Realistic | Realistic |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | Single-TRP (N=1) with Rel-17 PS TypeII | Rel-17 PS TypeII with CSI feedback periodicity: 5 ms Companies should state assumptions about any NW-side extrapolation |
| SRS modeling for UL channel estimation |  | SRS periodicity with 5ms SRS error modeling in Table A.1-2 in 36.897 with Δ=9 BW: same as CSI-RS Number of ports = 2 Tx power = 23 dBm | SRS periodicity with 5ms SRS error modeling in Table A.1-2 in 36.897 with Δ=9 BW: same as CSI-RS Number of ports = 2 Tx power = 23 dBm |
| FDD DL/UL calibration error model at gNB |  | According to R1-144943, with amplitude error (expressed in decibel of x=20log10a) and phase error are normal distribution with 0.7dB and 5 degrees standard deviation, respectively. Both amplitude/phase errors are assumed to be constant during a simulation drop at time, and constant per 4 PRB at frequency. Companies shall report the assumption of error modelling at frequency. | According to R1-144943, with amplitude error (expressed in decibel of x=20log10a) and phase error are normal distribution with 0.7dB and 5 degrees standard deviation, respectively. Both amplitude/phase errors are assumed to be constant during a simulation drop at time, and constant per 4 PRB at frequency. Companies shall report the assumption of error modelling at frequency. |