3GPP TSG RAN WG1#109-e			R1-2203892

e-Meeting, May 9th – May 20th, 2022

Agenda item:	9.1.3.2

Source: 	Samsung

Title: 	Views on SRS enhancements

Document for:	Discussion and Decision

Introduction

The Rel-18 WID [1] includes the following objectives regarding the SRS enhancements.

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences - Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

This contribution provides initial views on SRS enhancement for time-division duplex (TDD) coherent joint transmission (C-JT) and 8 Tx UL operations.

SRS Enhancement

  - Interference management for TDD CJT

In TDD, SRS transmissions from UEs are a main source for CSI acquisition at gNB as to both of UL and DL channels. SRS transmissions, however, can be more congested in a multi-TRP (mTRP) scenario wherein a gNB controlling mTRP capable of CJT can support more UEs (associated with a given cell ID) and need more frequent CSI acquisition. This can result in increasing the possibility of scheduling SRS resources for multiple UEs that are overlapping in given time-and-frequency resources. Therefore, potential interference across SRS transmissions from multiple UEs can be severe in congested mTRP scenarios, and thus an SRS enhancement could be needed to manage inter-TRP / cross-SRS interference targeting TDD CJT.

Under the three constraints described in Rel-18 WID, the following two directions can be considered to randomize or manage inter-TRP cross-SRS interference:

- Enhanced frequency hopping pattern

- Enhanced code-domain (e.g., cyclic shift, root sequence) hopping

Due to asymmetric distances from TRPs for a given UE, it could be beneficial to allow more flexible SRS resource allocation, for example, that enables different RB size allocation across symbols in frequency hopping SRS transmission. By assigning uneven (unequal) size of RBs across symbols, controlling stronger/weaker power allocations across symbols can be feasible to randomize/manage interference with consideration for distant/close TRPs in UL CSI acquisition. Notice that when SRS transmissions for multiple UEs in given time-and-frequency resources need to be scheduled, power-domain difference across the UEs and symbols can also be exploited through enhanced frequency hopping patterns, e.g., gNB can schedule SRS resources for multiple UEs to be overlapping on time-and-frequency domains but differentiating in power domain on purpose, considering advanced receiver methods (e.g., successive interference cancellation, SIC, advanced inter-/extrapolation methods) are utilized at gNB.

Figure 1. Example of different-size resource allocation across symbols in frequency hopping.

Figure 1 shows an example of different-size resource allocation across symbols in frequency hopping. As shown in the example, the gNB allocates different-size resource allocation for each UE with consideration for distant/close TRPs. Here, the gNB assigns orthogonal resources for UEs experiencing similar signal powers from TRPs (e.g., UE1/UE2 pair, UE3/UE4 pair) with allowing partially overlapping resources for UEs experiencing different signal powers from TRPs (e.g., UE1/UE3, UE2/UE4). The interference for the partially overlapping resources across the UEs can be handled by advanced receiver techniques at the gNB by exploiting signal power differences, which NW can consider before scheduling SRS resources for the UEs.

Proposal 1: Consider enhanced frequency hopping patterns, e.g., that support different RB-size allocations across symbols in frequency hopping SRS transmission.

In Rel-17 SRS enhancement, the supported number of symbol repetitions has been increased up to 14 for SRS coverage enhancement. This could be useful in TDD CJT scenarios wherein cell-edge UEs that usually need a larger number of symbol repetitions are targeted as CJT candidates. On the other hand, inter-SRS interference could be worse across such scheduled UEs due to that situations under limited time/frequency resources can further frequently happen. To reduce interference in such scenarios, code-domain hopping (e.g., cyclic shift, and sequence group/number) across time symbols/slots can be considered for interference randomization across scheduled UEs, as shown in Figure 2. Code-domain hopping across symbols in frequency hopping SRS transmission can also be considered

Figure 2. Example of code-domain hopping

Proposal 2: Consider code-domain hopping (e.g., cyclic shift, and root index) across time symbols/slots for interference randomization.

In multi-TRP scenario targeting TDD CJT, SRS transmissions are received by multiple distributed TRPs for DL CSI acquisition. It is mostly probable that the power control parameter setting is different across transmission to different TRP. Hence, in addition to consider frequency and code-domain hopping as above, one approach for multi-TRP CJT scenario to randomize cross-SRS interference is to dynamically indicate power control parameters for the SRS resource sets prior to transmission across TRPs accordingly. This dynamic update can be done either by MAC-CE in similar approach for Rel-16 MAC-CE based spatial relation information update for AP/SP-SRS resources or alternatively by RRC configuring multiple sets of power control parameters and then using DCI to indicate one of those configurations prior to transmission across TRPs. Based on this approach, interference randomization could be additionally achieved by controlling power control parameters for each TRP dynamically.

Proposal 3: Consider dynamic indication of power control parameters to randomize cross-SRS interference additionally.

  - 8 TX operation

To support SRS 8 Tx operations, frequency-domain (e.g., transmission comb) and code-domain (e.g., cyclic shifts) components can be considered, similar to that SRS 4 Tx operation does. This direction does not much impact on UE/gNB implementation and can provide simple specification impact. Further, the decoding complexity for SRS reception at gNB can be maintained as it is decodable within each symbol (cf. TD-OCC).

Proposal 4: Consider existing frequency-domain (e.g., transmission comb) and code-domain (e.g., cyclic shifts) components to enable SRS 8-port operation, similar to that SRS 4-port operation does.

Since transmission combs (2, 4, and 8) and the corresponding max cyclic shift (8, 12, and 6, respectively) values are already sufficiently many to support (nearly) orthogonal SRS sequences across 8 ports, it is less motivated to increase the maximum values of transmission comb and/or cyclic shifts. Also, considering that the need for SRS 8 Tx operation can occur less frequently than that for SRS x(<8) Tx in common scenarios, keeping the max values of Rel-17 could prevent from excessive optimization.

Proposal 5: Maintain the maximum values of transmission comb (up to 8) and cyclic shifts (up to 12) as in Rel-17 SRS.

Based on the current possible values of transmission comb and the corresponding maximum cyclic shift as mentioned above, the issue is how to allocate comb and cyclic shift for each port considering 8-port SRS resource. Since 8-port SRS resource would be used for UL traffics from CPE and/or FWA-like high-end devices, achieving better channel estimation performance might be necessary. For that purpose, appropriate allocation methods for comb and cyclic shift seems to be defined.

Proposal 6: For 8-port SRS resource, study on how to allocate comb and cyclic shift for each port.

For 8 TX operations of SRS, the potential impact on the current specification is SRS resource and SRS resource set configuration. When we consider 8 Tx operations of SRS, the possible configured values of usage for SRS resource set are codebook, non-codebook, and antenna switching.

For the usage of codebook, how to configure 8-port SRS resources can be discussed including full-power modes.

For the usage of non-codebook, in order to support up to 8TX and the corresponding 8 layers, the number of SRS resources is naturally extended to 8 within an SRS resource set based on the legacy Rel-15 solution. On the other hand, two SRS resource sets can provide more flexible operations, and can be also used for Rel-17 mTRP PUSCH repetition schemes. We can study further about the number of SRS resource sets for the case of 8TX whether one or two SRS resource sets for non-codebook based PUSCH transmission can be used.

For the usage of antenna switching, it is natural to extend antenna switching configuration up to 8 TX. So far, the current specification only covers the configurations for antenna switching supportedSRS-TxPortSwitch of xTyR where x = {1, 2, 4} and y is up to 8. Supporting up to 8TX of UL transmission requires new SRS resource and SRS resource set configurations for 8T8R antenna switching scenario. Hence, similar with the current configuration with x = y, the corresponding configuration for 8T8R can be supported.

Proposal 7: For 8TX operation of SRS for codebook, consider SRS resource configurations including full-power modes.

Proposal 8: For 8TX operation of SRS for non-codebook, consider one or two SRS resource sets and the corresponding SRS resource configuration.

Proposal 9: For 8TX operation of SRS for antenna switching, support SRS resource/set configuration for 8T8R.

Evaluation methodology (EVM)

The evaluation methodology (EVM) for SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT can be based on the evaluation assumptions from Rel-17 SRS. Also, additional views are summarized as follows.

  - Number of TRPs: 2, 3, 4

  - Baseline schemes to compare

    - Rel-17 SRS (with RPFS and/or Comb) to allocate non-overlapped REs between UEs

    - Rel-17 SRS (with RPFS and/or Comb) to allocate overlapped REs between UEs

Proposal 10: For evaluation methodology, support evaluation assumptions from Rel-17 SRS as a starting point considering a number of TRPs and some of Rel-17 SRS capacity enhancement schemes as baseline schemes.

Conclusion

In this contribution, the following observations and proposals are made:

Proposal 1: Consider enhanced frequency hopping patterns, e.g., that support different RB-size allocations across symbols in frequency hopping SRS transmission.

Proposal 2: Consider code-domain hopping (e.g., cyclic shift, and root index) across time symbols/slots for interference randomization.

Proposal 3: Consider dynamic indication of power control parameters to randomize cross-SRS interference additionally.

Proposal 4: Consider existing frequency-domain (e.g., transmission comb) and code-domain (e.g., cyclic shifts) components to enable SRS 8-port operation, similar to that SRS 4-port operation does.

Proposal 5: Maintain the maximum values of transmission comb (up to 8) and cyclic shifts (up to 12) as in Rel-17 SRS.

Proposal 6: For 8-port SRS resource, study on how to allocate comb and cyclic shift for each port.

Proposal 7: For 8TX operation of SRS for codebook, consider SRS resource configurations including full-power modes.

Proposal 8: For 8TX operation of SRS for non-codebook, consider one or two SRS resource sets and the corresponding SRS resource configuration.

Proposal 9: For 8TX operation of SRS for antenna switching, support SRS resource/set configuration for 8T8R.

Proposal 10: For evaluation methodology, support evaluation assumptions from Rel-17 SRS as a starting point considering a number of TRPs and some of Rel-17 SRS capacity enhancement schemes as baseline schemes.

References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, RAN#94-e