3GPP TSG RAN WG1#109-e			R1-2203894

e-Meeting, May 9th – May 20th, 2022

Agenda item:	9.1.4.2

Source: 	Samsung

Title: 	Views on TPMI/SRI enhancements for 8Tx UL transmission

Document for:	Discussion and Decision

# Introduction

The Rel.18 WID [1] includes the following objectives regarding the 8Tx UL operation.

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

This contribution provides Samsung’s view regarding the 8Tx UL operation.

# Discussion on device types

The WID includes four target device types, CPE/FWA/vehicle/Industrial devices. These device types are quite different in terms of use cases, deployment scenarios, and antenna structures. If we study all of them, then we will have to study them separately, which will be very challenging (if not impossible) given that we only 3TU in total for as many as 8 objectives.

Observation 1: four device types (i.e. CPE/FWA/vehicle/Industrial devices) are quite different (in use cases, deployment scenarios, antenna structures), hence require separate study, which will be very challenging in limited TU (~3/8 TU)

To ensure that the scope of this objective is manageable (within ~3/8 TU), we prefer to prioritize one device type for study and RAN1 discussions. For example, the one device type can be the one which is ‘most-likely’ to have 8 Tx antennae for UL operations. In our view, among the four device types, CPE can be one such device, followed by FWA. We therefore propose the following.

Proposal 1: for the study of 8 Tx UL operations,

- Prioritize one device type for efficient study and discussions

- The one device should be the one which is ‘most-likely’ to have 8 Tx antennae for UL operations

  - 1st priority: CPE

  - 2nd priority: FWA

# Potential enhancements

## Transmission scheme

In Rel. 15 NR, two transmission schemes are supported for PUSCH transmission, namely codebook (CB) based UL transmission and non-codebook (NCB) based UL transmission. For 8 Tx UL operations, both schemes should be supported due to the following reasons:

- The CB based UL transmission is currently implemented in real deployments, hence is likely to implemented for 8 Tx also.

- The NCB based UL transmission on the other hand targets a different use case, i.e., reciprocity-based UL operations. If UE-side reciprocity holds, then NCB based UL transmission can be beneficial.

The scope of potential enhancements for the two schemes, however, are quite different. In particular, CB based is likely to have a broader scope (e.g., including 8 Tx TPMI codebook design which often requires simulations), hence will require more time, when compared with NCB based. Due to limited TU, RAN1 can therefore adopt a serialized approach, and prioritize CB based UL transmission in first 1-2 meetings, and the discussion on NCB based can start after 1-2 RAN1 meetings.

Observation 2: regarding the UL transmission schemes

- CB-based UL is implemented in current deployments, hence is likely to be implemented for 8 Tx as well

- NCB based UL can be beneficial if UE-side reciprocity holds

Proposal 2: Regarding the transmission scheme for 8Tx UL operations,

- Support both CB and NCB based UL transmission

- For RAN1 work, adopt a serialized approach,

  - prioritize CB based discussion until RAN1#110

  - NCB based discussion starts after RAN1#110

## Codebook-based transmission

For CPE/FWA device types, the 8Tx antenna structure can be modelled based on the following aspects:

- Polarization: the antennae can be co-polarized, i.e., having a single polarization, or dual-polarized, i.e., having two polarizations, 4 antennae with one polarization (e.g. +45) and 4 antennae with another polarization (e.g. -45).

- Antenna groups: 8 antennae can be co-located at one place on the device or distributed at multiple places on the device (e.g. surfaces/planes, sides, edges, corners). Antennae at one place can be modelled as an antenna group. Hence, 8 antennae can comprise one group or multiple groups depending on whether they are located at one or multiple locations.

- Antenna panels and coherence types: the antenna groups can also be used to describe the antenna panel(s) and the coherence types of the UE. For a single panel UE, the number of antenna group can be one, and the coherence type can be full-coherence. For a multi-panel UE (MPUE), the number of antenna groups can be multiple (e.g. panel = group), and the coherence type can be partial-coherence or non-coherence.

Proposal 3: regarding the 8 Tx antenna structure, study the following aspects

- Polarization: co-pol or dual-pol

- Antenna groups: one group or multiple groups

- Antenna panels and coherence types:

  - Single panel: one antenna group and full-coherence

  - Multi-panel: multiple antenna groups and partial-/non-coherence

In Rel. 15 4Tx UL codebook, three types are precoders, i.e., full-coherent (FC), partial-coherent (PC), and non-coherent (NC) are included. The FC precoders are essentially a subset of DL Type I 4Tx codebook [1], (relevant RAN1 agreements are copied in Appendix A), the PC precoders correspond to a subset of LTE 4Tx codebook, and the NC precoders correspond to antenna selection vectors. Since these three types of precoders were taken from three different types of codebooks, they were difficult to describe using a unified approach. For efficient discussion on 8Tx, however we should avoid adopting similar (i.e. Rel.15) approach, and rather consider a unified approach for all 3 coherence types. The approach based on antenna groups can be one way to unify. The antennae within an antenna group can be coherent. Across two antenna groups, the transmission is non-coherent.

Proposal 4: support a single unified 8Tx codebook structure for different coherence types (i.e. FC, PC, and NC) based on antenna groups

- Antennae within a group are coherent

- Antennae across multiple groups are non-coherent

Figure 1

The baseline for 8Tx UL codebook should be Rel. 15 NR 8Tx Type I codebook. Similar to Type I single or multi-panel codebooks, different 8Tx antenna structures can be described based on the following parameters.

- P= number of antenna polarization; P=1 for co-pol, and P=2 for dual-pol.

- Mg= number of antenna groups.

- N1= Number of antennae in 1st dimension.

- N2= Number of antennae in 2nd dimension.

The candidate values for Mg can be Mg=1,2,4,8. For dual-pol, Mg=1,2,4, and for co-pol, Mg=1,2,4,8.

The number of groups (Mg) can also be used to describe antenna coherence types, i.e. full-coherent (FC), partial-coherent (PC), and non-coherent (NC). In particular,

- Full-coherent: Mg=1

- Partial-coherent: Mg=2,4

- Non-coherent: Mg=8

A few examples of the antenna structure are shown in Figure 1. When Mg=1 (one group), N1,N2,P can take a value as follows.

- Co-pol: N1,N2,P=8,1,1, 4,2,1

- Dual-pol: N1,N2,P=4,1,2, 2,2,2

When Mg=2, N1,N2,P can take a value as follows.

- Co-pol: Mg,N1,N2,P=2,4,1,1, 2,2,2,1

- Dual-pol: Mg,N1,N2,P=2,2,1,2,(4,1,1,2)

Proposal 5: regarding the 8Tx UL codebook,

- The baseline is Rel. 15 NR 8Tx DL Type I codebook

- Support the following parameters for 8Tx UL codebook

  - P= number of antenna polarization; P=1 for co-pol, and P=2 for dual-pol.

  - Mg= number of antenna groups.

  - N1= Number of antennae in 1st dimension.

  - N2= Number of antennae in 2nd dimension.

Depending on the assumptions on the antennae within different antenna groups (e.g. linear/regular vs irregular arrays, co-pol vs dual-pol), the codebook can be single-stage (similar to LTE 4Tx codebook) or dual-stage (similar to DL codebooks). Further discussion is needed in order to determine the need for single-stage or dual-stage codebook structures.

Proposal 6: investigate both single-stage and dual-stage codebook structures

The Rel. 16 supports 3 different full power modes. If PC or NC precoders are included in the 8Tx UL codebook, then the full power modes may need to be supported. However, the discussion on full power modes can start after the 8Tx codebook is designed.

Proposal 7: Discussion on full power modes can be start after the 8Tx codebook is designed

## Non-codebook-based transmission

For NCB based UL, the potential enhancements can include the following.

- Number of SRS resources: for 8Tx, the number of SRS resources (NSRS) can be increased to up to 8

- Number of SRS resource set: when NSRS≤4, the legacy (Rel.15) NCB based UL can be reused, and hence one SRS resource set with NSRS resources can be configured. When NSRS>4, then the NSRS SRS resources can be configured via one or two SRS resource sets. One resource set corresponds to simple extension of legacy Rel. 15 solution, whereas two SRS resource sets can provide more flexibility, and can also be used for mTRP schemes (e.g. Rel. 17 mTRP PUSCH). We therefore propose to exploit number of SRS resource sets, and consider both one and two SRS resource sets for NCB based UL.

- SRI indication: when NSRS≤4, the SRI indication remains the same as in Rel. 15, and when NSRS>4, the following aspects can be considered regarding the SRI indication.

  - Lmax: the configured value of the max number of UL layers (Lmax) can be 4 or >4. Whether Lmax>4 is needs further study.

  - Indication scheme: the SRI indication scheme can be one of the following.

    - Alt1: the legacy scheme based on the combinatorial index is used (extension of Rel. 15 SRI indication tables)

    - Alt2: a bitmap comprising NSRS bits is used, where the location of ‘1’s indicate the indicated SRS resource(s). The bitmap based scheme can be simpler (when compared with a combinatorial index based scheme).

Proposal 8: regarding 8Tx NCB based UL transmission,

- Support number of SRS resources (NSRS) up to 8

- Support both one SRS resource set and two SRS resource sets

- When NSRS≤4, the SRI indication follows legacy (Rel.15) scheme, and

- When NSRS>4,

  - Study the need for Lmax>4

  - study the following SRI indication schemes

    - Alt1: combinatorial index scheme

    - Alt2: bitmap based scheme

## Multi-panel UE

For a UE equipped with 2 antenna panels, and simultaneous transmission from 2 panels (STx2P), the UL transmission can correspond to one PUSCH transmitted towards one TRP (sTRP) or 2 PUSCHs transmitted towards two TRPs (mTRP). The STx2P to sTRP is akin to Rel. 17 NCJT CSI, and hence SRI can indicate a pair of SRS resources (e.g. in one set) for joint transmission from 2 panels to 1 TRP. The STx2P to mTRP on the other hand is akin to Rel. 17 mTRP PUSCH repetition, and hence 2 SRIs can be used to indicate 2 SRS resources (e.g. from 2 sets) for separate transmission from 2 panels to 2 TRPs. Further details on STx2P are provided in our contribution [3].

Proposal 9: for STx2P, support both

- Case 1 (1 PUSCH): one SRI indicating a pair of SRS resources (e.g. STx2P to sTRP)

- Case 2 (2 PUSCHs): two SRIs, each indicating a SRS resource for a TRP (e.g. STx2P to mTRP)

## Number of layers

Regarding the max number of UL layers, up to 4 layers is more important since it is likely in real deployments. We therefore prefer to prefer to prioritize it. If the need and use cases for >4 layers can be identified, we can be supportive of it.

Proposal 10: regarding max number of layers

- prioritize the RAN1 work for max 4 layers

- >4 layers can be supported, if its need and use cases can be identified

Evaluation methodology (EVM)

The evaluation methodology (EVM) for the study on CB based 8Tx UL operation can be based on Rel.15, with the following modifications.

- Dense urban (macro only), FR1 (2GHz)

- #Tx at UE=8

  - One antenna group

  - Multiple antenna groups: 2 or 4 groups

- Three evaluations can be conducted depending on max rank value.

  - Evaluation 1: up to rank 2

  - Evaluation 2: up to rank 4

  - Evaluation 3: up to rank 8

- #ports or Rx chains at the gNB = 2 (Evaluation 1), 4 (Evaluation 1), 8 (Evaluation 1)

The baseline for this study should be Rel. 15 CB based 4Tx UL transmission. The detailed EVM assumptions are tabulated in Table 1 in Appendix B.

Proposal 11: for the study of 8Tx UL operations, support EVM assumptions provided in Table 1.

Conclusion

In this contribution, the following observations and proposals are made:

Observation 1: four device types (i.e. CPE/FWA/vehicle/Industrial devices) are quite different (in use cases, deployment scenarios, antenna structures), hence require separate study, which will be very challenging in limited TU (~3/8 TU)

Proposal 1: for the study of 8 Tx UL operations,

- Prioritize one device type for efficient study and discussions

- The one device should be the one which is ‘most-likely’ to have 8 Tx antennae for UL operations

  - 1st priority: CPE

  - 2nd priority: FWA

Observation 2: regarding the UL transmission schemes

- CB-based UL is implemented in current deployments, hence is likely to be implemented for 8 Tx as well

- NCB based UL can be beneficial if UE-side reciprocity holds

Proposal 2: Regarding the transmission scheme for 8Tx UL operations,

- Support both CB and NCB based UL transmission

- For RAN1 work, adopt a serialized approach,

  - prioritize CB based discussion until RAN1#110

  - NCB based discussion starts after RAN1#110

Proposal 3: regarding the 8 Tx antenna structure, study the following aspects

- Polarization: co-pol or dual-pol

- Antenna groups: one group or multiple groups

- Antenna panels and coherence types:

  - Single panel: one antenna group and full-coherence

  - Multi-panel: multiple antenna groups and partial-/non-coherence

Proposal 4: support a single unified 8Tx codebook structure for different coherence types (i.e. FC, PC, and NC) based on antenna groups

- Antennae within a group are coherent

- Antennae across multiple groups are non-coherent

Proposal 5: regarding the 8Tx UL codebook,

- The baseline is Rel. 15 NR 8Tx DL Type I codebook

- Support the following parameters for 8Tx UL codebook

  - P= number of antenna polarization; P=1 for co-pol, and P=2 for dual-pol.

  - Mg= number of antenna groups.

  - N1= Number of antennae in 1st dimension.

  - N2= Number of antennae in 2nd dimension.

Proposal 6: investigate both single-stage and dual-stage codebook structures

Proposal 7: Discussion on full power modes can be start after the 8Tx codebook is designed

Proposal 8: regarding 8Tx NCB based UL transmission,

- Support number of SRS resources (NSRS) up to 8

- Support both one SRS resource set and two SRS resource sets

- When NSRS≤4, the SRI indication follows legacy (Rel.15) scheme, and

- When NSRS>4,

  - Study the need for Lmax>4

  - study the following SRI indication schemes

    - Alt1: combinatorial index scheme

    - Alt2: bitmap based scheme

Proposal 9: for STx2P, support both

- Case 1 (1 PUSCH): one SRI indicating a pair of SRS resources (e.g. STx2P to sTRP)

- Case 2 (2 PUSCHs): two SRIs, each indicating a SRS resource for a TRP (e.g. STx2P to mTRP)

Proposal 10: regarding max number of layers

- prioritize the RAN1 work for max 4 layers

- >4 layers can be supported, if its need and use cases can be identified

Proposal 11: for the study of 8Tx UL operations, support EVM assumptions provided in Table 1.

# References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, RAN1#103-e

- RAN1#91, Chairman’s notes

- R1-2203893, Views on UL precoding indication for STxMP, Samsung

# Appendix A: Rel. 15 4Tx UL codebook

Agreement:

Agreement:

Agreement:

Agreement:

# Appendix B

Table 1: EVM for CB based UL

| Parameters | Values |
| --- | --- |
| Traffic model | FTP model 1 with packet size 0.1Mbytes Medium load 50% Target RU |
| Duplex, Waveform | FDD, OFDM |
| Scenario | Dense urban (macro only) |
| Frequency Range | FR1, 2GHz |
| Inter-BS distance | 200m |
| Channel model | According to TR 38.901 |
| Antenna setup and port layouts at gNB | Evaluation 1 (up to rank 2): 2 ports or Rx chains: (8,1,2,1,1,1,1), (dH,dV) = (0.5, 0.8)λ Evaluation 2 (up to rank 4): 4 ports or Rx chains: (8,2,2,1,1,1,2), (dH,dV) = (0.5, 0.8)λ Evaluation 3 (up to rank 8): 8 ports or Rx chains: (8,2,2,1,1,2,2), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 8 Tx One antenna group: - (1,4,2,1,1,1,4), (dH,dV) = (0.5, 0.5)λ - (2,2,2,1,1,2,2), (dH,dV) = (0.5, 0.5)λ Multiple antenna groups: - 2 groups: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for each group - 4 groups: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for each group - companies provide their assumptions regarding the location of the groups on the device |
| gNB and UE antenna polarizations | gNB: (+45°,-45°); UE: (0°, 90°) |
| UE Tx power | 23 dBm |
| BS antenna height | 25m |
| UE antenna height & gain | Follow TR36.873 |
| BS receiver noise figure | 5dB |
| Modulation | Up to 256QAM |
| Coding on PUSCH | LDPC; Max code-block size=8448bit |
| Numerology | Slot: 14 OFDM symbol SCS: 15kHz |
| Number of RBs | 52 |
| Simulation bandwidth | 10 MHz |
| Frame structure | All uplink |
| MIMO scheme | SU-MIMO |
| SU pre-coding | Conjugate beamforming |
| Scheduling | SU, Proportional fair |
| Channel estimation | Non-ideal |
| Transmission rank | Evaluation 1: up to rank 2 Evaluation 2: up to rank 4 Evaluation 3: up to rank 8 |
| Receiver | MMSE-IRC |
| Baseline | Rel. 15 CB based 4Tx UL |