3GPP TSG RAN WG1#109-e			R1-2203895

e-Meeting, May 9th – May 20th, 2022

Agenda item:	9.1.5

Source: 	Samsung

Title: 	Initial SLS results on Type-II CSI enhancements for CJT

Document for:	Discussion and Decision

# Introduction

The Rel-18 WID [1] includes the following objectives regarding the CSI enhancements for C-JT.

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

This contribution provides initial system-level simulation (SLS) results for Type II CSI enhancements for C-JT under intra-cell and inter-cell scenarios. In particular, it compares user perceived throughput (UPT) vs. overhead for the proposed CJT codebooks (decoupled codebook: CB1 and joint codebook: CB2) in [2] along with the sTRP baseline using the Rel-16 Type II codebook.

# SLS results for intra-cell mTRP scenarios

Figure 1: Intra-cell scenario layouts for N=2,3,4

To simulate intra-cell multi-TRP (mTRP) scenarios, we considered an RMa (Rural Macro) channel model with 700 MHz carrier frequency (from TR 38.901). Figure 1 shows layouts for three intra-cell scenarios for N=2,3,4, where N is the number of TRPs within each cell. The detailed simulation assumptions are described in Table 1 of Appendix A.

The user throughput (UPT) vs. overhead trade-off results are provided for the following three codebooks.

- CB1

  - Rel-16 Type II codebook per TRP, i.e., (W1,W2,Wf) are independently selected for each TRP

  - Inter-TRP component: co-phase/co-amplitude to compensate for phase mismatch and power imbalance across TRPs.

    - 3-bit for amplitude and 3-bit for phase per SB are used.

- CB2

  - W1 per TRP: W1 is independently selected for each TRP

  - Single Wf: Wf is commonly selected for all TRPs.

  - Single W2: W2 is jointly selected across all TRPs

- Single-TRP (sTRP) with Rel-16 Type II codebook as a baseline

  - Rel-16 Type II codebook is used for the conventional-layout scenario (i.e., N=1, sTRP).

For parameters (L,M,β), the existing Rel-16 parameter combinations (paraComb) table (Table 5.2.2.2.5-1 in TS.38.214) is used.

Figures 2, 3, and 4 show the average UPT gain vs. the amount of feedback overhead for CB1, CB2, and Rel-16 T2 CB (as a baseline), with respect to paraComb=1,2,⋯,6, for N=2,3, and 4, (8 ports per TRP) respectively. The avg. UPT gain is shown with respect to sTRP with Rel-16 eType-II CB with paraComb=1 as reference.

It can be observed from Figure 2 that the proposed CB2 and CB1 yield 45% and 35% higher throughputs than that of sTRP with Rel-16 eType-II CB, respectively. Such large gain comes from the joint precoding operation across TRPs, which amounts to array, power, and beamforming gains from coherently using larger number of antenna elements. Between CB1 and CB2, the throughput of CB2 is higher than that of CB1, since CB2 provides a joint codebook structure that exploits the composite delay profile across all the TRPs. Another observation is that the throughputs of CB2 and CB1 do not change significantly as overhead increases. Notice also the high overhead of both codebooks, which implies that the parameter combinations can be refined for CB1/CB2 to further reduce the overhead.

Observation 1: CB2 and CB1 yield gain in throughout vs. overhead trade-off over Rel-16 T2 CB, with CB2 outperforming CB1.

Observation 2: The throughputs of CB2 and CB1 do not change significantly as overhead increases. The overhead for both codebooks is high. This implies that the set of parameter combinations can be refined for CB1/CB2 to further reduce the overhead.

Figure 2: Average UPT gain vs overhead when N=4 and 8 ports per TRP

As seen in Figures 3, and 4, similar trends are observed as Figure 2, i.e., CB2>CB1≫sTRP with Rel-16 eType-II CB for the avg. UPT gains, and CB2/CB1 are in high-overhead regime.

Observation 3: for varying number of TRPs (N∈2,3,4),

- CB2 outperforms CB1 for any N value

- The performance of CB2/CB1 remain similar as overhead is increased for the existing Rel-16 paraComb=1,2..,6.

Figure 3: Average UPT gain vs overhead when N=3 and 8 ports per TRP

Figure 4: Average UPT gain vs overhead when N=2 and 8 ports per TRP

Next, we compare average UPT gains of CB1/CB2 over sTRP with Rel-16 eType-II CB with respect to N=2,3,4 in one plot, as shown in Figure 5.

Observation 4: Significant performance gain (e.g.35-45% in avg. UPT with CB2 and 25-35% in avg. UPT with CB1) can be achieved with mTRP C-JT CSI (N=2,3,4) over sTRP CSI (N=1).

Figure 5: Average UPT gain of mTRP with N=2,3,4 vs overhead, and 8 ports per TRP

The next set of results are provided for the case of 4 ports per TRP. The simulation results, analogous to Figure 2, 3, 4, and 5, are provided in Figures 6, 7, 8, and 9, respectively. Since each TRP has 4 ports, the maximum L value is 4/2=2 for CB1 and CB2. Thus, we used paraComb=1 and 2, and modified paraComb=5 and 6 by replacing L=4 with L=2, and we denoted it by adjusted paraComb=1,2,3,4 in the figures.

As shown in Figures 6, 7, 8, and 9, similar trends of CB1, CB2, and Rel-16 T2 CB are observed, as we observed in the case of 8 ports per TRP.

Observation 5: the throughput-overhead trade-offs for 4 ports are similar to that for 8 ports.

Figure 6: Average UPT gain vs overhead when N=4 and 4 ports per TRP

Figure 7: Average UPT gain vs overhead when N=3 and 4 ports per TRP

Figure 8: Average UPT gain vs overhead when N=2 and 4 ports per TRP

As seen in Figures 7, 8, 9, and 10, it can be observed that the gains of CB1/CB2 over sTRP w/ Rel-16 eType-II CB becomes further significant. This is because the baseline method, sTRP w/ Rel-16 eType-II CB using 4 ports, yields limited performance that falls much short of the achievable UPT in a given traffic.

Observation 6: Further significant performance gain (e.g.70-110% in avg. UPT with CB2 and 50-90% in avg. UPT with CB1) can be achieved with mTRP C-JT CSI (N=2,3,4) over sTRP CSI (N=1).

Figure 9: Average UPT gain mTRP with N=2,3,4 vs overhead, and 4 ports per TRP

SLS results for inter-cell scenarios

To simulate inter-cell multi-TRP (mTRP) scenarios, we considered a UMa (Dense Urban) channel model with 2 GHz carrier frequency (from TR 38.901). Figure 10 shows the layout for inter-cell scenario where the three sectors of each site is assumed to be mTRP capable of CJT. Detailed simulation assumptions are described in Table 1 of Appendix A.

Figure 10: Inter-cell scenario layout

Figure 11 shows the average UPT gain over sTRP with Rel-16 eType-II CB for paraComb=1 vs the amount of feedback overhead for CB1, CB2, and Rel-16 eType-II CB, for the inter-cell scenario with 16 ports per TRP. As a baseline, the conventional sTRP scheme (i.e., no collaboration across the TRPs in each site) is assumed for sTRP with Rel-16 eType-II CB. As seen in Figure 11, a throughput vs overhead trade-off for the inter-cell scenario is similar to that observed for intra-cell scenarios.

Observation 7: A similar trend is observed that CB2 (55%) > CB1 (44%) ≫ sTRP with Rel-16 eType-II CB (0%) as the case of intra-cell scenarios.

Figure 11: Average UPT gain vs overhead for inter-cell scenario with 16 ports per TRP

Conclusion

In this contribution, the following observations are made:

Observation 1: CB2 and CB1 yield gain in throughout vs. overhead trade-off over Rel-16 T2 CB, with CB2 outperforming CB1.

Observation 2: The throughputs of CB2 and CB1 do not change significantly as overhead increases. The overhead for both codebooks is high. This implies that the set of parameter combinations can be refined for CB1/CB2 to further reduce the overhead.

Observation 3: for varying number of TRPs (N∈2,3,4),

- CB2 outperforms CB1 for any N value

- The performance of CB2/CB1 remain similar as overhead is increased for the existing Rel-16 paraComb=1,2..,6.

Observation 4: Significant performance gain (e.g.35-45% in avg. UPT with CB2 and 25-35% in avg. UPT with CB1) can be achieved with mTRP C-JT CSI (N=2,3,4) over sTRP CSI (N=1).

Observation 5: the throughput-overhead trade-offs for 4 ports are similar to that for 8 ports.

Observation 6: Further significant performance gain (e.g.70-110% in avg. UPT with CB2 and 50-90% in avg. UPT with CB1) can be achieved with mTRP C-JT CSI (N=2,3,4) over sTRP CSI (N=1).

Observation 7: A similar trend is observed that CB2 (55%) > CB1 (45%) ≫ sTRP with Rel-16 eType-II CB (0%) as the case of intra-cell scenarios.

# References

- RP-2133598, New WID: MIMO Evolution for Downlink and Uplink, RAN1#103-e

- R1-2203890, “Views on CSI enhancements”, Samsung

# Appendix A

Table 1: Simulation Assumptions for C-JT

| Parameter |  | Value (Intra-cell scenario) | Value (Inter-cell scenario) |
| --- | --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM | FDD, OFDM |
| Multiple access |  | OFDMA | OFDMA |
| Scenario |  | RMa (Rural Macro) N_TRP (#TRPs): 2, 3, 4 | Dense Urban (Macro only) N TRPs: 3 only |
| Frequency Range |  | FR1, 700Hz | FR1, 2GHz |
| Inter-BS distance |  | 1.7km | 200m |
| Channel model |  | According to the TR 38.901 | According to the TR 38.901 |
| Number of Rings |  | 2 rings (57 sectors) - Each sector has N TRP as a cooperating mTRP set. | 2 rings (57 sectors): - The three sectors of each site is a cooperating mTRP set. |
| Number of UEs per sector |  | 30 | 30 |
| Antenna setup and port layouts at gNB |  | For each TRP, - 4 ports: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.8)λ - 8 ports: (2,2,2,1,1,2,2), (dH,dV) = (0.5, 0.8)λ Total #ports in mTRP = N TRP x {4,8} | For each TRP, - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Total #ports in mTRP= N TRP x {16} |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |
| BS Tx power |  | 46 dBm in total, (normalized by N TRP, i.e., distributed across TRPs) | 41 dBm per TRP (not normalized) |
| BS antenna height |  | 25m | 25m |
| UE antenna height & gain |  | Follow TR36.873 | Follow TR36.873 |
| UE receiver noise figure |  | 9dB | 9dB |
| Modulation |  | Up to 256QAM | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot | 14 OFDM symbol slot |
|  | SCS | 15kHz | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS | 52 for 15 kHz SCS |
| Simulation bandwidth |  | 10 MHz | 10 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots | Slot Format 0 (all downlink) for all slots |
| MIMO and scheduling scheme |  | MU-MIMO PF scheduling (User Rank 1) | MU-MIMO PF scheduling (User Rank 1) |
| MIMO layers |  | Up to 4 | Up to 4 |
| CSI feedback |  | Feedback assumption - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms | Feedback assumption - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Based on CB1, CB2, Rel-16 eType-II | Based on CB1, CB2, Rel-16 eType-II |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | RU 40~50% | RU 40~50% |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC | MMSE-IRC |
| Feedback assumption |  | Realistic | Realistic |
| Channel estimation |  | Realistic | Realistic |
| Evaluation Metric |  | User perceived throughput and CSI feedback overhead | User perceived throughput and CSI feedback overhead |
| Baseline for performance evaluation |  | Single-TRP (N=1) with Rel-16 eType-II CB | Single-TRP with Rel-16 eType-II CB (no collaboration across the sectors in each site) |