3GPP TSG RAN WG1 #109-e	R1-2203898

e-Meeting, May 9th – May 20th, 2022

Agenda item:	9.2.2.2

Source: 	Samsung

Title: 	Other aspects on AI/ML for CSI feedback enhancement

Document for:	Discussion and Decision

# Introduction

In RAN#94-e, Rel-18 new study item on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” is endorsed. One of the objectives of the study item [1] is the following:

*** text omitted*** Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] *** text omitted*** For the use cases under consideration: - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

In this contribution, we will provide our views on the potential specification impact of the CSI feedback enhancement use case and finalization of its sub-use cases.

# CSI Prediction

## Motivation

A massive MIMO system relies on high-resolution CSI feedback from UEs to the gNB to facilitate multi-user MIMO (MU-MIMO) transmission. This can yield significant gains in terms of system throughput, where multiple users can be simultaneously supported.

Massive MIMO systems are affected by channel aging, where the channel varies between the time that 1) a UE computes CSI feedback and the time that 2) a gNB uses that CSI feedback to transmit to the UE.  For example, channel aging can create a significant mismatch between a DL beamforming vector that a UE recommends for transmission at time t and the optimal DL beamforming vector at time t.  Channel aging can be especially problematic in high-mobility scenarios, leading to significant degradation in MU-MIMO performance (e.g. user-perceived throughput).

Recent research results from both academia and industry indicate that AI-based CSI prediction strategies can significantly reduce prediction error beyond that achieved by the sample-and-hold strategy that is supported by Rel. 15-17.  Many of these results utilize deep learning techniques to learn the temporal channel correlations (and, in some instances, spatial-frequency channel correlations).

## Description

Fig. 1 shows one approach for this sub-use case which relies on a three-dimensional convolutional neural network (3D-CNN) and entails the following steps:

- The UE/gNB stores its channel observation H(tk) at each time instant tk in a buffer, where this buffer can hold three channel observations.

- At time instant t3, the UE/gNB passes all of the channel observations in its buffer (i.e. H(t3), H(t2), and H(t1)) to a 3D-CNN, which predicts the channel at the next time instant H(t4) as H(t4).

Figure 1: AI-based CSI prediction

Another approach for this sub-use case could entail UL-to-DL channel prediction at the gNB in an FDD system, where UL-to-DL channel reciprocity cannot be assumed.  In this scenario, the gNB could use one or multiple received SRS to predict DL CSI.

As this sub-use case can encompass a range of approaches, this sub-use case should be clearly defined (e.g. assumptions, required inputs, required outputs).  For example:

- CSI prediction at the gNB: one or multiple received UL SRS could be input to the CSI predictor, and the CSI predictor could output a prediction of the UL channel H

- CSI prediction at the UE: one or multiple received DL CSI-RS could be input to the CSI predictor, and the CSI predictor could output a prediction of the DL channel H.

Proposal 1-1: Study CSI prediction as one sub-use case for AI/ML for CSI feedback enhancement, including signaling requirements, input/output requirements, CSI configurations, and training strategies.

## Spec impact

In [2], four UE-gNB collaboration levels (labeled as (A), (B), (C), and (D)) are defined with varying degrees of spec impact.  A scenario where the gNB performs CSI prediction is expected to map to collaboration level (A), as the gNB can rely on implementation-based local training and inference (e.g. the gNB can rely on existing signals such as UL SRS to perform training and inference, so no new signaling would need to be defined).

A scenario where the UE performs CSI prediction is expected to map to collaboration level (B), though.  For example, when UL-to-DL channel reciprocity cannot be assumed (e.g. FDD systems), DL CSI prediction at the UE can yield improved performance, as the UE can perform training and inference while relying on limited information exchanges with the gNB:

- The gNB may enable or disable channel prediction at the UE.

- The gNB may configure the UE to apply a particular channel prediction method.

- The gNB may configure the UE to perform channel prediction at a given time offset.

- The gNB may configure the UE to report its channel prediction.

In particular, this scenario only requires AI/ML model training/storage/inference at the UE; AI/ML model exchanges with the gNB are not required.

Proposal 1-2: Study CSI prediction at the UE under collaboration level (B), where limited information exchanges are required to configure/enable AI/ML.

# CSI Compression

## Motivation

A massive MIMO system relies on high-resolution CSI feedback from UEs to the gNB to facilitate multi-user MIMO (MU-MIMO) transmission. This can yield significant gains in terms of system throughput, where multiple users can be simultaneously supported.

The overhead of sending high-resolution CSI feedback can be significant, especially in rich scattering environments. Several feedback reduction strategies have been adopted in Rel. 15-17, where channel eigenvectors are represented by a codebook. Such codebooks facilitate feedback overhead reduction by exploiting inherent correlations in the spatial and frequency domains (and, in Rel. 17, by exploiting angle-delay reciprocity when justified).

Recent research results from both academia and industry indicate that AI-based CSI compression strategies can yield further reductions in feedback overhead beyond those achieved by the Rel. 15-17 codebooks.  Many of these results utilize an autoencoder (AE), where a trained encoder at the UE transforms CSI into a latent space that captures the above-mentioned channel correlations. A trained decoder at the gNB then inverts this transformation to recover the original CSI.

## Description

Fig. 2 shows one approach for this sub-use case which relies on an AE and entails the following steps:

- The UE pre-processes its channel estimate H (e.g. by performing an SVD for each sub-band) to obtain a precoder Vstack, which is passed to the encoder.

- The encoder compresses the precoder Vstack into a feature vector z.

- The feature vector z is passed from the UE to the gNB (here, it is assumed that the feedback channel is noiseless), which passes it to the decoder.

- The decoder decompresses the feature vector z into an estimate Vstack of the precoder.

Figure 2: Autoencoder

This sub-use case should be clearly defined (e.g. assumptions, required inputs, required outputs).  For example, the UE could either:

- pre-process its channel estimate H to obtain a precoder Vstack that it passes to the encoder

- directly pass its channel estimate H to the encoder.

Similarly, the gNB could either:

- use its decoder to decompress the feature vector z into an estimate Vstack of the precoder

- use its decoder to decompress the feature vector z into an estimate H of H.

The data format of the required inputs/outputs, including the input to the encoder and the output of the encoder (i.e. the feedback that is sent by the UE to the gNB) should be clearly defined.  For example:

- the input to the encoder could consist of the channel matrices of a single resource element (RE)

- the input to the encoder could consist of the channel matrices of multiple REs.

Proposal 2-1: Study CSI compression as one sub-use case for AI/ML for CSI feedback enhancement, including signaling requirements, input/output requirements, CSI configurations, and training strategies.

## Spec impact

For model inference of AE-based CSI compression, the encoder and the decoder should be separately deployed at the UE and the gNB, respectively.  Therefore, this sub-use case maps to collaboration level (D), where new signaling to support model exchange is required.  This includes the following cases:

- Offline training-based methods:

  - For example, a gNB can either 1) use a specified AE(s) or 2) collect a dataset for cell-specific offline training using the current specifications (e.g. CSI measurements from SRS by exploiting DL-UL reciprocity, reconstructed CSIs from legacy CSI feedback, generated CSI from a channel model by applying second-order statistics that are measured from SRS, etc.). Thus, no signaling would be required to transfer a training dataset. After training the AE, the gNB transfers/configures the specified/trained encoder to the UE.

- Online training-based methods:

  - For example, if a gNB employs a proprietary training strategy, a UE could update the trained/specified encoder without knowledge of the gNB’s training strategy.  In this case, the gNB would also transfer/configure its specified/trained decoder to the UE, and the UE would report validation results for the updated encoder to the gNB.  If the UE updates the decoder, it would not need to transfer the updated decoder to the gNB, yielding a significant reduction in UL feedback overhead.  As another example, a gNB could leverage federated learning to train a cell-specific AE.  Initially, the gNB would transfer the cell-specific AE to each UE in a cell.  Each UE could then use a local dataset to update the AE model weights and send these updated weights to the gNB.  After that, the gNB could aggregate these updated weights and update the cell-specific AE.

Proposal 2-2: Study CSI compression at the UE under collaboration level (D), where model exchanges are required to configure/enable AI, and inference is performed at the UE and the gNB.

Proposal 2-3: Study and verify model update of the encoder at the UE, where the transferred/configured AE is required to keep gNB’s training strategy privacy.

# Joint CSI Prediction and Compression

## Motivation

The legacy codebooks (CBs) compress the CSI in spatial (angle) and frequency (delay) domains, i.e., spatial-domain compression in Rel. 15 CBs as well as spatial & frequency-domain compression in Rel. 16 and Rel. 17 CBs. Moreover, Rel. 17 port selection (PS) CB achieves further reduction in the CSI feedback overhead (higher compression) by exploiting angle-delay reciprocity. The remaining dimension (domain) that has not yet been exploited is the time-domain compression. Therefore, it is natural if AI/ML-based CSI feedback enhancement incorporates joint compression in spatial (angle), frequency (delay), and time (Doppler) domains. It can be conjectured that by utilizing the three-dimensional (spatial-frequency-time) compression, the CSI overhead can be reduced significantly, i.e., higher compression ratio can be achieved.

Figure 3: Non-linear compression of CSI via AI/ML

Additionally, one application area for joint prediction & compression is medium/high speed scenarios where CSI aging is observed in the legacy CSI feedback framework. Owing to the shorter channel coherence time/duration in medium/high speed scenarios, frequent CSI measurements and feedback might be required. However, frequent CSI measurements and feedback is inefficient in terms of both feedback overhead and computational complexity. In this regard, it may be helpful to investigate CSI feedback compression in the time domain. In Fig. 3, time-domain compressed CSI which can be applied to N coherence time intervals is depicted; in contrast, N CSI reports in legacy CBs would be required for this scenario.

Finally, another advantage of AI/ML-based CSI feedback enhancement is its ability to achieve non-linear compression. DFT basis vectors-based compression has been utilized in Rel. 15-17 CBs for spatial & frequency-domain compression. Basis vectors-based representation of precoding vectors is computationally advantageous. However, basis vectors-based representation may incur a non-trivial approximation error due to incomplete basis representation, fixed basis sampling, fixed (RRC-configured) number of basis vectors, etc. In particular, to reduce CSI feedback overhead and achieve efficient representation, the conventional schemes (Rel. 16-17 codebooks and their potential enhancement in Rel. 18 for Doppler-domain compression) represent the CSI with fewer number of basis vectors, i.e., L spatial, M[v] frequency and M[d] Doppler basis vectors, as compared to the full set of orthogonal basis vectors, the number of which are, N3, and N[4], respectively, where, and . This incomplete basis representation results in imperfect representation or approximation loss. Furthermore, the number of basis vectors, i.e., L, M[v] and M[d], and the dimension of the basis vectors (sampling), i.e., Ntx, N3 and N[4 ] are fixed (RRC configured), will result in lacking flexibility. Moreover, the existing codebooks rely on fixed (linear) quantization for amplitudes and phase coefficients reporting. In this regard, due to the non-linear approximation property of auto-encoders (AEs), AI/ML-based sampling and quantization may provide more flexibility (degrees-of-freedom) and reduce quantization/approximation error. The nonlinear compression of CSI is depicted in Fig. 3 by a broken yellow line as opposed to linear (basis-vector-based) compression along the axes of the three dimensions.

## Description

Figure 4: CSI compression from M measurements and prediction/extrapolation to N time instants/intervals

Joint CSI prediction and compression allows the UE to report a CSI that may be used by the gNB to derive multiple CSI which can be applied to multiple future time instants/intervals. Thus, the CSI in this sub-use case involves time-domain CSI compression in addition to spatial & frequency-domain compression.

Figure 5: Approach 1: prediction at gNB vs. Approach 2: prediction at UE

Proposal 3-1: Study joint CSI prediction and compression as a representative sub-use case of AI/ML based CSI feedback enhancement.

Two approaches can be considered for this sub-use case.

Approach 1: A UE measures CSI from M CSI-RS resources and derives a single compressed CSI report from these measurements. The gNB then reconstructs the M CSIs corresponding to the M measurements. Moreover, the gNB performs prediction/extrapolation to future time instants based on the reconstructed M CSIs.

TABLE 1: Input variants for AE: Approach 1

| Variants (Inputs to the Autoencoder) | Compression Domain for AI | Advantage |
| --- | --- | --- |
| Explicit Eigenvectors | Spatial, frequency and time domain | - Lower complexity (no need to compute PMI) - May achieve higher compression ratio |
| PMI information based on conventional codebook (Rel. 15, 16, 17 CBs) | Time domain | - May have better convergence and/or complexity |
| Full channel matrix (angle-delay domain) | Time domain | - May have better convergence and/or complexity - Lower processing at the UE. |

Approach 2: A UE measures CSI from M CSI-RS resources and predicts/extrapolates the CSIs corresponding to N predefined time intervals. One way to do such prediction/extrapolation is based on Doppler components estimation. The encoder at the UE then compresses the CSI corresponding to these N time intervals. As the N time intervals are ideally within the Doppler stationary time interval, i.e., the time interval where the Doppler components of the channel are constant, the N CSI are correlated thus compressible. The gNB then reconstructs the CSI for these N time instants (sub time units) from the compressed CSI feedback.

A comparison between these two approaches is provided in the table below.

TABLE 2: Comparison of Approach 1 and Approach 2

| No. | Metric | Approach 1 | Approach 2 |
| --- | --- | --- | --- |
| 1. | Burden (complexity) on UE | Moderate | High |
| 2. | Suitability for CQI reporting | May require study | Suitable (CQI can be derived from corresponding PMI) |
| 3. | Time granularity in predicted PMI | Flexible (gNB has control) | Configured application interval (subtime unit) |

## Spec impact

In case the encoder part of the AE is shared by the gNB via model transfer (collaboration level (D)) the values of M and/or N along with other parameters that describe the input to the AE such as the number of sub-bands, quantization bits per port, etc., can be configured as part of the model transfer. Thus, M and N may not be needed to be specified as part of CSI resource and reporting configurations. This facilitates the use of models with different input types without the need for specification support, i.e., inputs including eigenvectors, full channel matrix across angle-delay domains, even PMI information based on legacy CBs etc., giving vendors/operators flexibility in their AI/ML model development.

One of the essential configurations from the gNB is the size of the AE’s output. Of course, this information could be configured as part of the model transfer. Then, when the gNB triggers, activates or configures a CSI report based on a certain AI model, the UE will implicitly figure out the size of the AE’s output. One open issue, however, concerns the mapping of feedback bits to the UCI payload. Another open issue is whether an AE-based CSI feedback can be split into multiple parts, which would allow dropping of some feedback bits in case the UCI payload is not large enough to carry all of the feedback bits.

It is essential to spare the UE from the burden of heavy processing as much as possible. In this regard, it is preferred to perform essential training at the gNB. Various approaches can be considered to tackle scenario-sensitivity (generalization issues) of the AE including transfer learning, assistance information exchange in model selection, update, etc.

Proposal 3-2: Study joint CSI compression and prediction: gNB-side CSI prediction/extrapolation (Approach 1) and UE-side prediction/extrapolation for joint CSI prediction and compression (Approach 2) including signaling requirements, CSI configurations, and training strategies.

# Other Topics

If online training is supported at the UE (e.g. for the CSI compression sub-use case), the UE would need sufficient processing time to perform that task.  As RAN1 has already defined a timeline for PDSCH/PUSCH processing, an analogous timeline should be defined for model training, given that model training differs from PDSCH/PUSCH processing in several respects.

Proposal 4-1: Study UE processing time impact on online training for update, transfer, and download.

# Conclusions

In this contribution, we discussed the potential specification impact of the CSI feedback enhancement use case and finalization of its sub-use cases.  Our proposals are summarized as follows.

Proposal 1-1: Study CSI prediction as one sub-use case for AI/ML for CSI feedback enhancement, including signaling requirements, input/output requirements, CSI configurations, and training strategies.

Proposal 1-2: Study CSI prediction at the UE under collaboration level (B), where limited information exchanges are required to configure/enable AI/ML.

Proposal 2-1: Study CSI compression as one sub-use case for AI/ML for CSI feedback enhancement, including signaling requirements, input/output requirements, CSI configurations, and training strategies.

Proposal 2-2: Study CSI compression at the UE under collaboration level (D), where model exchanges are required to configure/enable AI, and inference is performed at the UE and the gNB.

Proposal 2-3: Study and verify model update of the encoder at the UE, where the transferred/configured AE is required to keep gNB’s training strategy privacy.

Proposal 3-1: Study joint CSI prediction and compression as a representative sub-use case of AI/ML based CSI feedback enhancement.

Proposal 3-2: Study joint CSI compression and prediction: gNB-side CSI prediction/extrapolation (approach 1) and UE-side prediction/extrapolation for joint CSI prediction and compression (approach 2) including signaling requirements, CSI configurations, and training strategies.

Proposal 4-1: Study UE processing time impact on online training for update, transfer, and download.

# References

[1] RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Dec, 2021.

[2] R1-2203896, Samsung, “General aspects of AI ML framework and evaluation methodology” 3GPP TSG RAN1#109-e