3GPP TSG RAN WG1 #109-e			R1-2203899

e-Meeting, May 09 – May 20, 2022

Agenda item:	9.2.3.1

Source:	Samsung

Title:	Evaluation on AI/ML for beam management

Document for:	Discussion and Decision

- Introduction

In RAN#94-e, Rel-18 new study item on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” is endorsed [1]. The objective of the study item is as follows, with bullets relevant to this contribution highlighted.

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment, model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this contribution, we will provide our view on the aspects of evaluation on AI/ML for beam management (BM), including the discussion on the evaluation methodology, KPI, and other relevant aspects.

- Evaluation methodology on AI/ML-based beam management

As presented in our accompanying discussion paper on general aspects of AI/ML framework [3], we expect the evaluation of candidate (sub) use cases to be conducted in Phase-I, in which various AI/ML models could be considered for each (sub) use cases to study the potential performance benefits and other aspects, as the input to determine the representative (sub) use cases in Phase II. Furthermore, the evaluation methodology (EVM) shall at least fulfill:

- Reuse/align the evaluation assumptions for conventional schemes in Rel 15/16/17, as much as possible;

- Comprehensiveness enough, to include various aspects of AI/ML operations including overhead associated with assistance information;

- Model agnostic EVM, allows different kinds of model or implementation assumptions.

Based on these principles, we would like to provide our views on the evaluation methodology for the use cases relevant to AI/ML for beam management. Specifically, in the approved SID [1], the potential sub-use cases of beam prediction in time, and/or spatial domain are brought up for AI/ML for beam management, with the expected benefits in terms of overhead and latency reduction, beam selection accuracy improvement. Based on the accompanying discussion paper with the analysis on the beam management use cases [2], it has been shown that the sub-use cases can be further classified by which entity to perform AI/ML-based prediction, i.e., either in a gNB side or in a UE side. Furthermore, one additional use case of beam measurement feedback compression is provided in [2], which makes the following sub use cases as the targets for this EVM analysis:

- Beam predication in the spatial domain (either in the UE side, or in the gNB side);

- Beam predication in the time domain (either in the UE side, or in the gNB side);

- Beam measurement feedback compression.

Accordingly in the following sections, after providing the preference on the common considerations on the evaluation methodology for all (sub) use cases of AI/ML for beam management for both system-level and link-level evaluation, we will elaborate the specific aspects for each (sub) use case.

As provided in SID, the evaluation methodology is expected to be based on statistical models (from TR 38.901), for link and system level simulations. Based on that, the relevant simulation methodologies should firstly be reviewed and analyzed, by especially considering the evaluation targets to provide the training, validation and testing dataset. Similar to the EVM discussion for NR eMIMO on multi-beam operation, it is proposed to consider Table A.2.5-2 (for SLS) and Table A.1.6.4 (for LLS) of TR 38.802 as the starting point for Rel-18 AI/ML for beam management evaluation, based on which necessary modifications/simplifications can be further discussed for the specific evaluation purpose and/or sub use case.

Proposal 1-1: Consider Table A.2.5-2 (for SLS) and Table A.1.6.4 (for LLS) of TR 38.802 as the starting point for Rel-18 AI/ML for beam management evaluation, based on which necessary modifications/simplifications can be further discussed for the specific evaluation purpose and/or sub use case.

In the following subsections, our analysis and preference will be provided for system-level, link-level and other EVM aspects for AI/ML-based beam management.

1.1 System-level evaluation methodology for AI/ML-based beam management

1.1.1 General evaluation assumptions

For SLS, by digging into the details of Table A.2.5-2 in TR38.802, the following highlighted points in the Table 1-1 are provided and marked which we believe can be further discussed, by considering the needs of AI/ML for beam management use cases and also the urgent timeline for Rel-18 AI/ML BM evaluation (especially for Phase-I). Particularly, by considering the two sub-use cases mentioned in SID (beam prediction in the spatial and time domain), the continuity in the spatial domain and the time domain is of importance, and without the continuity provided in the training/evaluation/testing dataset, the prediction is baseless and impossible, thus highlighting the newly added last row for spatial continuity model listed in the below Table 1-1.

Table 1-1: Proposed general evaluation assumptions for AI/ML beam management (for SLS)

| Parameters | Values provided in A.2.5-2 in TR 38.802 | Proposed for AI/ML BM |
| --- | --- | --- |
| Scenarios (Carrier Frequency) | Indoor hotspot :4 GHz, 30GHz; Urban macro: 4 GHz, 30GHz; Dense Urban: For 4 GHz: Evaluate macro layer For 30 GHz: Evaluate micro layer Note: other antenna configurations should be considered as well. | Down-selected to only consider Dense Urban: - For 30GHz: Evaluate macro layer Hex. Grid - with inter-BS distance = 200m - Details to be discussed later. |
| Mode | DL SU-MIMO/ MU-MIMO | Down-selected to only consider DL SU-MIMO: Note: For beam prediction to evaluate beam selection accuracy, DL SU-MIMO can be considered as starting point. |
| Simulation bandwidth | 4GHz: 20MHz (DL+UL) 30GHz: 80MHz.(DL+UL) or 40MHz.(DL+UL) | 30GHz: 80MHz.(DL+UL) or 40MHz.(DL+UL) |
| Subcarrier Spacing for data | For 4 GHz: 15kHz For 30 GHz: 120kHz, 60kHz (Other subcarrier spacings can be considered) | For 30 GHz: Only 120kHz Note: Only 120kHz is considered to save simulation efforts. |
| Channel Model | Following related assumption in TR 38.802 | Following related assumption in TR 38.802 |
| TXRU mapping to antenna elements | Companies explain the details of TXRU mapping to antenna elements. Notes: 30GHz: 2D DFT based beam per polarization as a baseline; 4GHz: 1D DFT per vertical dimension per polarization as baseline; | Companies explain the details of TXRU mapping to antenna elements. Notes: 30GHz: 2D DFT based beam per polarization as a baseline; |
| TXRU mapping weights | Companies explain the details of TXRU mapping weights. | Companies explain the details of TXRU mapping weights. |
| Criteria for selection for serving TRP | Companies explain the details of criteria for selection for serving TRP. | Based on the distance to gNB, rather than RSRP based. Note: for easier alignment and avoid the impact of different beambook. |
| Criteria for beam selection for serving TRP | Companies explain the details of criteria for beam selection for serving TRP. | Based on a detailed algorithm, which is the evaluation target for AI and non-AI based BM algorithm |
| Constraints for the range of selective beams per TRP sector | Companies explain what scheme is used | Companies explain what scheme is used |
| Scheduling algorithm | PF scheduler | PF scheduler |
| Link adaptation | Based on CSI-RS. | Based on CSI-RS. |
| Traffic Model | FTP model 1/3 with packet size 0.1 and 0.5Mbytes (other value is not precluded). Other traffic models including the full buffer are not precluded. | FTP model 1/3 with packet size 0.1 and 0.5Mbytes (other value is not precluded). Other traffic models including the full buffer are not precluded. |
| BS antenna configurations | For 4GHz: (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) as baseline. (dV, dH) = (0.8, 0.5) λ. For 30GHz: (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2). (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ Note: important to consider also other antenna configurations to maintain flexibility | Simplified BS antenna configuration with only 1 panel used, which is equivalent in beam selection accuracy performance to be evaluated in Phase-I. - For 30GHz: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1). (dV, dH) = (0.5, 0.5) λ. |
| BS antenna element radiation pattern | For 4GHz: According to TR36.873 For 30 GHz: According to TR38.802 | For 30 GHz: According to TR38.802 (Table A.2.1-6 for 3-Sector BS antenna radiation pattern) |
| UE antenna configurations | For 4GHz: Mg = 1, Ng = 1, P =2, (dV, dH) = (0.5, 0.5) λ, MxNxP<=8 (companies report M,N) For 30GHz: (M, N, P, Mg,Ng) = (2, 4, 2, 1, 2); (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (0, 0) λ. *Θmg,ng=90°; Ω0,1=Ω0,0+180°; Note: important to consider also other antenna configurations to maintain flexibility | For 30GHz: (M, N, P, Mg,Ng) = (2, 4, 2, 1, 2); (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (0, 0) λ. *Θmg,ng=90°; Ω0,1=Ω0,0+180°. Note: Number/location of panels: 2 panels (left, right) can be assumed based on the above parameters. |
| UE antenna element radiation pattern | For 4GHz: Omni-directional with 5dBi gain For 30GHz: See Table A.2.1-8 in TR 38.802 | For 30GHz: See Table A.2.1-8 in TR 38.802 |
| Inter-panel calibration | Ideal, non-ideal following 38.802 (optional) | Ideal, non-ideal following 38.802 (optional) |
| Beam correspondence | Companies report details of the assumptions | Companies to explain beam correspondence assumptions (in accordance to the two types agreed in RAN4) - UE capability of beam correspondence without SRS assistance is prioritized. |
| Control and RS overhead | Companies report details of the assumptions | Companies report details of the assumptions |
| Control channel decoding | Ideal or Non-ideal (Companies explain how is modeled) | Ideal or Non-ideal (Companies explain how is modeled) |
| UE receiver type | MMSE-IRC as the baseline, other advanced receiver is not precluded | MMSE-IRC as the baseline, other advanced receiver is not precluded |
| BF scheme | Companies explain what scheme is used | Companies explain what scheme is used |
| Transmission scheme | Multi-antenna port transmission schemes Note: Companies explain details of the using transmission scheme. | Multi-antenna port transmission schemes Note: Companies explain details of the using transmission scheme. |
| UE mobility feature | Follow Phase 3 calibration i.e. Add-on features including UE mobility, rotation, blockage, etc. can be considered. Note: Companies explain whether or which model is used in simulation evaluation. If used, the configuration details should be explained | UE mobility feature (features including UE mobility, rotation, blockage, etc) is of importance to AI/ML beam prediction, and to be chosen for different sub-use cases. Note: Companies explain whether or which model is used in simulation evaluation. If used, the configuration details should be explained |
| Spatial consistency | N/A in TR38.802 | Spatially-consistent UE mobility modelling: Procedure A in clause 7.6.3.2 in TR38.901 Note: Spatial consistency model is needed to evaluate beam prediction performance, by generating the correlated cluster power, delay, AOD/AOA/ZOD/ZOA. |

Proposal 1-2: Based on SLS assumption proposed in Table A.2.5-2 of TR 38.802, the following selection/simplification/modification are proposed as the starting point for Rel-18 AI/ML for beam management evaluation:

- Down-selected to only consider 30GHz, Dense Urban, macro layer Hex. Grid, inter-BS distance = 200m;

- Down-selected to only consider 120kHz SCS;

- Simplified criteria for selection for the serving TRP, i.e., based on the distance to the gNB.

- Simplified BS antenna configuration for BS antenna configuration with only 1 panel used, which is equivalent in beam selection accuracy performance to be evaluated in Phase-I:

  - For 30GHz: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ;

- Beam correspondence: Companies report details of the assumptions:

  - UE capability of beam correspondence without SRS assistance is prioritized

- UE mobility feature (features including UE mobility, rotation, blockage, etc) is of importance to AI/ML beam prediction, and to be chosen for different sub-use cases;

- Spatially-consistent UE mobility modelling: Procedure A in TR 38.901.

1.1.2 Assumptions and parameters for Dense Urban scenario

By following the principle of “reusing/aligning the evaluation assumptions for conventional schemes in Rel 15/16/17 as much as possible”, for the above-mentioned selected scenario (i.e., Dense Urban, 30GHz), the following detailed system-level parameters are provided based on Table A.2.1-1 of TR 38.802:

Table 1-2: Proposed SLS assumptions/parameters for Dense Urban for AI/ML beam management

| Parameters | Proposed SLS assumptions/parameters for Dense Urban for AI/ML BM |
| --- | --- |
| Layout | Single layer: Macro layer: Hex. Grid |
| Inter-BS distance | Macro layer: 200m |
| Carrier frequency | Macro layer: 30GHz |
| Channel model | Above 6GHz: UMa (Macro layer) |
| BS Tx power | Macro layer: Above 6GHz: 40 dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 40 dBm EIRP should not exceed 73 dBm and 68 dBm for the macro and micro layers respectively(*) |
| UE Tx power | 30GHz: 23dBm EIRP should not exceed 43 dBm (*) |
| BS antenna height | 25m for macro cells |
| BS receiver noise figure | Above 6GHz: 7dB |
| UE antenna height | Follow TR36.873 |
| UE antenna gain | Follow the modeling of TR36.873 |
| UE receiver noise figure | Above 6GHz: 10dB (high performance) |
| UE distribution | Simplified to 100% outdoor and only LoS case only as starting point. FFS the impact of NLoS based on company selected distribution. |
| UE speed | Fixed 30km/h as starting point. FFS random UE speed based on company selected distribution. |

Proposal 1-3: Based on SLS assumptions/parameters in Table A.2.1-1 of TR 38.802, the following simplification/modification are proposed as the starting point for the selected Dense Urban scenario:

- Down-selected to only consider 10 dB UE receiver noise figure;

- For UE distribution, simplified to 100% outdoor and only an LoS case only as a starting point.

  - FFS the impact of NLoS based on company selected distribution;

- For UE speed, fixed 30 km/h as a starting point:

  - FFS random UE speed based on company selected distribution.

1.1.3 Trajectory model for UE mobility

As mentioned above in Table 1-1, the UE mobility feature (features including UE mobility, rotation, blockage, etc.) is of importance especially for the evaluation on AI/ML beam prediction, for both time and spatial domains. At least the following kinds of UE mobility models need to be considered as candidates:

- Option #1: Linear and fixed trajectory model,

  - e.g., the intra-cell mobility model, provided in Table 2 of R1-2007151, which is proposed for the EVM on the enhancement of multi-beam operation in the Rel-17 NR FeMIMO work item. Here are some detailed descriptions:

    - Trajectory sampling at most spaced by decorrelation distance;

    - For each of the 21 cells: One UE is dropped as follows: For the upper right sector/cell (can be extended analogously to the upper left and lower sectors/cells) with d=30m, the UE starts at P and moves along the 120-deg line downward to Q;

    - Each sector is a cell and that the cell association for intra-cell mobility is geographic and not RSRP based.

    - Note: Optionally, if for some reason a company would like to simulate only one cell/sector, the company should clearly state this in the contribution, including the assumed interference model.

Figure 1-1. Linear and fixed trajectory model for UE intra-cell mobility [R1-2007151]

- Option #2: Linear trajectory model with random direction change:

  - In order to avoid the very simplification of the existing linear and fixed trajectory model, the randomness can be futher considered, with the below as an example:

    - Initial UE location, moving direction and speed: UE is randomly dropped in a cell, and a moving direction is randomly selected, with a fixed speed;

    - UE moving trajectory: UE will move straightly along the selected direction to the end of an time interval, where the length of the time interval is provided by using an exponential distribution with average interval length, e.g., 5s, with granularity of 100 ms.

      - UE moving direction change: At the end of the time interval, UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°].

      - UE move straightly within the time interval with the fixed speed.

- Option #3: Linear trajectory model with random and smooth direction change:

  - To avoid the sharp direction change in Option #2, the smooth direction change can be introduced for the UE mobility model. Take below as an example:

    - Initial UE location, moving direction and speed: The same as above example for Option #2.

    - UE moving trajectory: UE will change the moving direction by multiple steps within an time internal, where the length of the time interval is provided by using an exponential distribution with average interval length, e.g., 5s, with granularity of 100 ms.

      - UE moving direction change: At the end of the time interval, UE will change the moving direction with the angle difference A_diff from the beginning of the time interval, provided by using a uniform distribution within [-45°, 45°].

      - The time interval is further broken into N sub-intervals, e.g. 100ms per sub-interval, and at the end of each sub-interval, UE change the direction by the angle of A_diff/N.

      - UE move straightly within the time sub-interval with the fixed speed.

Observation 1-1: For the trajectory model for UE mobility, at least the following options exist:

- Option #1: Linear and fixed trajectory model, e.g., the intra-cell mobility model in Table 2 of R1-2007151.

- Option #2: Linear trajectory model with random direction change.

- Option #3: Linear trajectory model with random and smooth direction change.

Based on some initial analysis and evaluation, the mobility model could have the impact on prediction performance, for example, the UE’s sharp turning points which exists in the linear trajectory model (e.g., P and Q in the above Option #1 for intra-cell linear mobility model) gives the discontinuity which does not fully match with practical scenario and provided additional challenges to beam prediction.

Proposal 1-4: The impact of discontinuous UE trajectory model on the beam prediction performance shall be considered for UE mobility model selection.

1.1.4 gNB/UE beambook and beam measurement configuration

The gNB/UE beambook design is of importance for beam prediction in both spatial and time domains, which is expected to be aligned among companies, otherwise, the conclusion of AI/ML beam management can hardly be aligned. For example, for beam prediction in the spatial domain, the measurements of beams as input and the beam selection as an output of neural network can be greatly different, if the number of beams for input and output are differently assumed by companies. Furthermore, the various beam directions in different beambook designs will also make the alignment harder. On the other hand, we also noticed that in the existing evaluation on beam management in eMIMO/FeMIMO WI, the beambook design is up to companies’ choice, due to the difficulty of alignment, we propose to have the following two-stage approach for gNB/UE beambook alignment.

Proposal 1-5: The following two-stage approach is adopted for gNB/UE beambook design:

- Stage-1: Alignment on the number of beams (as input/output for AI/ML beam prediction)

  - Based on the initial evaluation results, and to be concluded by RAN1 #110.

- Stage-2: Based on the agreed number of beams (as input/output for AI/ML beam prediction), companies provide the simulation results:

  - Companies shall disclose the detailed beambook design: beam directions/beamwidth.

  - Companies shall disclose the detailed beam measurement and report configuration.

1.1.5 Consideration on AI/ML beam prediction in spatial domain

To evalute the beam predication in the spatial domain, the use case should be firstly clarified, which will be discussed separately [2], including the detailed procedure of the prediction, i.e., which node (either a UE or a gNB) to perform prediction, what is the input/ouput of the AI/ML model, how the prediction results will be applied in the beam management, etc. From an EVM perspective, it is observed that no matter the prediction is performed in a UE or gNB side, the fundamental target is to (1) perform measurements on beams in a given subset of the full beambook, and (2) perform the prediction for the beam performance concerning a beam which is not yet contained in the given measured subset of the full beambook.

Proposal 1-6: EVM on AI/ML beam prediction in spatial domain shall at least be able to evaluate the following spatial domain beam prediction:

(1) Measurements are performed on beams in a given subset of full beambook, and

(2) Spatial domain beam prediction is performed for the beam performance concerning a beam which is not yet contained in the given measured subset of full beambook.

1.1.6 Consideration on AI/ML beam prediction in time domain

To evaluate the beam prediction in the time domain, realistic channel conditions should be considered with UE-mobility and spatial characteristics of FR2. To do this, time variation and time-varied scatters should be modeled properly. As mentioned above, in the channel modeling in TR 38.901, spatially-consistent mobility modeling was defined, and our preference and analysis has already be provided in above Section 1.1.1.

To generate a dataset for evaluation for AI/ML beam prediction in the time domain, UE trajectory modelling as mentioned in Section 1.1.3 is of great significance in the system-level evaluation. In addition to that, antenna boresight, moving direction, and speed of UE should also be considered. For example, uniform distributions can be used for antenna boresight and directions with fixed UE speed. In addition, blockage [Section 7.6.4 in TR 38.901] and UT rotating [Section 7.6.7 in TR 38.901] can be considered together to reflect the changes in the beam direction and gain over time.

Proposal 1-7: For evaluation of beam prediction in the time domain, in addition to the spatially-consistent mobility modelling and UE trajectory modelling, the selection of UE speed, blockage [Section 7.6.4 in TR 38.901], and/or UT rotating [Section 7.6.7 in TR 38.901] should be considered.

1.1.7 Consideration on AI/ML beam measurement feedback compression

From an EVM perspective, the beam measurement feedback compression sub-use case can reuse the assumptions for EVM of beam prediction in the spatial domain. One important aspect to consider here is the accuracy of the compressed feedback in terms of selecting the Top-N beams. This has a direct implication in the system’s performance as failing to indicate the beams in their correct order of strength may result in sub-optimal application of beams by the gNB. The second aspect is the accuracy of the RSRP estimates after the beam measurement feedback is decompressed. This aspect combined with the possibility of compressing the measurement feedback for the entire beam-set, as opposed to Top-N beam report, may allow the system to employ predictors in the spatial and/or time domain.

Proposal 1-8: EVM on AI/ML based beam measurement feedback compression shall at least be able to evaluate the system performance while condisdering one or both of the following aspects:

(1) the compressed beam measurement feedback has lower feedback overhead as compared to the legacy feedback for a given number of reported beams

(2) the number of reported beams in the compressed beem measurement feedback is larger than the legacy feedback for the same beam measurement feedback overhead.

1.2 Link-level simulation methodology for AI/ML-based beam management

1.2.1 General evaluation assumption

For the evaluation of AI/ML-based beam management, link-level simulation should also be considered. Especially for the beam prediction in the time domain, the dataset generated by link-level simulation can be used to simulate the time variation properties, based on which the prediction is performed.

Observation 1-2: Link-level simulation can be used for the evaluation of beam prediction in the time domain.

As mentioned in Proposal 1-1, the Table A.1.6.4 (LLS) of TR 38.802 can be used as the starting point, while necessary modifications/simplifications can be further discussed for sub use case of AI/ML-based beam prediction in the time domain, with the changes and highlights provided in the following Table 1-3.

Table 1-3: Proposed general simulation assumptions for AI/ML beam management (for LLS)

| Parameter | Value |
| --- | --- |
| Frequency | 30GHz. |
| Subcarrier spacing | 120kHz |
| Data allocation | 8 RBs First 2 OFDM symbols for PDCCH, and following 12 OFDM symbols for data channel |
| PDCCH decoding | Ideal or Non-ideal (Companies explain how is modeled) |
| Channel model | CDL-D extension Indoor: DS = 20ns Outdoor: DS = 100ns |
| BS antenna configurations | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1). (dV, dH) = (0.5, 0.5) λ. Note: Other antenna configurations are not precluded. |
| BS antenna element radiation pattern | Table A.2.1-4 in TR 38.802 |
| BS/UE beambook design | Candidate total beam number: 32 beams at BS and 4 beams at UE. The DFT beam candidate in beam selection is generated according to the uniform vertical and horizontal angular distribution shown as follows: , for i=1,…,rN, where r (=1) denotes the oversampling factor, N denotes the number of vertical/horizontal antennas. |
| TXRU mapping to antenna elements | Companies explain details of the using TXRU mapping to antenna elements. Notes: 30GHz: 2D DFT based beam per polarization as a baseline; |
| TXRU mapping weights | Companies explain details of the using TXRU mapping weights. |
| Procedure of beam sweeping | Companies explain details of procedure of beam sweeping. |
| Criteria for beam selection | Companies explain details of criteria for beam selection. |
| BS antenna height and antenna array downtile angle | Indoor: 3m, 180° Outdoor: 25m, 110° |
| UE antenna configurations | (M, N, P, Mg, Ng) = (1, 4, 2, 1, 1). (dV, dH) = (0.5, 0.5) λ. UE boresight: Uniform random [0,360] in azimuth |
| UE antenna element radiation pattern | TR 36.873 |
| UE mobility model | Option #1: Fixed trajectory. Indoor: Dmin=10m, Ds=20m. Outdoor: Dmin=100m, Ds=200m. Option #2: Randomly model. random starting location, random initial moving direction, random initial moving speed Note: Other UE mobility models are not precluded. Companies should explain the details of UE mobility model. |
| UE distribution and UE speed | Indoor: Fixed 3km/h as starting point. Outdoor: Fixed 30km/h as starting point. FFS random speed |
| Raw data collection format | Depends on sub-use case and companies’ choice. |

Proposal 1-9: Based on LLS assumption proposed in Table A.1.6-4 of TR 38.802, the following selection/simplification/modification are proposed as the starting point for Rel-18 AI/ML for beam management evaluation:

- Down-selected to only consider 30GHz, 120kHz SCS

- For a channel modoel, simplified to an LoS model with CDL-D extension as a starting point;

  - DS = 20ns for a indoor scenario (for a low UE speed);

  - DS = 100ns for a outdoor scenario (for a medium UE speed);;

  - FFS the impact of NLoS models with CDL-A/B/C extension;

- Simplified BS antenna configuration for BS antenna configuration with only 1 panel used, which is aligned with proposed general evaluation assumptions for SLS in Table 1-1:

  - For 30GHz: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ;

- Simplified UE antenna configuration for UE antenna configuration with only 1 panel used;

  - For 30GHz: (M, N, P, Mg, Ng) = (1, 4, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ;

- UE mobility feature (features including UE mobility, rotation, blockage, etc) is of importance to AI/ML beam prediction, and to be chosen for different sub-use cases;

- Simplified spatially-consistent UE mobility modelling: the CDL extension used in the Rel-17 HST-SFN evaluation as a good starting point;

  - For a low UE speed, fixed 3km/h in a simplified indoor scenario as a starting point;

    - Option #1 (fixed trajectory): Dmin=10m, Ds=20m;

    - FFS Option #2 (randomly model);

  - For a medium UE speed, fixed 30km/h in a simplified outdoor scenario as a starting point;

    - Option #1 (fixed trajectory): Dmin=100m, Ds=200m;

    - FFS Option #2 (randomly model);

  - FFS random UE speed based on company selected distribution.

1.2.2 Consideration on LLS AI/ML beam prediction in time domain

One approach to model the spatial consistency is reusing the evaluation methodologies in the Rel-17 HST-SFN. In the evaluation methodologies in the Rel-17 HST-SFN, the CDL extension was used to model the time-varying Doppler effect with considering a UE trajectory. However, in a CDL channel model from TR 38.901, angles for scatters are fixed to generate for channel coefficient and will not change. This means only UE speed is considered in the CDL model and not possible to use this as mobility when we directly use Table A.1.6-4 of TR 38.802. The other approach to model the spatial consistency is to use a similar methodology for the Rel-17 M-TRP evaluation methodology for the blockage model (i.e., combining a CDL channel with the spatially-consistent mobility modeling).

Proposal 1-10: For the evaluation of beam prediction in the time domain, the CDL extension used in the Rel-17 HST-SFN evaluation should be a good starting point in link-level evaluation.

One issue for reusing the CDL extension used in the Rel-17 HST-SFN evaluation is modelling of a random UE trajectory that is discussed in Section 1.1.3. To do this, if we have random Dmin, random Ds, and random desired mean angles, it will result in a random starting location and a random initial moving direction.

Observation 1-3: For the simplified random trajectory model in the CDL extension, at least random Dmin, random Ds, and random desired mean angles can be considered.

1.3 Other aspects for EVM on AI/ML-based beam management

There are other aspects of EVM to be noted, which is not yet considered or relevant in the existing evaluation for FR2 beam management. For example, when the node is required to performed the prediction, the length of observation time window will plays the important role. Specifically, the tradeoff between longer and shorter observation window length should be considered for the beam prediction in the time domain. To illustrate the potential benefits of different use case, companies may propose to use different lengths of observation windows for different purpose, e.g., longer observation time window shall be employed for time-domain prediction to include more historical measurement samples, while the long observation time window is not of necessity if the prediction is only performed in spatial domain. It is not suggested to restrict the observation time window length selection especially for Phase-I evaluation, but the relevant information should be explained by companies.

Proposal 1-11: The length of observation time window should be selected by companies for the particular sub use case and evaluation purpose for AI/ML-based beam management EVM.

- The relevant information should be explained by companies.

It should be noted that different AI/ML model could be used for various sub use case and evaluation purpose: for example, it is expected that the time-series of measurement results can be used as the input for time-domain beam prediction, for which LSTM (long short-term memory) may be considered as the neural network for this purpose, but it may not be suitable for spatial-domain beam prediction, in which the spatially correlation information should be considered so it is not suitable to use the measurement results as time-series input. Similarly, for loss function or reward function used for AI/ML model training, we found it is hard to have a certain baseline design for companies to follow and calibrate, therefore it is suggested to leave the freedom to companies but the utilized model and loss/reward function design should be reported and explained by companies in their evaluation results.

Proposal 1-12: For AI/ML training and inferring, the detailed AI/ML model, and loss/reward function design should be decided by companies, while the relevant information should be explained by companies:

- AI/ML model selection: e.g., DNN, CNN, LSTM with the number of layers, and the number of nodes in each layer, and some necessary description for the node connection, etc.

- Input of AI/ML model.

- Output of AI/ML model.

- KPI on AI/ML for beam management

As we discussed in our accompanying contribution on the general discussion of KPI [3], the relevant KPI on AI/ML for beam management can be categorized into the following two categories:

- Performance-related KPIs: These are KPIs that are directly related to the performance of an AI/ML based solutions with respect to the use case they are applied for.

- Capability-related KPIs: This falls into a category of KPIs that are related to AI/ML operation but does not directly indicate the performance of AI/ML algorithm. The KPIs in this category may rather indicate the required capability or states the UE may have to performance a given AI/ML operation.

In the following analysis, we will follow the above two categories by investigating each relevant sub use cases.

2.1 Performance-related KPI

2.1.1 Performance-related KPI for beam prediction in time/spatial domain

For beam predictions in both time and spatial domains, it is to perform the prediction for the beam performance concerning a beam which is not yet measured as conducted in the regular non-AI/ML beam management reference algorithm. For example, for spatial domain prediction, it is expected to perform the prediction for a beam which is not yet contained in the given measured subset of the full beambook. For the time domain prediction, on the other hand, it is expected to perform the prediction for a beam’s future performance which can be directly derived from the existing measurement occasion directly.

To evaluate the above targets of beam prediction, at least the following two performance-related KPIs for beam prediction performance can be considered, which is particularly useful for quick performance evaluation:

- Accuracy of predicted best beam within the Top-N from genie-aided beam measurement: The percentage of the predicted best beam within the top-N beams from the genie-aided beam measurement, provided that the genie-aided beam measurement is available based upon simulation.

- Performance gap from the genie-aided beam measurement: For the beam prediction result, the performance gap, i.e., x dB degradation from the genie-aided beam measurement case, can also be used as the metrics for beam prediction performance.

Proposal 2-1: For the performance-related KPI for AI/ML based beam prediction (in both time and spatial domains), the following two KPIs can be utilized, especially for Phase-I:

- Accuracy of predicted best beam within the Top-N from genie-aided beam measurement.

- Performance gap from the genie-aided beam measurement.

Similar as EVM for other cases, user-perceived throughput (UPT) in the system-level simulation can be also considered as performance-related KPI, e.g., average UPT, 5% UPT, 50% UPT, 95% UPT. Theoretically, UPT should be a better KPI to evaluate the performance benefits obtained by AI/ML-based beam measurement.

Proposal 2-2: For Phase-II, user-perceived throughput (UPT) can be utilized as the KPI to evaluate the performance benefits obtained by AI/ML-based beam measurement.

On the other hand, we also observed some practical difficulties for AI/ML-based algorithm evaluation: Usually, AI/ML training/inferring is likely to be conducted in the independent AI/ML platform (e.g., with Tensorflow) which is different from the system-level simulator (e.g., in C/C++ or Matlab). With two independent platforms, it can be easier by employing the “two-step” method:

- Step-1: Run system level simulator to generate training/evaluation dataset;

- Step-2: perform AI/ML training and evaluation based on the data generated in Step-1.

Given this “two-step” method, the beam management accuracy KPIs given in Proposal 2-1 can be evaluated, while UPT is hard to be provided, because the actual beam management decision given in Step-2 will have impact to obtain UPT in the system-level simulator, and additional step should be needed to feedback the beam management decision obtained in Step-2 into the SLS.

Observation 2-1: Provided two independent platforms for system-level simulator and AI/ML platform, by employing the “two-step” method to generate dataset and perform AI/ML training/inferring sequentially,

- the beam management accuracy KPIs given in Proposal 2-1 can be evaluated;

- UPT is difficult to be evaluated, because the beam management decision will impact on SLS for UPT.

One alternative simplified model based on Shannon capacity usually used in academic paper, to avoid the above difficulty for SLS+AI/ML, and by considering the maximum rate achieved from a MCS table in the spec when we draw a CDF of UPT, we at least have the below three options:

- Option 1: Tput=α∙B∙log2(1+SNR),

- Option 2: Tput'=min(Tput, Tput,MCSmax),

- Option 3. Tput''= α∙B∙log21+11SNR+1SNRMCSmax,

where Tput,MCSmax = α∙B∙log21+SNRMCSmax and SNRMCSmax can be obtained from the minimum SNR to achieve maximum rate in a MCS table in the spec.

- Note: Option 3 is a smooth version of Option 2.

Proposal 2-3: Shannon capability based simiplified model for UPT can be employed in the EVM of AI/ML-based beam management.

As described in WID, the overhead is of importance to be considered as one of the performance-related KPI. For the beam prediction in both time and spatial domains, accordingly, the reference signal overhead (SSB or CSI-RS) and reduced latency for the specific beam management procedure should be considered as performance-KPI for beam prediction in both time and spatial domains. In addition, when we evaluate the large latency, performance degradation due to the measurement aging can be considered.

Proposal 2-4: Reference signal overhead (SSB or CSI-RS) and reduced latency for the specific beam management procedure should be considered as performance-KPI for beam prediction in both time and spatial domains.

2.1.2 Performance-related KPI for beam measurement feedback compression

As described in our accompanying discussion paper on AI/ML beam management use case [2], the beam measurement feedback compression is one of the important use cases. By utilizing auto-encoder and decoder to compress the beam measurement results in UE and gNB-side respectively, at least the following performance-related KPI should be considered for this specific use case.

- NMSE (normalized mean squared error) of reconstructed vector for measurement results, which is used to measure the error in reconstructed vector from the ideal measurement vector obtained from the genie-aided measurement results.

- Compression ratio compared to conventional reporting, which is the ratio of the size (in terms of bits) of the compressed feedback and a baseline legacy feedback, while keeping the NMSE of the measurement vector is below a specific threshold.

- Top-N hit-ratio: The probability of Top-N beams derived from the reconstructed feedback are in Top-N beams from genie-aided beam measurement (ground truth).

Proposal 2-5: For the use case of beam measurement feedback compression, at least the following performance-realted KPI shall be considered:

- NMSE (normalized mean squared error) of reconstructed vector for measurement results;

- Compression ratio compared to conventional reporting;

- Top-N hit-ratio.

2.2 AI/ML capability-related KPI

Similar to other use cases for AI/ML based operation, the AI/ML capblity related KPI shall be considered in the EVM for AI/ML based beam management. Although depending on the detailed discussion for use case analysis, AI/ML model may not be needed for transfer between UE and gNB, which alleviate the burden introduced by the large size of AI/ML model. However, the size of AI models is still of importance (especially for AI/ML at UE side), because it is closed linked to the required storage and computation complexity of AI/ML operation.

Furthermore,the generalization is also one of the main aspects to evaluate AI/ML model. In the Phase-II evaluation, the mixed scenarios shall be considered as a test case to verify the generalization of a AI model. It is hard to find quantized metric to evaluate generalization, but at least the performance degradation should be considered if the test case with mixed sceanrios is employed.

For the complexity of training and inference, relevant KPI selection should be considered in the general framework discussion. From our understanding, there is no special consideration for beam management case, so the conclusion in the general framework should be followed.

Proposal 2-6: For the use case of AI/ML based beam management, at least the following capability-realted KPI shall be considered:

- Size of AI/ML model;

- Generalization of AI/ML model by considering the performance degradation for test case with mixed scenarios.

- Complexity of traning and inference of AI/ML operation.

- Preliminary results for AI/ML beam management

3.1 AI/ML beam prediction in spatial domain

For AI/ML-based beam prediction in the spatial domain, the preliminary evaluation results is provided in this section. By employing system-level simulation, L1-RSRP measurement reports are periodically reported to gNB, based on which the time sequence of the best 4 beams’ measurement reports are used as AI/ML model input. Accordingly, the beam prediction is performed by which the beam signal quality of a larger beambook is about to be obtained, as the output of AI/ML model, as illustrated in the below Figure 3-1.

Figure 3-1. Illustration of beam prediction in the spatial domain.

Based on our study, the following AI/ML model with both structural and size information is provided for this particular AI/ML use case for beam prediction in the spatial domain:

- the input data will be input into a 3 layer bi-directional LSTM model firstly, each LSTM layer has 256 cells;

- the output of LSTM model will be input into a full connection (FC) layer, with 512 cells;

- the output of FC layer will go through a residual network (ResNet);

- the output of ResNet will be input into another FC layer, whose cell number is equal to the total beambook size, i.e. the final output length is equal to the total beambook size.

Figure 3-2. Flow chart of AI/ML model design for AI/ML beam prediction in spatial domain

By assuming an evenly distributed beambook design with total 56 beams, in which only 14 beams (evenly selected among the total 56 beams) transmitted with SSB and TRS to be measured and reported for AI/ML based beam prediction, it is shown that AI/ML prediction can achieve the performance comparable to legacy beam management with all 56 beams to be measured, but compared with legacy beam management over 14 beams only, the average RSRP of the selected beam is increased by 4.61dB (from 6.56dB gap reduced to 1.95dB gap), with the same overhead of reference signals and measurement burden.

Table 3-1: Performance of AI/ML beam prediction in spatial domain

|  | Accuracy of predicted best beam within genie Top-N |  |  | Performance gap from the genie-aided beam measurement (Average RSRP difference) | Number of Beams with Configured SSB/TRS Measurement |
| --- | --- | --- | --- | --- | --- |
|  | Top-1 | Top-2 | Top-3 |  |  |
| Legacy BM over 56 beams with SSB/TRS | 85.8% | 96.7% | 98.7% | 0.32dB | 56 |
| Legacy BM over 14 beams with SSB/TRS | 32.9% | 56.3% | 71.1% | 6.56dB | 14 |
| AI/ML Beam Prediction over 14 beams with SSB/TRS | 70.4% | 86.6% | 91.8% | 1.95dB | 14 |

Observation 3-1: AI/ML beam prediction in the spatial domain shows some levels of good prediction accuracy, with much reduced reference signal overhead and alleviated measurement burden.

Proposal 3-1: In this SI, study and evaluate AI/ML beam prediction in the spatial domain as a representive sub-use case.

3.2 AI/ML beam prediction in time domain

Figure 3-3. Illustration of RSRP measurements for beam predictor.

As discussed in the approved SID [1], the benefits of AI/ML beam prediction in the time domain may be the overhead and latency reduction for beam management, which means AI/ML may be enable to predict the desired future beam direction for a moving UE from the histrocial measurement data with reduced or no beam sweeping.

For an preliminary simulation to verify the ability of the future beam prediction, we evaluate the top N beam prediction accuracy over 32 beams at the future time occassion. In figure 3-3, we assume that K narrow beams associated with K periodic CSI-RSs are used for measuring and reporting historical RSRPs (from t1 to t3) where we set K as 1 (i.e., no beam sweeping). Here, t3 refers the latest time for RSRP reporting for the beam prediction and t3 = t1 + 2P or t3 = t2 + P, where P is the periodicity of the CSI-RS for the beam prediction. The DNN-based beam predictor is located at the gNB and it predicts the future beam set at t4 (= t3 + P). The beam predictor consists of 3 hidden layers with 128, 64, and 32 nodes for each layer. We use LLS assumptions in Table 1-3 adopting the outdoor scenario with the fixed trajectory to depict the beam management for a UE on a road for the preliminary simulation. We also assume P = 80ms and N=1 (i.e., top 1 prediction).

As shown in Table 3-2, the evaluation results show that the top 1 beam accuracy is 37.7% for the AI-input only with the legacy RSRP feedback from t1 to t3 where the reported RSRP is quantized with the 1dB gap. It shows the potential ability of AI/ML beam prediction in the time domain without beam sweeping since the accuracy is higher than the random beam selection without beam sweeping (3.1% accuracy). For the AI-input with assistant information on top of the legacy RSRP feedback, it shows that higher accuracy can be achieved depeding on type of information. For example, for the RSRP with the higher qunatization reolution, 46.6% of the top 1 beam accuracy can be achieved while 96.9% accuracy can be achieved with the position information. Here, we use the latest CSI-RS ID for the CSI-RS for beam prediction as the position information, since it can represent the direction of the UE’s desired beam at t3 from the gNB knowledge. In the real-world deployment, the UE’s trajectory will be random (e.g., Option #2 in Table 1-3), so various assistant information (e.g., high resolution RSRP, positioning information, a UE speed, beam information, etc.) would be required to improve the beam prediction accuracy.

Table 3-2: Top N beam accuracy of AI/ML beam prediction in the time domain

| Assistant information | Legacy RSRP feedback (No assistant information) | Higher resolution RSRP | Position information (CSI-RS ID) |
| --- | --- | --- | --- |
| Top N beam accuracy (N=1) | 37.7% | 46.6% | 96.9% |

Observation 3-2: AI/ML beam prediction in the time domain shows some levels of good prediction accuracy.

Observation 3-3: AI/ML beam prediction in the time domain can reduce the CSI-RS overhead for beam sweeping.

Observation 3-4: Assistant information (e.g., high resolution RSRP, positioning information, a UE speed, beam information, etc.) can improves the beam prediction accuracy.

Proposal 3-2: In this SI, study and evaluate AI/ML beam prediction in the time domain as a representive sub-use case.

Proposal 3-3: In this SI, study and evaluate the performance of AI/ML beam prediction in the time domain in terms of the top N beam prediction accuracy and the overhead and latency reduction.

Proposal 3-4: In this SI, study and evaluate the performance impact of various assistant information (e.g., high resolution RSRP, positioning information, a UE speed, beam information, etc.).

3.3 AI/ML beam measurement feedback compression

Beam measurement feedback compression is considered in this section as a sub-use case for AI/ML based beam management. Here, we consider a scenario where a UE measures K beams from the corresponding reference signal resources (SSB or CSI-RS for BM). Then, instead of reporting the Top-N beams, where , it reports a compressed feedback corresponding to all K beams. The RSRP measurements of each measured gNB beam at UE is fed as input to the encoder network as a K-length vector. The output of the encoder is then quantized and reported to the gNB. At the gNB, the received bits are fed to the decoder and used to reconstruct the original measurements in to full length (K-length) RSRP vector. This enhanced reporting can help in spatial and time prediction of beams at gNB using the full length vector.

Figure 3-4. Illustration of compression of RSRP measurement vector feedback.

To evaluate the recovery performance of the proposed beam measurement compression scheme, the NMSE is computed over the TOP-N beams. Let  be the set of indices of Top-N beams and  be the ground-truth RSRP value for  n-th beam, then the NMSE for Top-N beams, denoted as

where is the recovered RSRP value corresponding to the n-th beam.

In this simulation setup, we have set K=24 beams at gNB and 8 beams at UE. At the UE, the best UE Rx beam measurement for each gNB Tx beam is considered for reporting. A 24 length RSRP vector representing all the gNB beams is used as input to the autoencoder. After the quantization, reported bits are used at gNB to decompress the measurement vector of length 24.

Moreover, the compression ratio with respect to reporting the Top-N beams is reported. Here, we assumed 5 bits suffice to report the reference resource index (CRI/SSBRI) from measurments over K-24 beams (resources) and the number of bits for legacy reporting is computed as  . Moreover, as per the legacy beam groups reporting in TS 38.214, a 7-bits RSRP reporting for the strongest beam and 4-bits RSRP reporting for the rest N-1 beams is consider. The the compression ratio is computed as

Table 3-3: NMSE of decompressed Top-K RSRPs at gNB.

| Feedback bits | M=1 | M=2 | M=4 | Compression ratio % Compared to Top-4 reporting |
| --- | --- | --- | --- | --- |
| Without Quantization | 0.0039 | 0.0037 | 0.0034 |  |
| 42 bits | 0.0047 | 0.0044 | 0.0041 | 107% |
| 36 bits | 0.0046 | 0.0044 | 0.0040 | 92% |
| 30 bits | 0.0055 | 0.0051 | 0.0047 | 76% |
| 24 bits | 0.0052 | 0.0049 | 0.0046 | 61% |
| 18 bits | 0.0090 | 0.0082 | 0.0077 | 46% |

Table 3-4: Hit ratio of Top-N actual and Top-N reported beams

| Feedback bits | N = 1 | N=2 | N=4 |
| --- | --- | --- | --- |
| Without Quantization | 89.15 | 83.1 | 70.15 |
| 42 bits | 88.8 | 83.6 | 68.42 |
| 36 bits | 88.4 | 82.85 | 67.67 |
| 30 bits | 88.1 | 83.5 | 67.67 |
| 24 bits | 86.1 | 80.5 | 64.25 |
| 18 bits | 79.8 | 73.95 | 59.62 |

Observation 3-5: Results from AI/ML beam measurement feedback compression shows the possibility of compression and enables full length RSRP vector reporting. Such full length RSRP vectors can further be used by gNB to accurately predict the spatial beam prediction.

- Conclusion

The observations and proposals made in this contribution are summarized below:

Proposal 1-1: Consider Table A.2.5-2 (for SLS) and Table A.1.6.4 (for LLS) of TR 38.802 as the starting point for Rel-18 AI/ML for beam management evaluation, based on which necessary modifications/simplifications can be further discussed for the specific evaluation purpose and/or sub use case.

Proposal 1-2: Based on SLS assumption proposed in Table A.2.5-2 of TR 38.802, the following selection/simplification/modification are proposed as the starting point for Rel-18 AI/ML for beam management evaluation:

- Down-selected to only consider 30GHz, Dense Urban, macro layer Hex. Grid, inter-BS distance = 200m;

- Down-selected to only consider 120kHz SCS;

- Simplified criteria for selection for the serving TRP, i.e., based on the distance to the gNB.

- Simplified BS antenna configuration for BS antenna configuration with only 1 panel used, which is equivalent in beam selection accuracy performance to be evaluated in Phase-I:

  - For 30GHz: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ;

- Beam correspondence: Companies report details of the assumptions:

  - UE capability of beam correspondence without SRS assistance is prioritized

- UE mobility feature (features including UE mobility, rotation, blockage, etc) is of importance to AI/ML beam prediction, and to be chosen for different sub-use cases;

- Spatially-consistent UE mobility modelling: Procedure A in TR 38.901.

Proposal 1-3: Based on SLS assumptions/parameters in Table A.2.1-1 of TR 38.802, the following simplification/modification are proposed as the starting point for the selected Dense Urban scenario:

- Down-selected to only consider 10 dB UE receiver noise figure;

- For UE distribution, simplified to 100% outdoor and only an LoS case only as a starting point.

  - FFS the impact of NLoS based on company selected distribution;

- For UE speed, fixed 30 km/h as a starting point:

  - FFS random UE speed based on company selected distribution.

Observation 1-1: For the trajectory model for UE mobility, at least the following options exist:

- Option #1: Linear and fixed trajectory model, e.g., the intra-cell mobility model in Table 2 of R1-2007151.

- Option #2: Linear trajectory model with random direction change.

- Option #3: Linear trajectory model with random and smooth direction change.

Proposal 1-4: The impact of discontinuous UE trajectory model on the beam prediction performance shall be considered for UE mobility model selection.

Proposal 1-5: The following two-stage approach is adopted for gNB/UE beambook design:

- Stage-1: Alignment on the number of beams (as input/output for AI/ML beam prediction)

  - Based on the initial evaluation results, and to be concluded by RAN1 #110.

- Stage-2: Based on the agreed number of beams (as input/output for AI/ML beam prediction), companies provide the simulation results:

  - Companies shall disclose the detailed beambook design: beam directions/beamwidth.

  - Companies shall disclose the detailed beam measurement and report configuration.

Proposal 1-6: EVM on AI/ML beam prediction in spatial domain shall at least be able to evaluate the following spatial domain beam prediction:

(1) Measurements are performed on beams in a given subset of full beambook, and

(2) Spatial domain beam prediction is performed for the beam performance concerning a beam which is not yet contained in the given measured subset of full beambook.

Proposal 1-7: For evaluation of beam prediction in the time domain, in addition to the spatially-consistent mobility modelling and UE trajectory modelling, the selection of UE speed, blockage [Section 7.6.4 in TR 38.901], and/or UT rotating [Section 7.6.7 in TR 38.901] should be considered.

Proposal 1-8: EVM on AI/ML based beam measurement feedback compression shall at least be able to evaluate the system performance while condisdering one or both of the following aspects

(1) the compressed beam measurement feedback has lower feedback overhead as compared to the legacy feedback for a given number of reported beams

(2) the number of reported beams in the compressed beem measurement feedback is larger than the legacy feedback for the same beam measurement feedback overhead.

Observation 1-2: Link-level simulation can be used for the evaluation of beam prediction in the time domain.

Proposal 1-9: Based on LLS assumption proposed in Table A.1.6-4 of TR 38.802, the following selection/simplification/modification are proposed as the starting point for Rel-18 AI/ML for beam management evaluation:

- Down-selected to only consider 30GHz, 120kHz SCS

- For a channel modoel, simplified to an LoS model with CDL-D extension as a starting point;

  - DS = 20ns for a indoor scenario (for a low UE speed);

  - DS = 100ns for a outdoor scenario (for a medium UE speed);;

  - FFS the impact of NLoS models with CDL-A/B/C extension;

- Simplified BS antenna configuration for BS antenna configuration with only 1 panel used, which is aligned with proposed general evaluation assumptions for SLS in Table 1-1:

  - For 30GHz: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ;

- Simplified UE antenna configuration for UE antenna configuration with only 1 panel used;

  - For 30GHz: (M, N, P, Mg, Ng) = (1, 4, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ;

- UE mobility feature (features including UE mobility, rotation, blockage, etc) is of importance to AI/ML beam prediction, and to be chosen for different sub-use cases;

- Simplified spatially-consistent UE mobility modelling: the CDL extension used in the Rel-17 HST-SFN evaluation as a good starting point;

  - For a low UE speed, fixed 3km/h in a simplified indoor scenario as a starting point;

    - Option #1 (fixed trajectory): Dmin=10m, Ds=20m;

    - FFS Option #2 (randomly model);

  - For a medium UE speed, fixed 30km/h in a simplified outdoor scenario as a starting point;

    - Option #1 (fixed trajectory): Dmin=100m, Ds=200m;

    - FFS Option #2 (randomly model);

  - FFS random UE speed based on company selected distribution.

Proposal 1-10: For the evaluation of beam prediction in the time domain, the CDL extension used in the Rel-17 HST-SFN evaluation should be a good starting point in link-level evaluation.

Observation 1-3: For the simplified random trajectory model in the CDL extension, at least random Dmin, random Ds, and random desired mean angles can be considered.

Proposal 1-11: The length of observation time window should be selected by companies for the particular sub use case and evaluation purpose for AI/ML-based beam management EVM.

- The relevant information should be explained by companies.

Proposal 1-12: For AI/ML training and inferring, the detailed AI/ML model, and loss/reward function design should be decided by companies, while the relevant information should be explained by companies:

- AI/ML model selection: e.g., DNN, CNN, LSTM with the number of layers, and the number of nodes in each layer, and some necessary description for the node connection, etc.

- Input of AI/ML model.

- Output of AI/ML model.

Proposal 2-1: For the performance-related KPI for AI/ML based beam prediction (in both time and spatial domains), the following two KPIs can be utilized, especially for Phase-I:

- Accuracy of predicted best beam within the Top-N from genie-aided beam measurement.

- Performance gap from the genie-aided beam measurement.

Proposal 2-2: For Phase-II, user-perceived throughput (UPT) can be utilized as the KPI to evaluate the performance benefits obtained by AI/ML-based beam measurement.

Observation 2-1: Provided two independent platforms for system-level simulator and AI/ML platform, by employing the “two-step” method to generate dataset and perform AI/ML training/inferring sequentially,

- the beam management accuracy KPIs given in Proposal 2-1 can be evaluated;

- UPT is difficult to be evaluated, because the beam management decision will impact on SLS for UPT.

Proposal 2-3: Shannon capability based simiplified model for UPT can be employed in the EVM of AI/ML-based beam management.

Proposal 2-4: Reference signal overhead (SSB or CSI-RS) and reduced latency for the specific beam management procedure should be considered as performance-KPI for beam prediction in both time and spatial domains.

Proposal 2-5: For the use case of beam measurement feedback compression, at least the following performance-realted KPI shall be considered:

- NMSE (normalized mean squared error) of reconstructed vector for measurement results;

- Compression ratio compared to conventional reporting;

- Top-N hit-ratio.

Proposal 2-6: For the use case of AI/ML based beam management, at least the following capability-realted KPI shall be considered:

- Size of AI/ML model;

- Generalization of AI/ML model by considering the performance degradation for test case with mixed scenarios.

- Complexity of traning and inference of AI/ML operation.

Observation 3-1: AI/ML beam prediction in the spatial domain shows some levels of good prediction accuracy, with much reduced reference signal overhead and alleviated measurement burden.

Proposal 3-1: In this SI, study and evaluate AI/ML beam prediction in the spatial domain as a representive sub-use case.

Observation 3-2: AI/ML beam prediction in the time domain shows some levels of good prediction accuracy.

Observation 3-3: AI/ML beam prediction in the time domain can reduce the CSI-RS overhead for beam sweeping.

Observation 3-4: Assistant information (e.g., high resolution RSRP, positioning information, a UE speed, beam information, etc.) can improves the beam prediction accuracy.

Proposal 3-2: In this SI, study and evaluate AI/ML beam prediction in the time domain as a representive sub-use case.

Proposal 3-3: In this SI, study and evaluate the performance of AI/ML beam prediction in the time domain in terms of the top N beam prediction accuracy and the overhead and latency reduction.

Proposal 3-4: In this SI, study and evaluate the performance impact of various assistant information (e.g., high resolution RSRP, positioning information, a UE speed, beam information, etc.).

Observation 3-5: Results from AI/ML beam measurement feedback compression shows the possibility of compression and enables full length RSRP vector reporting. Such full length RSRP vectors can further be used by gNB to accurately predict the spatial beam prediction.

References

[1] RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm.

[2] R1-2203900, “Representative sub use cases for beam management”, Samsung.

[3] R1-2203896, “General aspects of AI ML framework and evaluation methodogy”, Samsung.

Appendix

Appendix A.1 Simulation assumptions for beam management (SLS) in TR 38.802

The following system level simulation assumptions to evaluate beam management are provided in Table A.2.5-2 in TR 38.802:

Table A.2.5-2: Evaluation assumptions for beam management in TR 38.802

| Parameters | Values |
| --- | --- |
| Scenarios (Carrier Frequency) | Indoor hotspot :4 GHz, 30GHz; Urban macro: 4 GHz, 30GHz; Dense Urban: For 4 GHz: Evaluate macro layer For 30 GHz: Evaluate micro layer Note: other antenna configurations should be considered as well. |
| Mode | DL SU-MIMO/ MU-MIMO |
| Simulation bandwidth | 4GHz: 20MHz (DL+UL) 30GHz: 80MHz.(DL+UL) or 40MHz.(DL+UL) |
| Subcarrier Spacing for data | For 4 GHz: 15kHz For 30 GHz: 120kHz, 60kHz (Other subcarrier spacings can be considered) |
| Channel Model | Following related assumption in TR 38.802 |
| TXRU mapping to antenna elements | Companies explain the details of TXRU mapping to antenna elements. Notes: 30GHz: 2D DFT based beam per polarization as a baseline; 4GHz: 1D DFT per vertical dimension per polarization as baseline; |
| TXRU mapping weights | Companies explain the details of TXRU mapping weights. |
| Criteria for selection for serving TRP | Companies explain the details of criteria for selection for serving TRP. |
| Criteria for beam selection for serving TRP | Companies explain the details of criteria for beam selection for serving TRP. |
| Constraints for the range of selective beams per TRP sector | Companies explain what scheme is used |
| Scheduling algorithm | PF scheduler |
| Link adaptation | Based on CSI-RS. |
| Traffic Model | FTP model 1/3 with packet size 0.1 and 0.5Mbytes (other value is not precluded). Other traffic models including the full buffer are not precluded. |
| BS antenna configurations | For 4GHz: (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) as baseline. (dV, dH) = (0.8, 0.5) λ. For 30GHz: (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2). (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ Note: important to consider also other antenna configurations to maintain flexibility |
| BS antenna element radiation pattern | For 4GHz: According to TR36.873 For 30 GHz: According to TR38.802 |
| UE antenna configurations | For 4GHz: Mg = 1, Ng = 1, P =2, (dV, dH) = (0.5, 0.5) λ, MxNxP<=8 (companies report M,N) For 30GHz: (M, N, P, Mg,Ng) = (2, 4, 2, 1, 2); (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (0, 0) λ. *Θmg,ng=90°; Ω0,1=Ω0,0+180°; Note: important to consider also other antenna configurations to maintain flexibility |
| UE antenna element radiation pattern | For 4GHz: Omni-directional with 5dBi gain For 30GHz: See Table A.2.1-8 in TR 38.802 |
| Inter-panel calibration | Ideal, non-ideal following 38.802 (optional) |
| Beam correspondence | Companies report details of the assumptions |
| Control and RS overhead | Companies report details of the assumptions |
| Control channel decoding | Ideal or Non-ideal (Companies explain how is modeled) |
| UE receiver type | MMSE-IRC as the baseline, other advanced receiver is not precluded |
| BF scheme | Companies explain what scheme is used |
| Transmission scheme | Multi-antenna port transmission schemes Note: Companies explain details of the using transmission scheme. |
| UE mobility feature | Follow Phase 3 calibration i.e. Add-on features including UE mobility, rotation, blockage, etc. can be considered. Note: Companies explain whether or which model is used in simulation evaluation. If used, the configuration details should be explained |
| MCS | Use LTE MCS |
| Metric | Spectral efficiency (evaluated under full buffer) 5%,50% UPT(evaluated under FTP model) Outage Beam management latency Proponents are encouraged to provide additional observations on beam failure rate, SINR and RSRP. |

Appendix A.2 Simulation assumptions for beam management (LLS) in TR 38.802

The following link level simulation assumptions to evaluate beam management are provided in Table A.1.6-4 in TR 38.802:

Table A.1.6-4: Simulation assumptions for beam management in TR 38.802

| Parameters | Values |
| --- | --- |
| Carrier Frequency | 4 GHz, 30 GHz |
| Subcarrier Spacing for data | For 4 GHz: 15kHz For 30 GHz: 120kHz, 60kHz (Other subcarrier spacings can be considered) |
| Data allocation | 8 RBs First 2 OFDM symbols for PDCCH, and following 12 OFDM symbols for data channel |
| PDCCH decoding | Ideal or Non-ideal (Companies explain how is modeled) |
| Channel Model | CDL-A /B/C model - delay spread =100ns - UE speed=3km/h. - The angles of BS, i.e., AoD, ZoD, are uniformly distributed within [-60, 60] degrees in azimuth domain and [90, 135] degrees in zenith domain, and those of UE, i.e., AoA, ZoA, are uniformly distributed within [-180, 180] degrees in azimuth domain and [45, 90] in zenith domain, via applying uniform-distribution desired mean angle in subclause 7.7.5.1 in TR 38.901 [15] accordingly. |
| TXRU mapping to antenna elements | Companies explain details of the using TXRU mapping to antenna elements. Notes: 30GHz: 2D DFT based beam per polarization as a baseline; 4GHz: 1D DFT per vertical dimension per polarization as baseline; |
| TXRU mapping weights | Companies explain details of the using TXRU mapping weights. |
| Procedure of beam sweeping | Companies explain details of procedure of beam sweeping. |
| Criteria for beam selection | Companies explain details of criteria for beam selection. |
| UE reporting | Companies explain details of criteria for UE reporting. |
| BS antenna configurations | For 4 GHz: (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) as baseline. (dV, dH) = (0.8, 0.5)λ. For 30 GHz: (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2). (dV, dH) = (0.5, 0.5)λ. (dg,V, dg,H) = (2.0, 4.0)λ Note: important to consider also other antenna configurations to maintain flexibility |
| BS antenna element radiation pattern | For 4GHz: According to TR36.873 For 30 GHz: According to TR38.802 |
| UE antenna configurations | For 4 GHz: Mg = 1, Ng = 1, P =2, (dV, dH) = (0.5, 0.5)λ, MxNxP<=8 (companies report M,N) For 30 GHz: (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV, dH) = (0.5, 0.5)λ. (dg,V, dg,H) = (0, 0)λ. * Θmg,ng=90°; Ω0,1=Ω0,0+180°; Note: important to consider also other antenna configurations to maintain flexibility |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |
| UE array orientation | ΩUT,a uniformly distributed on [0, 360] degree, ΩUT,b = 0°, ΩUT,g = 0° |
| UE antenna element radiation pattern | For 4 GHz: Omni-directional with 5dBi gain For 30 GHz: See Table A.2.1-8 in TR 38.802 |
| Transmission scheme | Multi-antenna port transmission schemes Note: Companies explain details of the using transmission scheme. |
| MIMO mode | SU-MIMO |
| UE receiver type | MMSE-IRC as baseline; other advanced receiver is not precluded. |
| MCS | LTE MCS |
| Link adaptation | Based on CSI-RS |
| Metrics | BLER w/ beamforming Proponents are encouraged to provide additional observations on SINR and RSRP |