3GPP TSG-RAN WG1 Meeting #109-e	R1-2203903

e-Meeting, May 9th – 20th 2022

Agenda item:	9.3.1

Source:	Samsung

Title:	Deployment scenario and evaluation methodology for NR duplex evolution

Document for:	Discussion and Decision

- Introduction

RAN#94-e endorsed the new Rel-18 study items on “Study on evolution of NR duplex operation” [1]. The objectives for this SI is shown below:

In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, we discuss deployment scenarios and evaluation methodologies for Rel-18 duplex evolution SI.

- Applicable Scenarios

The technical report on “Study on scenarios and requirements for next generation access technologies”, TR38.913 [2], includes 12 deployment scenarios representing different deployment topologies, different frequency ranges, and different commercial demands. Even though each deployment scenario is attractive and reflects a unique commercial use case, RAN1 should focus on a minimal set of basic deployment scenarios to capture fundamental benefits and drawbacks on duplex evolution by considering RAN1 evaluation workload. Among 12 deployments scenarios, the following 3 deployment scenarios, can be considered as a baseline deployment scenarios and the remaining 8 deployments scenarios (e.g., High speed, Extreme long distance coverage in low density areas, Urban coverage for massive connection, Highway Scenarios, Urban Grid for Connected Car, Commercial Air to Ground scenario, Light aircraft scenario, Satellite extension to Terrestrial) can be deprioritized.

- Indoor hotspot (FR/ FR2): The indoor hotspot deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density and consistent user experience indoor [2].

- Dense urban (FR1/FR2): The dense urban microcellular deployment scenario focuses on macro TRxPs with or without micro TRxPs and high user densities and traffic loads in city centres and dense urban areas. The key characteristics of this deployment scenario are high traffic loads, outdoor and outdoor-to-indoor coverage. This scenario will be interference-limited, using macro TRxPs with or without micro TRxPs. A continuous cellular layout and the associated interference shall be assumed [2].

- Urban macro (FR1): The urban macro deployment scenario focuses on large cells and continuous coverage. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas. This scenario will be interference-limited, using macro TRxPs (i.e. radio access points above rooftop level) [2]. FR2 can be excluded for evaluation since the urban macro can be mainly used for FR1 scenario.

Given the 3 baseline deployment scenarios, we need to identify duplex operation scenarios to reflect commercial network deployments.

| Duplex operation scenarios | Figures |
| --- | --- |
| 1) Static TDD deployment (legacy), where all gNBs have a static TDD configuration, i.e., DDDSU slot format. This configuration is widely deployed in the commercial network in national-wide, and global-wide so that it can be used for reference network performance for comparison. |  |
| 2) SBFD-only deployment, where all gNBs have a SBFD capability, i.e., all gNBs can use UL subband in TDD DL slots or DL subband in TDD UL slot. It is worth noting that we don’t consider a new dedicated frequency band for duplex evolution. Instead, it is a reasonable assumption that the duplex evolution techniques are applied in the already deployed TDD band. It means that some of slots have UL subband and DL subband, but other slots are full DL slot or full UL slot as used in the TDD band. In SBFD deployment, we can distinguish two sub-deployments according to UL subband configuration as shown in figure 1. a) Aligned subband deployment, where all gNBs’ UL subband is aligned, i.e., starting frequency position and bandwidth of UL subband/DL subband are same across all gNBs. The main benefit of this scenario is that it can avoid intra-subband CLI interference so that each gNB can schedule UL transmission/DL transmission within the UL subband/DL subband independently, without sharing subband information. However, since some of resources are dedicated to UL subband/DL subband without considering of traffic, it degrade resource efficiency and also block flexible scheduling. b) Non-aligned subband deployment, where different gNBs have different UL subband configurations, i.e., starting frequency position and bandwidth of UL subband/DL subband can be different according gNBs’ DL/UL traffics or channel environment. In this deployment, intra-subband CLI interference should be taken into account. | a) Alinged subband b) Non-aligned subband |
| 3) Dynamic/flexible TDD-only deployment, where all gNBs have a dynamic/flexible TDD capability. Note that in order to enable dynamic/flexible TDD, gNB should configure flexible symbols and its direction can be indicated by group-common PDCCH carrying DCI format 2_0 or UE-specific PDCCH scheduling PDSCH (DCI format 1_0/1_1/1_2) or PUSCH (0_0/0_1/0_2). The DL/UL ratio is dynamically changed across TDD configuration periodicity. |  |
| 4) Coexistence deployment of static TDD and SBFD or dynamic/flexible TDD, where some of gNBs are static TDD while other gNBs follow a new duplex operation such as SBFD or dynamic flexible TDD. Since gNB supporting a new duplex operation will be deployed in a band where TDD system has been already deployed, it is important to evaluate coexistence deployment scenarios. For evaluation, we can consider the following two sub deployment scenarios. a) Single-layer deployment, where some gNBs in a single layer are evolved to support a new operation, but other gNBs only support static TDD operation. This 1-layer can be evaluated in 1-layer Dense Urban or Urban Macro. b) Two-layer deployment, which is only defined in 2-layer Dense Urban scenario. In this deployment, gNBs in the first layer can use a new operation and gNBs in the second layer can use static TDD operation. The first layer (or the second layer) can be Macro layer and the second layer (or the first layer) can be Micro layer or indoor hotspot layer. | a) Single-layer deployment b) Two-layer deployment |
| 5) Coexistence deployment of SBFD and dynamic/flexible TDD deployment, where all gNBs in a network support both new duplex operations, SBFD and dynamic/flexible TDD operation. In general, a network operator would adopt one duplex operation and probably there is no deployment scenario to deploy both duplex operation at least for 1-layer deployment. For 2-layer deployment, we may consider a gNB in Macro layer supports SBFD operation and a gNB is Micro layer supports dynamic/flexible TDD operation. It is because dynamic/flexible TDD operation is suitable to adjust DL/UL ratio flexibly without any impacts on gNB’s RF circuit in accordance with DL/UL traffic and SBFD operation can be used to enhance UL coverage in large macro cells. |  |

Given 3 deployment scenarios and 5 duplex operation scenarios, we summarize the considered applicable scenarios in Table 1. In the table, for each frequency range, up to 15 applicable scenarios (if 1-layer dense urban and 2-layer dense urban are considered, the number of applicable scenarios is 20) should be taken into account so that to total number of applicable scenario is too large to be evaluated within the Rel-18 SI. Therefore, we need to prioritize some applicable scenarios based on deployment scenarios and duplex operation scenarios. In Table 1, the initial assessments for all deployment scenarios and duplex operation scenarios are shown. Here, ‘High’ means the high priority, ‘Medium’ means the medium priority, ‘Low’ means the low priority, and ‘not necessary’ means there is no use case to evaluate the deployment scenario and the duplex operation scenario.

Table 1. Candidate Applicable Scenarios

| Deployment scenario Duplex operation scenario | Indoor Hotspot | Dense Urban | Urban Macro |
| --- | --- | --- | --- |
| Scenario 1. Static TDD deployment (legacy) | Baseline performance | 1-layer: Baseline performance 2-layer: not necessary | Baseline performance |
| Scenario 2. SBFD-only deployment | High | 1-layer: High 2-layer: not necessary | High |
| Scenario 3. Dynamic/flexible TDD-only deployment | High | 1-layer: High 2-layer: not necessary | High |
| Scenario 4. Coexistence deployment of static TDD and SBFD or dynamic/flexible TDD | not necessary | 1-layer: Medium Sub-case 1) 1 gNB or 7 gNBs have capability of SBFD or dynamic/flexible TDD 2-layer: Medium Sub-case 1) Macro: static TDD, Micro/indoor hotspot: SBFD or dynamic/flexible TDD Sub-case 2) Macro: SBFD, Micro: static TDD | not necessary |
| Scenario 5. Coexistence deployment of SBFD and dynamic/flexible TDD deployment | not necessary | 1-layer: not necessary 2-layer: Low Sub-case 1) Macro: SBFD, Micro/indoor hotspot: dynamic/flexible TDD | not necessary |

In order to evaluate a new duplex operation, we suggest to evaluate SBFD-only deployment and Dynamic/flexible TDD-only deployment as a high priority, where any impacts on coexistence of legacy static TDD are not considered. As a medium priority, the 1-layer dense urban or 2-layer dense urban scenarios can be evaluated by considering coexistence of legacy static TDD systems. For 1-layer dense urban, 1 gNB or 7 gNBs with a new duplex operation are surrounded by gNBs with static TDD operation. For 2-layer dense urban, we can deploy that 1) Macro gNBs supports static TDD operation and Micro/indoor hotspot gNB supports SBFD or dynamic/flexible TDD or 2) gNBs supports SBFD operation and Micro/indoor hotspot gNB supports static TDD operation. As a low priority, we can evaluate coexistence deployment of two duplex operations. For 1-layer, we think a network operator does deploy two duplex operations in the neighbour cell sites. For 2-layer dense urban, we can deploy Macro gNBs supports SBFD operation and Micro/indoor hotspot gNB support dynamic/flexible TDD.

Note that in coexistence deployments, dynamic/flexible TDD operation is not deployed in Macro layer, since it is much suitable for micro/indoor hotspot layer where DL/UL traffic changes more dynamically. Also, for SBFD operation, we need to prioritize the aligned subband deployment since it is reasonable that subband related information cannot be exchanged across gNBs with SBFD capability.

Base on the discussion, the followings are proposed.

For evaluation purpose, the following three categories of applicable scenarios can be evaluated with their priorities.

    - The non-coexistence applicable scenario with highest priority

    - The coexistence applicable scenario of static TDD and a new duplex operation with medium priority

    - The coexistence applicable scenario of two new duplex operations with low priority

For non-coexistence applicable scenario, RAN1 takes the following applicable scenarios as a baseline

    - Indoor hotspot (FR1/FR2) with SBFD-only deployment and dynamic/flexible TDD-only deployment

    - 1-layer Dense Urban (FR1/FR2) with SBFD-only deployment and dynamic/flexible TDD deployment

    - Urban Macro (FR1) with SBFD-only deployment and dynamic/flexible TDD-only deployment

    - Note: down-selection/prioritization of non-coexistence applicable scenarios can be done in Rel-18

For the coexistence applicable scenario of static TDD and a new duplex operation, RAN1 takes the following applicable scenarios as a baseline

    - 1-layer/2-layer Dense Urban (FR1/FR2) with coexistence deployment of static TDD and SBFD or dynamic/flexible TDD.

      - For 1-layer, 1 gNB or 7 gNBs with capability of SBFD or dynamic/flexible TDD are surrounded by gNBs with static TDD

      - For 2-layer, Macro layer is for static TDD and Micro/indoor hotspot layer is for SBFD or dynamic/flexible TDD or Macro layer is for SBFD and Micro/indoor hotspot layer is for static TDD

    - Note: down-selection/prioritization of the coexistence applicable scenario of static TDD and a new duplex operation can be done in Rel-18

For the coexistence applicable scenario of two new duplex operations, RAN1 takes the following applicable scenarios as a baseline

    - 2-layer Dense Urban (FR1/FR2) with coexistence deployment of SBFD and dynamic/flexible TDD deployment

    - For 2-layer, Macro layer is for SBFD and Micro/indoor hotspot layer is for dynamic/flexible TDD

    - Note: down-selection/prioritization of the coexistence applicable scenario of two new duplex operations can be done in Rel-18

For evaluation purpose, RAN1 to prioritize the aligned subband configuration for SBFD operation.

Adjacent channel deployment

In the SID objective, it is stated that RAN1 studies the impact on legacy operation assuming their co-existence in co-channel and adjacent channels. So, to evaluate the potential impact by adjacent channels, we need to define adjacent channel deployment.

The adjacent channel can be occupied and operated by the same operator or by the different operator. For the same operator, the position of gNBs operated in the adjacent band are exactly same as the gNBs using a new duplex operation so that we can reuse the deployment scenario, i.e., the identical gNB position. For the different operator, the positions of gNBs of different operators may be different so that the deployment scenarios should be defined. TR38.828 defines grid shift to evaluate different operator’s adjacent channel interference. We can reuse this deployments, which are shown in Figure 2.

Figure 2. Multi-operator deployment

As shown in Figure 2, the second operator’s gNBs (operated in adjacent channel, red) can be located at the edge of the first operator’s gNBs, i.e., 100% grid shift. It is the most optimistic scenarios, i.e., the lowest adjacent channel interference may be observed. The two operators’ gNBs are located at the same location, i.e., 0% grid shift, which is the same as the single operator case. Also, we can take a middle between 0% grid shift and 100% grid shift.

To evaluate adjacent channel interference in Macro layer, it is sufficient to consider two scenarios, 0% grid shift and 100% grid shift. Note that the worst-case scenario in Figure 2 is only applicable to Macro layer (Urban macro deployment, or 1-layer Dense urban) and not applicable to indoor hotspot.

For indoor scenarios, RAN1 needs to discuss how to shift grid since the room size is limited.

The adjacent channel can use different duplex operations, for example, static TDD, SBFD, or dynamic/flexible TDD. It is too hard to consider all duplex operations for adjacent channel impact so that we propose to consider static TDD operation only. It is worth noting that UL subband for SBFD operation is configured in the middle of cell bandwidth, its adjacent channel interference is similar as that of the static TDD.

To evaluate impacts on adjacent channel, consider the followings as a baseline

- For Macro layer (in Urban macro or Dense urban),

  - Two operators’ gNBs are located at the same (0% grid shift)

  - The second operator’s gNBs are located at edge of the first operator’s gNB (100% grid shift)

- FFS: indoor hotspot deployment

- Adjacent channel uses static TDD operation.

- Evaluation Methodology

3.1 Performance metric

Careful selection of performance metric is important to draw a correct conclusion and recommendation for duplex evolution SI. Multiple performance metrics can be adopted, but, at the same time, it might increase evaluation workload. So, we provide some observations on the performance metrics widely used in RAN1 system level evaluation.

DL/UL UPT CDF and (5%-tile, 50%-tile, 95%-tile UPT):

DL/UL UPT (in unit of bps) of a UE is defined as total size of transmitted packets divided by total packet transmission time. By dropping multiple UEs and iterating multiple snapshots, we can obtain DL/UL UPT CDF. Since DL/UL UPT CDF is a statistical distribution of all UEs in all possible geometrical realizations, it would be proper performance metric to measure system impacts. As per DL/UL UPT CDF, RAN1 could conclude overall network performance of static TDD, SBFD and dynamic/flexible TDD operations. Note that 5%-tile UPT is called user experience date rate as defined in TR37.910.

Latency CDF and (5%-tile, 50%-tile, 95%-tile UPT):

The latency (in unit of sec) can be defined as a packet transmission time which includes scheduling time and re-transmission time with a given traffic model. One of key performance target of a new duplex operation is to reduce latency resulted by long DL symbol period within a TDD UL/DL periodicity. Note that User-plane/Control-plane latency of static TDD and FDD is measured by analytic analysis in TR37.910 and the same approach can be reused. However, it does not take into account interference impacts (higher interference, higher transmission error probability), we suggest to use system-level-simulation based latency observation.

Received SINR (at gNB, at UE)

RAN1 may need to align the evaluated results across different companies. For this objective, received SINR is widely used. Also, it is a good performance metric to check potential benefits on CLI handling schemes in each geometry. However, it is just an intermediate performance a specific link and the final conclusion and recommendation should be drawn from the system-wise performance metric such as UPT or Latency.

Peak spectral efficiency, Peak data rate

Both performance metrics are used in TR37.910 where peak spectral efficiency and peak data rate are derived by equations, not by simulation. These performance metrics are not suitable to a new duplex evolution since the new duplex evolution is not targeted to increase peak spectral efficiency or peak data rate. Also, it might cause misleading because it does not take into account traffic model in a cell and interference, which are the key component in the evaluation.

Coverage (worst case required SINR)

In Rel-17 Coverage Enhancement WI, RAN1 defined coverage performance metric as the required received SINR of the worst case channel. It is beneficial to identify which channel is bottleneck in terms of the received SINR. To identify the worst-case channel, channels need to be separately simulated, which means that evaluation workload is inevitably high. Therefore, RAN1 selects this coverage performance metric for duplex evaluation of channels already identified as worst-case channels in static TDD scenarios in Rel-17.

URLLC (with reliability and/or latency requirements)

In Rel-16 URLLC WI, the percentage of users satisfying reliability and latency requirements is used for a key performance metric. URLLC-specific deployments are evaluated. Since it is for URLLC scenarios and the new duplex operation can be applied for eMBB scenarios, this is not a proper performance metric.

Based on the discussion, we propose the following.

For evaluation, RAN1 takes DL/UL UPT CDF and Latency CDF as a performance metric to draw a conclusion and recommendation in TR.

    - DL/UL received SINR can be used for calibration

    - FFS: other performance metrics

3.2 Evaluation Assumptions for SBFD and dynamic/flexible TDD

Deployments related evaluation parameters

As we described above, we focus on Indoor hotspot (FR1/FR2), Dense Urban (FR1/FR2), Urban Macro (FR1). The details for each deployment scenario related parameters (cell layout, gNB-to-UE/gNB-to-gBN/UE-to-UE channel model, gNB’s transmission power, antenna configuration for each scenario, etc) are shown in Tables 2-4, which considered as starting point for duplex evolution.

Table 2. Indoor Hotspot

| Cell layout | Room size (W x L x H) = 120m x 50m x 3m (ISD = 20m), 12 gNBs |
| --- | --- |
| Pathloss model | InH : LoS and NLoS (TR38.901) |
| Channel model | gNB-to-UE : InH office (TR38.901) gNB-to-gNB : InH office (TR38.901) UE-to-UE : A.2.1.2 in TR36.843 |
| Min. UE – UE distance | 1~3m as in TR38.828 |
| BS Tx power | FR1 : 24 dBm FR2 : 23 dBm *DL power control scheme is not applied |
| UE Tx power | Maximum 23 dBm *UL power control scheme is applied |
| BS antenna configuration (M,N,P,Mg,Ng) | See Table 6 |
| BS antenna height | 3 m |
| BS receiver and noise figure | MMSE-IRC and 5 dB MMSE-IRC and 10 dB |
| UE antenna configuration (M,N,P,Mg,Ng) | FR1 : (1, 1, 1, 1, 2) FR2 : (2, 2, 1, 1, 2) |
| UE antenna height | 1.5 m |
| UE receiver and noise figure | FR1 : MMSE-IRC and 9 dB FR2 : MMSE-IRC and 10 dB |

Table 3. Dense urban

| Cell layout | 1-layer deployment : Hexagonal grid, 19 sites, 3 sectors per site (ISD = 200m) 2-layer deployment : - Macro – Micro Macro layer : Hexagonal grid, 19 sites, 3 sectors per site (ISD = 200m) Micro layer: random drop in a site (3 drops) - Macro – Indoor hotspot Macro layer : Hexagonal grid, 19 sites, 3 sectors per site (ISD = 200m) Indoor hotspot: random drop in a site (1 drop), Indoor hotspot deployment is used |
| --- | --- |
| Pathloss model | UMi : LoS and NLoS (TR 38.901) |
| Channel model | gNB-to-UE : UMi (TR38.901) gNB-to-gNB : UMi (TR38.901) UE-to-UE : A.2.1.2 in TR36.843 |
| Min. UE - UE distance | 3m |
| BS Tx power | For 4 GHz : 44 dBm For 30 GHz : 40 dBm *DL power control scheme is not applied |
| UE Tx power | Maximum 23 dBm *UL power control scheme is applied |
| BS antenna configuration (M,N,P,Mg,Ng) | See Table 6 |
| BS antenna height | 10 m |
| BS receiver and noise figure | For 4 GHz : MMSE-IRC and 5 dB For 30 GHz : MMSE-IRC and 7 dB |
| UE antenna configuration (M,N,P,Mg,Ng) | FR1 : (1, 1, 1, 1, 2) FR2 : (2, 2, 1, 1, 2) |
| UE antenna height | according to UE distribution |
| UE receiver and noise figure | For 4 GHz : MMSE-IRC and 7 dB For 30 GHz : MMSE-IRC and 10 dB |

Table 4. Urban Macro

| Cell layout | Hexagonal grid, 19 sites, 3 sectors per site (ISD = 500m) |
| --- | --- |
| Pathloss model | UMa : LoS and NLoS (TR 38.901) |
| Channel model | gNB-to-UE : UMa (TR 38.901) gNB-to-gNB : UMa(TR 38.901) UE -to-UE : A.2.1.2 in TR36.843 |
| Cell layout | Hexagonal grid, 19 micro sites, 3 sectors per site (ISD = 500m) |
| Min. BS - UE distance (2D) | 10m |
| Min. UE – UE distance | 3m |
| BS Tx power | 49 dBm *DL power control scheme is not applied |
| UE Tx power | Maximum 23 dBm *UL power control scheme is applied |
| BS antenna configuration (M,N,P,Mg,Ng) | See Table 6 |
| BS antenna height | 25 m |
| BS receiver and noise figure | MMSE-IRC and 5 dB |
| UE antenna configuration (M,N,P,Mg,Ng) | FR1 : (1, 1, 1, 1, 2) FR2 : (2, 2, 1, 1, 2) |
| UE antenna height | according to UE distribution |
| UE receiver and noise figure | MMSE-IRC and 7 dB |

For evaluation purpose, RAN1 takes the deployment related parameters in Tables 2-4 as a starting point.

- Antenna configuration

When a gNB adopts SBFD operation, the antenna structure may be changed. In Table 5, potential antenna structures for SBFD operation are listed. Here, we identify two cases.

Case 1. Shared antenna structure. Each antenna element is used for both DL transmission and UL reception. All antenna elements are used for DL transmission when gNB transmits a physical channel or signal toward a UE and also all antenna elements are used for UL reception when gNB receives a physical channel or signal from a UE. To transmit and receive at the same time, some of antenna elements are used for DL transmission and other antenna elements are used for UL reception. Since the number of antenna elements during SBFD operation is decreased so that antenna configuration for SBFD may be different from that for static TDD or dyanmic TDD operation.

Case 2. Dedicated antenna strucure. Each antenna element is used for either of DL transmission or UL reception. Some antenna elements are used for DL transmission only when gNB transmits a physical channel or signal toward a UE and the remaining antenna elements are used for UL reception only when gNB receives a physical channel or signal from a UE. Since the number of antenna elements during SBFD operation is kept so that antenna configuration for SBFD is same as that for static TDD or dyanmic TDD operation.

Table 5. Antenna structures

|  | Antenna structures | RX antenna elements for DL transmission (blue) | TX antenna elements for UL reception (red) | TX/RX antenna elements for simultaneous DL transmission and UL reception |
| --- | --- | --- | --- | --- |
| Case 1. Shared antenna structure |  |  |  |  |
| Case 2. Dedicated antenna structure |  |  |  |  |

For evaluation of SBFD operation, RAN1 takes the following two cases for BS antenna configuration.

    - Case 1. Half number of antenna elements are used to simultaneously perform DL transmission and UL reception

    - Case 2. The same number of antenna elements are used to simultaneously perform DL transmission and UL reception

Table 6. BS antenna configuration

| Deployment scenarios | BS antenna configuration (M,N,P,Mg,Ng) |
| --- | --- |
| Indoor hotspot | FR1, - static TDD : (4,4,2,1,1) - SBFD : Case 1: for SBFD slots, (2,4,2,1,1), for non-SBFD slots, same as static TDD Case 2: for all slots, (4,4,2,1,1) - dynamic/flexible TDD: same as static TDD FR2, - TDD : (4,8,2,1,1) - SBFD : Case 1: for SBFD slots, (4,4,2,1,1), for non-SBFD slots, same as static TDD Case 2: for all slots, (4,8,2,1,1) - dynamic/flexible TDD: same as static TDD |
| Dense Urban/Urban Macro | FR1, - static TDD : (8,8,2,1,1) - SBFD : Case 1: for SBFD slots, (4,8,2,1,1), for non-SBFD slots, same as static TDD Case 2: for all slots, (8,8,2,1,1) - dynamic/flexible TDD: same as static TDD FR2, - TDD : (8,16,2,1,1) - SBFD : Case 1: for SBFD slots, (8,8,2,1,1) for non-SBFD slots, same as static TDD Case 2: for all slots, (8,16,2,1,1) - dynamic/flexible TDD: same as static TDD |

Note1: (dH,dV)=(0.5,0.8)λ for FR1, (dH,dV)=(0.5,0.5)λ for FR2

Note 2: SBFD slot is a slot containing DL subband and UL subband. Non-SBFD slot is a slot with one direction DL or UL

For evaluation, RAN1 takes the parameters for BS antenna configurations shown in Table 6 as a starting point.

Key Evaluation parameters for duplex evolution

- System parameters and Frame structure

For three duplex operations, carrier frequency is assumed with 4GHz for FR1 and 30GHz for FR2. FR1 band has 100MHz bandwidth and 30kHz subcarrier spacing (273 RBs) and FR2 band has 200MHz bandwidth and 120kHz subcarrier spacing (132RB).

For three duplex operations, the following frame structures can be considered as shown in Figure 3.

- Static TDD configuration is DDDSU where S slot has 10 DL symbols, 2 Flexible symbols, and 2 UL symbols. For evaluation purpose, the flexible symbols is dedicated to use RX-to-TX switching so that the flexible symbols is not used for DL transmission and UL reception.

- For SBFD operation, the frame structure is DXXXU, where X slot containing DL subband and UL subband. Note that the UL subband is centered at cell bandwidth to suppress adjacent channel interference. Note that the first D slot is used to schedule wideband (e.g., full bandwidth) DL transmission or for legacy UE not supporting SBFD operation. Given the cell bandwidth, the bandwidth of UL subband is [20]% of the cell bandwidth. For example, the cell bandwidth is 100MHz at FR1, [20]MHz UL subband is centered at 100MHz cell and the cell bandwidth is 200MHz at FR2, [40]MHz UL subband is centered at 200MHz cell. Note that the exact number (including maximum and minimum) should be determined based on RAN4’s RF requirements. In addition, guard band can be inserted between UL subband and DL subband.

- For Dynamic TDD configuration, the frame structure is DFFFU, where F slot contains flexible symbols only and their direction can be indicated by Dynamic SFI (in DCI format 2_0) and DL/UL scheduling DCI. For evaluation purpose, it is assumed that 2 OFDM symbols are used for RX-to-TX switching so that 2 OFDM symbols between DL transmission and UL reception should be reserved, i.e., not used for DL transmission and UL reception.

Figure 3. Frame structures for static TDD, SBFD, and Dynamic/flexible TDD operation

- Traffic model

For evaluation, FTP model 3 can be used, where size of a packet is assumed to be 0.1 Mbyte or 0.5 Mbyte. For DL/UL packet arrival rates (λDL,λUL), the following two traffic models can be used.

- Fixed DL/UL packet arrival rates (λDL,λUL), where ratio of λDL and λUL is 4:1, 2:1, or 1:1.

- DL/UL packet arrival rates (λDL,λUL) to meet a given target resource utilization, where the target resource utilization is 20% (light-loaded), 50% (medium-loaded), and 80% (heavy-loaded)

Table 7. Key evaluation parameters for duplex evolution

| Duplex | Static TDD, SBFD, Dynamic/flexible TDD |
| --- | --- |
| Frequency range and numerology | FR1 : 4 GHz with 30kHz SCS FR2 : 30 GHz with 120kHz SCS |
| System bandwidth and # of RBs | FR1 : 100 MHz (273 RBs) FR2 : 200 MHz (132 RBs) *DL BWP and UL BWP include all RBs in a cell |
| Frame structure | Static TDD : DDDSU, where S = 10 DL: 2 Gap: 2 UL. SBFD: DXXXU * X slot contains BWUL MHz for UL at the center and remaining bandwidth for DL, where BWUL = [20] MHz for FR1 and [40] MHz for FR2. * Guard band can be further considered to suppress inter-subband interference Dynamic/flexible TDD : DFFFU * F slot can be used for DL or UL. 2-symbol Gap is assumed |
| Traffic model | FTP model 3 with packet size - DL : 0.5, 0.1 Mbytes - UL : 0.5, 0.1 Mbytes Packet arrival rate - λDL:λUL = 4:1 or 2:1 or 1:1 - Adjusted to meet a given resource utilization |
| Resource Utilization | 20% (light loaded), 50% (medium loaded), 80% (heavy loaded) |

RAN1 takes the parameters for duplex evolution in Table 7.

Interference Modelling for SBFD operation

For UL case, the victim gNB’s UL signal reception may suffer from aggressor gNBs’ DL signal and aggressor UE’s UL signal. The interference measured at subcarrier k (note that the subcarrier k is associated in UL subband at the victim gNB) is expressed as

IULk=Iinter-gNB/inter-subbandk +IUE-to-gNB/intra-subbandk+Iselfk,

where Iinter-gNB/inter-subbandk denotes the received power of gNB-to-gNB (DL-to-UL) interference at subcarrier k from all aggressor gNBs (①), IUE-to-gNB/intra-subbandk denotes the received power of UE-to-gNB (UL-to-UL) interference at subcarrier k from all aggressor UEs (②), and Iselfk denotes the residual self-interference power after self-interference cancelation at subcarrier k caused from the victim gNB’s DL transmission (③). Note that IUE-to-gNB/intra-subbandk follows the legacy interference model as used for dynamic/flexible TDD operation.

Table 8. Interference in UL case of SBFD operation (Aligned UL subband configuration)

| UL case (at victim gNB) | ①gNB-to-gNB interference (DL-to-UL) | ②UE-to-gNB interference (UL-to-UL) | ③Self-interference |
| --- | --- | --- | --- |
| Intra-subband interference | No interference | Legacy interference model (no RAN4 feedback is required) | RAN4 feedback is required for the residual interference level |
| Inter-subband interference | RAN4 feedback is required for the suppression level at gNB, f | No interference |  |

①Iinter-gNB/inter-subbandk

The Iinter-gNB/inter-subbandk can be modelled as the sum of interference power from all interference DL subbcariers, i.e., Iinter-gNB/inter-subbandk=j∈DL subcarriersIinter-gNB/inter-subbandj→k. By assuming frequency response of a filter to reduce the interference power Iinter-gNB/inter-subbandj→k at the victim gNB side, the interference Iinter-gNB/inter-subbandj→k caused from DL subcarrier j and measured at UL subcarrier k is can be modelled as

Iinter-gNB/inter-subbandj→k=all aggressor gNBsPCLI,gNBj→k

where PCLI,gNBj→k=DL Transmission power at subcarrier j of aggressor gNB - effective channel gain between the aggressor gNB and victim gNB - fj→k.

The effective channel gain consist of large-scaling channel gain (e.g., pathloss and shadowing factor) and small-scale channel gain (e.g., wireless channel and transmit beamformer selected at the aggressor gNB and receive beamformer selected at the victim gNB). The value of fj→k is a representative value for suppression level after filtering the inter-subband interference caused by subcarrier j and measured at subcarrier k, and its value should be reported by RAN4.

Figure 4. ①gNB-to-gNB interference (DL-to-UL)

The DL transmission power at subcarrier j may be varied according to DL signal or channel. For example, if gNB would transmit SS/PBCH at subcarrier j, the PSS EPRE can be larger than the SSS EPRE by 3 dB. Also, the ratio of PDCCH DMRS EPRE to SSS EPRE is within -8 dB and 8 dB. Such a different DL transmission power control is hard to be implemented in SLS, so we can simply assume the same DL transmission power at subcarrier j if any DL transmission is scheduled on subcarrier j.

To simplify the evaluation, we can adopt much simpler model. In the previous interference model, it is assumed that all DL subcarriers in DL subband interfere all UL subcarrier in UL subband. However, if gNB uses a filter with sharp roll-off factor, some of DL subcarriers far from the UL subband does not cause substantial inter-subband interference so that we can only take into account DL subcarriers near the UL subband. Also, For the representative value for suppression level, fj→k, can be defined as one-level value or two-level values.

For one-level value, the representative value for suppression level is independent to the index of interfering DL subcarrier index j and the UL subcarrier index k, i.e., fj→k=f. The one-level value f can be defined as the ratio of the filtered mean power centred on the UL subband to the filtered mean power centred on frequency in DL subband. Note that its definition is similar as ACLR definition.

For two-level values, according to the distance between two subcarriers j and k, two values are defined for suppression level, i.e., fj→k=f1 if |j-k| is less than a threshold, fj→k=f2, otherwise. Here, f1<f2. This two-level values model can capture the impact of distance between the interfering DL subcarrier and the UL subcarrier. The two values can be defined as the ratio of the filtered mean power centred on the UL subband to the filtered mean power centred on the first frequency and the second frequency in DL subband, where the first frequency is closer to the UL subband, than the second frequency. Note that such two-level ACLR values are used to evaluate adjacent channel interference in TR36.942.

To model the received power of gNB-to-gNB (DL-to-UL) interference at subcarrier k from all aggressor gNBs (Iinter-gNB/inter-subbandk), RAN1 takes the following model as a starting point.

Iinter-gNB/inter-subbandk=all aggressor gNBsj∈DL subcarriersPCLI,gNBj→k

- where PCLI,gNBj→k=DL Transmission power at subcarrier j of aggressor gNB - effective channel gain between the aggressor gNB and victim gNB - fj→k.

    - The suppression value fj→k should be asked to RAN4

    - RAN1 further to study the simplified model for the suppression level fj→k and its feasibility should be asked to RAN4

③ Iselfk

In order to suppress the self-interference at the victim gNB, the gNB might use technical components such as physical antenna isolation between TX antenna elements and RX antenna elements, RF-domain self-interference cancelation unit, and digital domain self-interference cancelation method. The design target of the self-interference cancelation units is to make the residual interference level comparable to noise level under all possible transmission and reception conditions. At this end, the worst-case residual self-interference level can be a representative value for Iselfk, which is independent to transmission power, transmit beamforming, receive beamforming, or downlink scheduling, etc. So, the representative value for Iselfk is considered as additional noise level in addition to the original noise level. The (worst-case) residual self-interference value should be asked to RAN4.

To model the residual self-interference power after self-interference cancelation at subcarrier k (Iselfk), the representative value for Iselfk is considered as additional noise level in addition to the original noise level.

    - The (worst-case) residual self-interference value should be asked to RAN4

For DL case, the victim UE’s DL signal reception may suffer from aggressor gNBs’ DL signal and non-intended UE’s UL signal. The interference measured at DL subcarrier k (note that the subcarrier k is associated in DL subband at the victim gNB) is expressed as

IDLk=Iinter-UE/inter-subbandk +IgNB-to-UE/intra-subbandk,

where Iinter-UE/inter-subbandk denotes the received power of UE-to-UE (UL-to-DL) interference at subcarrier k from all aggressor UEs (④), and IgNB-to-UE/intra-subbandk denotes the received power of gNB-to-UE (DL-to-DL) interference at subcarrier k from all aggressor gNBs (⑤). Note that IgNB-to-UE/intra-subbandk follows the legacy interference model as used for dynamic/flexible TDD operation.

Table 9. Interference in DL case of SBFD operation (Aligned UL subband configuration)

| DL case (at victim UE) | ④UE-to-UE interference (UL-to-DL) | ⑤gNB-to-UE interference (DL-to-DL) | Self-interference |
| --- | --- | --- | --- |
| Intra-subband interference | No interference | Legacy interference modelling (no RAN4 feedback is required) | No interference |
| Inter-subband interference | RAN4 feedback is required for the suppression level at UE, g | No interference |  |

④Iinter-UE/inter-subbandk

Similarly as in Iinter-gNB/inter-subbandk, Iinter-UE/inter-subbandk can be modelled as the sum of interference power from all interferer UL subcarriers, i.e., Iinter-UE/inter-subbandk=j∈UL subcarriersIinter-UE/inter-subbandj→k. By assuming frequency response of a filter to reduce the interference power Iinter-UE/inter-subbandj→k at the victim UE side, the interference Iinter-UE/inter-subbandj→k caused from DL subcarrier j and measured at UL subcarrier k is can be modelled as

Iinter-UE/inter-subbandj→k=all aggressor UEsPCLI,UEj→k

where PCLI,UEj→k=UL Transmission power at subcarrier j of aggressor UE - effective channel gain between the aggressor UE and victim UE - gj→k. The value of gj→k is a representative value for suppression level after filtering the inter-subband interference caused by subcarrier j and measured at subcarrier k, and its value should be reported by RAN4.

The main difference between PCLI,gNBj→k and PCLI,UEj→k is that the UL transmission power in PCLI,UEj→k is controlled by open-loop power control and closed-loop power control so that the evaluation should take into account UL power control scheme. Also, fj→k in PCLI,gNBj→k could be determined according to gNB’s capability and its capability can be enhanced when SBFD operation is supported at gNB. However, gj→k in PCLI,UEj→k cannot be changed or enhanced even if gNB supports SBFD operation. It is because it is assumed that the UE operates in half-duplex mode in Rel-18. So that, we expect much smaller value of gj→k in PCLI,UEj→k than that of fj→k in PCLI,gNBj→k. Also, to reduce evaluation workload, we can also consider one-level or two-level model for the suppression level gj→k.

To model the received power of UE-to-UE (UL-to-DL) interference at subcarrier k from all aggressor UEs (Iinter-UE/inter-subbandk), RAN1 takes the following model as a starting point.

Iinter-UE/inter-subbandj→k=all aggressor UEsj∈UL subcarriersPCLI,UEj→k

- where PCLI,UEj→k=UL Transmission power at subcarrier j of aggressor UE - effective channel gain between the aggressor UE and victim UE - gj→k.

    - The suppression value gj→k should be asked to RAN4

    - RAN1 further to study the simplified model for the suppression level gj→k and its feasibility should be asked to RAN4

- Conclusion

In this contribution, we made the following observations and proposals:

- For evaluation purpose, the following three categories of applicable scenarios can be evaluated with their priorities.

    - The non-coexistence applicable scenario with highest priority

    - The coexistence applicable scenario of static TDD and a new duplex operation with medium priority

    - The coexistence applicable scenario of two new duplex operations with low priority

For non-coexistence applicable scenario, RAN1 takes the following applicable scenarios as a baseline

    - Indoor hotspot (FR1/FR2) with SBFD-only deployment and dynamic/flexible TDD-only deployment

    - 1-layer Dense Urban (FR1/FR2) with SBFD-only deployment and dynamic/flexible TDD deployment

    - Urban Macro (FR1) with SBFD-only deployment and dynamic/flexible TDD-only deployment

    - Note: down-selection/prioritization of non-coexistence applicable scenarios can be done in Rel-18

For the coexistence applicable scenario of static TDD and a new duplex operation, RAN1 takes the following applicable scenarios as a baseline

    - 1-layer/2-layer Dense Urban (FR1/FR2) with coexistence deployment of static TDD and SBFD or dynamic/flexible TDD.

      - For 1-layer, 1 gNB or 7 gNBs with capability of SBFD or dynamic/flexible TDD are surrounded by gNBs with static TDD

      - For 2-layer, Macro layer is for static TDD and Micro/indoor hotspot layer is for SBFD or dynamic/flexible TDD or Macro layer is for SBFD and Micro/indoor hotspot layer is for static TDD

    - Note: down-selection/prioritization of the coexistence applicable scenario of static TDD and a new duplex operation can be done in Rel-18

For the coexistence applicable scenario of two new duplex operations, RAN1 takes the following applicable scenarios as a baseline

    - 2-layer Dense Urban (FR1/FR2) with coexistence deployment of SBFD and dynamic/flexible TDD deployment

    - For 2-layer, Macro layer is for SBFD and Micro/indoor hotspot layer is for dynamic/flexible TDD

    - Note: down-selection/prioritization of the coexistence applicable scenario of two new duplex operations can be done in Rel-18

For evaluation purpose, RAN1 to prioritize the aligned subband configuration for SBFD operation.

To evaluate impacts on adjacent channel, consider the followings as a baseline

- For Macro layer (in Urban macro or Dense urban),

  - Two operators’ gNBs are located at the same (0% grid shift)

  - The second operator’s gNBs are located at edge of the first operator’s gNB (100% grid shift)

- FFS: indoor hotspot deployment

- Adjacent channel uses static TDD operation.

For evaluation, RAN1 takes DL/UL UPT CDF and Latency CDF as a performance metric to draw a conclusion and recommendation in TR.

    - DL/UL received SINR can be used for calibration

    - FFS: other performance metrics

For evaluation purpose, RAN1 takes the deployment related parameters in Tables 2-4 as a starting point.

For evaluation of SBFD operation, RAN1 takes the following two cases for BS antenna configuration.

    - Case 1. Half number of antenna elements are used to simultaneously perform DL transmission and UL reception

    - Case 2. The same number of antenna elements are used to simultaneously perform DL transmission and UL reception

For evaluation, RAN1 takes the parameters for BS antenna configurations shown in Table 6 as a starting point.

RAN1 takes the parameters for duplex evolution in Table 7.

To model the received power of gNB-to-gNB (DL-to-UL) interference at subcarrier k from all aggressor gNBs (Iinter-gNB/inter-subbandk), RAN1 takes the following model as a starting point.

Iinter-gNB/inter-subbandk=all aggressor gNBsj∈DL subcarriersPCLI,gNBj→k

- where PCLI,gNBj→k=DL Transmission power at subcarrier j of aggressor gNB - effective channel gain between the aggressor gNB and victim gNB - fj→k.

    - The suppression value fj→k should be asked to RAN4

    - RAN1 further to study the simplified model for the suppression level fj→k and its feasibility should be asked to RAN4

To model the residual self-interference power after self-interference cancelation at subcarrier k (Iselfk), the representative value for Iselfk is considered as additional noise level in addition to the original noise level.

    - The (worst-case) residual self-interference value should be asked to RAN4

To model the received power of UE-to-UE (UL-to-DL) interference at subcarrier k from all aggressor UEs (Iinter-UE/inter-subbandk), RAN1 takes the following model as a starting point.

Iinter-UE/inter-subbandj→k=all aggressor UEsj∈UL subcarriersPCLI,UEj→k

- where PCLI,UEj→k=UL Transmission power at subcarrier j of aggressor UE - effective channel gain between the aggressor UE and victim UE - gj→k.

    - The suppression value gj→k should be asked to RAN4

    - RAN1 further to study the simplified model for the suppression level gj→k and its feasibility should be asked to RAN4.

References

RP-220633 “Study on Evolution of NR Duplex Operation,” RAN#95-e

TR38.913 “Study on scenarios and requirements for next generation access technologies (Release 14)”

Regarding the deployment scenarios related to UAV, UAM, or Satellite (Commercial Air to Ground scenario, Light aircraft scenario, Satellite extension to Terrestrial), these scenarios are getting higher interest but not deployed in commercial communication networks yet. And it is still questionable whether coverage improvement and latency improvement targeted by duplex evolution SID are necessary. Therefore, it can be deprioritized in Rel-18 SI.

Note that since the subcarrier k (which is associated in UL subband) is not used for DL transmission at the aggressor gNB, so that transmit beamforming at subcarrier j is used for subcarrier k. Since we assume no advanced coordinated beamforming between the aggressor gNB and the victim gNB, the transmit beamforming for subcarrier k can be selected arbitrary and its impacts could be marginal.