3GPP TSG-RAN WG1 Meeting #109-e	R1-2203905

e-Meeting, May 9th – 20th 2022

Agenda Item:	9.3.3

Source:	Samsung

Title:	Dynamic and flexible TDD for NR duplex evolution

Document for:	Discussion

Introduction

The SID for NR duplex evolution aims to identify solutions and evaluate their feasibility to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation in unpaired spectrum.

In the R18 study item, it is assumed that the duplex enhancements are deployed at the gNB side and that the TDD UE operates in half-duplex. With respect to dynamic/flexible TDD operation, the SID for NR duplex evolution [1] specifies the following RAN1 objectives:

- Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1) - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1) - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1)

In this contribution, we summarize the existing R15-R17 NR features and earlier evaluation work for supporting dynamic/flexible TDD operation in Section 2. We then present our views on possible CLI enhancements in Section 3 to better exploit the configuration flexibility already provided by existing core specifications. We first consider the intra-operator and then the inter-operator cases for co-deployed TDD networks in the NR band. Note that our accompanying contributions discuss CLI enhancements in the context of SBFD [2][3].

Background

The R15 NR TDD network of an operator and the co-deployed networks of different operators in an operating band are assumed to be synchronized in the same coverage area. Rural, macro or suburban micro cell deployments of networks of different operators usually must align their UL/DL split. Often, site engineering solutions and inter-operator agreements are necessary to coordinate the TDD deployments. Similar to 3G TDD and 4G LTE TDD, avoidance of UL and DL interference is necessary also in 5G NR TDD. gNB-to-gNB adjacent channel interference is of particular concern due to the usually high transmitter power of macro base stations deployed for coverage and their above-rooftop deployment. Current 3GPP UE and network-side RF and RRM performance requirements have been derived using the assumption of synchronized and aligned TDD operation in the same coverage area.

However, a difference between earlier generations of 3G and 4G TDD network deployments and NR TDD is that NR TDD uses the higher FR1 frequency bands, e.g., n78 (3.5 GHz). Signal propagation in the higher NR mid-band is more attenuated. NR TDD also benefits from the availability of massive MIMO and the use of DL/UL beamforming. Better spatial separation and isolation between TDD transmitters becomes possible. FR2 operation is subject to even larger signal attenuation. There is more potential for 5G NR TDD to use flexible TDD operation in FR1 or FR2 in which the de-facto requirement on UL/DL alignment can be relaxed. Note that in many countries, regulatory requirements currently still mandate inter-operator TDD frame and time synchronization to avoid interference for TDD network deployments in the same band.

R15 NR design inherently provides much configuration flexibility to configure the UL/DL split according to the needs of the gNB, e.g., scheduling-based UL/DL assignments, symbol-based D/F/U configuration, DL and UL TDRA signaling, the use of UL asynchronous HARQ, flexible A/N timing relationships or the short PUCCH formats 0/2. Practical UE TDD implementation imposes more constraints. The number of supported TDD Tx-Rx/Rx-Tx switching points per slot are best limited. UE power consumption greatly benefits when the cell-specific TDD UL-DL configuration allows to not monitor for DL signals in known UL slots/symbols.

Note that quite many techniques to support TDD duplexing flexibility and CLI mitigation were discussed and evaluated already during the NR SI [4]., e.g., techniques falling in the categories of advanced receiver design (IC/IS), hybrid dynamic/static UL/DL resource assignment, scheduling coordination, beam coordination, link adaptation, power control, sensing, cell/TRP clustering, OTA signaling, co-channel multiple connectivity, load- or link-based resource or resource scheduling adaptation.

The R16 WI CLI and RIM provides additional specification support to mitigate inter-cell TDD interference. The evaluations and findings from the R16 CLI coexistence study are summarized in [5].

The CLI feature is primarily intended for dynamic TDD operation in small cell deployments. gNBs can exchange and coordinate the intended TDD UL-DL configuration over the Xn and F1 interfaces. The gNB can then decide the transmission and reception pattern to either avoid creating CLI towards a neighbor cell or to incur CLI from a neighbor cell. Two different types of UE reportable CLI measurements were specified: CLI-RSSI and SRS- RSRP. For CLI-RSSI measurements, the victim UE measures the total received power over the configured CLI-RSSI measurement resource(s). For SRS-RSRP measurements, the victim UE measures the RSRP over configured SRS resource(s) which are transmitted from one or multiple aggressor UEs. These measurements are L3 filtered. Event triggered and periodic reporting are supported. Note that it cannot be assumed that the UE supports concurrent DL reception while performing CLI measurements in the same time-domain resources even if the UE supports the CLI measurement features, e.g., FGs 17-1 and 17-2.

RIM was introduced as feature to deal with atmospheric duct conditions experienced at times by large cell TDD deployments for purpose of gNB-to-gNB interference management. RIM allows for coordination between victim and aggressor gNBs either using OTA or backhaul-based solutions. The gNBs in a victim set simultaneously transmit an identical RIM reference signal carrying the victim set ID over the air. For use by OTA solutions, RIM-RS type 1 and RIM-RS type 2 were specified. Upon reception of the RIM-RS type 1 from the victim set, the aggressor gNBs perform RIM measurements and can send back a RIM-RS type 2 signal carrying the aggressor set ID when configured. Interference mitigation is gNB-implementation based, e.g., a larger GP can be employed. When backhaul-based solutions are used, upon reception of the RIM-RS type 1 from the victim set, the aggressor gNBs perform RIM measurements and use backhaul coordination towards the victim gNB set. The backhaul messages which carry the RIM detection or disappearance indications are aggregated at gNB-CU via the F1 interface and sent from individual aggressor gNBs to individual victim gNBs via the NG interface. Note that the signaling is transparent to the core network. Another alternative is the use of OAM. Note that the R16 RIM-RS were only specified for 15 and 30 kHz SCS.

CLI enhancements for Dynamic/Flexible TDD

Deployment aspects

Due to the high transmit power of wide area base stations and over-the-rooftop deployment for coverage, gNB-to-gNB interference dominates co- and adjacent channel CLI in urban macro cell deployments. This applies to TDD deployments with co-located or co-sited gNBs from different operators, but also to non-collocated TDD deployments. Without mitigation measures such as operator coordination, the resulting small coupling loss between the base stations of different operators in adjacent channels can still result in an increase of the noise floor of several dBs even at distances of several km away. For TDD macro cell deployments across different sites, avoidance of LoS conditions is one particular concern. Therefore, most TDD macro cell deployments in the NR mid-band are likely to continue using synchronized and UL/DL aligned TDD operation.

- gNB-to-gNB interference dominates co- and adjacent channel CLI in TDD urban macro deployments

TDD urban micro cell deployments usually use smaller transmit power level when compared to wide area base stations. One deployment challenge is the base station density with ISD of 200-250m in the urban outdoor deployment grid. gNB-to-gNB interference cannot be easily avoided due to network density and interference limited DL and UL performance. In these types of TDD deployments, harvesting the gains possible with dynamic TDD operation is therefore particularly challenging to implement in practice. It should also be considered that urban micro cells often carry the highest traffic load of the network in average over time, so they benefit least from average short or mid-term UL/DL traffic ratio variations. A high number of UEs are typically being served. Similar to TDD urban macro cell deployments, the continued use of synchronized and UL/DL aligned TDD operation is a practical necessity.

- TDD urban micro deployments experience high CLI and less variation of the offered UL/DL traffic ratio

Hotspot and factory floor TDD gNB deployments are often more isolated due to being located indoors. gNBs use much reduced transmit power in many, but not all cases. Small form factors of network equipment often result in more balanced UL and DL link budgets when compared to the macro or micro deployment cases. Cells typically experience much higher variations in the ratio of offered UL/DL traffic during scheduling. Fewer UE are present especially in the indoor hotspot case and when a UE is scheduled, adaptation of the UL/DL split by the gNB to the traffic being served can bring very significant improvements to achievable latency and peak throughput. Factory deployments are more divers. The number of UEs can vary more widely as can their required service requirements, e.g., in a range from low or intermittent DL or UL data transfer, video upload to the need to support time-synchronized networking. Inter-operator arrangements and site solutions may still be required in some cases, e.g., to avoid LoS conditions with respect to the high-power macro sites. Synchronization may be required for the needs of TSN, but not necessarily for purpose of co-deployment with the TDD macro coverage layer of an operator. TDD small cell deployments in the capacity layer of the deployment grid are therefore of particular interest for improved configuration flexibility to use dynamic TDD operation.

- TDD indoor hotspot and factory deployments offer most potential for improved configuration flexibility to use dynamic TDD operation

- Another important consideration for potential CLI enhancements in adjacent channel TDD deployments in the inter-operator case is that any such solution cannot rely on inter-operator signaling. In many cases the received signal strength and interference measurements for purpose of OAM support and network configuration can be measured and reported by TDD base stations to OAM using implementation. 3GPP does not specify gNB-side measurements on DL signals transmitted by neighbor base stations. However, TDD base stations often implement DL-based measurement solutions for a variety of purposes. The gNB can create silent intervals where no DL and UL transmissions are scheduled (or configured) in the serving cell and measure the DL signals from co- and adjacent channel neighbor cells in the deployment using SSBs or NZP CSI-RS or the transmitted DL signals. In principle, gNB-to-gNB CLI interference is observable by the base station. One drawback is that such gNB-specific measurements in the inter-operator-case can’t rely on a priori knowledge of the DL signal and when it will be transmitted from the measured base station.

- TDD base stations can measure DL signals received from neighbor cells of the same or different operator using implementation techniques

When considering the potential enhancements to CLI mitigation in scope of the R18 SID, e.g., measurements in support of CLI estimation and interference mitigation techniques, it is important to consider in which type of TDD deployments such techniques are feasible and can potentially be useful.

High gNB-to-gNB interference characterize urban macro TDD deployments due to high gNB transmit power levels and over-the-rooftop site deployment in the coverage layer. High rise over thermal from network densification characterize many TDD urban micro deployments. Dynamic TDD operation in these environments is challenging. SBFD has the potential to support much more flexibility to allocate the UL/DL transmission resources in these deployments. The SBFD UL subband when placed at the center of the NR carrier provides additional frequency-domain isolation towards the adjacent channel(s) in the inter-operator case for adjacent channel protection. SBFD operation when available in the TDD cell does not require inter-operator coordination. SBFD is therefore inherently suitable as potential CLI mitigation technique for urban macro and micro TDD deployments. Note that we assume that SBFD operation only uses the DL slots for purpose of scheduling UL transmissions in the SBFD UL subband from UEs when needed. Dynamic TDD operation can be seen as a generalization of SBFD, e.g., all transmission resources in the symbol/slot are allocated to either DL or UL based on the gNB scheduling decision. In that case, no gNB-side SIC is necessary due to spatial isolation between base stations.

- SBFD is a potential CLI mitigation solution for urban macro and micro TDD deployments

Intra-operator CLI mitigation

For UE-to-UE (UL-to-DL) CLI measurements and the associated UE reporting the existing R16 CLI feature can in principle be used “as is”. This applies both to the case of intra- and inter-cell CLI reporting when SRS transmissions from interfering (aggressor) UEs transmitting in the same or neighbor cell are reported by the victim UEs. Note that we assume that the D/U/F symbols and slots are configured for UEs by SIB1 tdd-UL-DL-ConfigCommon and/or tdd-UL-DL-ConfigDedicated for purpose of SRS transmission and CLI-RSRP reporting in Figure 1.

- The existing R16 CLI features can be re-used for intra-cell and inter-cell CLI measurements and associated UE-based measurement reporting

For gNB-to-gNB (DL-to-UL) CLI estimation by the gNB, providing a known CLI-RS reference signal (Figure 1) inserted into the DL transmissions from the aggressor gNB allows the victim gNB to estimate CLI and adjust its scheduling behavior. In principle, it is possible to consider the reuse of the existing RIM-RS type 1 or 2. Both types of RS were designed to be distinguishable from any other type of UL RS, e.g., using a different pseudo-random sequence.

However, transmission of the RIM-RS incurs much overhead which is warranted in the macro TDD deployment case for which the R16 feature was developed, but which penalizes the DL spectral efficiency when used more frequently in the context of supporting dynamic TDD operation in small cells such as the indoor hotspot or indoor factory deployments. RIM-RS occupy 2 consecutive symbols and occupy 96 RBs for 15 or 30 kHz SCS or can be configured using 48 RBs for the 30 kHz SCS. For many indoor factory deployments, 60 kHz SCS support is desirable, which RIM-RS do not currently support. Only up to 4 RIM-RS resources can be configured in frequency-domain.

The R18 SID aims to evaluate duplexing enhancements for both FR1 and FR2. Due to high signal attenuation and narrow beamwidth by lots of antenna elements in FR2, gNB-to-gNB CLI is not so significant, compared to FR1 macro or micro TDD deployment. However, in a case where transmit beam from aggressor gNB is aligned to receive beam at victim gNB, gNB-to-gNB CLI could be increased. Therefore, RIM-RS can be extended to FR2, where 60kHz and 120kHz SCS are supported.

In summary, we think that for purpose of intra-operator gNB-to-gNB (DL-to-UL) CLI measurements, the existing R16 RIM feature is not easily re-purposed and a new DL-based CLI RS signal should be considered. Existing R16 RIM-RS type 1 or 2 design as specified can be taken as a starting point. The CLI-RS should allow for less overhead when used in TDD small deployments when transmitted more frequently by the gNBs, allow for higher multiplexing capacity and support 60 kHz SCS in addition to 30 kHz SCS and extension to FR2.

- The existing R16 RIM-RS type 1 or 2 are not suitable for purpose of intra-operator gNB-to-gNB (DL-to-UL) CLI measurements in NR mid-band small deployments

RAN1 to study and evaluate the benefits of a new DL reference signal design to support intra-operator gNB-to-gNB (DL-to-UL) CLI measurements

Figure 1: UE-based intra- and inter-cell CLI reporting

In the intra-operator case, inter-cell coordination using the Xn interface is possible. The exchange of the Intended TDD DL-UL Configuration NR is already supported by existing specifications. When considering NR mid-band deployments, beam coordination/avoidance by the gNBs is one very promising CLI mitigation technique to reduce gNB-to-gNB (DL-to-UL) interference. Note that R17 eIAB introduced signaling support to indicate desired/prohibited beam indications for purpose of better supporting SDM at the IAB-DU/MT. In our view, the existing Xn-AP signaling should be extended with similar functionality for purpose of dynamic TDD operation.

RAN1 to study and evaluate the benefits of providing desired/prohibited beam indications using Xn-AP to support intra-operator gNB-to-gNB (DL-to-UL) CLI mitigation

Conclusion

In this contribution, we made the following observations and proposals:

- gNB-to-gNB interference dominates co- and adjacent channel CLI in TDD urban macro deployments

- TDD urban micro deployments experience high CLI and less variation of the offered UL/DL traffic ratio

- TDD indoor hotspot and factory deployments offer most potential for improved configuration flexibility to use dynamic TDD operation

- TDD base stations can measure DL signals received from neighbor cells of the same or different operator using implementation techniques

- SBFD is a potential CLI mitigation solution for urban macro and micro TDD deployments

- The existing R16 CLI features can be re-used for intra-cell and inter-cell CLI measurements and associated UE-based measurement reporting

- The existing R16 RIM-RS type 1 or 2 are not suitable for purpose of intra-operator gNB-to-gNB (DL-to-UL) CLI measurements in NR mid-band small deployments

- RAN1 to study and evaluate the benefits of a new DL reference signal design to support intra-operator gNB-to-gNB (DL-to-UL) CLI measurements

RAN1 to study and evaluate the benefits of providing desired/prohibited beam indications using Xn-AP to support intra-operator gNB-to-gNB (DL-to-UL) CLI mitigation

- References

RP-220633 “Study on Evolution of NR Duplex Operation,” RAN#95-e

R1-2203903 “Deployment scenarios and evaluation methodology for NR duplex evolution,” Samsung

R1-2203904 “Subband non-overlapping full duplex for NR duplex evolution,” Samsung

TR 38.802 “Study on New Radio Access Technology Physical Layer Aspects (Release 14)”

TR 38.828 “CLI handling and RIM for NR (Release 16)”