3GPP TSG RAN WG1 Meeting #109-e		             R1-2203906

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.4.1.1

Source: 	Samsung

Title: 	On channel access mechanism for sidelink on FR1 unlicensed spectrum

Document for:	Discussion and Decision

# Introduction

NR sidelink operating on unlicensed band is introduced in Rel-18 to extend use scenarios and satisfy increasing requirements of wireless traffic. A new NR SL work item was approved in RAN#94e meeting with the following objective in [1]:

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In this contribution, the possible aspects related to channel access mechanisms of NR SL on unlicensed spectrum are discussed.

# Channel access on unlicensed spectrum

Wireless communication systems operating on shared spectrum needs to comply with regulation requirement. From the channel access perspective, legacy NR SL techniques need to be enhanced to satisfy the key requirements of energy detection (ED), including dynamic channel access mode and semi-static channel access mode, where the dynamic channel access procedure could further include Type1 channel access procedure (e.g. random sensing duration) and Type 2 channel access procedure (e.g. deterministic sensing duration). There are also features of LBT bandwidth, ED threshold adaptation, and channel occupancy sharing associated with the channel access procedure.

For channel access of SL-U on FR1, whether and how to reuse the NR-U mechanisms can be studied as a starting point, and RAN1 could further investigate the necessity of enhancement to the NR-U mechanisms, especially for the applicable use cases that are unique for sidelink operation (e.g. resource allocation schemes).

Proposal 1:

- Support NR-U dynamic channel access mechanism as a baseline for SL-U, including:

  - Type 1 channel access procedure with random sensing duration;

  - Type 2 channel access procedure with deterministic sensing duration.

- Further study NR-U semi-static channel access mechanism for SL-U.

Another aspect to allow flexible channel access is channel occupancy sharing. There exists two types of channel occupancy initiating and sharing mechanisms in NR-U:

- gNB initiates a channel occupancy and transmits DL channels to UEs at the beginning of the initiated channel occupancy. In addition, gNB can share the channel occupancy to its UE, then the UE can transmits UL channels to gNB in the shared channel occupancy.

- UE initiates a channel occupancy and transmits UL channels to gNB in the initiated channel occupancy. In addition, UE can share the channel occupancy to gNB, then gNB can transmit DL channels, at least including this UE as the receiver, in the shared channel occupancy.

The difference with Uu is that the SL communication topology is more complicated and dynamic, therefore the channel occupancy initiating and sharing mechanism in different scenarios needs to be discussed. For SL-U on FR1, it is necessary to support UE-1 initiating a channel occupancy and sharing the channel occupancy via sidelink to another UE-2. On top of the basic scenario, it needs further study whether UE-1 can share initiated channel occupancy to a group of UE-2s, and after being shared with channel occupancy, how the UE-2 could use the shared channel occupancy, e.g. whether UE-2 can transmit to UE-3, which is different as UE-1, in the shared channel occupancy, and whether UE-2 can further share channel occupancy to UE-3 for UE-3’s transmission.

At the starting phase of SL-U study, RAN1 should list potential scenarios and reach common understanding on each feasibility. Some valid cases from our side are provided in Figure 1. In our understanding, the examples in Figure 1(a) and 1(b) can be considered as typical cases for SL-U channel occupancy sharing. For the examples in Figure 1(c), further investigation on their compliance with regulation and justification of application scenarios is needed.

Proposal 2: Support UE sharing channel occupancy to other UE(s) via sidelink. RAN1 should study the potential scenarios and justify the feasibility, includes:

- Channel occupancy sharing between a single TX-RX UE pair is studied as a baseline

- Further study if TX UE could share channel occupancy to a group of RX UEs

- Further study whether channel occupancy between multiple sidelink transmission pairs are supported

(a) UE-1 shares CO to a single UE-2          (b) UE-1 shares a CO to a group of UE-2s

(c) CO sharing between multiple SL transmission pairs

Figure 1. Typical cases of sidelink channel occupancy sharing

According to the WID, gNB assistance and mode-1 resource allocation is supported by SL-U system, where Uu operation for mode 1 is limited to licensed spectrum only. Therefore, how could gNB involve in SL-U systems needs further discussion, e.g. whether to support the case of gNB determining the UE that initiates channel occupancy and UE(s) that shares the channel occupancy, whether UE provided information of initialized/shared channel occupancy to gNB and receive SL grants scheduling resource in the channel occupancy, etc.

Proposal 3: Study how gNB could be involved in channel occupancy sharing schemes and support mode 1 operation on unlicensed spectrum.

Channel occupancy indication (including the time domain channel occupancy and frequency domain channel occupancy) should also be studied as a basic functionality of channel occupancy sharing. It is conveyed by GC-PDCCH in Rel-16 NR-U system to achieve flexible and fast channel occupancy information exchanged. Similarly, it can be conveyed by 1st/2nd stage SCI in SL-U system. Both SCI formats used to convey channel occupancy and detailed field design in the SCI format need to be studied and specified in Rel-18.

In addition, considering that gNB could be involved in initiating and sharing the channel occupancy for SL UEs, and the case of UE initiating channel occupancy and requesting gNB scheduling in the initiated channel occupancy should be supported for mode 1 operation, the indication method from SL UE to gNB with intention of providing channel occupancy information rather than sharing it to gNB is also necessary to be studied.

Proposal 4: Study the indication method of channel occupancy sharing between SL UEs, and the indication method of providing channel occupancy information from SL UE to gNB.

# Enhancement on resource allocation schemes

Listen before talk (LBT), which is also referred as short-term sensing, is the key functionality of NR-U to allow spectrum sharing between different RATs with manageable interference. Different LBT types e.g. Type-1/2A/2B/2C were defined with corresponding length to satisfy the regulation requirement of initializing channel occupancy and transmitting within the channel occupancy after a gap duration e.g. for switching points. With performing LBT, UE could decide whether the channel is unoccupied and start to transmit if LBT procedure is passed.

However, LBT has its own limitation in the decentralized sidelink network, which may limit the overall performance and impact throughput. If only LBT is performed in SL-U, a UE can hardly predict intra-system interference, thus collision due to LBT failure may increase. In addition, the LBT bandwidth is 20MHz, which is much larger than a subchannel size in SL resource pool. Consequently it is important to support multiple users simultaneously sending FDMed signals on a LBT band to improve spectrum efficiency, but the goal is hard to be achieved in LBT-only system without a centralized scheduler. There is another limitation that LBT is hard to properly handle the periodic transmissions, which may lead to noticeable performance degradation considering that periodic traffic is typical in SL communications.

Sidelink channel sensing in Rel-16 and Rel-17, which is referred to as long-term sensing (although in Rel-17 it also introduced a short-term sensing component), is an important method to avoid intra-system collision in legacy NR SL. Sidelink sensing supports UE to autonomously select resources that are more likely to be unoccupied by SCI detection and reservation based resource exclusion, thus it naturally allows FDMed transmissions from different users to enhance resource efficiency.

According to the respective pros/cons of LBT and legacy sidelink sensing, the two methods can hardly replace each other, therefore RAN1 needs to study how the two methods work in combination for Rel-18 SL-U system.

Proposal 5: Both Rel-16/17 sidelink sensing and LBT and supported for channel accessing in Rel-18 SL-U. Study how the two methods work with each other and potential enhancements if necessary.

SL transmissions on unlicensed spectrum have higher uncertainty than legacy ones. SL transmission on the resources selected in legacy sensing procedure may be dropped or postponed if LBT procedure fails, thus SL reliability and latency are strongly impacted due to LBT failure in SL-U channel access.

In order to compensate the performance degradation, one possible solution is to introduce multiple transmission occasions for a given (re-)transmission, as shown in Figure 2. UE could try to pass LBT procedure in each occasion, and after LBT procedure passed, the remaining occasions can be released or used for next (re-)transmission. Correspondingly, UE needs to determine multiple resources during sensing procedure. The difference with legacy procedure is that the multiple occasions are for same transmission thus is not restricted by some processing delay e.g. HARQ gap duration. In addition, to reduce intra-system collision, UE could also reserve the multiple occasions by SCI indication. The potential drawback is that SL transmission may be overbooked thus unnecessarily increases congestion level of resource pool. Hence, how to evaluate and solve the overbooking issue needs to be carefully discussed.

Figure 2. Selection of multiple candidate PSSCH occasions for a given (re-)transmission in RA procedure

Another possible way is to introduce a new trigger of LBT failure for legacy resource allocation procedure, as illustrated in Figure 3. After LBT procedure failed, UE re-starts the resource allocation procedure and select new transmission resource. It is beneficial to reduce overbooked resources, but sacrifices latency with introducing more processing delay, and is also risky to impact the SL collision avoidance mechanism from system level, since resources selected in the newly triggered procedure is lack of previous reservation.

Figure 3. Resource reselection triggered by LBT failure

Furthermore, RAN1 also needs to study whether legacy resource allocation procedure can work well under channel access structure on unlicensed spectrum. The legacy steps, e.g. determination of RSW, monitoring of sensing window, and resource exclusion and selection, may be further restricted by channel occupancy. RAN1 should further study if there exists some problem, and if any, discuss if that can be handled in an efficient way within WID scope.

Proposal 6: Study the applicability of further enhancement on channel access schemes for unlicensed spectrum, including:

- Multiple sidelink transmission occasions, e.g. selection of multiple candidate PSSCH occasions for potential LBT procedure, and resource reservation of multiple candidate PSSCH occasions in SCI

  - Resource overbooking issue needs to be further considered

- Resource reselection triggered by LBT failure

  - Study how to reduce the latency impact and potential collision

- Potential issues on legacy mode-2 resource allocation procedure and how to enhance it under WID scope

Considering the potential impact due to LBT failure, it is beneficial to reduce the use of LBT procedure as much as possible. In an initiated/shared channel occupancy, LBT procedure occurs mainly due to gap duration between different transmissions. With a larger gap, UE is more probable to lose channel occupancy thus has to drop or postpone the subsequent transmissions. Even if LBT procedure is passed, the overhead of LBT duration is still not negligible overhead from overall system level. Therefore, how to improve resource determination principle to reduce gap duration is also important issue for SL-U.

For UEs with multiple SL transmissions using different HARQ process or HARQ disabled, one simple solution is to select consecutive TX resources during resource selection procedure. UE can send signals on the guard and/or AGC symbols to eliminate transmission gap and maintain channel contiguously being occupied (see accompany contribution [2] for details), then LBT procedure only happens at the beginning of first transmission. An example is illustrated in Figure 4.

Proposal 7: Study the possibility and methods of selecting consecutive resources in time domain to minimize the overhead of LBT and avoid potential LBT failure.

Figure 4. Selection of consecutive resources to reduce LBT

The feasibility and performance of legacy resource reservation on unlicensed bands also needs to be carefully evaluated. According to the typical channel occupancy length in NR-U, it is possible that inter-period reservation will cross channel occupancy boundary, i.e. resource reserved in next period is after the ending point of current channel occupancy. For the last few slots in channel occupancy, regardless of its length, there always exists a potential risk that intra-period reservation as well as PSFCH position will cross channel occupancy boundary in a similar manner.

Considering the uncertainty of acquiring channel occupancy right after current channel occupancy finished, and also the length of LBT duration, it is highly possible that inter/intra-period reservations and PSFCH resources cannot be occupied for transmission. It is possible to dropping such transmission but significantly degrades system performance. Therefore, it is necessary to study some methods to make resource reservation more flexible and reduces the impact of unoccupied resources.

Proposal 8: Study flexible data/feedback resource reservation to improve reliability and availability of short/long term resource reservations.

# Conclusions

In this contribution, the issues for channel access of sidelink operating on FR1 unlicensed spectrum are discussed with the following proposals:

Proposal 1:

- Support NR-U dynamic channel access mechanism as a baseline for SL-U, including:

  - Type 1 channel access procedure with random sensing duration;

  - Type 2 channel access procedure with deterministic sensing duration.

- Further study NR-U semi-static channel access mechanism for SL-U.

Proposal 2: Support UE sharing channel occupancy to other UE(s) via sidelink. RAN1 should study the potential scenarios and justify the feasibility, includes:

- Channel occupancy sharing between a single TX-RX UE pair is studied as a baseline

- Further study if TX UE could share channel occupancy to a group of RX UEs

- Further study whether channel occupancy between multiple sidelink transmission pairs are supported

Proposal 3: Study how gNB could be involved in channel occupancy sharing schemes and support mode 1 operation on unlicensed spectrum.

Proposal 4: Study the indication method of channel occupancy sharing between SL UEs, and the indication method of providing channel occupancy information from SL UE to gNB.

Proposal 5: Both Rel-16/17 sidelink sensing and LBT and supported for channel accessing in Rel-18 SL-U. Study how the two methods work with each other and potential enhancements if necessary.

Proposal 6: Study the applicability of further enhancement on channel access schemes for unlicensed spectrum, including:

- Multiple sidelink transmission occasions, e.g. selection of multiple candidate PSSCH occasions for potential LBT procedure, and resource reservation of multiple candidate PSSCH occasions in SCI

  - Resource overbooking issue needs to be further considered

- Resource reselection triggered by LBT failure

  - Study how to reduce the latency impact and potential collision

- Potential issues on legacy mode-2 resource allocation procedure and how to enhance it under WID scope

Proposal 7: Study the possibility and methods of selecting consecutive resources in time domain to minimize the overhead of LBT and avoid potential LBT failure.

Proposal 8: Study flexible data/feedback resource reservation to improve reliability and availability of short/long term resource reservations.

# References

[1] RP-213678, New WID on NR sidelink evolution, OPPO, LG Electronics

[2] R1-2203907, On physical channel design framework for sidelink on FR1 unlicensed spectrum, Samsung